3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216187

# Electronic Meeting, Oct. 10 – 19, 2022

Source:	LG Electronics

Title:	MSD evalueation considering the high PCB isolation for CA n1-n3

Agenda item:	6.6.4.1

Document for:	Discussion

Introduction

In this paper, MSD values of CA_n1-n3 is analyzed considering high PCB isolation based on the WF[1] agreed in RAN4#104-e meeting.

Discussion

MSD values are analyzed for CA_n1-n3 based on Table1 and Table2. Table 1 shows the component linearity parameter and Table 2 shows the isolation factor used in this analysis.

Table 1 Front-end component linearity parameter

| Component | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| --- | --- | --- | --- | --- |
| Ant. Switch | 112 | 68 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 |
| PA | 28.5 | 30 | 30 | 28 |
| LNA | 10 | 0 | 0 | -10 |

Table 2 Isolation factor

| Isolation parameter | Case1 | Case2 |
| --- | --- | --- |
| Antenna to antenna | 10 dB | 10 dB |
| PCB | 60 dB | 70 dB |

The MSD value for CA_n1-n3 in TS 38.101-1[2] is shown in Table 3, and the analyzed MSD values in this paper are shown in Table 4.

Table 3 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA [TS 38.101-1]

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |

Table 4 Analyzed MSD values considering two types of PCB isolation

| Isolation factor | MSD |
| --- | --- |
| Case1 | 23.1 dB |
| Case2 | 5 dB |

In case 1, assuming PCB isolation of 60 dB, the MSD value is 23.1 dB, which is similar to the MSD value of TS38.101-1. In case 2, assuming PCB isolation of 70 dB, the MSD value is 5 dB, which is 18.1 dB lower than case 1.

observation1: In CA_n1-n3, when the PCB isolation is 70 dB, it has a much lower MSD value than 60 dB PCB isolation. Further analysis is required for other combinations using 70 dB PCB isolation.

Proopsal1: 70 dB PCB isolation is feasible for lower MSD.

Proposal2: Further MSD analysis using 70 dB PCB isolation is needed for other combinations.

Conclusion

observation1: In CA_n1-n3, when the PCB isolation is 70 dB, it has a much lower MSD value than 60 dB PCB isolation. Further analysis is required for other combinations using 70 dB PCB isolation.

Proopsal1: 70 dB PCB isolation is feasible for lower MSD.

Proposal2: Further MSD analysis using 70 dB PCB isolation is needed for other combinations.

- References

# [1] R4-2214452, “WF on study for lower MSD”, Huawei

# [2] TS 38.101-1