3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216215

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on remaining RRM issues for RedCap UEs

Agenda item:	4.6.3.1.1	General 	[NR_redcap-Core]

Document for:	Discussion

Introduction

The discussion on RedCap core requirements related to general measurement requirements was continued at RAN4 #104-e [1]. With regard to Small Data Transmission, the following agreements and open issues are captured in the WF [2].

Agreements: SDT FR2 requirements Reuse the FR2 requirements for SDT for legacy NR devices defined in clause 5.5.3 for RedCap UE in clause 5.2B.2.1, where: - X1 in FR2 TA validation rules is set to max{480ms, 8*SMTC periodicity}. - Z1 is set to 640 ms.

Open issues: SDT for RedCap with eDRX _FFS: TA validation requirements for RedCap CG-SDT is defined for UE configured with eDRX in RRC_INACTIVE state by reusing the principles from DRX requirements introduced in R17 SDT WI. Note: eDRX in RRC_INACTIVE state does not have PTW.

In this contribution, we discuss the above open issue from last meeting.

Discussion

The above open issue from last meeting is discussed in this section.

TA validation requirements for CG-SDT and eDRX

At RAN4#103-e, the discussion on TA validation requirements for small data transmission for RedCap UE was progressed in the endorsed draft CR [3]. The impact of eDRX was discussed but no specific requirements for eDRX were agreed to be defined.

At RAN4#104-e, the discussion on TA validation requirements for SDT under eDRX was then continued. It was agreed to reuse the same principles as for DRX by replacing TDRX with TeDRX-RAN , i.e. the eDRX cycle for RAN paging in RRC_INACTIVE. The CR 38.133 [4] was agreed.

In our view, TA validation requirements for SDT should not depend on whether the UE is configured with or without eDRX. Thus, the same timing requirements for ensuring the two RSRP measurements are appropriate related to the two instants, i.e., reception of last TA command and TA validation before transmitting on preconfigured uplink resources, should apply. Hence, there is no justification to specify different TA validation requirements to DRX when eDRX is configured and we supported the agreement for CR [4]. Some refinements on references and terms related to TA validation requirements for SDT are still found necessary and are contributed to this meeting in [5].

The following proposal is made.

Agree changes in R4-2216216 correcting references to the respective SDT section in 38.321, adding term definition for TeDRX-RAN , and removing brackets for FR2-1 aligned to FR2-1 requirements for SDT in clause 5.5.3.

Conclusion

The open issue from last meeting on TA validation requirements for SDT under eDRX has been addressed in this contribution and the following proposal is made.

- Agree changes in R4-221xxxx correcting references to the respective SDT section in 38.321, adding term definition for TeDRX-RAN , and removing brackets for FR2-1 aligned to FR2-1 requirements for SDT in clause 5.5.3.

- References

- R4-2214274, “Email discussion summary for [104-e][223] NR_redcap_RRM_1”, Moderator (Ericsson)

- R4-2214485, “WF on RedCap RRM requirements”, Ericsson

- R4-2211039, "Draft CR on SDT requirements for RedCap for TS 38.133", RAN4#103-e, Ericsson

- R4-2214626, "CR on changes to SDT requirements for NR RedCap for TS 38.133", RAN4#104-e, Ericsson

- R4-2216216, "CR on corrections to SDT requirements for RedCap for TS 38.133", RAN4#104-bis-e, Nokia, Nokia Shanghai Bell