GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216261

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.5.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements maintenance for FR2-2

Document for:	Discussion

# Introduction

In RAN4#103-e meeting, it was agreed to define Rx beam sweeping scaling factor as 12 for FR2-2. In RAN4#104-e, CR to update the requirements for beam sweeping for FR2-2 was discussed. Some remaining requirements are left over. In this paper, we provide our views on how to update the requirements for FR2-2, e.g. SCell activation and the remaining issue in last meetings.

# Discussion

In last meeting, the CR to update the beam sweeping factor for requirements of FR2-2 was discussed and agreed. During the discussions, companies commented that the changes on SCell activation were not discussed and should be removed in the CR. Thus, in the latest spec, the requirements are not differentiated for FR2-1 and FR2-2. For instance, for SCell activation requirements in 8.3.2, the related requirements are shown as follows:

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

It could be observed that the requirements for FR2-2 are defined with the assumption that the beam sweeping factor is 8. Thus, it is reasonable to update the requirements accordingly.

Thus, it is proposed to have following changes:

If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 6ms + TFirstSSB_MAX + (2*N-1)*TSMTC_MAX + N*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). If the PCell/PSCell and the target SCell are configured as FR1-FR2 CA or if the PCell/PSCell and the target SCell are in a FR2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is: - 3ms + TFirstSSB_MAX + (2*N-1)*TSMTC_MAX + N*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}. …… N = 8 for FR2-1 and N= 12 for FR2-2.

Similarly, for requirements with CCA, the requirements are suggested to be updated as:

- If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_withCCA is: - 6ms + TFirstSSB_MAX + ((2*NRxBeam-1)+[N*L4,1])*TSMTC_MAX + (NRxBeam +[ N *L4,2])*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP). - If the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA or if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management, and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time_withCCA is: - 3ms + TFirstSSB_MAX + ((2*NRxBeam-1)+[ N*L5,1])*TSMTC_MAX + (NRxBeam +[ N *L5,2])*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}. …… NRxBeam = 12

Proposal 1: Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12.

Another remaining issue is whether to have a capability to reflect the cell detection relaxation for 480 kHz and 960 kHz. As analysed in last meeting, we found that even at the worst case where UE is required to monitor all SSB indexes, the number of samples that UE needs to proceed is only increase by 30%-40% compared with FR2-1. But now, the requirements are extend by 2 and 3 times which will leads to extremely long delay compared with FR2-1. Companies against having a UE capability argued that RAN4 only define minimum requirements. However, we cannot entirely agree with the argument. Minimum requirements don't mean the requirement can be as long as possible without limitation. As mentioned above, the detection delay for 480 kHz and 960 kHz are 2/3 times of that for 120 kHz, however the number of samples are increased by around 30% at the worst case, which means the delay is unnecessarily over extended. Besides, it will have impacts on NW behaviour. The timetotrigger shall be configured with different value as the expected delay is much longer than 120 kHz. With the capability, if UE needs longer processing time, then the extension can apply. For other UE to whom the extension is not needed, legacy requirements can apply.

Proposal 2: Introduce UE capability on relaxed requirements for cell detection.

We provide the corresponding capability table in the Annex.

# Conclusions

Proposal 1: Update the SCell activation requirements for FR2-2 with beam sweeping scaling factor of 12.

Proposal 2: Introduce UE capability on relaxed requirements for cell detection.

# References

[1] R4-2210590 WF on NR extension to 71 GHz RRM requirements (Part 1), Qualcomm

[2] R4-2214982 Draft CR on beam sweeping factor for RRM requirements of FR2-2, Huawei, HiSilicon

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15. NR_ext_to_71GHz | 15-x | Cell detection requirements without relaxation for FR2-2 | UE capable of this feature shall support the cell detection requirements in FR2-2 without relaxation as defined in TS 38.133; |  | Yes | No | The performance of RRM requirements in FR2-2 cannot be guaranteed. | Per band | N/A | Applicable to FR2-2 only | N/A |  | Optional with capability signaling |

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15. NR_ext_to_71GHz | 15-x | Cell detection requirements without relaxation for FR2-2 | UE capable of this feature shall support the cell detection requirements in FR2-2 without relaxation as defined in TS 38.133. |  | Yes | No | The performance of RRM requirements in FR2-2 cannot be guaranteed. | Per band | N/A | Applicable to FR2-2 only | N/A |  | Optional with capability signaling |