GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216267

# Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.3.6.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance requirements for NR operation extending to 71GHz

Document for:	Discussion

# Introduction

In last RAN4 meeting, the performance requirements for FR2-2 are discussed with agreements and issues captured in [1]. In this paper, we further provide our views on some remaining issues.

# Discussion

In last RAN4 meeting, most of the test configurations are settled and draft CR for test cases are also endorsed. One of the remaining issues is about the CCA model in the test cases. The agreements are shown as follows:

Agreement: For CCA model in test cases, an unavailable SSB/SMTC group can be modelled as that there is exactly one SSB not transmitted by TE in N consecutive SSB/SMTC occasions - Shift SSB index in each N consecutive SSB/SMTC occasions rather than keeping one fixed SSB index - FFS: Exact shifting pattern

Based on the discussion in previous meeting, the CCA model is defined on SSB/SMTC group basis which is consist of 12 SSB/SMTC occasions. The remaining point is how to shift the SSB index in each N consecutive SSB/SMTC occasions. Firstly, from our understanding, SSB index here is intended for the index of one SSB burst in N SSB bursts. For instance, the 1st/2nd SSB burst within the 12 SSB bursts. It should not be the SSB index within one SSB burst. As previously agreed, there will be only 1 or 2 SSB index configured for test cases.

Thus, based on the analysis above, we proposed to model the CCA procedure as follows:

- Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probability PCCA_DL.

- If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group.

- If the CCA attempt is determined to be unsuccessful, one of the SSB shall not be transmitted by the test equipment. The SSB within the SSB/SMTC group shall be randomly chosen from all SSBs within the group. The test equipment shall transmit rest transmissions for the SSB/SMTC group.

Proposal 1:

Define CCA model as follows:

- Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probability PCCA_DL.

- If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group.

- If the CCA attempt is determined to be unsuccessful, one of the SSB shall be not be transmitted by the test equipment. The SSB within the SSB/SMTC group shall be randomly chosen from all SSBs within the group. The test equipment shall transmit rest transmissions for the SSB/SMTC group.

Then RAN4 shall define the value of PCCA_DL. In Rel-16 NR-U, PCCA_DL is define with different value for LBE and FBE mode. For FBE, the value is defined as 0.9375. For LBE, the value is define as 0.75/0.75 for different candidate SSB index. For FR2-2, there is no need to differentiate LBE and FBE, and the CCA model is defined on SSB instead of each SSB index. It means the status of all SSB index within one SSB burst are same which is different from Rel-16 NR-U. Then, PCCA_DL is defined as the probability that all SSB are available within one group. Considering that, we think PCCA_DL = 0.9 is a reasonable value for FR2-2, which means the probability of CCA failure for each single SSB is very low.

Proposal 2: Define PCCA_DL = 0.9 in each test case, which is the probability that all SSBs are available within one SSB/SMTC group.

# Conclusions

Proposal 1:

Define CCA model as follows:

- Prior to each SSB/SMTC group which is consist of 12 SSB/SMTC, the test equipment shall determine whether the CCA attempt is successful based on probability PCCA_DL.

- If the CCA attempt is determined to be successful, then the test equipment shall transmit remaining transmissions for the SSB/SMTC group.

- If the CCA attempt is determined to be unsuccessful, one of the SSB shall not be transmitted by the test equipment. The SSB within the SSB/SMTC group shall be randomly chosen from all SSBs within the group. The test equipment shall transmit rest transmissions for the SSB/SMTC group.

Proposal 2: Define PCCA_DL = 0.9 in each test case, which is the probability that all SSBs are available within one SSB/SMTC group.

# References

[1] R4-2214478 WF on NR extension to 71 GHz RRM performance requirements, Qualcomm