3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216295

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on Extended DRX enhancements for Inactive RedCap UE

Source: 	Huawei, HiSilicon

Agenda item:	4.6.3.2

Document for:	Discussion

Introduction

This contribution provides further analysis on Extended DRX enhancements for Inactive RedCap UE.

Discussion

RAN4 achieved the following agreements for the serving cell, intra-frequency and inter-frequency measurement requirements at RAN4#103e meeting [1]. For RedCap UE in RRC inactive mode when idle eDRX is configured, the measurement period on serving cell is based on paging monitoring cycle which is denoted as T.

Issue 1-3-1: Inactive state eDRX intra-frequency cell reselection requirements (combine original issue 1-3-1 and 1-3-2) - Option 1: Measurement requirements for inactivate state (serving cell, intra/inter frequency) Redcap UE are based on inactive DRX or inactive eDRX when inactive eDRX is configured - Option 2: Based on the paging monitoring cycle of T agreed in RAN2 IDLE eDRX[s]; Inactive eDRX[s]; Outside CN PTW or during CN PTW; T. >10.24; Not configured; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; Not configured; Outside CN PTW; RAN paging cycle.. >10.24; ≤10.24; During CN PTW; Shortest value of default paging cycle and UE specific DRX cycle if configured by upper layer. >10.24; ≤10.24; Outside CN PTW; INACTIVE eDRX cycle. Agreement (from 1st round GTW on inactive state eDRX requirements): - For serving cell measurement, agree on Option 2 - For inter-frequency and inter-RAT measurement, agree on Option 1. - FFS whether Option 1 or Option 2 can be agreed for intra-frequency measurement. Agreement (2nd round discussion): Option 1 is agreed for intra-frequency cell reselection requirements for eDRX inactive state

Based on the agreements, the requirements for inactive redcap with eDRX_idle are specified as below (requirements for FR1 are duplicated).

Table 5.1B.2.2-1: Nserv_RedCap for inactive Redcap UE configured with eDRX_IDLE cycle, (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX or eDRX INACTIVE cycle length[s] | T [s] | Nserv _RedCap [number of T ] |
| --- | --- | --- | --- |
| 2.56 ≤eDRX_IDLE cycle length ≤10485.76 | 0.32 ≤DRX_Inactive cycle length ≤2.56; or 2.56 ≤eDRX_Inactive cycle length ≤10.24 if inactive eDRX is configured | 0.32 | 4*M1 |
|  |  | 0.64 | 4*M1 |
|  |  | 1.28 | 2 |
|  |  | 2.56 | 2 |
|  |  | 5.12 | 2 |
|  |  | 10.24 | 2 |
| Note1: T is dertermined according to clause 7.1 in [1]. Note2: M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second. |  |  |  |

As we know that UE performs measurements both within PTW and outside PTW when idle_eDRX is configured. However as per RAN2 definition, the T length within PTW and outside PTW will be different.

| IDLE eDRX | Inactive eDRX | T |
| --- | --- | --- |
| >10.24 | N/A | Within PTW: Min{UE specific DRX cycle(if configured by upper layer), RAN paging cycle, default paging cycle} |
|  |  | Outside PTW: RAN paging cycle |
|  | <=10.24 | Within PTW: min{UE specific DRX cycle (if configured by upper layer) , INACTIVE eDRX cycle and default paging cycle}. |
|  |  | Outside PTW: INACTIVE eDRX cycle |
| <=10.24 | N/A | min{ RAN paging cycle, IDLE eDRX cycle} |
|  | <=10.24 | min{IDLE eDRX cycle, INACTIVE eDRX cycle} |

It is observed that there would be some issues to determine Nserv _RedCap, when UE transmits from within PTW to outside PTW or vise versa during one measurement period. In the following, two examples are given:

- If T within PTW is 0.64s, Nserv _RedCap is defined as 4* M1, if T outside PTW is 2.56, Nserv _RedCap is defined as 2. When UE transmits from within PTW and to outside PTW during one measurement period, T is changed and the evaluation number Nserv _RedCap is changed accordingly. The example is presented in Figure 1 where M1 is assumed as 1. In this scenario shall UE use 4* M1 paging cycles (where Twithin PTW=0.64s) or use 2 paging cycles (where Toutside PTW=2.56s)?

- The existing M1 is specified as:

“The UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second,

-	otherwise M1=1.”

As the logic of serving cell evaluation measurement is based on T (paging occation), it is straight forward to update the existing definition of M1 to “M1=2 if SMTC periodicity (TSMTC) > 20 ms and T ≤ 0.64 second, otherwise M1=1.”

Then the similar issue raises. When T within PTW is less than 0.64s, and T outside PTW is larger than 0.64s, M1 would be changed from 2 to 1. Then Nserv _RedCap is changed from 4*2 to 4. When When UE transitions from measurements within PTW and to measurements outside PTW during one measurement period, how does UE behaviour?

In summary, the identified issue is that both the number of paging cycles and paging cycle length (i.e., T) would be changed from within PTW to outside PTW (and vice versa) during one measurement period eDRX_IDLE is configured for inactive RedCap UE. To address the above  identified issues, it is proposed to make the below clarification:

When UE transitions from measurements within PTW and to measurements outside PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement requirements before or after the transition.

Proposal 1: When UE transitions from measurements within PTW and to measurements outside PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement requirements before or after the transition.

Conclusions

This contribution provides our analysis on eDRX requirements for inactive RedCap UE. The following proposal is provided:

Proposal 1: When UE transitions from measurements within PTW and to measurements outside PTW or vice versa during one measurement period, the UE measurement requirements apply based on the longer measurement requirements before or after the transition.

Reference

- R4-2210596, WF on eDRX and RRM measurement relaxations requirements for Redcap UE, vivo