3GPP TSG-RAN WG4 Meeting # 104bis-e	R4-2216309

Electronic Meeting, October 10 – October 19, 2022

Title: 	Discussion on	improvement on FR2 SCell/SCG setup/resume

Source: 	Huawei, HiSilicon

Agenda item:	6.20.2

Document for:	Discussion

Introduction

One objective of R18 WI on Further NR Mobility Enhancements is improvement on FR2 scell/SCG setup/resume. The corresponding objective is duplicated as below [1]. In last meeting, some discussion were carried out [2]. This contribution provides further analysis on this objective.

- To study the following, with completion targeted by RAN#98 meeting [RAN4]: - The impact of FR2 RRM mobility measurement acquisition and reporting on FR2 SCell/SCG setup/resume delay for a UE connecting from idle/inactive mode. - The level of feasible improvement in FR2 SCell/SCG setup delay from defining new UE measurement procedures and RRM core requirements, and whether additional information from the network would help the UE to perform those measurements effectively. The following sequence of events should be assumed. - The UE initiates and performs improved measurements when it requests RRC connection setup/resume. - After acquiring those improved measurements, the UE subsequently reports those measurements to the network to support SCell/SCG setup.

Discussion

In R16, UE can perform EMR (early measurement report) measurement on neighbour and serving cells during idle/inactive mode. The EMR is reported to UE immediately when UE enters to RRC connected mode. The motivation of R16 EMR is to reduce delay setting up MR-DC / CA. In R18 the objective of improvement on FR2 scell/SCG setup/resume is to let UE perform FR2 L3 measurement during a transition time from idle/inactive mode to connected mode. In other words, UE initiates and performs the improved EMR measurement during RRC connection access procedure. The motivation of this R18 objective is to avoid outdated FR2 measurement result reporting which may exist in R17 EMR as FR2 measurement period in idle/inactive mode is large.

Time duration of “UE connecting from idle/inactive mode”

In last meeting, there are some discussion on the concept of “when it requests RRC connection setup/resume” which is in the WID. We think, for MT originating call, the measurement can be started from the point of UE paging reception. For MO originating call, the measurement can be started from the point when upper layers request RRC connection. The duration from paging reception to PRACH transmission (MSG1) in MT call and the duration from upper layers request RRC connection to PRACH transmission (MSG1) in MO call are similar, and both are the uncertainty time in acquiring the first available PRACH occasion, which is up to 10ms.

Recalling R16 EMR measurement, in idle mode, the available indication of EMR measurement can be included in RRCSetupComplete (MSG 5).  It means that before MSG5, the EMR measurement result shall be available. In inactive mode, the EMR measurement results can be included in RRCResumeComplete. Similar, the EMR measurement result shall be available before MSG5 as well. Therefore regardless idle mode or inactive mode, the ending point of UE perform “improved measurement” is MSG4 in RACH procedure.

Proposal 1: In idle mode and inactive mode, the time duration during RRC connection setup/resume procedure for enhanced measurement,

- For MT originating call: from receiving paging to MSG4;

- For MO originating call:  from the point when upper layers request RRC connection to MSG4.

The latency of RACH procedure (control plane latency) from RRC_Idle/RRC_INACTIVE state to RRC_CONNECTED state was ever widespread evaluated in R16 study on self-evaluation towards IMT-2020 submission [3]. The conclusion is achieved in TS37.910 clause 5.7.2.1:

“It is observed that NR fulfils the control plane latency requirement of 20ms in a wide range of configurations. If, in control plane procedure, the latency of step 7 and step 9 can be further reduced, the 10ms target as encouraged by ITU-R can be achieved in some cases.”

UE

gNB

1. Delay for RACH Scheduling Period

3. Processing delay in gNB

5. Processing delay in UE

7. Processing delay in gNB

9. Processing delay in UE

2. RACH Preamble

4. RA response

6. RRC Resume Request

8. RRC Resume

10. RRC Resume Complete

Control plane procedure

Figure 5.7.2.1-1 C-plane procedure (example for NR Rel-15) [TS37.910]

It can be observed that in wide range of configuration, the time duration from MSG1 to MSG 4 is 20ms in self-evaluation towards IMT-2020 submission.  Additional step 1 corresponds to the time uncertainty in acquiring first available PRACH occasion, this is up to 10ms. Based on the above analysis, the time duration in which UE can perform enhanced measurement during RRC connection setup/resume is from the paging reception/ upper layers requesting RRC connection to MSG4, therefore it is at most 30ms.

Proposal 2: The time duration during RRC connection setup/resume procedure for enhanced measurement is at most 30ms.

Feasibility

As we know, UE shall sweep the RX beams to perform FR2 L3 mobility measurement. For FR2 L3 measurement in both idle/inactive mode and connected mode, the RX beam number is 8. We don’t think UE will apply a special implementation to reduce the RX beam sweeping number during the RRC connection setup/resume procedure, as the time duration is very short and afterwards UE will go back to normal implementation.

Observation 1: Not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure.

To avoid the instant channel impact, several physical measurement samples shall be linear averaged. Using one or less physical measurement samples to present of the cell quality is not preferred, as such measurement accuracy is low and it would degrade the performance robustness. Even there may have some prior information from the EMR measurement during idle/inactive mode, but these information can’t be regarded as always valid and available, because this R18 objective (the improved measurement) is based on the assumption that the measurement results derived from EMR measurement in idle/inactive mode is outdated. In addition, L3 measurement based on one shot is extremely radical, which may mistakenly setup CA/DC.

Observation 2: Not to reduce number of samples during the RRC connection setup/resume procedure.

In legacy, L3 measurement periodicity depends on DRX cycle in idle/inactive mode. It is agreed in last meeting that DRX is not in use during RRC connection setup/resume procedure for enhanced measurement. Then SMTC is supposed to be used to specify RRM requirements. To further reduce the measurement delay, some companies proposed to use SSB periodicity. In typical configuration, SSB periodicity is 20ms. Considering the total time duration for RRC connection setup/resume is at most 30ms, 20ms RS periodicity is relatively large.

Observation 3: Considering the total time duration for RRC connection setup/resume is at most 30ms, either SMTC or SSB periodicity is relatively large.

Therefore it is doubted that whether UE can complete a FR2 L3 measurement during RRC connection setup/resume procedure for enhanced measurement.

Proposal 3: It is doubted that whether UE can complete a FR2 L3 measurement during RRC connection setup/resume procedure for enhanced measurement.

In last meeting, one view proposed that two active RF chains are used when performing enhanced measurement. However in our understanding, when UE will setup CA/DC after it enters to connected mode depends on the subsequent traffic load. It is likely that network doesn’t configure CA/DC for the UE after it reports the enhanced measurement results achieved during RRC connection setup/resume procedure. So from UE implementation perspective, it is not desired to open two active RF chains only during RRC connection setup/resume while before and after this procedure, one RF chain is used.

Observation 4: One active RF chain is used during RRC connection setup/resume procedure, as it is likely that network doesn’t configure CA/DC when UE enters to connected mode.

If one RF chain is used, as we know UE shall sweep the RX beams to perform FR2 L3 mobility measurement. Simultaneously during RACH procedure, UE is request to receive MSG2 and MSG4. If UE has tuned its beam to perform neighbour cell measurement, there is a risk that MSG2/MSG4 is not correctly received.

Even if two RF chains are used, a UE with IBM capability may use one RF chain for SCG setup/resume on PCell, while the other RF chain is used for inter-band measurement. However if there are more than one frequency to be measured, the interruption will happen on PCell, as frequency switching procedure resembles the SCell addition/change procedure. Then it may also interrupt MSG2/MSG3/MSG4 in RACH procedure. Moreover when network doesn’t reconfigure UE to setup CA/DC after UE enters to RRC connected mode, UE would fall back to one RF chain, which will introduce additional interruptions furtherly.

Observation 5:

- If one RF chain is used, Msg2/3/4/5 may be impacted during RRC connection setup/resume due to RF retuning.

- Even if two RF chains are used, for IMB capable UE, for inter-band inter-frequency measurement, Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning. Moreover if no CA/DC is setup after UE enters to RRC connected mode, additional interruption would be furtherly introduced.

Based on the above analysis, some potential issues are identified. We suggest to further discuss the feasibility of improvement in FR2 SCell/SCG setup delay before specifying related requirements.

Proposal 4: Further study is needed before RAN4 confirms the feasibility of early measurement during connection setup.

Conclusions

This contribution provides the consideration on improvement of FR2 SCell/SCG setup/resume in R18 mobility enhancement. The following proposals are provided:

Proposal 1: In idle mode and inactive mode, the time duration during RRC connection setup/resume procedure for enhanced measurement,

- For MT originating call: from receiving paging to MSG4;

- For MO originating call:  from the point when upper layers request RRC connection to MSG4.

Proposal 2: The time duration during RRC connection setup/resume procedure for enhanced measurement is at most 30ms.

Observation 1: Not to reduce the scaling factor of Rx beam sweeping during the RRC connection setup/resume procedure.

Observation 2: Not to reduce number of samples during the RRC connection setup/resume procedure.

Observation 3: Considering the total time duration for RRC connection setup/resume is at most 30ms, either SMTC or SSB periodicity is relatively large.

Proposal 3: It is doubted that whether UE can complete a FR2 L3 measurement during RRC connection setup/resume procedure for enhanced measurement.

Observation 4: One active RF chain is used during RRC connection setup/resume procedure, as it is likely that network doesn’t configure CA/DC when UE enters to connected mode.

Observation 5:

- If one RF chain is used, Msg2/3/4/5 may be impacted during RRC connection setup/resume due to RF retuning.

- Even if two RF chains are used, for IMB capable UE, for inter-band inter-frequency measurement, Msg2/3/4/5 may be impacted if there are more than one frequency to measure during RRC connection setup/resume due to RF retuning. Moreover if no CA/DC is setup after UE enters to RRC connected mode, additional interruption would be furtherly introduced.

Proposal 4: Further study is needed before RAN4 confirms the feasibility of early measurement during connection setup.

Reference

- RP-213565, New WID on Further NR mobility enhancements, MediaTek

- R4-2214348, WF on Further NR Mobility Enhancements, MediaTek

- TS37.910, Study on self evaluation towards IMT-2020 submission (Release 16)