3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216320

Electronic Meeting, 10 – 19 October, 2022

Title: 	Discussion on cell reselection test for NTN

Source: 	Huawei, HiSilicon

Agenda item:	4.2.6.2

Document for:	Discussion

Introduction

RRM test cases for NTN cell reselection are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- SMTC setup and scaling factor “K_multi”

- Time triggered measurement

- Location triggered measurement

- Other

In this paper we will provide our views on cell reselection test cases for NTN.

Discussion

SMTC setup and scaling factor “K_multi”

- Issue 3-1: General test design for intra/inter-frequency cell reselection. - Agreement: - RAN4 needs to study proper values of signal quality/power parameters to model the satellite communication condition. - To verify time based cell reselection measurement (time based), the signal level of active cells should be larger than Srxlev or Squal threshold in the test design. - RAN4 not include the trigger cases: “t-service+10” and “T1+10” in TCs. - FFS whether to include “t-service” configuration in TCs. - FFS on whether and how to include “K_multi” parameter in the tests

In our view, t-Service impacts the requirements in such a way that when it is configured, UE should complete neighbour cell measurement on all carriers before it, which means it is effectively a measurement trigger. As RAN4 agreed to define dedicated TCs for time triggered measurement in TC 1-3 and 1-7, we do not see the need to configure t-Service in other TCs.

Proposal 1: t-Service is configured in TC 1-3 and 1-7, but not configured in other TCs.

On scaling factor K_multi, it is defined in the measurement delay requirements when UE is configured to measure multiple overlapping SMTCs or multiple LEO satellites in one SMTC. As such it should be included in the test requirements depending on whether the test setup involves multiple overlapping SMTCs or multiple LEO satellites in one SMTC.

With two satellites in the test, at most two SMTCs can be configured. We would have following cases:

- One satellite per SMTC, two SMTCs are overlapping. K_multi = 2 for this case.

- One satellite per SMTC, two SMTCs are not overlapping. K_multi = 1 for this case.

- Two satellites per SMTC. K_multi = 1/2 for this case with GEO/LEO.

In our view, it is meaningful to test different STMC configurations with different TCs.

Proposal 2: Use the following SMTC configurations for TC 1-1 – 1-4.

- TC 1-1: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 non-overlapping

- TC 1-2: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 overlapping

- TC 1-3 and 1-4: serving cell in SMTC1, neighbor cell in SMTC1

- Scaling factor “K_multi” is taken into account in the testing requirement for TC 1-1 – 1-4.

It is noted that for TC 1-5 – 1-8, the two cells from two satellites are on different carriers, so they can follow the current SSMTC configuration in A.6.1.1.2, and K_multi should not be considered in the test requirement.

Proposal 3: Use the same SMTC configuration as in A.6.1.1.2 for TC 1-5- 1-8, and scaling factor “K_multi” is not taken into account in the testing requirement.

Time triggered measurement

- Issue 3-3: Test case for intra/inter-frequency cell reselection with timer trigger. - Agreement: - Regarding test cases for reselection measurement based on time trigger, the time instant fulling Tserivce is after T3, time period between the end of T3 (refer to A.6.1.1.1.3) and Tserivce shall be equal to or longer than measurement delay (Tdetect, NR_Intra or Tevaluate, NR_ intra defined in A.6.1.1.1.3) for reselection.

In [1] it was agreed that t-Service is after T3, and the gap between t-Service and start of T3 should be larger than Tdetect. In our view, for TC 1-3 and 1-7 only two time periods T1 and T2 are needed. In particular, the 3 time periods in A.6.1.1.1 are used as follows.

- T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

- T2: cell2 is powered on and better than cell1, UE shall reselect to cell2 which is a newly detectable cell

- T3: cell1 is better than cell2, UE shall reselect to cell2 which is an already detected cell

It can be seen that the purpose of T3 is to verify UE can evaluate an already detected cell within Tevaluate. This can be already verified with TC 1-1 and 1-5. For TC 1-3 and 1-7, the purpose is to verify UE can trigger measurement timely and complete measurement before t-Service. We think having T2 is enough, otherwise t-Service needs to be set for both cell1 (during T2) and cell2 (during T3) which makes the test case complex.

We suggest to define the test case based on the setup shown in Figure 1.

Figure 1: Test setup for time triggered measurement in cell reselection

Proposal 4: TC 1-3 and 1-7 consists two time periods T1 and T2:

- Before test: UE camps in cell1, and t-Service is included in SIB19 of cell1

- T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

- T2: cell2 is powered on, T2 is 40s, t-Service is pointed to the time point (start of T2 + 36s)

- UE should reselect to cell2 before t-Service

Location triggered measurement

- Issue 3-4: Test case for intra/inter-frequency cell reselection with location trigger. - Agreement: - Dmargin is not added to the distance threshold, and it is reflected in the test configuration. - Open issues: - FFS on whether to consider the response time for UE to response when DdistanceThresh condition is met

For similar reasons as discussed in section 2.2, we suggest to define TC for location triggered measurement with two time periods, as shown in Figure 2.

Figure 1: Test setup for location triggered measurement in cell reselection

One open issue from [1] is whether response time is needed when the location trigger condition is met. This should depend on the test method for changing UE GNSS location. If the whole GNSS test setup is used, then some response time is needed for UE to obtain the location change, and RAN4 may need to discuss required response time, e.g. the 2s for moving test in 38.171. If special test procedure “Update UE Location Information” is used to artificially change the UE GNSS location, the response time may be quite small or even not required.

Proposal 5: TC 1-4 and 1-8 consists two time periods T1 and T2:

- Before test: UE camps in cell1, and Ref-location is included in SIB19 of cell1

- T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

- T2: cell2 is powered on, T2 is 40s, UE location is changed at the time point (start of T2 + 4s)

- UE should reselect to cell2 before time point (start of T2 + 36s + Δ), Δ is the response time depending on how UE GNSS location is changed in the test.

Other

In the test requirements, UE shall camp on the new cell and start PRACH transmission for TA update. UE can only camp on the cell and do RA after UE read necessary system information in the new cell. For TN, the mainly refers to SIB1, but for NTN SIB19 is also needed besides SIB1.

Current test requirement for TSI-NR is 1280ms. SIB19 reading time should be considered in the testing requirement for TSI-NR, or alternatively SIB19 scheduling period is small enough for the current requirement.

Proposal 6: SIB19 reading time should be considered in the testing requirement for TSI-NR, or alternatively SIB19 scheduling period is small enough for the current requirement.

Conclusions

In this paper we provided our views on cell reselection test cases for NTN.

Proposal 1: t-Service is configured in TC 1-3 and 1-7, but not configured in other TCs.

Proposal 2: Use the following SMTC configurations for TC 1-1 – 1-4.

- TC 1-1: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 non-overlapping

- TC 1-2: serving cell in SMTC1, neighbor cell in SMTC2, SMTC1 and SMTC2 overlapping

- TC 1-3 and 1-4: serving cell in SMTC1, neighbor cell in SMTC1

- Scaling factor “K_multi” is taken into account in the testing requirement for TC 1-1 – 1-4.

Proposal 3: Use the same SMTC configuration as in A.6.1.1.2 for TC 1-5- 1-8, and scaling factor “K_multi” is not taken into account in the testing requirement.

Proposal 4: TC 1-3 and 1-7 consists two time periods T1 and T2:

- Before test: UE camps in cell1, and t-Service is included in SIB19 of cell1

- T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

- T2: cell2 is powered on, T2 is 40s, t-Service is pointed to the time point (start of T2 + 36s)

- UE should reselect to cell2 before t-Service

Proposal 5: TC 1-4 and 1-8 consists two time periods T1 and T2:

- Before test: UE camps in cell1, and Ref-location is included in SIB19 of cell1

- T1: cell2 is powered off, T1 is long enough to make UE have no information about cell2

- T2: cell2 is powered on, T2 is 40s, UE location is changed at the time point (start of T2 + 4s)

- UE should reselect to cell2 before time point (start of T2 + 36s + Δ), Δ is the response time depending on how UE GNSS location is changed in the test.

Proposal 6: SIB19 reading time should be considered in the testing requirement for TSI-NR, or alternatively SIB19 scheduling period is small enough for the current requirement.

Reference

- R4-2214473, WF on performance part for NR NTN, Xiaomi