3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216323

Electronic Meeting, 10 – 19 October, 2022

Title: 	Discussion on measurement delay TCs for NTN

Source: 	Huawei, HiSilicon

Agenda item:	4.2.6.7

Document for:	Discussion

Introduction

RRM measurement delay test cases for NTN are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- SMTC configuration

- Test case reduction

In this paper we will provide our views on measurement delay test cases for NTN.

Discussion

SMTC configuration

- Issue 2-4: Test coverage regarding multiple SMTCs. - Agreement: - Test cases shall take multi-SMTC and multi-satellite tests into account. - Minimize the test case number. - RAN4 to define the following configurations for SMTC and satellite in clause A.3 RRM test configuration. - Config. 1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites - FFS further split into overlapping and non-overlapping cases - Config. 2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites - FFS Config. 3: 2 SMTC per MO, each SMTC contains 2 SSB/Satellites - FFS define the configurations for measurement gap in clause A.3 RRM test configuration. - Config.1: 1 measurement gap configuration; (reuse the exiting measurement gap configuration) - Config.2: 2 measurement gap configurations (FNO case); - Config.3: 2 measurement gap configurations (PPO case); - FFS Config.4: 2 measurement gap configurations (FO case with MGRP = 160ms);

The discussion on multiple SMTC is for measurement delay TCs. We have a companion paper on the cell reselection TCs, and in this paper we will focus on event triggered reporting TCs i.e. TC 10-x and 11-x.

In our view, it is important to limit the number of satellites in the test otherwise the design and implementation of the testing can increase, so we suggest to not further consider Config 3 which involves 4 satellites in total.

For Config 1, we think it is necessary to define separately test overlapping and non-overlapping case. When two SMTCs are non-overlapping, UE should be able to measure the two cells without any scaling, i.e. scaling factor for each cell is 1. When two SMTCs are overlapping, UE should be able to measure the two cells alternately and with scaling factor for each cell as 2. The two cases requires different UE behaviours.

It should be noted that above analysis are for intra-frequency TCs (10-x) where there is only one MO in the test. For inter-frequency TCs (11-x), as the two cells/satellites are on different carriers, there can be only one SMTC per MO, and each SMTC contains 1 satellite. It is also meaningful to separately test overlapping and non-overlapping case. For overlapping case, only a single MG is required, and for non-overlapping case, con-MGs are required.

Proposal 1: Use the following SMTC configurations in the measurement delay TCs.

- For intra-frequency TCs (10-x),

  - Config.1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites

    - Config.1a: two SMTCs are overlapping

    - Config.1b: two SMTCs are non-overlapping

  - Config.2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites

- For inter-frequency TCs (11-x), use the following SMTC configurations:

  - Config.0: 1 SMTC per MO, each SMTC contains 1 SSB/Satellites

    - Config.0a: two SMTCs are overlapping

    - Config.0b: two SMTCs are non-overlapping

Test case reduction

Intra-frequency

Current there are 9 TCs for intra-frequency measurement delay.

| 10-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 10-2 | Event triggered reporting tests without gap under DRX |
| 10-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 10-4 | Event triggered reporting tests with measurement gap under non-DRX |
| 10-5 | Event triggered reporting tests with measurement gap under DRX |
| 10-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading |
| 10-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 10-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 10-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

For measurement without gap (TC 10-1 – 1-3), we are using 3 different SMTC configurations, i.e. Config.1aq, Config.1b and Config 2 for different TCs. In our view, this allows RAN4 to test all the possible configurations corresponding to different requirements while keeping the TC number as minimum.

For measurement with gap (TC 10-4 – 10-9), we assume Config.2 will be used in TC 10-4 – 1-6 as only single MG is used, and Config.1b will be used in TC 10-7 – 1-8 as FNO con-MG is used. It is a question why the TC number for different SMTC configurations is different, and more importantly, whether we need multiple TCs for one SMTC configuration. It is noted that in A.6.6.18, all the TCs for con-MG are defined without DRX, and it is unclear to us why FNO case need to be tested with both DRX and non-DRX (TC 10-7 and 10-8) for NTN.

Our preference is to follow the same principle as for measurement without gap.

Proposal 2: Update the TC list for intra-frequency measurement delay as follows.

| 10-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 10-2 | Event triggered reporting tests without gap under DRX |
| 10-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 10-4 | Event triggered reporting tests with single measurement gap under non-DRX |
| 10-5 | Event triggered reporting tests with FNO concurrent measurement gap under DRX |
| 10-6 | Event triggered reporting tests with PPO concurrent measurement gap under non-DRX with SSB index reading |
| 10-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 10-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 10-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

Inter-frequency

Current there are 9 TCs for inter-frequency measurement delay.

| 11-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 11-2 | Event triggered reporting tests without gap under DRX |
| 11-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 11-4 | Event triggered reporting tests with measurement gap under non-DRX |
| 11-5 | Event triggered reporting tests with measurement gap under DRX |
| 11-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading |
| 11-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 11-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 11-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

For measurement without gap (TC 11-1 – 11-3), we assume TC 11-3 can be removed because for TN only two TCs are defined for inter-frequency without gap in clause A.6.6.2.

For measurement with gap (TC 11-4 – 11-9), we assume TC 11-8 can be removed because for TN only two TCs are defined for inter-frequency with con-MG in clause A.6.6.18.

Proposal 3: Update the TC list for inter-frequency measurement delay as follows.

| 11-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 11-2 | Event triggered reporting tests without gap under DRX |
| 11-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 11-4 | Event triggered reporting tests with measurement gap under non-DRX |
| 11-5 | Event triggered reporting tests with measurement gap under DRX |
| 11-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading |
| 11-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 11-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 11-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

Conclusions

In this paper we provided our views on measurement delay test cases for NTN.

Proposal 1: Use the following SMTC configurations in the measurement delay TCs.

- For intra-frequency TCs (10-x),

  - Config.1: 2 SMTC per MO, each SMTC contains 1 SSB/Satellites

    - Config.1a: two SMTCs are overlapping

    - Config.1b: two SMTCs are non-overlapping

  - Config.2: 1 SMTC per MO, each SMTC contains 2 SSBs/Satellites

- For inter-frequency TCs (11-x), use the following SMTC configurations:

  - Config.0: 1 SMTC per MO, each SMTC contains 1 SSB/Satellites

    - Config.0a: two SMTCs are overlapping

    - Config.0b: two SMTCs are non-overlapping

Proposal 2: Update the TC list for intra-frequency measurement delay as follows.

| 10-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 10-2 | Event triggered reporting tests without gap under DRX |
| 10-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 10-4 | Event triggered reporting tests with single measurement gap under non-DRX |
| 10-5 | Event triggered reporting tests with FNO concurrent measurement gap under DRX |
| 10-6 | Event triggered reporting tests with PPO concurrent measurement gap under non-DRX with SSB index reading |
| 10-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 10-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 10-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

Proposal 3: Update the TC list for inter-frequency measurement delay as follows.

| 11-1 | Event triggered reporting tests without gap under non-DRX |
| --- | --- |
| 11-2 | Event triggered reporting tests without gap under DRX |
| 11-3 | Event triggered reporting tests without gap under non-DRX with SSB index reading |
| 11-4 | Event triggered reporting tests with measurement gap under non-DRX |
| 11-5 | Event triggered reporting tests with measurement gap under DRX |
| 11-6 | Event triggered reporting tests with measurement gap under non-DRX with SSB index reading |
| 11-7 | Event triggered reporting tests with FNO concurrent gaps under non-DRX |
| 11-8 | Event triggered reporting tests with FNO concurrent gaps under DRX |
| 11-9 | Event triggered reporting tests with PPO concurrent gaps under non-DRX |

Reference

- R4-2214473, WF on performance part for NR NTN, Xiaomi