3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2216335

Electronic Meeting, 10 – 19 October, 2022

Title: 	Discussion on RRM requirements for MUSIM gaps

Source: 	Huawei, HiSilicon

Agenda item:	6.21.2

Document for:	Discussion

Introduction

RRM requirements for MUSIM gaps are discussed in RAN4#104-e, and the outcomes are captured in [1]. Based on [1], the following issues need to be further discussed.

- Gap collision handling

- NW A requirements

- NW B requirements

- Other requirements

In this paper we will provide our views on RRM requirements for MUSIM gaps.

Discussion

Gap collision handling

Case 1: MUSIM gap and legacy MG

Issue 2-3-2-1: Clarification on the scope of Rel-17 legacy gap - Proposals: - Option 1: Discuss if concurrent MUSIM and other Rel17/18 measurement gap types is in the scope of this WID or NR_MG_enh2 (Nokia) - Option 2: In case 1, gaps to be considered include all gaps defined till Rel-17 including Pre-MG, NCSG and legacy gaps for measurement and other purposes (MTK Apple Charter CMCC Huawei vivo xiaomi Qualcomm oppo) - Option 2a (Ericsson): Use Option 2 with the following note: Note 1: The group needs to further consider how to handle Pre-MG/NCSG and MUSIM gaps. Note 2: The Pre-MG/NCSG and concurrent gaps are discussed in parallel in Rel-18 WI further MG enh. Tentative agreements: No

We support the understanding of option 2 and 2a.

When MUSIM gaps are configured, NWA could also configure normal MG, pre-MG or NCSG for RRM measurement, so it is meaningful to consider collision between MUSIM gaps and these MGs.

The term normal MG may need some clarification. A normal MG means the MG is not a pre-MG or NCSG, and a normal MG can be configured in two ways as below. Both cases should be considered for collision with MUSIM gaps in our view.

- #1: via GapConfig, or

- #2: via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17

In our view, MUSIM gaps are similar to normal MGs, i.e. they cannot be (de)activated like pre-MG and they have only MGL instead of VIL and ML as NCSG. As noted in option 2a, the concurrency between normal MG and pre-MG/NCSG is being discussed in Rel-18 feMG WI, and as such the conclusion can be re-used for concurrency between MUSIM gaps and pre-MG/NCSG. There is no need to start a completely new and separate discussion in Rel-18 MUSIM WI for concurrency between MUSIM gaps and pre-MG/NCSG.

Proposal 1: “Legacy MG” in Case 1 includes pre-MG, NCSG and ‘normal MG’ (MG configured via GapConfig, or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17).

- Re-use the conclusions from Rel-18 feMG WI on concurrency between MUSIM gaps and pre-MG/NCSG.

Issue 2-3-2-3: Priority of MUSIM against other legacy gaps - Proposals: - Option 1: MUSIM gaps should have high priority in the event of a collision (Charter Qualcomm) - Option 2: MUSIM gaps can be defined as the lowest priority, and periodic MUSIM gaps will be dropped once the gap dropping rule defined in Con-MGs is met () - Option 3: Up to NW configuration (Apple CMCC Huawei Xiaomi Charter oppo vivo) - Option 3a: Up to UE request (MTK) - Option 4: FFS (Ericsson Nokia) Tentative agreements: No

Issue 2-3-1: General principles on gap collision handling - Proposals: - Option 1: For priority based solution, priorities can be allocated to each existing gap patterns and when two or more gap collide, only the highest priority gap is kept and all other gaps are dropped (Apple Huawei Xiaomi vivo) - Option 2: Apply gap-group priority to handle collisions between different gaps groups (i.e., MUSIM gaps group and legacy MGs group). Then, within each gap group, apply different priorities to handle the collision between the gaps within the same group (Ericsson Charter) - Option 3: Agree at high-level that applying priority rule to handle collisions, but the way how to apply it can be FFS (oppo MTK CMCC vivo) - Option 4: priority-based scheme for (a) Collisions between a MUSIM gap and measurement gaps and (b) Collisions between MUSIM gaps, but the definition of collisions may be different for cases a and b. (Qualcomm) - Option 5: FFS (Nokia) Tentative agreements: No

Issue 2-3-1-1: On network A priority assignment scheme - Proposals: - Option 1: RAN4 to study the issue when the priority is all assigned by NW A, under the current signalling framework, which might lead to missing significant activities in NW B due to MUSIM gap collision handling (e.g., reading the paging in NW B, which are unknown to NW A) (MTK Apple Ericsson Huawei QC oppo vivo) - Option 2: Clarification is needed for option 1 (Nokia) Tentative agreements: Further study on the issue in option 1.

In last meeting, it was agreed in Issue 2-3-2-2 that the Rel-17 priority rule will be used as baseline for handling collision between MUSIM gaps and legacy MGs. The remaining issue is how to determine the priority of MUSIM gaps.

First, we think the priority of MUSIM gaps should be configured by NW A. In Rel-17, the MG priority are configured by the NW, and we suggest to re-use the same approach as this gives flexibility to the NW A, who configures both MUSIM gaps and legacy MGs. MUSIM can be in general considered as one purpose for configuring gaps, and NW A may consider it more important than some of the RRM measurements while less important than some other RRM measurements.

In last meeting, some companies suggested to study whether NW A could provide different priority for different MUSIM gaps, or a kind of group priority should be used for MUSIM gaps.

In our view, NW A may provide different priority for different MUSIM gaps even with current signalling. For example, NW A may consider the periodicity of requested MUSIM gaps, i.e. if the periodicity of a MUSIM gap is very large, NW A may give it higher priority than legacy MGs because the impact of this MUSIM gap to NW A is small while dropping it may cause very long delay to NW B operation.

It is also noted that allowing NW A to configure different priority for different MUSIM gaps does not preclude NW A to configure same priority for all MUSIM gaps. Adopting the group priority may impose unnecessary restriction to NW A, and may negatively limit the use of MUSIM gaps.

Another issue discussed in last meeting is how NW A could determine the priority of MUSIM gaps. Based on current signalling framework, NW A is not aware of the exact purpose of each MUSIM gaps, e.g. whether it is used for paging or for RRM measurement. Some companies proposed to allow UE to inform the usage or priority of MUSIM gaps to NW A.

In our understanding, reporting purpose or usage of MUSIM gaps has been discussed in Rel-17 but it was not agreed by RAN2. Technically, it is NW A who decides whether to configure MUSIM gaps or not, and it is also NW A who makes the decision on priority of MUSIM gaps.

- NW A may not care how MUSIM gaps are used for NW B operations. In this case, even UE reports a particular use case or requests high priority for a particular MUSIM gap, NW A may still configure it with low priority.

- If NW A wants to configure different priorities for different MUSIM gaps, it is more likely to consider the periodicity of a MUSIM gap, or in other words, the impact to NW A. On the other hand, we assume important operations in NW B such as paging or SI reading typically require MUSIM gaps with large periodicity or AP gaps, while MUSIM gaps with small periodicity are usually used for measurement, which may not be regarded a high priority compared to NW A’s own measurement.

Based on above discussion, we do not see strong need to allow UE to inform the usage or priority of MUSIM gaps to NW A. It is noted that the above discussions are for collision between MUSIM gaps and legacy MG, while the collision between MUSIM gaps are discussed in section 2.1.3, and our proposal is that the priority configured by NW A does not impact collision between MUSIM gaps.

Proposal 2: Priority of MUSIM gaps are up to NW A configuration. Different priorities can be configured for different MUSIM gaps.

Issue 2-3-2-5: Definition on MUSIM gap collides with legacy gaps - Proposals: - Option 1: The gap proximity condition of concurrent gap collision could be reused for MUSIM gap collision with other gaps (Ericsson Apple Charter Huawei Qualcomm Xiaomi MTK vivo) - Option 2: FFS (Nokia) Tentative agreements: No

Another open issue for Case 1 is the definition on MUSIM gap collides with legacy MGs. We think it is straightforward to re-use the proximity condition for con-MGs in Rel-17, and so far we have not seen clear reason to use a different definition.

Proposal 3: For collisions between MUSIM gaps and legacy MG, the proximity condition of Rel-17 concurrent gap is reused.

Issue 2-3-2-4: Order for applying the priority when number of colliding MGs is larger than 2 - Proposals: - Option 1: For collisions between MUSIM gap and legacy measurement gap (i.e. Rel-15 to Rel-17 measurement gaps), RAN4 to discuss the order for applying the priority when number of colliding MGs is larger than 2. (Huawei) - Option 2: The gap with the highest priority is kept when colliding (Apple vivo) - Option 3: FFS (Apple Ericsson MTK Huawei xiaomi QC Nokia) Tentative agreements: FFS

It is noted that based on Rel-17, multiple MGs can be already configured for NW A RRM measurement, and it was agreed to consider up to 2 priorities because in the same FR the number of MGs that may collide is up to 2. With MUSIM gaps, the number of colliding gaps can go beyond 2.

With MUSIM gaps, the number of colliding gaps can be larger than 2, e.g. two MUSIM gaps and one legacy MG. For this case, RAN4 has to discuss the order for applying the priority e.g. when the 3 gaps are configured with 3 different priorities. Similar issue has been discussed in RAN4#103-e for concurrent MGs in Issue 2-2-2 in [2], and RAN4 can consider the options from that discussion for handling Case 1.

Proposal 4: For collisions between MUSIM gaps and legacy MG, RAN4 to discuss the order for applying the priority when number of colliding gaps is larger than 2.

Case 2: MUSIM gap and NW A RRM procedure

Issue 2-3-3-1: Definiton of collisions between MUSIM gap and SMTC and other L3/L1 measurement resources - Proposals: - Option 1: Condition “SMTC is overlapping with MUSIM gap“ and “L1 measurement resource is overlapping with MUSIM gap”could be used as baseline for MUSIM gap collision with SMTC an L1 measurement resources (Apple) - Option 2: “Condition “SMTC is overlapping with MG” and “L1 measurement resource is overlapping with MG”could be used as baseline for MUSIM gap collision with SMTC and L1 measurement resources. (MTK Huawei xiaomi) - Option 3: RAN4 to discuss the proximity condition for the following cases: “SMTC is overlapping with MG” and “L1 measurement resource is overlapping with MG” (Ericsson oppo Nokia) - Option 4: FFS (Huawei Qualcomm Xiaomi MTK vivo) - Option 5: Further consider the proximity discussed in NTN other than only the fully/partially overlapping case (Ericsson) Tentative agreements: No

We support option 1 and option 2, i.e. collision between SMTC/L1 resource of NW A and MUSIM gaps is based on their “physical” overlap in time domain. The collision between SMTC/L1 resource of NW A and MG configured by NW A is based on “physical” overlap in time domain. We think it is straightforward to re-use the same definition for MUSIM gaps.

In last meeting, some companies suggested to consider the collision definition for one SMTC outside MG and the other SMTC within MG, as discussed in Issue 3 for NTN [3]. We understand the scenarios are different. For NTN, what is discussed is the collision between two SMTCs for the same MO, and the context is that UE is not required to measure two colliding SMTCs for the same MO. It is not for collision between an SMTC and a MG.

Proposal 5: Condition “XXX is overlapping with MG” is used for defining MUSIM gap collision with SMTC and L1 measurement resources in NW A.

Issue 2-3-3-2: Priority of MUSIM against SMTC, and other L3/ L1 measurement resources - Proposals: - Option 1: MUSIM gaps should have high priority against SMTC and L1 measurement resources (Apple xiaomi oppo Qualcomm Huawei MTK vivo) - Option 2: NW-A’s RRM procedure, including DL SMTC should have higher priority than MUSIM gaps. The MUSIM periodic gaps should be dropped once the gap proximity rule is met. (Ericsson) - Option 3: As baseline solution, UE can only perform gap-less L3 measurement and L1 operation outside MUSIM gap. Other solutions are not precluded to handle collision between MUSIM gap and SMTC/RS for L1 operation. (Apple oppo xiaomi) - Option 4: FFS (Ericsson Nokia CMCC vivo) Tentative agreements: No

Issue 2-3-3-3: Priority of MUSIM against uplink signals, such as PRACH, CSI-RS reporting etc. - Proposals: - Option 1: NW-A’s RRM procedure, including UL CSI-RS, PRACH, should have higher priority than MUSIM gaps. The MUSIM periodic gaps should be dropped once the gap proximity rule is met. (Ericsson) - Option 2: PRACH procedure can be higher priority than MUSIM gaps (MTK) - Option 3: FFS (Huawei Ericsson Apple Nokia CMCC Qualcomm Xiaomi MTK vivo) Tentative agreements: No

As a general principle, we understand the priority of MUSIM gaps are higher than transmission or reception in NW A, including SMTC and L1 measurement. In any case, it is NW A who configures MUSIM gaps, and from interruption point of view, MUSIM gaps have no difference than the MG configured for NW A’s RRM measurement, which always has higher priority than other transmission or reception in NW A.

### 9.1.2 Measurement gap During the per-UE measurement gaps the UE: - is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

Following the same principle as legacy MG, UE is in general not expected to transmit or receive signals for NW A (including SMTC and L1 measurement resources) during MUSIM gaps, except for signals used for random access procedure.

In last meeting, some companies suggested to further consider the priority between MUSIM gaps and certain RRM procedure in NW A, e.g. SCell activation, HO, TCI state switching, or some important transmissions or receptions in NW A, e.g. paging, SI, UCI. While we can understand the point, we are not sure if RAN4 needs to specify such details for MUSIM gaps when the corresponding prioritization has not been specified for legacy MGs, i.e. it is already possible to have legacy MG colliding with those procedures, but RAN4 has not defined how to handle the collision between NW A MG and all the procedures.

For example, the SCell activation delay can be several SMTC periods, and when MG is configured, it could happen that MG is colliding with SMTC of the SCell. It is clear that UE cannot perform MG based RRM measurement and SCell activation at the same time, and as such the requirements for one of the procedures need to be relaxed, but so far RAN4 has not defined the what happens in this case. We believe collision with legacy MG is more relevant than collision with MUSIM gaps in real deployment.

In addition, besides legacy MG and MUSIM gaps, there are also other sources of interruption, e.g. measurement of deactivated SCell, SRS carrier or antenna switching, BWP switching, etc. The question is whether RAN4 should define requirements or prioritization for all collisions between all interruption sources and all important RRM procedures/important transmissions or receptions.

Based on above, we suggest not to define specific handling for collision between MUSIM gaps and specific RRM procedures / transmissions or receptions in NW A.

Proposal 6: UE is in general not expected to transmit or receive signals for NW A (including SMTC and L1 measurement resources) during MUSIM gaps, except for signals used for random access procedure.

- RAN4 not to define specific handling for collision between MUSIM gaps and specific RRM procedures / transmissions or receptions in NW A.

Case 3: MUSIM gap and MUSIM gap

Issue 2-3-4: Collisions between different MUSIM gaps - Proposals: - Option 1: priority rule can be used as baseline (Apple oppo CMCC Huawei Xiaomi MTK vivo) - Option 2: RAN4 will discuss separately how to define and resolve collisions between MUSIM gaps (Ericsson Huawei Qualcomm) - Option 2a: When the time duration between the two closest gap occasions within the two measurement gap patterns is shorter than [4]ms and the second gap occasion is for paging, UE should keep both gap occasions instead of dropping any of them. (Ericsson) - Option 3: Aperiodic gap should have higher priority than periodic gaps once collision happens within MUSIM gaps. (Ericsson MTK) - Option 4: It is UE’s responsibility not to request colliding MUSIM gaps from NW-A (Ericsson Nokia) - Option 5: Option 2 can be discussed if option 1 is agreed (Charter MTK) - Option 5a: Option 3 can be discussed if option 1 is agreed (Charter) Tentative agreements: No

In our view, MUSIM gaps should not be dropped due to collision with another MUSIM gap.

All the MUSIM gaps are requested by the UE, and if two MUSIM gaps are colliding based on UE request, it would mean that UE needs to use both of them. As mentioned by some companies in last meeting, one MUSIM gap may be used for serving cell measurement (sync/AGC) and another MUSIM gap may be used for paging, and depending on the distance between PO and SSB in NW B, it could happen that those two gaps are colliding, and in this case no gap should be dropped.

In addition, the use of MUSIM gaps is up to UE, so it would be unreasonable for NW A to decide which gap should be kept and which one to drop, based on the configured priority. In other words, NW A could decide the priority between MUSIM gaps and legacy MG for NW A’s own RRM measurement, but it should not decide the priority between different MUSIM gaps.

Proposal 7: MUSIM gaps are not dropped due to collision with another MUSIM gap.

- No definition for collision between MUSIM gaps is needed.

Aperiodic gap

Issue 2-3-5-1: On aperiodic gap priority - Proposals: - Option 1: UE can request aperiodic MUSIM gap with a higher priority. (Ericsson Charter) - Option 2: Option 1 is up to UE implementation (vivo) - Option 3: FFS (Apple CMCC Huawei Qualcomm Xiaomi MTK) Tentative agreements: No

In last meeting some companies raised up the issue of AP MUSIM gap, and we think it is valid issue as the handling of AP gap can be different from periodic gap.

In our view, AP gap by its nature should be always prioritized. It is noted that AP gap is still configured by NW A, i.e. NW A can choose to configure the requested AP gap or not. If NW A chooses to configure the AP gap, it does not make much sense to associate it with a lower priority than legacy MG, as it will cause the AP gap to be dropped, and it is same as not configuring the AP gap.

Based on above, we suggest AP MUSIM gap is always prioritized over legacy MG in NW A. This also means there is no need to configure a priority for AP MUSIM gap.

Proposal 8: AP MUSIM gap is always prioritized over legacy MG in NW A.

NW A requirements

Issue 2-2-3: Principle on layer 3 measurement requirements after gap collision handling - Proposals - Option 1: The principle of defining scaling factor Kp and Kgap for multi-concurrent gaps are applied to the calculation of Kp and Kgap for layer 3 measurement (Apple xiaomi oppo MTK vivo) - Option 1a: re-use the ‘counting’ approach defined for Rel-17 concurrent MGs to define scaling factor for the impacts of MUSIM gaps (Apple xiaomi vivo) - Option 2: Define requirements after solving gap collision issue (CMCC Huawei vivo MTK Qualcomm Nokia) - Option 3: Too early to discuss this issue (Ericsson) Tentative agreements: No Issue 2-2-4: Principle on L1 measurement requirements after gap collision handling - Proposals - Option 1: The principle of defining P value for L1 measurement and RLM/BFD measurement in Rel-17 cam be reused (Apple xiaomi oppo) - Option 1a: re-use the ‘counting’ approach defined for Rel-17 concurrent MGs to define scaling factor for the impacts of MUSIM gaps (Apple xiaomi oppo) - Option 2: Define requirements after solving gap collision issue (CMCC xiaomi vivo Huawei Qualcomm Nokia) - Option 3: Too early to discuss this issue (Ericsson) Tentative agreements: No

Issue 2-3-5-2: On the time window W for aperiodic gap - Proposals: - Option 1: Discuss whether and how to determine the time window W when aperiodic MUSIM gap with higher priority is involved in collision (oppo) - Option 2: W could be the largest periodicity among all the periodic gaps + Time margin [M] for the one-shot aperiodic gap (MTK) - Option 3: FFS (oppo Ericsson Apple Nokia Charter CMCC Huawei Qualcomm xiaomi MTK vivo) Tentative agreements: No

In last meeting, there were some discussions on measurement requirements for NW A. The common understanding is that the requirements would depend on the outcome for collision handling, e.g. what happens when MUSIM gaps collide with legacy MG and SMTC/L1 resource for NW A. It may be too early to discuss the measurement requirements for NW A at this stage.

Proposal 9: RAN4 to first work on gap collision handling before working on measurement requirements for NWA.

Issue 2-5-1: MUSIM overhead - Proposals: - Option 1: RAN4 to define MUSIM gap overhead for MUSIM gap(s) (xiaomi) - Option 2: wait for concurrent gap conclusion (Nokia) - Option 3: FFS (Apple oppo Nokia Charter Huawei Qualcomm Xiaomi MTK vivo) - Option 4: Reuse conclusion in concurrent gap if MUSIM gaps can be viewed as “one gap”(Ericsson) Tentative agreements: No

In last meeting, some companies proposed to define overhead cap for MUSIM gaps. We do not think it is necessary.

In Rel-17 con-MG, overhead cap is defined to limit the data interruption and measurement burden for the UE. One difference between legacy MG and MUSIM gaps is that legacy MGs are fully controlled by the NW, and as such it make sense to define some restrictions in the spec to make sure the MG configuration to the UE is reasonable. MUSIM gaps are requested by the UE. UE should be well aware of the consequent data interruption in NW A and measurement burden in NW B, and take them into account when making the request. There is no need to define additional restriction in the spec.

Proposal 10: RAN4 not to define overhead cap for MUSIM gaps.

One related issue is the total number of gaps when MUSIM gaps are configured.

For MUSIM alone, up to 3 periodic gaps and 1 AP gap can be configured. It is clearly too restrictive if no legacy MG can be configured for NW A’s RRM measurement, when MUSIM gaps are configured. As such, we assume at least the number of MGs in Rel-15 should be supported, i.e. UE can be configured with one per-UE MG or two per-FR MGs in different FRs, when MUSIM gaps are configured.

In Rel-17 the number of MGs is enhanced with con-MGs, so the question is whether UE can be configured with con-MGs for RRM (e.g. two per-UE MGs) and the MUSIM gaps. We assume this may be defined as an optional feature, but we are open to further discussions.

Proposal 11: RAN4 to discuss total number of gaps when MUSIM gaps are configured.

NW B requirements

Issue 2-4-1: Whether to define network B requirements - Proposals: - Option 1: Define the requirements for Network B in RRC idle/inactive (Ericsson) - Option 2: No measurement requirements in network B will be defined by RAN4 (Apple Nokia Huawei Qualcomm MTK vivo) - Option 3: If there is a consensus to specify network B requirement, its priority should be lower compared with the work for network A requirements and could be carried out at the second phase in the WI time frame (Apple vivo) Tentative agreements: No

A basic principle in defining MUSIM gap patterns in Rel-17 is that what MUSIM gaps to request and how to use them are up to UE implementation, so it may be difficult to define requirements for measurements in NW B. On other hand, during WID discussion, some companies mentioned that the reason for NW A to configure MUSIM gaps is to allow UE to do correct measurement in NW B, so without the requirements, there will be no motivation for NW A to configure the MUSIM gaps.

If requirements for measurements in NW B are to be defined, the existing measurement requirements for IDLE/INACTIVE should be used as baseline. We do not think UE is expected to meet a more stringent requirements just because the measurement is performed based on MUSIM gaps, e.g. UE should not be assumed to perform more than one measurements per DRX cycle.

As the periodicity of MUSIM gaps can be up to 5.12s, it is possible that the periodicity of MUSIM gaps is larger than the DRX cycle in NW B. In this case, the requirements should be based on the maximum between the DRX cycle and the periodicity of MUSIM gaps.

Proposal 12: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP).

Conclusions

In this paper we provided our views on RRM requirements for MUSIM gaps.

Proposal 1: “Legacy MG” in Case 1 includes pre-MG, NCSG and ‘normal MG’ (MG configured via GapConfig, or via GapConfig-r17 but without preConfigInd-r17 or ncsgInd-r17).

- Re-use the conclusions from Rel-18 feMG WI on concurrency between MUSIM gaps and pre-MG/NCSG.

Proposal 2: Priority of MUSIM gaps are up to NW A configuration. Different priorities can be configured for different MUSIM gaps.

Proposal 3: For collisions between MUSIM gaps and legacy MG, the proximity condition of Rel-17 concurrent gap is reused.

Proposal 4: For collisions between MUSIM gaps and legacy MG, RAN4 to discuss the order for applying the priority when number of colliding gaps is larger than 2.

Proposal 5: Condition “XXX is overlapping with MG” is used for defining MUSIM gap collision with SMTC and L1 measurement resources in NW A.

Proposal 6: UE is in general not expected to transmit or receive signals for NW A (including SMTC and L1 measurement resources) during MUSIM gaps, except for signals used for random access procedure.

- RAN4 not to define specific handling for collision between MUSIM gaps and specific RRM procedures / transmissions or receptions in NW A.

Proposal 7: MUSIM gaps are not dropped due to collision with another MUSIM gap.

- No definition for collision between MUSIM gaps is needed.

Proposal 8: AP MUSIM gap is always prioritized over legacy MG in NW A.

Proposal 9: RAN4 to first work on gap collision handling before working on measurement requirements for NWA.

Proposal 10: RAN4 not to define overhead cap for MUSIM gaps.

Proposal 11: RAN4 to discuss total number of gaps when MUSIM gaps are configured.

Proposal 12: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP).

Reference

- R4-2214349, WF on RRM requirements for Rel-17 MUSIM gaps, Moderator (vivo)

- R4-2210478, Email discussion summary for [103-e] [209] NR_MG_enh_1, Moderator (MediaTek inc.)

- R4-2214471, WF on NR NTN RRM requirements, Moderator (Qualcomm Incorporated)