3GPP TSG-RAN4 Meeting #104bis-e	R4-2216372

# Electronic Meeting, October 10th – October 19th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on above 10GHz NTN band

Agenda item: 	6.22.1.1

Document for:	Discussion and Approval

# Introduction

The NTN WI, as presented in [1], includes the following objectives related to NTN deployment in above 10GHz bands:

4.1.2 NR-NTN deployment in above 10 GHz bands The following assumptions are taken a baseline for this work: - GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered - ESIM scenarios for NGSO in Ka band are not considered in this WI. - Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study. - FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2 - The ITU-R harmonized Ka band will serve as reference - Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification. The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022. - Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4] - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4] - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4] - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4] - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4]. - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4] - Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4] - Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed.as follows [RAN4]: - time relationship related enhancement (e.g. K_offset) - subcarrier spacing for different UL/DL signals/channels - PRACH configuration index for FDD above 10 GHz.

In this contribution we present further considerations for the introduction of NTN deployment in above 10 GHz bands.

# Discussion

#### Main challenges with the introduction of NTN deployment in above 10 GHz bands

During previous RAN4 meetings progress have been made on FR1 bands for NTN deployment. However, for FR2 work were postponed at RAN#92 and RAN#93 to prioritize fundamental NTN support. At RAN#96 a new WI were approved including a specific objective for NTN deployment in above 10 GHz bands.

The hesitation with introduction of NTN deployment in above 10 GHz bands are due to multiple aspects:

- Currently there are no NR FR2 bands already defined corresponding to ITU defined bands for IMT NTN deployments.

- ITU defined bands/frequency ranges for IMT NTN deployments suggested for “FR2” NR NTN bands either fully or partially fall in the 7-24 GHz frequency range for which a RAN4 a study-item (SI) have been concluded. However, the SI concluded without any decision on how to split its relation to the FR1 and FR2 range. There have been no Work-Item (WI) approved by 3GPP RAN addressing this issue. Hence, there are no baseline specification for this frequency range currently in RAN4.

- Of the existing NR FR2 bands in e.g., TS 38.101-2, all are of the time division multiplexed (TDD) type. Satellite based NTN operation are mainly utilizing frequency division multiplex (FDD) systems. Hence, if a FR2 FDD band shall be defined all RAN4 requirements related to FDD FR2 operation are missing in the RAN4 specification.

Almost all of these aspects have been taken into account when formulating the objectives of the NTN enhancements WI.

#### Frequency range for NTN deployment in above 10 GHz bands

Currently the 3GPP specification defines the frequency ranges as below.

38.101-2 - Table 5.1-1: Definition of frequency ranges

| Frequency range designation |  | Corresponding frequency range |
| --- | --- | --- |
| FR1 |  | 410 MHz – 7125 MHz |
| FR2 | FR2-1 | 24250 MHz – 52600 MHz |
|  | FR2-2 | 52600 MHz – 71000 MHz |

As seen from above there are no definition of the frequency range 7125 MHz to 24250 MHz (i.e 7-24 GHz) meaning there are no fundamental requirements captured in any of the 3GPP standards for this part of the spectrum.

Observation 1:	There are no definition of the frequency range 7125 MHz to 24250 MHz (i.e 7-24 GHz).

As mentioned there have already been a RAN4 SI on the 7-24 GHz frequency range. This SI did not conclude on how to define the this range and also not if parts of it could be included to the existing frequency ranges (i.e. FR1 or FR2). The NTN WID also clearly mentions that “definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region”, meaning any decisions related to a new band above 10 GHz shall only be applicable for NTN deployments.

This RAN agreement shall be respected in RAN4.

Proposal 1:	RAN4 to confirm that any decisions related to a new band above 10 GHz shall only be applicable for NTN deployments.

Since this means there shall be no change to the Definition of frequency ranges in the specification there is a need to discuss how to capture potential support for a NTN band above 10 GHz.

#### Naming of a NTN frequency band above 10GHz

At RAN4#100 it was agreed that for NTN the band numbering shall start from the largest band number in the FR1 range which fully within FR1 frequency range, in a decreased order with first come, first service. At RAN4#101 is was further agreed to use “n” as prefix. This resulted in the introduction of n256 (1980 MHz – 2200 MHz and n255 (1525 MHz – 1660.5 MHz). Following this logic the next available NTN band number is n254.

Observation 2:	Following previous agreements the next available NTN band number is n254.

Since this band number is in between the number range from FR1 and FR2 we believe it is fitting to follow this naming convention also for a NTN band above 10 GHz.

Proposal 2:	RAN4 shall consider n254 as the band number for the next NTN band.

#### NTN frequency band above 10GHz candidates

As mentioned in the WID RAN4 are to consider the “Ka band” as a reference band for the development of a NTN band above 10 GHz. The “Ka band” referes to the frequency range 17.3 to 31.0 GHz. For 3GPP RAN4 work this range needs to be further downscoped as also mentioned in the WID.

When considering available regulations and the relation to excisting bands, especially in FR2, some proposals for consideration have been listed below together with excisting FR2 bands.

| Example NTN satellite band # | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| A | 27500 MHz – 31000 MHz | 17700 MHz – 21200 MHz | FDD |
| B | 29500 MHz – 30000 MHz | 19700 MHz – 21200 MHz | FDD |
| C | 28600 MHz – 29100 MHz | 18800 MHz – 19300 MHz | FDD |

Option A – is to our understanding often used for so-called High Throughput Satellites (HTS). The main deployments are GSO. The frequency range is large and subject to multiple regional and country based restrictions.

Option B – is to our understanding often used for so-called High-Density Fixed-Satellite Service (HDFSS). The main deployments are GSO. A benefit of this frequency range is that it is globally allocated on a primary basis in the ITU RR.

Option C – were to our understanding intended for commercial broadband satellite internet. A benefit of this frequency range is that, according to ITU RR, GSO and non-GSO satellite networks have equal rights.

If an objective is to avoid overlap with existing NR bands option B would be straight forward.

Observation 3:	Option B (UL 29.5-30 GHz DL 19.7-21.2) GHz) – does not overlap any existing NR bands.

For reference the existing FR2 bands have been included below.

38.101-2 - Table 5.2-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – | 43500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| n262 | 47200 MHz | – | 48200 MHz | 47200 MHz | – | 48200 MHz | TDD |
| n263 | 57000 MHz | – | 71000 MHz | 57000 MHz | – | 71000 MHz | TDD1 |
| NOTE 1: [This is for unlicensed band operation] |  |  |  |  |  |  |  |

Proposal 3:	RAN4 shall discuss and decide a specific frequency range for a NTN band above 10 GHz.

# Conclusion

This contribution discusses aspects related to NTN and has the following proposal and observations:

Observation 1:	There are no definition of the frequency range 7125 MHz to 24250 MHz (i.e 7-24 GHz).

Proposal 1:	RAN4 to confirm that any decisions related to a new band above 10 GHz shall only be applicable for NTN deployments.

Observation 2:	Following previous agreements the next available NTN band number is n254.

Proposal 2:	RAN4 shall consider n254 as the band number for the next NTN band.

Observation 3:	Option B (UL 29.5-30 GHz DL 19.7-21.2) GHz) – does not overlap any existing NR bands.

Proposal 3:	RAN4 shall discuss and decide a specific frequency range for a NTN band above 10 GHz.

# References

- RP-222654, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- RP-211596, Moderator Proposed Extended Round WF