3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216389

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item:	4.5.3.2

Source:	Ericsson

Title:	Discussion on the gamma value of PMI reporting tests for multi-TRP transmission

Document for:	Discussion

# 1	Introduction

After the last RAN4 #104 meeting’s discussion, all open issues for PMI reporting tests for multi-TRP transmission are settled. In this meeting, companies are encouraged to provide simulation results to get them aligned and therefore set requirements.

In this contribution, we are going to give our views on selecting the gamma value for both FDD and TDD requirements.

# 2	Discussion

Following the simulation assumptions and parameter configurations agreed previously, we have summarized our simulations results here:

Table 2-1: SNR point and TP ratio at 90% max Tput

|  | SNR point(dB) | TP ratio (following Type I PMI /random Type I PMI) |
| --- | --- | --- |
| FDD, 2Rx | 9.3 | 2.36 |
| FDD, 4Rx | 4.4 | 2.82 |
| TDD, 2Rx | 7.7 | 2.92 |
| TDD, 4Rx | 3.3 | 3.12 |

Based on the simulation results we propose the following gamma value as the requirement:

Proposal 1:

Table 2-2: Proposed gamma values

|  | TP ratio (following Type I PMI /random Type I PMI) | Proposed Gamma value |
| --- | --- | --- |
| FDD, 2Rx | 2.36 | 1.6 |
| FDD, 4Rx | 2.82 | 2.1 |
| TDD, 2Rx | 2.92 | 2.2 |
| TDD, 4Rx | 3.12 | 2.4 |

# 3	Summary

In summary, we listed our observations and proposals here:

Proposal 1:  Consider following gamma values:

|  | TP ratio (following Type I PMI /random Type I PMI) | Proposed Gamma value |
| --- | --- | --- |
| FDD, 2Rx | 2.36 | 1.6 |
| FDD, 4Rx | 2.82 | 2.1 |
| TDD, 2Rx | 2.92 | 2.2 |
| TDD, 4Rx | 3.12 | 2.4 |

# References

[1]	R4-2214397: WF on UE demodulation and CSI requirements for FeMIMO, Samsung

## Annex: Simulation configurations

Here we provide our simulation assumptions for information.

Common simulation assumption - Channel and correlation models: TDLA30-10 with XP High with statistically independent for each TRP - Pc setting: Same Pc ratios for each TRP in defining requirement - SNR setting: The SNRs for TRP #1 and TRP #2 are assumed to be balanced with a scaling factor of 1/sqrt(2) for the transmitted signal from each TRP General test set-up for CSI reporting - 2 TPs configured with fully overlapping resource allocation - One CSI-RS resource with Ks = 2 - TP1 associated with NZP-CSI-RS resource 1 - TP2 associated with NZP CSI-RS resource 2 - CSI reporting: One CSI associated with multi-TRP measurement hypothesis and X=0 CSI associated with single-TRP measurement hypothesis (CSI reporting mode 1 with X=0) - CMR group 1 {CMR a} corresponding to NZP CSI-RS resource 1, K1=1 - CMR group 2 {CMR b} corresponding to NZP CSI-RS resource 2, K2=1 - CMR pair (N=1): CMR {a,b} for M-TRP measurement hypothesis - No time/frequency offset between two TPs - WB PMI reporting for mode 1 with X=0 - RAN4 apply the above test setup as baseline for CSI reporting requirement definition CSI resource configuration - Configure two resources in a resource pair in the same slot for CSI reporting requirements for mTRP. Antenna configuration - 8x2, XP high (per TRP) Number of layers - 2 (1MIMO layer per TRP) Test metric for PMI reporting - TP ratio for follow PMI and random PMI with m-TRP reporting. Test point - 90% MCS - MCS 13