3GPP TSG-RAN WG4 Meeting #104bis-e                                                    R4-2216395

Electronic Meeting, 10 Oct. – 19 Oct. 2022

Agenda Item: 	4.2.7.3.1

Source:	Ericsson

Title:	Simulation results for PDSCH demodulation requirement for NTN

Document for:	Information

# 1	Introduction

In this contribution, we share our initial simulation results for PDSCH NTN. Simulation assumption can be found in the annex.

# 2	Discussion

## 2.1 Simulation results for PDSCH with NTN-TDLA

Table 2.1-1: Simulation results for the test cases for NLOS, NTN-TDLA

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | HARQ process | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | Disable HARQ | 70 | -2.1 |
| 1-2 |  |  |  |  |  | HARQ 32 |  | -2.7 |
| 1-3 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLA100-200 | 1x2, ULA Low | Disable HARQ |  | 6.1 |
| 1-4 |  |  |  |  |  | HARQ 32 |  | 5.2 |

## 2.2 Simulation results for PDSCH with NTN-TDLC

Table 2.2-1 Simulation results for the test case of LOS, NTN-TDLC

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | HARQ process | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLC3.5-200 k-factor: 8.05dB | 1x2, ULA Low | Disable HARQ | 70 | -3.4 |
| 2-2 |  |  |  |  |  | HARQ 32 |  | -3.9 |
| 2-3 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLC3.5-200 k-factor: 8.05dB | 1x2, ULA Low | Disable HARQ | 70 | 4.6 |
| 2-4 |  |  |  |  |  | HARQ 32 |  | 4.0 |

# 3	Summary

In this contribution, we provided our simulation results for NTN PDSCH based on the previously agreed parameter configurations.

# References

[1]	R4-2210661 WF on NTN Demod-general and PDSCH, Qualcomm Incorporated

# Annex

Simulation assumption

Table 1: Simulation parameters for PDSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Number of SAN antennas/UE antennas |  | 1x2 |
| Carrier frequency |  | 2 GHz |
| Duplex mode |  | FDD |
| Bandwidth (MHz) / Subcarrier spacing (kHz) |  | 10 / 15 |
| Residual frequency offset |  | No FO, 0.1ppm (200Hz) |
| Channel model |  | NTN-TDLA (NLOS) NTN-TDLC (LOS) |
| Channel model k-factor |  | Refer to the test cases |
| MCS |  | 4 (QPSK) 13 (16QAM) |
| Reference measurement channel |  | R.PDSCH.1-1.1 FDD (QPSK) R.PDSCH.1-2.1 FDD (16QAM) |
| HARQ | Maximum number of HARQ transmissions | 1(no retransmissions) and enable HARQ |
| DM-RS | DM-RS configuration type | Type 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Depends on RMC |
|  | Number of DM-RS CDM group(s) without data | 1 |
| Time domain resource assignment | PDSCH mapping type | A |
|  | Start symbol | 2 |
|  | Allocation length | 12 |
| K_offset | Slot | [8] |