3GPP TSG-RAN WG4 Meeting #104bis	R4-2216400

Electronic meeting, October 10th – 19th 2022

Agenda Item:	6.13.4

Source:	Ericsson

Title:	ATG BS requirements

Document for:	Approval

# 1	Introduction

During RAN4#104-e, a WF was agreed on ATG BS requirements. Many BS requirements are already resolved. This document provides some views on remaining issues.

# 2	Discussion

## 2.1	BS class and type for ATG BS

It was agreed to define a BS class for ATG. The proposed description is as follows:

ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]

This description of the ATG deployment scenario is reasonable for a class definition.

Confirm the following description for the ATG BS class: “ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]”

Assuming that all but a few requirements for the ATG BS class are the same as the wide area (WA) BS class, it may be sufficient to include a note together with the ATG class description that unless otherwise stated, ATG BS requirements are the same as WA requirements, and add additions for ATG BS only where needed.

Consider adding a note after the class definition that ATG BS requirements are, unless otherwise stated, the same as WA BS requirements.

BS types 1-H and 1-O (i.e., AAS) were agreed, but 1-C (i.e., non-AAS) was left FFS. According to our link budget analysis [1], it is necessary to use a beamforming array for the ATG BS. Thus, in our view 1-H and 1-O are sufficient.

1-H and 1-O (AAS) are sufficient for the ATG BS requirements

## 2.2	Transmitter requirements

The only transmitter requirements that were not fully agreed were the EVM and the ACLR and OBUE.

The ACLR and OBUE depend on the outcome of the co-existence study.

Regarding the EVM, the open issue is whether to include support for 256QAM. Link level studies [1] suggest that high SNR can be achieved on the ATG link. Furthermore, it is necessary to provide high capacity, since the ATG link is effectively a backhaul link covering all of the passengers in the aircraft. Considering the good available SNR and the need to provide high capacity, 256QAM support is necessary.

Support 256QAM in the ATG BS specification.

## 2.3	Receiver requirements

The RX ACS and blocking requirements depend on the outcome of the co-existence study.

During RAN4#104-e, it was agreed to not define RX IM requirements as there will not be interfering UEs close to the ATG BS. Whilst this may be true, in fact RX IM sets a minimum requirement on LNA linearity. Without the requirement, there is no requirement directly relating to receiver IIP3 and an ATG BS receiver may not perform as well as a terrestrial BS. We encourage RAN4 to double check whether it is good to remove RX IM.

RAN4 to double check whether it is good to remove the RX IM requirement, since then there is no requirement relating to receiver IIP3 performance.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm the following description for the ATG BS class: “ATG Base Stations are characterized by requirements derived from ATG scenarios with a ground BS to air UE with typical vertical altitude range [TBD km]”

Proposal 2	Consider adding a note after the class definition that ATG BS requirements are, unless otherwise stated, the same as WA BS requirements.

Proposal 3	1-H and 1-O (AAS) are sufficient for the ATG BS requirements

Proposal 4	Support 256QAM in the BS specification.

Proposal 5	RAN4 to double check whether it is good to remove the RX IM requirement, since then there is no requirement relating to receiver IIP3 performance.

# References

R4-2262617, Air to Ground link budget considerations, Ericsson, RAN4#104-e, August 2022