3GPP TSG-RAN WG4 Meeting # 104bis-e		R4-2216415

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.5.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on test methodology for FR2 UE with multi-Rx

Document for:	Approval

# 1 Introduction

In RAN4#104bis-e, the test methodology for UE RF, RRM and Demodulation were discussed and the WF on NR FR2 OTA testing enhancements were approved in [2]. In this paper, we will provide our views on test methodology for FR2 UE with multi-Rx.

# 2 Discussion

## 2.1 Full degree of rotation freedom for 2AoAs

When the workplan was discussed, it was agreed that RAN4 to further discuss the feasibility of supporting the full degree of rotation freedom with 2AoA and check the status by RAN4 #106 meeting. Below we will analyze the benefits of supporting full degree of rotation freedom for 2AoAs in multi-Rx testing.

From UE RF testing perspective, even though the side condition of selecting DL AoAs pairs are still under discussion in core requirements, independently controlling the 2AoAs with full degree will help to verify the UE RF requirements without restriction. To support full degree of rotation freedom for 2AoAs, several options were discussed in last meeting. The following two options which are enhanced based on IFF can be considered as the starting point to further investigate the feasibility.

Figure 1: Example of support full degree of rotation of 2AoAs: IFF+ rotating UE and anchor probe as a whole [2]

Figure 2: Example of support full degree of rotation of 2AoAs: IFF+DFF, DFF antennae as the second AoA NR [3]

From RRM testing perspective, as of now, it is not clear on how to define the RRM requirements for multi-Rx UE. But the legacy RRM test setup, i.e., 30°, 60°, 90°, 120°, 150° and 180° angular relationships, might not be proper for since more AoAs pairs need to be considered to test the UE beam management.

From demodulation testing perspective, it was agreed that the virtual cable approach should be the baseline and only pure baseband performance shall be tested [1]. In theory, the OTA channel can be equalized whichever the test directions are selected by running the procedure of achieving wireless cable mode isolation. But it should be noted that, as we used in legacy demodulation testing, the selected test directions should satisfy the REFSENSE requirements otherwise testable SNR would be very low. With this, supporting the full degree of rotation is also important when finding the test directions for demodulation testing.

Observation 1: The test setup supporting full degree of rotation freedom with 2AoA is beneficial for UE RF, RRM and Demodulation testing for FR2 multi-Rx UE.

Proposal 1: It is preferred to support full degree of rotation freedom for 2AoAs for Multi-Rx testing which is beneficial for RF, RRM and Demodulation testing.

Meanwhile, in RAN4#104-e, TE vendors shared the concerns on the high complexity, high cost with supporting a brand-new test system. It is expected the test system is to use legacy IFF/DFF as the baseline and enhance to support the full degree of rotation freedom for 2AoAs considering the complexity, cost, testing time, etc.

Proposal 2: RAN4 to analyze the feasibility of supporting full degree of rotation freedom for 2AoAs considering the complexity, cost, testing time, etc., aspects.

## 2.2 Test methodology for UE RF requirements

Per UE RF objectives, RAN4 will define the test methodology enabling testing with at least 2 simultaneous AoAs. For each AoA, dual-polarized measurement antenna is needed. As discussed in section 2.1, it is preferred to have a test system to support the full degree of rotation freedom for 2AoAs for UE RF testing. RAN4 should investigate the feasibility based on the input from TE vendors. If it is concluded that it is not feasible to support full degree of rotation for 2AoAs, the simplified approach supporting full degree of rotation freedom for AoA1, and fixed AoA2 with more flexible direction can be considered as the alternative method.

Proposal 3: The simplified approach supporting full degree of rotation freedom for AoA1, and fixed AoA2 with more flexibility can be considered as alternative method if supporting full degree of rotation freedom for 2AoAs is not feasible.

## 2.3 Test methodology for UE RRM requirements

For RRM, the target should be to allow testing of 4 AoAs with 2 simultaneously active AoAs. Therefore, the baseline is TDM transmission between 2 AoAs and 2 AoAs pairs. In the legacy measurement setup for RRM testing, the test setup shall enable 30°, 60°, 90°, 120°, 150° and 180° angular relationships between simultaneously active 2 AoAs. While for multi-Rx UE, as shown in Figure 3, there will be 4 AoAs and the angular offset should be defined for two beam pairs rather than two AoAs. Therefore, the definition of angular offset for multi-Rx UE RRM testing would be different from legacy RRM test setup. And how to select the beam pair would depend on the RRM core requirements discussion.

Proposal 4: RRM test setup should support at least 2 simultaneously active AoAs.

Observation 2: The definition of angular offset for multi-panel UE RRM testing would be based on the beam pairs which is different from legacy RRM test setup. The legacy measurement setup with 30°, 60°, 90°, 120°, 150° and 180° angular would lead to improper beam pair selection.

Proposal 5: RAN4 to consider more flexibility on the angular offset for multiple panels for UE RRM requirements testing.

Figure 3: Example for UE RRM requirements testing

## 2.4 Test methodology for UE demodulation requirements

For UE demodulation requirements testing, the virtual cable approach should be the baseline for multiple panels UE demodulation testing was agreed in [1]. In legacy demodulation testing, there are following two options to select test direction.

- EIS based: Select the known Rx beam peak direction reused from RF testing if available, as far as it satisfies the minimum isolation requirement defined in TS 38.521-4 and rank number in TS 38.521-4 corresponding to the test cases.

- RSRPB based: Otherwise select one direction which satisfies the REFSENS defined in TS 38.101-2, minimum isolation requirement defined in TS 38.521-4 and rank number in TS 38.521-4 corresponding to the test cases.

In 4-layer demodulation testing, it is straightforward to consider both EIS based and RSRPB based options. The examples of finding the test directions are given by:

  - EIS based

    - AoA1 from beam peak direction (TCI 1)

    - 3D scan for AoA2 when fixing AoA1, to select AoA2 which has the best EIS (TCI 2)

    - [Run the procedure to achieve wireless cable mode isolation]

    - [Run the isolation check for dual polarizations and cross 2AoAs]

  - RSRP based

    - AoA1 with top [5] RSRP for TCI 1

    - 3D scan for AoA2 when fixing AoA1, to select AoA2 from the directions which have top [5] RSRP for TCI 2

    - Check whether AoA1&AoA2 could pass the REFSENS

    - [Run the procedure to achieve wireless cable mode isolation]

    - [Run the isolation check for dual polarizations and cross 2AoAs]

With above, it is observed that the test directions for UE demodulation should satisfy REFSENSE requirements and the min. isolation between dual polarizations and between 2AoAs needs to be studied.

Proposal 6: The selected two test directions for UE demodulation should satisfy REFSENSE requirements.

Proposal 7: The min. isolation between dual polarizations and between 2AoAs needs to be studied.

# 3 Conclusions

This contribution provided our considerations on test methodology for FR2 UE with multi-Rx and the following observations and proposals are made:

Observation 1: The test setup supporting full degree of rotation freedom with 2AoA is beneficial for UE RF, RRM and Demodulation testing for FR2 multi-Rx UE.

Proposal 1: It is preferred to support full degree of rotation freedom for 2AoAs for Multi-Rx testing which is beneficial for RF, RRM and Demodulation testing.

Proposal 2: RAN4 to analyze the feasibility of supporting full degree of rotation freedom for 2AoAs considering the complexity, cost, testing time, etc., aspects.

Proposal 3: The simplified approach supporting full degree of rotation freedom for AoA1, and fixed AoA2 with more flexibility can be considered as alternative method if supporting full degree of rotation freedom for 2AoAs is not feasible.

Proposal 4: RRM test setup should support at least 2 simultaneously active AoAs.

Observation 2: The definition of angular offset for multi-panel UE RRM testing would be based on the beam pairs which is different from legacy RRM test setup. The legacy measurement setup with 30°, 60°, 90°, 120°, 150° and 180° angular would lead to improper beam pair selection.

Proposal 5: RAN4 to consider more flexibility on the angular offset for multiple panels for UE RRM requirements testing.

Proposal 6: The selected two test directions for UE demodulation should satisfy REFSENSE requirements.

Proposal 7: The min. isolation between dual polarizations and between 2AoAs needs to be studied.

# 4	References

R4-2214357, “WF on NR FR2 OTA testing enhancements”, Qualcomm Incorporated

R4-2211991, “Considerations on FR2 multiple AoA test”, Samsung

R4-2211549, “Views on FR2-1 RF OTA test for a device with multi-panel reception”, Anritsu Corporation