3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216418

# Electronic Meeting, October. 10-19, 2022

Agenda Item:	7.5.3

Source: 		Qualcomm Incorporated

Title: 	Coexistence simulation results for TN-NTN NB IoT

Document for:	Discussion

# Introduction

In this paper, we provide our simulation results for TN with NTN NB IoT in two cases. For instance, we provide simulation results for case 1 and case 4 as captured in [1].

# 2. Discussion

## 2.1 Simulation scenarios

In this document we present our simulation results between TN and NTN NB IoT for case 1 and case 4 which are the worst case for IoT NTN UE ACS and ACLR requirements respectively. The simulated cases are summarized in Table 1.

Table 1. List of study cases

| No. | Aggressor | Victim |
| --- | --- | --- |
| S1 | TN DL | NB IoT NTN DL |
| S4 | NB IoT NTN UL | TN UL |

## 2.2. Simulation parameters

The simulation parameters and assumptions as captured in [1] are used in the coexistence analysis between TN and NTN. For scenario 1, we have used 1UE with 20 MHz bandwidth for TN DL, and 1 UE with 1 RB (180 kHz) for NB IoT NTN DL. For scenario 4, we have used 1 RB bandwidth for NB IoT NTN UL with 12 UEs (15 kHz for each UE) and 36 UEs (3.75 kHz for each UE), for TN UL we used 20 MHz bandwidth with 3 UEs.  Furthermore, the ACIR model is adopted as summarized in Table 2 for the NB IoT simulations. Rest of assumptions and parameters didn’t change from [1].

Table 2. ACIR model for each case

| No. | Aggressor | Victim |  |
| --- | --- | --- | --- |
| S1 | TN DL (1 UE, 20 MHz) | NB IoT NTN DL (1 UE, 180 kHz) | Flat model with adjusting the ACLR according to TR 36.942 and adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS. FACLR = ~20.5 dB |
| S4 | NB IoT NTN UL (12 UE, 15 kHz each) Or (36 UE, 3.75 kHz each) | TN UL (3 UE, 20/3 MHz each) | Symmetric bandwidth ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat ACLR model and FACLR = 0 dB, assuming flat ACS |

## 2.2. Network layout model

The same network layout model is used as captured in [1].

## 2.3 Simulation results

The coexistence simulation results for the case 1 and case 4 which are mentioned in Section 2.1 are discussed in this section. In the following, the simulation results for the worst-case scenario which is GEO in the Urban environment are captured in Table 3 for the case 1 and case 4.  For case 1, we assume isolation distance of 1.5 km as captured in [1].  Furthermore, in case that the NTN NB IoT is the victim, the SINR loss is also computed as in [2] in dB as a function of the ACIR for different CDF points e.g., 5%, 50% and 95%-ile as seen in Table 4.

Table 3: Summary of ACIR as a function of Throughput Loss

| Environment |  | Urban |  |
| --- | --- | --- | --- |
|  |  | Average Throughput loss ACIR [dB] | Cell edge Throughput loss ACIR [dB] |
| Satellite |  | GEO @ EL 90 | GEO @ EL 90 |
| 4 | NTN UL to TN UL # UEs = 12 | 11 | 20 |
| 4 | NTN UL to TN UL # UEs = 36 | 19 | 27 |

Table 4: Summary of ACIR as a function of SINR Loss

| No. |  | ACIR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Environment / Satellite |  | Urban |  |  |  |
|  |  | GEO @ EL 9 |  |  |
| SINR Loss @x%-ile <1dB |  | 5 | 50 | 95 |
| 1 | TN DL to NTN DL Isolation distance = 1.5 km | 0 | 8.5 | 20 |

From the simulation results in Table 3 and Table 4, we have the following comments as follows:

- Case 1 (TN DL to NTN DL)

As we agreed that the criteria of SINR loss<1dB should be used when NTN IoT is the victim [2], the ACIR requirement is around 20 dB shown in Table 4.

- Case 4 (NTN UL to TN UL)

The simulation results for case 4 is showing around 20 dB when considering 12 UEs. On the other hand, there is around 7 dB increase in the ACIR requirement when considering 36 UEs as a worst-case scenario for NTN UL to TN UL. Note that we did not consider the isolation of 1.5km in case 4. If the isolation of 1.5km is applied in case 4, the more relaxed ACIR requirements can be achieved.

Observation 1: In case 1, the required ACIR requirement is around 20dB which indicates the legacy UE ACS requirements can be reused for NTN NB IoT UE with enough margin [3].

Observation 2: In case 4, the required ACIR requirement is around 27 dB when considering the 36 UEs as worst case. That is below the current requirements of NB-IoT requirements specified in [3]

Based on the above, we propose the following:

Proposal 1: The legacy requirements in TS36.101 can apply for NTN NB IoT UE ACLR and NTN NB IoT UE ACS even when considering the worst-case scenario with 36 UEs for case 4.

# 3.	Conclusion

In this paper, we provided simulation results between TN and NB IoT NTN for case 1 and case 4 coexistences scenarios. We have the following observations and proposal:

Observation 1: In case 1, the required ACIR requirement is around 20dB which indicates the legacy UE ACS requirements can be reused for NTN NB IoT UE with enough margin [3].

Observation 2: In case 4, the required ACIR requirement is around 27 dB when considering the 36 UEs as worst case. That is below the current requirements of NB-IoT requirements specified in [3]

Proposal 1: The legacy requirements in TS36.101 can apply for NTN NB IoT UE ACLR and NTN NB IoT UE ACS even when considering the worst-case scenario with 36 UEs for case 4.

# 4.	References

- 3GPP TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects

- 3GPP TR 36.802, Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception.

- 3GPP TS 36.101, User Equipment (UE) radio transmission and reception