3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216433

Electronic Meeting, 10 – 19 Oct, 2022

Title:	R18 Discussion on Tx switching

Source:	OPPO

Agenda item:	6.19.2

Document for:	Approval

- Introduction

In Rel-18, Tx switching is extended to 3 or 4 candidate bands but keep 2 simultaneous transmission chain numbers. RAN1 send LS [1] to RAN4 ask questions about the switching period, complexity and also Tx interruption between chains. Then in RAN4#104 meeting, the reply LS [2] was sent with answers for some of the questions. This paper will discuss the left issues.

- Discussion

  - Tx switching for single TAG

For single TAG, the only issue left is about whether to use same or different switching period values for each band pair among different releases, as seen below.

Issue 2-1-3: Exact value of Tx switching period - Further discuss the two options in the next meeting: - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching. - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

Comparing with Rel-16/17 Tx switching the Rel-18 Tx switching will choose the switching carriers from several candidate band groups. For the same band pair, if everything like the applied PLL and PAs are same then the switching time could be same. However, once they are not same then the Tx switching time could be different. One example as shown in figure 1 where LO1 and LO2 are for Band A and Band B respectively in Rel-16/17, when the additional candidate Band C comes in Rel-18, there is possibility that Band C will be allocated for a separate Tx chain and Band A/B are to be used in the other Tx chain for the reason of large frequency distance with Band C. In this case the switching time will be longer in Rel-18 comparing with Rel-16/17 for the switching between Band A and B.

Figure 1 Tx switching of Band A and B comparison between R16/17 and 18

Observation 1:    Different Tx chain could be applied for the same band pair in Rel-18 comparing to Rel-16/17.

Proposal 1:         Propose to allow UE report different switching period for the same band pair in Rel-18 comparing to Rel-16/17 within the set of switching periods already defined, i.e. {35us, 140us, 210us}.

  - Tx switching for Multi TAG

Multi TAG is another topic that has been discussed a lot in last meeting with open issues in below table. And it was agreed that the UL switching time should not include timing difference up to MTTD between two TAGs. How to handle the TA differences is unclear.

Issue 3-1-1 UL switching time RAN4 further discuss if the UL switching time is the same for single TAG and 2 TAGs FFS: UE may omit the uplink transmissions corresponding to any TAG during the UE switching time. Issue 3-1-2: UL outage time WF: Factors for UL outage time discussed in RF session, to be further checked in RRM session - UL switching time (UE capability) - The difference between the TA on the two TAGs, up to MTTD - Timing and measurement error

In the Rel-17 V2X concurrent operation requirement definition, similar issue has been discussed a lot. And it was recognized that TA difference between two TAGs is NW configuration dependent especially whether the two gNBs can exchange the TA information or not, while the switching period is UE hardware capability which is restricted with the time mask requirements. So in RAN4 spec, only switching period was defined as minimum requirement, and the handling of TA differences are left to NW scheduling. In principle, same approach can be applied to multi TAG in Tx switching scenario.

Observation 2:   TA difference is NW configuration dependent and is not UE hardware requirement. In Rel-17 V2X the time mask for switching between uu and SL only contains the switching period and no TA difference included.

Proposal 2:         Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included.

To cope with the additional time that added by TA difference between two TAG, the “outage time” was brought up in last meeting, and it includes UL switching time (UE capability), the difference between the TA on the two TAGs (up to MTTD), Timing and measurement error. From the factors it can be seen that in this “outage time”, only UL switching time is RF parameter, but the others like MTTD, Timing and measurement errors are all RRM parameters. In our view, this should not be discussed in RF session due to no such expertise here.

Observation 3:   MTTD, Timing and measurement errors are all RRM parameters which are to be considered in the outage time, should be discussed in RRM session.

Proposal 3:         The “Outage time” discussion should be moved to RRM session considering the parameters other than switching time are all RRM parameters like MTTD, Timing and measurement errors.

Besides, in last meeting WF it also includes the FFS on “UE may omit the uplink transmissions corresponding to any TAG during the UE switching time”. In principle this is correct that the UL transmission during UE switching time should be omitted, however, it is no clear whether this is relied on UE monitoring the TA differences between two TAGs and decide when the UL transmission to be omitted or it is relied on NW to avoid scheduling UE in the switching period. If NWs ignore the TA differences in the non-collocated UL Tx switching and scheduling UE at anywhere they want, then this multi-TAG will cause UE much trouble in Tx transmission. When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling.

Observation 4:   It is unclear when multi-TAG UL Tx switching is configured to UE whether TA difference will be considered in NW scheduling.

Proposal 4:         When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling.

- Conclusion

In this paper, we discussed the Tx switching time for single TAG comparing to multi-TAG, and the outage time. And got the following observations and proposals.

Observation 1:    Different Tx chain could be applied for the same band pair in Rel-18 comparing to Rel-16/17.

Proposal 1:         Propose to allow UE report different switching period for the same band pair in Rel-18 comparing to Rel-16/17 within the set of switching periods already defined, i.e. {35us, 140us, 210us}.

Observation 2:   TA difference is NW configuration dependent and is not UE hardware requirement. In Rel-17 V2X the time mask for switching between uu and SL only contains the switching period and no TA difference included.

Proposal 2:         Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included.

Observation 3:   MTTD, Timing and measurement errors are all RRM parameters which are to be considered in the outage time, should be discussed in RRM session.

Proposal 3:         The “Outage time” discussion should be moved to RRM session considering the parameters other than switching time are all RRM parameters like MTTD, Timing and measurement errors.

Observation 4:   It is unclear when multi-TAG UL Tx switching is configured to UE whether TA difference will be considered in NW scheduling.

Proposal 4:         When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling.

- References

[1] R1-2205502, LS on UL Tx switching across 3 or 4 bands in Rel-18, R1->R4

[2] R4-2214464, Reply LS on UL Tx switching across 3 or 4 bands, R4->R1