3GPP TSG-RAN WG4 #104bis-e	R4-2216506

Electronic Meeting, October 10 – October 19, 2022

Source: 	Ericsson

Title:  	RRM requirements for enhanced HST FR2

Agenda Item:	6.12.5

Document for:	Discussion

# Introduction

WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2) [1] is introduced in Rel-18 release. In this contribution, we present our viewpoints and proposals on the RRM requirements for this feature.

# Disucssion

## General

In WID [1], the following objective is listed for study:

- Specify the requirement for simultaneous multi-panel operation for train roof-mounted FR2 high power devices [RAN4]: - Maximum 2 active panels supporting the multi-panel simultaneous reception. - NOTE: Focus on FR2 HST specific requirements, and avoid the overlap with the scope of FR2 multi-Rx DL reception

The interpretation of multi-panel simultaneous reception is ambiguous, doubling RF and baseband (e.g. two separated DPS in opposite directions) or doubling RF but single baseband (e.g. SFN/JT) both are possible with pros and cons.  We suggest waiting for scenario analysis and demodulation analysis if applicable.

Proposal 1:  Postpone simultaneous multi-panel operation in RRM session until clear use case or scenario is fixed.

## Measurement requirements on CA enhancement for HST FR2

In WID [1], CA scenario is one of important aspects shall be studied:

- Specify the RF requirements for intra-band carrier aggregation (CA) scenario, and investigate and specify the RRM requirements for intra-band carrier aggregation (CA) scenario [RAN4]

| RRC mode | Type of requirements | Impact |
| --- | --- | --- |
| IDLE mode mobility | Neighbor cell measurements | Inter-frequency measurements enhancement shall be added. |
| CONNECTED mode mobility | Intra-frequency measurements | Deactivated SCell enhancement shall be added |
|  | inter-frequency measurements | Inter-frequency measurements enhancement shall be added. |

Proposal 2: To support CA enhancement for HST FR2, below requirements shall be added

| RRC mode | Type of requirements | Impact |
| --- | --- | --- |
| IDLE mode mobility | Neighbor cell measurements | Inter-frequency measurements enhancement shall be added. |
| CONNECTED mode mobility | Intra-frequency measurements | Deactivated SCell enhancement shall be added |
|  | inter-frequency measurements | Inter-frequency measurements enhancement shall be added. |

Another issue about the HST signaling raises upon CA enhancement. One approach is CA enhancement reuse existing HST FR2 signaling, on the contrary, another approach is CA enhancement needs new dedicated HST FR2 signaling.

Proposal 3: RAN4 shall study HST signaling for CA enhancement.

- Option 1: Reuse PCell signaling

- Option 2: New signaling

## Reference tunnel deployment scenario

Tunnel deployment scenario is introduced in WID in below:

- Study on reference tunnel deployment scenario for FR2 HST and specify the channel model and corresponding core requirements if any [RAN4]

Even scenario shall not be studied by RRM session firstly, it’s still beneficial to do some preliminary theoretical analysis here, below is our understanding from RRM perspective. For the tunnel scenario, there isn’t significant difference to the scenario A. For scenario A, the RRH is located close to the track and has LoS to the UE with a narrow beam. Inside the tunnel, the RRH will also be located close to the track and have LoS to UE. Channel characteristics may include possibly a very small delay/Doppler spread if there are some reflections may occur but with a narrow beam pointing along the track and considering reflections from tunnel walls may have very little impact.

Proposal 4: postpone discussion on tunnel deployment scenario until exact impact to RRM can be identified clearly.

## UL timing adjustment solution

- Specify UL timing adjustment solution, including explicit NW signalling assistance, for FR2 HST scenario with large UL/DL propagation delay difference from different RRHs/TRPs to UE [RAN4, RAN2]

UL timing adjustment solution has been discussed comprehensively in Rel-17 WI for HST FR2. It can be seen that one of the factors that leads to complexity and difficulty is that there is no discernible difference between intra-RRH and inter-RRH.

It can be beneficial to overall performance improvement and remove ambiguous and pointless supplements in the current UL timing adjustment technique implemented in Rel-17 if UE can distinguish TCI state switches between intra-RRH and inter-RRH.

Proposal 5: Support network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH.

## TCI state switch on SCell

In Rel-17, below statements in [2] capture UL timing adjustment impacts on TCI state switch.

### 8.10.3A MAC-CE based TCI state switch delay in HST FR2 scenarios For FR2 power class 6 UE, if the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the symbol m of the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ + TOk*(Tfirst-SSB + TSSB-proc + Trs + Trs-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3Nslotsubframe,µ .Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]; - Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; - TSSB-proc = 2 ms; - Trs is time to the first TRS or SSB transmission after the SSB transmission in the definition of Tfirst-SSB is processed by the UE; - Trs-proc = 2 ms; - TOk = 1, m = 0 if target TCI state is not in the active TCI state list for PDSCH; otherwise TOk = 0, m = 1. For FR2 power class 6 UE, if the target TCI state is unknown, the same requirement for unknown target TCI state case specified in clause 8.10.3 applies.

Since SCell may reuse timing of PCell to some extent, it is doable to check if TCI state switch delay can be enhanced.

Proposal 6: RAN4 shall check the necessity of enhancements on TCI state switch in SCell.

## SCell activation

At the least, Scell activation may follow the existing requirements in [2] in below provided conditions are fulfilled,  since Pcell in HST is alwasy in FR2, i.e. Scell and Pcell are on same band.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided: - The UE is provided with SMTC for the target SCell, and - The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in clause 3.6.3, - The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell. - SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell

Based on that, it is doable to investigate if further enhancements can be applied in HST FR2 scenario.

Proposal 7: RAN4 shall check the necessity of enhancements on SCell activation.

## Link recovery on SCell

In current specification in [2], RLM should apply to PCell or PSCell, and Link recovery shall apply to Pcell, PScell, and SCell.

Given the situation that deployments of PCell and SCell in HST scenario may be identical including site location, direction and so on, it is valuable to investigate enhancements on LR and RLM.

Proposal 8: RAN4 shall study possible optimization on link recovery on SCell, link recovery on SCell.

# Conclustion

Proposal 1:  Postpone simultaneous multi-panel operation in RRM session until clear use case or scenario is fixed.

Proposal 2: To support CA enhancement for HST FR2, below requirements shall be added

| RRC mode | Type of requirements | Impact |
| --- | --- | --- |
| IDLE mode mobility | Neighbor cell measurements | Inter-frequency measurements enhancement shall be added. |
| CONNECTED mode mobility | Intra-frequency measurements | Deactivated SCell enhancement shall be added |
|  | inter-frequency measurements | Inter-frequency measurements enhancement shall be added. |

Another issue about the HST signaling raises upon CA enhancement. One approach is CA enhancement reuse existing HST FR2 signaling, on the contrary, another approach is CA enhancement needs new dedicated HST FR2 signaling.

Proposal 3: RAN4 shall study HST signaling for CA enhancement.

- Option 1: Reuse PCell signaling

- Option 2: New signaling

Proposal 4: postpone discussion on tunnel deployment scenario until exact impact to RRM can be identified clearly.

Proposal 5: Support network assisted information, i.e., enable network assisted signaling of SSB index and order per RRH.

Proposal 6: RAN4 shall check the necessity of enhancements on TCI state switch in SCell.

Proposal 7: RAN4 shall check the necessity of enhancements on SCell activation.

Proposal 8: RAN4 shall study possible optimization on link recovery on SCell, link recovery on SCell.

# References

- RP-222272 “New WID on enhanced NR support for high speed train scenario in frequency range 2 (FR2)”, Samsung

- TS 38.133, Ver.17.6.0