3GPP TSG-RAN WG4 Meeting#104bis-e                               R4-2216557

E-meeting,11th Oct – 19th Oct,2022

Agenda item:	6.22.2

Source: 	ZTE Corporation

Title: 	Discussion on coexistence evaluation for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In this contribution, we want to share some initial views on the coexistence study for NTN in Ka-band.

The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022.

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4]

  - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4]

  - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4]

  - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4]

  - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4].

  - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4]

Discussion

## 2.1. Coexistence scenario

As indicated in the WID, RAN4 need to consider the coexistence study between Ka band and the TN bands. The detailed spectrum status between Ka band and surrounding TN bands are shown as following.

Figure 2.1-1. coexistence scenario for Ka band and other TN bands.

Based on the above adjacent spectrum for NTN Ka-band, then the coexistence scenario could be summarized as following.The aggressor and victim combinations are listed in Table 2.1-1.

Table 2.1-1: Coexistence scenario between Ka band NTN and TN

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | NTN UL | TN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 3 | TN with NTN | NTN UL | TN DL | Applicable for n257, n258 and n261 | Phase 1 |
| 4 | TN with NTN | TN DL | NTN UL | Applicable for n257, n258 and n261 | Phase 1 |
| 5 | TN with NTN | TN DL | NTN DL | FFS | Phase 2 |
| 6 | TN with NTN | NTN DL | TN DL | FFS | Phase 2 |
| NOTE 1: For coexistence between Ka band DL and surrounding TN bands, this need more discussions since currently there are no 3GPP defined TN bands specified. |  |  |  |  |  |

The frequency and bandwidth are listed in Table 2.1-2.

Table 2.1-2: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | FRF |
| --- | --- | --- | --- | --- |
| TN Rural1 | 30 GHz | 400MHz/200MHz | TDD | 1 |
| TN Urban macro | 30 GHz | 400MHz/200MHz | TDD | 1 |
| GEO | 30 GHz | 400MHz/200MHz3 | FDD | 1, 32 |
| [MEO] | 30GHz | 400MHz/200MHz3 | FDD | 1, 32 |
| LEO | 30 GHz | 400MHz/200MHz3 | FDD | 1, 32 |
| NOTE 1: For mmWave frequency, the rural area deployment does not consider in general. Whether we need to consider the rural area deployment can be further discussed. NOTE 2: Only FRF=1 has been used in co-existence studies for simplification. NOTE 3: Max BW per beam is 400MHz for Ka-band according to Table 6.1.1.1-5 and Table 6.1.3.2-1 of 38.821. |  |  |  |  |

Proposal 1: to consider the coexistence scenario in Table 2.1-1 and proposed frequency and bandwidth configuration in Table 2.1-2 for Ka band NTN coexistence study in Rel-18.

## 2.2. System parameters

### 2.2.1	NTN BS parameters

The general parameters for NTN network coexistence study are listed in the following Table 2.2.1-1 and Table 2.2.1-2.

Table 2.2.1-1: Set-1 satellite parameters for system level simulation

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | 40 dBW/MHz | 10 dBW/MHz | 4 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 40 km | 20 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| G/T |  | 28 dB K-1 | 13 dB K-1 | 13 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 2.2.1-2: Set-2 satellite parameters for system level simulation

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 2 m | 0.2 m | 0.2 m |
| Satellite EIRP density |  | 32 dBW/MHz | 2 dBW/MHz | -4 dBW/MHz |
| Satellite Tx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| 3dB beamwidth |  | 0.4412 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter (Note 2) |  | 280 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 1.33 m | 0.13 m | 0.13 m |
| G/T |  | 20 dB K-1 | 5 dB K-1 | 5 dB K-1 |
| Satellite Rx max Gain |  | 50.5 dBi | 30.5 dBi | 30.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. |  |  |  |  |

### 2.2.2	NTN UE parameters

NTN UE parameters in Ka band are given in Table 2.2.2-1

Table 2.2.2-1: NTN UE characteristics for system level simulation

| Characteristics | VSAT (Note 2) |  |  |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) |  |  |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter of TR 38.821 |  |  |
| Polarisation | circular |  |  |
| Rx Antenna gain | 39.7 dBi |  |  |
| Antenna temperature | 150 K |  |  |
| Noise figure | 1.2 dB |  |  |
| Tx transmit power | 2 W (33 dBm) |  |  |
| Tx antenna gain | 43.2 dBi |  |  |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

### 2.2.3	TN parameters

TN parameters for co-existence study are given in Table 2.2.3-1, 2.2.3-2 and 2.2.3-3.

| -Parameters | Urban macro | Dense Urban | Indoor |
| --- | --- | --- | --- |
| Channel bandwidth | 400MHz/200MHz | 400MHz/200MHz | NA |
| Scheduled channel bandwidth per UE (DL) | 400MHz | 400MHz/200MHz | NA |
| Scheduled channel bandwidth per UE (UL) | 400MHz/200MHz | 400MHz/200MHz |  |
| The number of active UE (DL) | Same as the number of BS beam | Same as the number of BS beam |  |
| The number of active UE (UL) | Same as the number of BS beam | Same as the number of BS beam |  |
| Traffic model | Full buffer | Full buffer |  |
| DL power control | NO | NO |  |
| UL power control | YES | YES |  |
| BS max TX power in dBm | 30dBm | [30dBm] |  |
| UE Peak EIRP in dBm | 22.4dBm | 22.4dBm |  |
| UE min TX power in dBm | -40dBm | -40dBm |  |
| BS Noise figure in dB | Note 1 | Note 1 |  |
| UE Noise figure in dB | Note 1 | Note 1 |  |
| Handover margin | 3dB | 3dB |  |
| Note 1: For deriving ACIR/ACS values which are included in respond to WP5D, following NF are used in co-existence simulation study for both UE and BS. 30GHz: 9 and 11 dB |  |  |  |

Table 2.2.3-2: Deployment-related parameters of TN

|  | Urban Macro | Dense Urban | Remarks |
| --- | --- | --- | --- |
| ISD in meters | 200 | 200 | ITU-R Report M.2292[11] |
| BS Antenna height in meters | 25 | 10 |  |
| UE Outdoor/indoor | 100% Outdoor |  |  |
| UE height in meter | 1.5 | 1.5 | RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13] |

Table 2.2.3-3: ACLR/ACS for TN

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28 dB |
|  | ACS | 24 dB |
| UE | ACLR | 17dB |
|  | ACS | 23dB |

### 2.2.4	TN UE parameters

TN UE parameters are given in Table 2.2.4-1

Table 2.2.4-1: TN UE characteristics for system level simulations

| Antenna element vertical radiation pattern (dB) |  |
| --- | --- |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 5dBi directivity and 2dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees Note 3 Fixed elevation: 90 degrees |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note3: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. Note 4: A 90 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation |  |

## 2.3.  Antenna and beamforming pattern modelling

### 2.3.1	NTN BS and UE antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811[5].

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθkasinθ2        for 0<θ≤90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument;

-	x, a is the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency.

| Scenario | Scenario A, C2 and D2 |
| --- | --- |
| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| Number of beams | Baseline: 19-beam layout considering wrap-around mechanism (i.e. 18 beams surrounding the central beam and allocated on 2 distinct "tiers") |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: Central beam boresight direction computed based on elevation angle target |

Table 2.3.1-1: System Level Simulation assumptions for calibration

| Configuration scenario | A, C2 and D2 |
| --- | --- |
| Frequency band | Ka-band (i.e. 20 GHz DL, 30 GHz UL) |
| Maximum Bandwidth per beam (DL + UL) | Ka-band: DL 400 MHz and UL 400 MHz The bandwidth per beam must be adapted based on the frequency factor and the polarization re-use option considered. |
| Satellite characteristics (G/T, EIRP density, antenna diameter) | See Table 6.1.1.1-1 and Table 6.1.1.1-2 Note: Same satellite characteristics should be considered for both single and multi-satellite simulations |
| Satellite antenna pattern | See section 6.4.1 in [2]: Bessel function |
| Satellite polarization configuration | Circular |
| Beam layout definition | For singles satellite simulation: See Table 6.1.1.1-4 For multi satellites simulation: FFS |
| Frequency re-use factor | Option 1: 1 Option 2: 3 Option 3: 2 if polarization re-use is enabled |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| Channel model | Large scale model of [2] (Note 2) |
| Deployment scenarios | Base-line: Rural Additional deployment scenario results can be provided |
| Propagation conditions | Base-line: Clear Sky Line of sight |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | Base-line for calibration: at least X=10 UEs per beam with uniform distribution in all the Voronoi cell area associated to each beam. The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE configuration | Ka-band: VSAT Others (optional for scenario A) See Table 6.1.1.1-3 |
| UE orientation | VSAT and Others: Ideal Tracking serving beam; Handheld: Random |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Metrics for calibration | Base-line: Coupling loss, Geometry Note: Coupling loss is defined as the signal loss from the antenna port to the antenna port |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). The atmospheric absorptions loss shall be considered. |  |

### 2.3.2	TN BS and UE antenna and beam forming pattern modelling

Antenna configuration for FR2 BS are listed in the following table 2.3.2-1 and table 2.3.2-2.

Table 2.3.2-1: FR2 BS antenna modelling for urban macro scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| (Mg, Ng, M, N, P) | For 30 GHz: (1, 1, 8, 16, 2) Note1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results. |  |

Table 2.3.2-2: FR2 BS antenna element pattern for dense urban scenario

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8dB loss) |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: TX power is specified per polarization, a single polarization may be simulated under the assumption of polarization match. Note 3: A 65 degree element beamwidth was assumed for simulations, even though the physically correct beamwidth would be 130 degrees. The difference in assumption does not substantially impact the simulation results |  |

## 2.4. ACIR model

ACIR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942 is used as baseline.

## 2.5. Propagation model

### 2.5.1 Propagation model between NTN BS and UE

Propagation model between NTN BS and UE should reference to section 6.6 in TR 38.811[5].

### 2.5.2	Propagation model between TN BS and UE

Propagation model between TN BS and UE should reference to section 7.4 in TR 38.901[10].

### 2.5.3	Propagation model between NTN BS and TN BS

Propagation model between NTN BS and TN BS should reference to TS 38.811[5] which is used for DL-UL cross link interference for band n1.

### 2.5.4	Propagation model between NTN UE and TN UE

Propagation model between NTN UE and TN UE should reference to the following propagation model,

- UE-to-UE: UMi (h_BS=1.5 m ~ 22.5 m) 
	  + penetration loss see TR 38.803 [5]

## 2.6. Transmission power control model

### 2.6.1	NTN and TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

### 2.6.2	NTN and TN UL TPC

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted

### 2.6.3	NTN and TN DL TPC

For downlink scenario, no power control scheme is applied.

## 2.7. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

## 2.8. Performance metric

For TN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN, the average throughput loss and 5%-ile throughput loss should be less than 5%.

## 2.9. Throughput ~ SNR mapping

The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Where:

S(SNIR)	Shannon bound, S(SNIR) =log2(1+SNIR) [bps/Hz]
		Attenuation factor, representing implementation losses
SNIRMIN  	Minimum SNIR of the code set, dB
SNIRMAX 	Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Conclusions

In this contribution, we shared some further views on coexistence study for NTN coexistence in Ka-band and proposals are made as following:

Proposal 1: propose to use the simulation assumption in this contribution as baseline for NTN Ka band coexistence study.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828