3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216578

Electronic Meeting, October 10 – October 19, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	General considerations on RRM requirements for multi-RX RRM

Agenda item:	6.8.3.1

Document for:	Discussion

Introduction

In this paper we discuss RRM aspects related to the multi-RX chain reception in FR2 for Rel-18. In the last RAN4 meeting we had the initial discussions related to the WI and in this is a continuation of the discussion. We address the objectives of the work item and based on those, observations and proposals are derived concerning assumed architecture, deployment, and beam management considerations for RRM measurement enhancements.

Discussion

## Deployment considerations

Figure 3: Distributed m-TRP deployment consideration

In the last RAN4 meeting, the scenarios for Rel-18 multi-RX work item were discussed, and the following was captured in the WF:

Issue 1-1-1-8: Scenarios for Rel-18 multi-Rx DL reception - Proposals - Option 1: support both intra-cell and inter-cell operation with TRPs located within reasonable intercell distance. - Option 2: Working on inter-cell operation with TRPs located within reasonable intercell distance after intra-cell multi-TRP operation work is completed. - Option 3: intra-cell multi-TRP operation only

Rel-17 already discussed and introduced both intra-cell and inter-cell multi-TRP operation, so in our view, there is no reason not to include the scenario in Rel-18 work and this work item.

- Rel-17 already discussed and introduced both intra-cell and inter-cell multi-TRP operation.

Support both intra-cell and inter-cell multi-TRP operation.

Until Rel-17, the assumption is that the propagation delay difference between the two TRPs is within the CP length. In practice, this restricts the deployment scenarios, since it assumes that the distance between the two TRPs is negligible.

Until Rel-17 the assumption is that the propagation delay difference between two TRPs to the UE is within the CP length.

- Our view is that the multi-RX UE can support independent time and frequency tracking for each Rx chain and can handle larger propagation delay differences than what is currently supported up to Rel-17 using a single Rx chain. Therefore, there would be no need to restrict the supported scenarios to propagation differences within CP.

The multi-RX UE can support independent time and frequency tracking for each Rx chain.

The MRTD is defined as the timing alignment error (TAE) plus the propagation delay difference (∆Tprop). Therefore, up to and including Rel-17, there are no MRTD requirements for operations the same carrier frequency (intra-carrier MRTD for multi-TRP reception).

In 3GPP TS 38.104, the TAE for MIMO ranges between 65ns to 260ns depending on the feature, and on the CP duration. For carrier aggregation and dual connectivity, the MRTD on FR2 goes up to 8.5 μs, allowing for some propagation delay difference between the transmission sources and the UE. These requirements are not applicable operation on the same carrier.

- The existing MRTD requirements are not applicable to intra-frequency operation.

Consider receive time differences larger than CP.

## Architecture considerations

The scope of this work depends on the assumptions related to the UE measurement capability with multiple RX chains. This will impact the requirements for RRM measurements, TCI switching, etc. The WID has defined the terms ‘multiple RX chains’, that does not simply mean multiple antenna panels. It is possible that multi-RX chains assumes multiple UE implementations options including simultaneous operation on multiple antenna panels, frond-end, demodulation and RRM. There can be various UE implementation options, and the WI scope include them for requirement study.

Independent timing and frequency tracking assumption impacts the possible deployment, i.e., if UE does not support independent tracking, the inter-TRP distance, deployment scenarios and WI scope will be limited.

- Supporting separate baseband processing per Rx chain, enables simultaneous measurements and data path processing while receiving from distributed TRPs.

Figure 4: Architectural overview of BB/RF blocks involved in multiple RX chain capabilities

In the above figure both RF chains will need independent timing control due to distributed TRPs.

According to the below highlighted RAN1#109 Agreement [4], two independent timing loops are needed for multi-TRP scenarios when assuming support of 2 TAs.

RAN1 #109 (NR_MIMO_evo_DL_UL) Agreement - Support two TA enhancement for both intra-cell and inter-cell multi-DCI multi-TRP scenarios in Rel-18.

- MIMO evolution is already assuming that 2 TAs can be supported, which implies separate time and frequency tracking.

Multi Rx architecture with above consideration, to assume that each Rx chain will need to process at an independent FFT window.

Multi Rx architecture to assume that each Rx chain can perform independent RRM measurements or demodulation tasks.

Multi Rx architecture to assume that each Rx chain can perform independent RRM measurements on 2 Rx chains.

Independent timing loops are supported in multi-RX for intra and inter cell mobility measurements.

## Beam management

In the previous RAN4 meeting, we have discussed beam management with the following open proposals for discussion [3]

Issue 1-1-8: Beam management - Proposals - P1: RAN4 can consider two options for beam management for UE with multiple RX chains - Option-(i) Reuse independent beam management concept (IBM) from Inter-band CA study - Option-(ii) define panel or RX chain specific behaviors with RX panel control signal for DL - P2: Study independent beam managements for different QCL-D sources with multiple TRPs, and also study other QCL channel property impacts of {average gain, delay shift and doppler shift} for multi-RX chain requirements.

IBM support is currently according to the following schematic, in Rel-17. This framework has been used to support intra/inter-cell mobility in multi-TRP scenarios with different bands.

Figure 1: IBM architecture in Rel-16/Rel-17

Since the IBM framework in Rel-17 is defined for the support intra/inter-cell mobility on 2 Rx chains and different bands, IBM shall be extended to intra-band multi-TRP scenarios as well.

During the last RAN4 meeting, we have agreed on the number of searchers for cell detection and RRM measurements [3]:

RAN4#104 Agreement Issue 1-1-1-2: Number of searchers for cell detection and measurements - 2 searchers are assumed.

In view of the above agreement, 2 searchers would improve RRM measurement requirements. Since the searchers may need independent control of the spatial filters, it can be assumed that IBM must be extended to intra-band multi-TRP scenarios.

Figure 2: Proposed IBM architecture for intra Band m-TRP operation in Rel-18

Independent beam management between multiple Rx chains on the same carrier is assumed in this work item.

Conclusion

This paper has presented Nokia’s views on Rel 18 multi-RX chain architecture, deployment, and beam management general considerations in FR2, for improving RRM measurement requirements. As part of this discussion, we have reached the following observations and proposals are made

- Rel-17 already discussed and introduced both intra-cell and inter-cell multi-TRP operation.

- Support both intra-cell and inter-cell multi-TRP operation.

Until Rel-17 the assumption is that the propagation delay difference between two TRPs to the UE is within the CP length.

- The multi-RX UE can support independent time and frequency tracking for each Rx chain.

The existing MRTD requirements are not applicable to intra-frequency operation.

Consider receive time differences larger than CP.

Supporting separate baseband processing per Rx chain, enables simultaneous measurements and data path processing while receiving from distributed TRPs.

- MIMO evolution is already assuming that 2 TAs can be supported, which implies separate time and frequency tracking.

Multi Rx architecture with above consideration, to assume that each Rx chain will need to process at an independent FFT window.

Multi Rx architecture to assume that each Rx chain can be performing independently RRM measurements or demodulation tasks.

Multi Rx architecture to assume that each Rx chain can be performing independently RRM measurements on 2 Rx chains.

Independent timing loops are supported in multi-RX for intra and inter cell mobility measurements.

Independent beam management between multiple Rx chains on the same carrier is assumed in this work item.

- References

- R4-2212688, “Discussion on RRM requirements for FR2 multi-Rx chain reception”, Nokia, Nokia Shanghai Bell

- RP-221753, "Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception", Qualcomm

- R4-2214344, “WF on FR2 multi-Rx RRM”, Vivo

- Draft_Minutes_report_RAN1#109-e_v030.docx