3GPP TSG-RAN4 #104-bis-e	R4-2216585

Electronic Meeting, 10th –19th Oct 2022

Title:	Views on AoA pairs for FR2 Multi-Rx chain DL reception

Source:	Anritsu Limited

Agenda Item:	6.8.2.2

Document for:	Discussion

# Introduction

During RAN4 #104-e meeting, the topic of AoA pairs for setting the UE RF requirement was discussed and several solutions were proposed and listed in the WF [1], which proposed to further discuss them and study other solutions as can be seen below.

## 2.3 Test setups

### 2.3.1. How to determine candidate AoA pairs for setting the UE RF requirement

Agreement:

- Further discuss on the on the candidate AoA pairs for setting the UE RF requirement

  - One Fixed AoA1 (e.g. Peak) + Full set AoA2.

  - Multiple AoA1 + Full set AoA2.

  - Fixed offset between the two AoAs, both probes swept simultaneously.

  - Full set AoA1 + Full set AoA2

Other solutions are not precluded. Companies are also encouraged to bring the analysis on how to quantify the Refsens value when receiving multiple signals.

# Discussion

In this section, we present the choice of AoA pairs in two sub-sections:

- with the focus of defining UE RF requirements with almost no trade-off with Test system complexity, test system reuse, test time. While also listing the comments from different companies during 104-e meeting.

- with the focus of defining UE RF requirements considering trade-offs with feasibility, test system complexity, test system reuse, test time.

## AoA pairs and UE RF requirements (solely ensuring real-life performance)

In this section are listed based on the discussions in [2] the different solutions of AoA pairs and their flexibility offered for setting UE RF requirements, listed also is the feedback and comments from different companies regarding about those different AoA pair options.

Table 1 - Comparison of the different candidate AoA pairs for setting the UE RF requirements.

| OPTION | Flexibility for setting UE RF requirements Companies considering option as possible choice based on R4-2214245 | Comments from Email Discussion Summary for [104-e][134] FR2_multiRx_UERF [2] |
| --- | --- | --- |
| #0 All | LGE (1), Lenovo (1), Oppo (1) | LGE: “Support proposal with all candidates. For Core requirements, we need to consider all candidates at first. If the difference is not found, then we can remove some candidates.” Lenovo: “Support proposal with all candidates for now. How can a measurement method be selected before spherical coverage for two directions is defined? What is the formal definition?” Oppo: “Support all the options as candidates and down-selection can be done based on further study/discussion.” |
| #1 One Fixed AoA1 (e.g. Peak) + Full set AoA2 | vivo (1), Samsung (1), Nokia (1), Rohde & Schwarz, | Sony: “One concern on having one AoA fixed [...] is how we can ensure the AoA is fixed to UE orientation if the UE is rotating during the test. To our understanding, this is not always possible for all kinds of test equipment.” Samsung: “already brings double test efforts compared with legacy spherical coverage test” “We agree with Nokia that the 1st option is the best compromise.” Rohde & Schwarz: “For sequential measurements as e.g. outlined by Samsung (find AoA1, fix and transfer to link/anchor, scan AoA2), Option 1 or Option 2 could be feasible as well. This may however require a specific test function that allows fixing one of the beams (similar to current UBF).” Ericsson: “It is not obvious how to test the proposed options assuming a test with a rotating DUT and two probes – measure spherical coverage with different separations between the probes?” Nokia: “The 1st option is the best compromise in our view.” Lenovo: “For proposal 1, the measurement result will be highly dependent on selection of AOA1. If AOA1 is selected as peak EIRP, then measurement is already conditioned on AOA1 being in coverage (which is a biased result). Conversely, if AOA1 is selected as direction which does not meet some minimal requirement for EIS, then joint reception will fail regardless of the value of AOA2 that is selected (also biased).” vivo: “Option 1 is currently our first preference.” |
| #2 Multiple AoA1 + Full set AoA2 | QC (1), vivo, Rohde & Schwarz | Sony: “One concern on having one AoA fixed [...] is how we can ensure the AoA is fixed to UE orientation if the UE is rotating during the test. To our understanding, this is not always possible for all kinds of test equipment.” Qualcomm: “Best compromise.” in terms of flexibility to set proper RF requirements while test system complexity.” Samsung: “option 2, the test efforts is not affordable, and actually is not so necessary.” Rohde & Schwarz: “For sequential measurements as e.g. outlined by Samsung (find AoA1, fix and transfer to link/anchor, scan AoA2), Option 1 or Option 2 could be feasible as well. This may however require a specific test function that allows fixing one of the beams (similar to current UBF).” Ericsson: “It is not obvious how to test the proposed options assuming a test with a rotating DUT and two probes – measure spherical coverage with different separations between the probes?” Nokia: “The 2nd option leads to very large test time. We could see it reasonable with two AoA1.” |
| #3 Fixed offset between the two AoAs, both probes swept simultaneously | Rohde & Schwarz Ok for multiple ‘fixed offsets’: Qualcomm Ok for two offsets (narrow and wide): Sony, Nokia | Qualcomm: “ok […] having multiple ‘fixed offsets’, but a single offset may not work for all future UEs.” vivo1: “fixed offset is problematic since no one typical value is reasonable. If multiple offsets considered, the test burden with both swept can still be unduly high.” Sony “For [that] option, it is possible to verify under two offset values for PC3, to mimic wide and small angel separation of TRPs, which only requires two sweepings. We think the test time of such a scheme would be similar at least comparing to option 2.” Samsung: “we think there are some issues. For the Back to Back 2 panel implementations, the test could not be done with fixed offset between 2AoAs. Moreover, How to set the direction of the fixed offset between the two AoAs (vertical, horizontal, or other directions) will cause different relative position between UE and the two AoAs” Rohde & Schwarz: “Option [3] could be a reasonable approach for testing, when measurement on both directions at the same time is required and can be realized in e.g. the RRM 2AoA system today. Here also different fixed offsets would be possible.” Nokia: “The 3rd option might delay the work because of the choice of offset between the two AoAs. One alternative might be to choose two offsets (narrow and wide) which is then equivalent to the 2nd option” |
| #4 Full set AoA1 + Full set AoA2 | Most companies do not consider as a possible choice | Nokia: “The 4th option leads to very large test time.” Apple: “We can discuss further at the next meeting. Perhaps the agreement we can reach at this meeting is to remove “Full set AoA1 + Full set AoA2” from further consideration because it is unrealistic and incurs excessively high testing burden..” Qualcomm: “We agree with the test burden aspect of proposal 4, but it may be useful to understand the relation between the results from an N2 search, versus a k*N search (option 2) during the study phase. This may help with validating or identifying ‘real life performance’ concerns.” Moderator: “We agree with the test burden aspect of proposal 4, but it may be useful to understand the relation between the results from an N2 search, versus a k*N search (option 2) during the study phase. This may help with validating or identifying ‘real life performance’ concerns.” Samsung: “For the 4th option, it is not affordable. Note that verification test will follow core requirements. We don’t think it reasonable to define requirement with full set pairs while the test only cover few of them regarding spherical coverage test case.” vivo2: “Option 4 might be useful for some theoretical analysis, since it is the most complete set of all possibilities. However, it is basically impossible to be really used. Since the requirements are closely related to verification framework including this AoA pair, it is questionable to consider this option in the requirements.” |
| #5 One point AoA1 (not fixed) + Full set AoA2 |  | Not discussed |

Observation 1: At some point, the choice of the AoA pair for setting the UE RF requirements should be based on a preselection of the simplest AoA pairs allowing to set requirements that allow guarantying FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. It is understood that “Full set AoA1 + Full set AoA2” is the most complicated AoA pair.

Observation 2: Due to test time, test system complexity and test system reuse, refinement of the RF requirements should be looked at such that one AoA pair can be chosen while still guarantying real-life performance of the FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. That decision should happen after the preselection of AoA pairs have been studied properly.

Observation 3: The best and final choice of an AoA pair may be influenced by discussions outside the “FR2 multi-Rx chain DL reception” WI ones, such as related as a to RF/RRM/Demodulation including Tx related requirements and may still vary within Rel-18 timeframe. It may be necessary to prepare specific UE RF requirements or one set of UE RF requirements compatible with different plausible AoA pairs.

## AoA pairs and TE setups (+ practical aspects)

In this section are listed the different solutions of AoA pairs and possible TE setups, the following points are compared: feasibility, test system complexity, test system reuse and test time.

Table 2 - Comparison of the different candidate AoA pairs in terms: feasibility, test system complexity, test system reuse and test time.

| OPTION | Tdoc# of a solution | Feasibility | Test system complexity | Test system reuse | Test time |
| --- | --- | --- | --- | --- | --- |
| #1 One Fixed AoA1 (e.g. Peak) + Full set AoA2 | R4-2212806 [5], similar view in R4-2211993 [6] | Keeping the AoA1 towards the UE beam peak direction is complicated without deteriorating the QZ as the UE is rotating |  |  | Similar to single AoA |
| #2 Multiple AoA1 + Full set AoA2 | R4-2212806 [5] Option 3 in [3] (discussed in R4-2211549 [4]): IFF+DFF, fixed DFF antennae as NR anchor | However, number of AoA combinations is limited. | Less complicated configuration than option 3 and 4 with almostsufficiently full rotational degrees of freedom from 1 AoA. | Enhancement of existing 1AoA FR2 OTA test systems. High chance to reuse the existing 2AoA RRM test configuration. | Require sequential measurements for each AoA (longer test time) |
| #3 Fixed offset between the two AoAs, both probes swept simultaneously | R4-2213568 [7] |  |  |  |  |
| #4 Full set AoA1 + Full set AoA2 | Option 1 in [3] (discussed in R4-2211549 [4]): IFF+IFF with moving reflectors, Test 2 AoAs simultaneously with 2 IFF Option 2 in [3] (discussed in R4-2211549 [4]): IFF+DFF, DFF antennae as the second AoA NR anchor | Full rotational degrees of freedom cannot be supported (reflectors proximity, interference issue) [3] | Most complicated structure from the existing RF OTA test systems in the market which will delay the release of dedicated test systems (since the current OTA test system for 1AoA is basically designed based on the combined axes system) [3]. Still complex design from the existing FR2 OTA test systems | Completely new design | Enormous |
| #5 One point AoA1 (not fixed) + Full set AoA2 | Option 5 in [3] (discussed in R4-2211991 [8]): IFF+ rotating UE and anchor probe as a whole, the probes are divided into test probe and anchor probe |  |  |  |  |
| #6 Sequential testing | Option 4 in [3] discussed in R4-2211549 [4]): Sequential tests by introducing a new test command to fix an active antenna in the DUT |  |  |  |  |

Observation 4: Choice of dedicated FR2 OTA test system configuration may still vary depending on assumptions of test conditions and scenarios for minimum requirements within Rel-18 timeframe.

Observation 5: Sequential tests as in Option 6 (Sequential tests by introducing a new test command to fix an active antenna in the DUT) and Option 5 (IFF+ rotating UE and anchor probe as a whole) should be considered.

Observation 6: To consider option 6 as equivalent to other test methods, further studies are necessary on the way to process the obtained sequential test results.

Proposal 1: Sequential measurement principle should be studied at top level and for different AoA pairs, and conclusion should be made if it can be considered or not.

# Conclusion

Based on the discussion above, we propose the following:

Observation 1: At some point, the choice of the AoA pair for setting the UE RF requirements should be based on a preselection of the simplest AoA pairs allowing to set requirements that allow guarantying FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. It is understood that “Full set AoA1 + Full set AoA2” is the most complicated AoA pair.

Observation 2: Due to test time, test system complexity and test system reuse, refinement of the RF requirements should be looked at such that one AoA pair can be chosen while still guarantying real-life performance of the FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. That decision should happen after the preselection of AoA pairs have been studied properly.

Observation 3: The best and final choice of an AoA pair may be influenced by discussions outside the “FR2 multi-Rx chain DL reception” WI ones, such as related as a to RF/RRM/Demodulation including Tx related requirements and may still vary within Rel-18 timeframe. It may be necessary to prepare specific UE RF requirements or one set of UE RF requirements compatible with different plausible AoA pairs.

Observation 4: Choice of dedicated FR2 OTA test system configuration may still vary depending on assumptions of test conditions and scenarios for minimum requirements within Rel-18 timeframe.

Observation 5: Sequential tests as in Option 6 (Sequential tests by introducing a new test command to fix an active antenna in the DUT) and Option 5 (IFF+ rotating UE and anchor probe as a whole) should be considered.

Observation 6: To consider option 6 as equivalent to other test methods, further studies are necessary on the way to process the obtained sequential test results.

Proposal 1: Sequential measurement principle should be studied at top level and for different AoA pairs, and conclusion should be made if it can be considered or not.

# References

[1] R4-2214457, “WF on multi-Rx chain DL reception”, Sony, RAN4 #104-e, Electronic meeting

[2] R4-2214245, “Email Discussion Summary for [104-e][134] FR2_multiRx_UERF”, Moderator (Qualcomm Incorporated), RAN4 #104-e, Electronic meeting

[3] R4-2215711, “Views on FR2-1 RF OTA test for a device with multi-panel reception”, Anritsu Corporation, RAN4 #104-bis-e, Electronic meeting

[4] R4-2211549, “Views on FR2-1 RF OTA test for a device with multi-panel reception”, Anritsu Corporation, RAN4 #104-e, Electronic meeting

[5] R4-2212806, “General analysis on Multi-Rx requirements”, vivo, RAN4 #104-e, Electronic meeting

[6] R4-2211993, “Requirements for FR2 simultaneous DL reception from different directions”, Samsung, RAN4 #104-e, Electronic meeting

[7] R4-2213568, “Views on multi-Rx chain DL reception in FR2”, Sony, Ericsson, RAN4 #104-e, Electronic meeting

[8] R4-2211991, “Considerations on FR2 multiple AoA test”, Samsung, RAN4 #104-e, Electronic meeting