3GPP TSG-RAN WG4 Meeting #104bis-e                                    R4- 2216587

Electronic Meeting, October 10 – 19, 2022

Source: 	Huawei, HiSilicon

Title: 	On FR1 8Rx UE RF requirements

Agenda Item:	6.6.3

Document for:	Approval

Introduction

In the RAN#96, the WI for Rel-18 further RF requirements enhancement for NR and EN-DC in frequency range 1 has been approved [1]. The 8Rx will be discussed for certain types of UE like CPE/FWA.

Enable 8Rx for CPE/FWA/vehicle/industrial devices [RAN4] - Example bands: - TDD bands: n41, n77/ n78 - FDD bands: n7 - Note 1: the total number of example band should be limited to 3. n77/n78 are considered as one band during the study. - Note 2: other bands to be introduced in the release independent way later on from Rel-18 - Note 3: specifying requirements for TDD bands has first priority - Specify the UE RF requirements to support 8Rx - Study and specify the requirements to support SRS antenna switching for t1r8, t2r8, t4r8 - Discussion on t4r8 shall start after at least one PC for 4Tx is completed - NOTE: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

In the RAN4#104-e, initial discussion regarding 8Rx RF requirements have been done with following fruitful outcomes (and also some FFS points) which are captured in the WF [2]:

- ΔRIB, 8R: - Study possible performance of ΔRIB, 8R in NR devices compared to LTE. - Considering the fact NR 8Rx aims for CPE/FWA/vehicle/industrial devices while LTE 8Rx aims for any types of UE. - As impacted factors, ILs, couplings among Rx paths, and antenna correlation should be studied. Other factors are not precluded. - 1T8R/2T8R AS-SRS: - Further study ΔTRxSRS for 1T8R/2T8R AS-SRS considering the following aspects: - ILs, form factor and the number of Rx/Tx antennas as impacted factors. - CPE/FWA/vehicle/industrial devices specific assumption. - The impact of larger SRS IL on the NW performance. - Further study if the assumption that zero delta is applied for the transmission power calculation for the first SRS resource still valid. - ΔTRxSRS for 4T8R AS-SRS will be discussed after 4Tx requirement is completed. - Methods to improve the SRS output power given the large delta SRS value: - For a UE that supports 2Tx and 1T8R SRS AS, further study whether 3dB power back off at main antenna defined for TxD (ΔPPowerClass) is applied for PCMAX_H,f,c or not. - Whether guard period between xTyR (y>=4) AS-SRS resources within one resource set needs to be kept: - Further study if remove the requirement for a guard period between two SRS resources transmitted in different symbols of the same slot belonging to the same SRS resource set with ‘antennaSwitching’ usage. - Check if this has RAN1 spec impact.

In this contribution, we would like to share our views regarding some 8Rx RF discussion points.

Discussion

SRS antenna switch additional IL for 8R

As an important feature to facilitate better DL channel estimation accuracy, the SRS antenna switch has been considered for NR UE RF design since Rel-15. Due to some UE implementation factors like space or power consumption, normally the number of Tx branch, which is equipped with PA, could be smaller than the number of Rx branch. In order to realise the antenna switch function, multiple RF components, e.g. RF switch, need to be implemented. Meanwhile the total insertion loss for each RF branch will be increased. To better explain the situation, we provide following examples for possible RF architectures of ‘t2r4’ and ‘t1r4’ capable UE, respectively.

Figure 1. Example for possible RF architecture of ‘t2r4’ (left) and ‘t1r4’ (right) capable UE

As depicted in the above figure, under one typical RF architecture, where LPAF is the module includes LNA/PA/Filter/switch, and DRX-M is the diversity receive module which includes the LNA and filters, the total insertion loss is different between main branch (Tx branch) and other branch (diversity receive branch). And it could be contributed by the RF switches which are implemented for adapting different physical antenna location of different branch. For instance, for a ‘t2r4’ capable UE, only one SPDT is implemented on the main branch, while additional IL can be expected since additional SPDT is introduced. Similar situation can be proved for a ‘t1r4’ capable UE. We believe that is the reason why specification introduced the non-zero transmission power relaxation i.e. ΔTRxSRS for 4Rx.

In last meeting, some inputs regarding the ΔTRxSRS for 8Rx were presented in [3]. Although UE capability on SRS antenna switch is involved as another factor, there is still a single IL value for all diversity branch at a certain frequency range which is the same as the existing ΔTRxSRS for 4Rx.

Consider that the IL imbalance between the main branch and the other branch especially for 8Rx could be more serious, if we still apply single value to present the SRS IL for an 8Rx UE, which is not well aligned with the actual UE implementation, maybe it is enough for the UE to pass some kind of test which might be specified for AS-SRS in the future, but it is definitely not beneficial for the overall network performance due to the reason that the SINR of SRS from each antenna is critical to the accuracy of channel estimation from BS perspective, which will consequently impact e.g. the choose of precoder.

Observation 1: For t1r8/t28r AS-SRS, if single non-zero transmission power relaxation ΔTRxSRS still be applied for all branches other than the main branch, the actual SRS IL for each branch determined by UE implementation cannot be known by BS. Consequently the channel estimation accuracy at BS side will not be guaranteed and the overall network performance will be degraded.

Thus we propose to allow the UE for reporting multiple ΔTRxSRS which could cover the different IL imbalance between the main branch and all other branches. The details can be further discussed.

Proposal 1: For t1r8/t28r AS-SRS, allow the UE for reporting multiple ΔTRxSRS values in order to cover all different ILs between the main branch and all other branches.

Increasing main branch IL due to 8Rx

Apart from the ΔTRxSRS, which is used for reflecting the non-zero IL difference between the main branch and other branches, another issue is that the IL on main branch becomes more serious especially for 8Rx. The example for RF architectures comparison between ‘t2r8’ and ‘t2r4’ can be found as below.

Figure 2. Example for possible RF architecture between ‘t2r8’ (left) and ‘t2r4’ (right) capable UE

Also, the example for RF architectures comparison between ‘t1r8’ and ‘t1r4’ is given as below.

Figure 3. Example for possible RF architecture between ‘t1r8’ (left) and ‘t1r4’ (right) capable UE

Regardless the SRS antenna switch capability (which is mapping to the Tx branch number) of a UE, it can be observed for the main branch that more IL can be expected considering different RF switch shall be used (SPDT -> SP4T for 2Tx case and ‘SP4T -> SP8T’ for 1Tx case).

Observation 2: For the SRS antenna switch capable 8Rx UE, more IL could be expected on the main branch.

But in the current specification, non-zero relaxation is only defined for other branches rather than main branch:

Some typical RF switch IL values are presented in the following table. Obviously, the main branch’s zero IL assumption may not be valid at least under 8Rx RF architecture. In conclusion, we propose 1.5dB (IL difference between SP8T and SPDT)

Table. Typical IL value of RF switch at 2.5GHz and 3.5GHz

|  | @2.5GHz | @3.5GHz |
| --- | --- | --- |
| SPDT | 0.6 | 0.7 |
| SP4T | 0.9 | 1 |
| SP8T | 1.5 | 1.8 |

Proposal 2: Non-zero transmission power relaxation for the main branch shall be applied for the 8Rx UE that capable of SRS antenna switch.

- 1.5dB can be considered for PCMAX_L,f,c.

Methods to improve the SRS output power given the large delta SRS value

- ΔPPowerClass

For the highlighted part, it can be understood that this 3dB relaxation for both PCMAX_L,f,c and PCMAX_H,f,c is introduced for TxD UE, since the antenna virtualization would be disabled for AS-SRS transmission with 1T1R/1T2R/1T4R capability.

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {PEMAX,c, PPowerClass – ΔPPowerClass }

As for 8R, we think such relaxation could be removed for the PCMAX_H,f,c, considering that additional insertion loss is already existed comparing to 4Rx which needs to be compensated and more performance degradation might be introduced from this transmission power boundary limitation. Thus we have the following proposal.

Proposal 3: For a UE indicating the support of TxD and 1T8R AS-SRS, the ΔPPowerClass applied for PCMAX_H,f,c could be removed.

Conclusion

In this contribution we discussed on the FR1 UE 8Rx RF requirements. According to the analysis, we have the following observations and proposals:

Observation 1: For t1r8/t28r AS-SRS, if single non-zero transmission power relaxation ΔTRxSRS still be applied for all branches other than the main branch, the actual SRS IL for each branch determined by UE implementation cannot be known by BS. Consequently the channel estimation accuracy at BS side will not be guaranteed and the overall network performance will be degraded.

Observation 2: For the SRS antenna switch capable 8Rx UE, more IL could be expected on the main branch.

Proposal 1: For t1r8/t28r AS-SRS, allow the UE for reporting multiple ΔTRxSRS values in order to cover all different ILs between the main branch and all other branches.

Proposal 2: Non-zero transmission power relaxation for the main branch shall be applied for the 8Rx UE that capable of SRS antenna switch.

- 1.5dB can be considered for PCMAX_L,f,c.

Proposal 3: For a UE indicating the support of TxD and 1T8R AS-SRS, the ΔPPowerClass applied for PCMAX_H,f,c could be removed.

- References

- RP-221496, “Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (RF1)”, Huawei, HiSilicon, 3GPP TSG-RAN Meeting #96, Budapest, Hungary, June 6 – 9, 2022.

- R4-2214451, “WF on FR1 8Rx UE RF requirements”, NTT DOCOMO, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.

- R4-2213312, “R18 Discussion on 8Rx FWA”, OPPO, 3GPP TSG-RAN WG4 Meeting #104-e, Electronic meeting, August 15 – 26, 2022.