3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216596

Electronic Meeting, October 2022

Source	: Nokia, Nokia Shanghai Bell

Title	: Remaining issues on unified TCI switching delay requirement

Agenda item	: 4.5.1.1

Document for	: Discussion

Introduction

RAN4 #104 discussed the remaining issues of Rel-17 feMIMO WI with RRM core requirements. The outcome was captured in two WFs:

- FeMIMO RRM impact for unified TCI [1]

- FeMIMO RRM requirements for inter-cell beam management [2]

In this contribution we discuss the remaining aspects of the unified TCI.

Discussion

Joint TCI state switching requirement

In RAN4 #103, it was agreed that the DL and UL requirements can be applicable independently.

RAN4#103 WF R4-2211203 Issue 1-1-3a Joint TCI state switching requirement - RAN4 #101bis GTW Agreements - No extra requirement needed for Joint TCI mode, DL and UL requirements can be applicable independently - Note: it is not expected that UE will be required to make DL reception or UL transmission before UE completes the DL or UL TCI state switching, respectively

In RAN4 #104 agreed on the following:

Issue 1-2-1 Joint TCI switching delay requirement - Agreement: - For UL TCI state switching, - In case of joint TCI state switch, UE is not expected to transmit on UL before UE completes the DL and UL TCI state switch. - Proposal: - For DL TCI state switching, - [In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch.]

Our interpretation of the agreement in RAN4 #103 is that the DL and UL TCI state switch requirements could be applicable independently.

For the joint TCI state switch, there could be different delays considering whether the UL or DL states are known or unknown or the PL-RS is maintained or not maintained.

In DL TCI state switch delay, the UE is expected to receive as soon as the DL TCI state switch is completed.

The proposal in brackets in the WF from the last RAN4 meeting suggests that in order to receive on DL, the UE needs also to wait for the completion on the UL. However, DL TCI state switching and reception using the new TCI state in DL is independent from UL TCI state switch. Hence, the DL reception is feasible before UL TCI switch is completed.

Considering that among the terms to be considered in the UL TCI state switch there is the PL-RS maintenance, which can lead to a very long delay in case the PL-RS is not maintained by the UE, the network would not be able schedule the UE in DL for a very long time in case of joint TCI state switch. Additionally, it is currently not clear exactly what PL-RS refers to and how ‘maintained’ is defined.

For UL TCI state switch, the network is not aware of whether the PL-RS is maintained or not maintained at the UE in case the number of activated TCI states is greater than four.

- Therefore, in joint TCI state switch, if the UE can only be scheduled in DL when the UL TCI state switch is completed, the network needs to assume a very relaxed TCI state switch delay and the UE cannot be scheduled for a long period in both DL and UL.

Our understanding is that the UE can receive in DL when the DL TCI state switching is completed. Independently of the UL TCI state switch status.

- However, for the joint TCI state switch, after the DL TCI state switch is complete, the UL TCI state switch status, which the UE will for UL transmission including HARQ feedback, is unclear until the UL TCI state switch is completed.

In the last RAN4 meeting it has been discussed in the summary whether to use the old UL TCI state to send HARQ feedback until the UL TCI state is not complete, but the proposal was not agreed.

As the UE would be ready to receive in DL after the DL TCI state switch is completed such DL reception could be initiated at least THARQ before the UL TCI state is completed and UE would be able to send HARQ feedback for the received DL data. This will reduce the scheduling loss and TCI switch impact for joint TCI state changes.

For joint TCI state switch, if the UL TCI state switch delay exceeds the DL TCI state switch delay, the UE is required to receive in DL up to THARQ before it completes UL TCI state switch.

MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

In RAN4 #104 meeting, different proposals were discussed, and the following has been captured in the WF:

Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2 - Proposals - Option 1: - Longer delay is expected. - Option 2: - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

Currently, the delay is specified as follows in TS 38.133 (8.16.3):

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell, - If target TCI state is known, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms). - If target TCI state is unknown, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms. Where, - THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise. - Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown. - Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State. - Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell - Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell - T L1-RSRP is the time for Rx beam refinement in FR2, defined as - TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1, - with the assumption of M=1 - with TReport = 0 - TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2 - CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell - configured with higher layer parameter repetition set to ON - with the assumption of M=1 for periodic CSI-RS - for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam - with TReport = 0 Editor note: when PL-RS is SSB in FR2, the delay requirement is FFS.

Companies argued that the beam sweeping scaling factor shall be assumed for PL-RS measurement in FR2 when the PL-RS signal is SSB.

In Rel-16, PL-RS maintenance relies on DL and UL channel reciprocity. If DL reception is triggered using a beam selection, UL beam is selected by the channel reciprocity. In Rel-17, the unified TCI framework also allows for scenarios in which the UL TCI states are switched independently from DL TCI states. The UL TCI switch delay would depend on whether the DL RS associated with the UL TCI state is acquired by the UE when the switch occurs. The UE cannot finalize UL switch before DL timing has been acquired. Therefore, the DL path loss measurement for an UL TCI in the UE side should be aligned with network.

Regarding, the number of PL-RS samples at UL switch, it is not clear to always require 5 samples measurement. The number of samples would depend on that accurate PL-RS measurement can be acquired and is a pre-condition to make UL transmission. However, it is not clear if the 5 sample PL-RS measurement delay is always necessary for activating UL TCI. RAN4 does not have accuracy requirement for PL-RS measurement and the conditions when PL-RS is maintained, but the specification addresses 5 samples are unconditionally if PL-RS is not maintained. And this is a concern.

For the DCI-based UL TCI switching, the activation of UL TCI should be quicker than MAC-CE based switching. In DCI-based TCI switching requirements, we are not sure if the ‘active TCI list’ in the box is the same TCI list equal to ‘active TCI list’ for UL. The UE behaviors regarding the active UL TCI state should be clear to address in 8.16.4. At least the ‘active’ TCI list seems different between DL and UL cases because of PL-RS maintenance in UL.

Observation 3: Regarding ‘active UL TCI list’ we have following concerns:

(i) Network does not know which PL-RS is maintained by UE. PL-RS being maintained is not well defined and network does not know if path loss measurement is available for UL transmission in inter-cell scenarios. PL-RS maintenance is executed by UE selection.

(ii) A UE does not know which TCI(s) in the 8 active TCI states will be used for UL transmission by network scheduling. A UE also does not know which PL-RS should be maintained.

(iii) If PL-RS is not maintained among the 8 active TCI states by a UE, network experiences UL switching latency due to PL-RS 5 sample measurement delay.

Here is the insight to reduce the 5 PL-RS sample measurement period. In the specification, the sample usage boundary between L1-RSRP measurement and PL-RS measurement is not well defined. Regarding the PL-RS measurement, TS38.213 specifies as

= referenceSignalPower – higher layer filtered RSRP, where referenceSignalPower is provided by higher layers and RSRP is defined in [7, TS 38.215] for the reference serving cell and the higher layer filter configuration provided by higher layer parameter QuantityConfig is defined in [12, TS 38.331] for the reference serving cell

RAN4 just takes a baseline UL behavior assuming two different steps of pathloss measurement:

- step-(i): PL-RS measurement beam selection through L1-RSRP, and

- step-(ii) derives ‘higher layer filtered RSRP’ from PL-RS (5 samples)’.

However, if the two steps are performed on the same RS, that needs not to make two separate steps. The process measuring higher layer filtered RSRP can be combined to obtain the filtered received power.

As stated above, we find two problems.

- If a UE needs to maintain multiple PL-RSs for multiple UL TCI, which PL-RS will be maintained or not maintained and how network knows if an UL-TCI is with maintained PL-RS or not?

- There is no separate indication to make such activation of PL-RS measurement in the current specification.

Also, the number of PL-RS samples (M) will not always need to be fixed at 5 samples. If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, then we think the number of samples can be less than 5. In Rel-17, assuming PL-RS with different QCL-D cannot be measured simultaneously, a scaling factor will be applied, but the number of PL-RS under pathloss measurement should be scheduled and managed.

This issue assumes when SSB is indicated as PL-RS in UL TCI state, then TSSB is referred in this discussion. However, the PL-RS configuration is possible with both SSB and CSI-RS in case, the notation in the spec should be ‘TPL_RS’ that makes more general and generic requirements.

when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples.

-  If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

-  If a UE has reported L1-RSRP measurement on a PL-RS within a time window, the PL-RS is regarded as maintained.

Whether UE need to track time/frequency  for the UL active TCI state

In RAN4 #104 it was discussed whether the UE needs to track the time for the UL active TCI state. The discussion was captured in the following issues:

Issue1-1-1a Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with serving cell - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list Issue1-1-1b Whether UE need to track UL time/frequency for UL TCI state activation when DL-RS is associated with cell with different PCI - Proposal: - Option 1: - No, UL timing for cell with different PCI if derived from DL timing of serving cell in Rel-17 - Option 2: - Depends on whether source RS in active UL TCI state is a subset of source RS in DL active TCI list

In Rel-17, the scenario for UL m-TRP considers that the propagation delay difference between the signals from different TRPs is at most the CP length (±CP/2). This guarantees that the UE is capable of correctly receiving from two TRPs. Assumption is that these two TRPs are not located at the same physical location.

Currently, for the DL TCI state switching and time tracking, according to specification TS 38.306, the UE is required to perform time/frequency tracking for each active TCI state in the list of active TCI states. However, it is a UE capability how many TCI state the UE support in the list of active TCI states. Our understanding is that this can be between 1 and 8.

Currently, the same requirements are not defined for the UL TCI states. In Rel-16, the DL/UL relied in channel reciprocity but in Rel-17 the following situation is possible: the DL and UL TCI states are associated to different TRPs.

Figure 1 - Propagation delay difference in an intra-cell m-TRP scenario

Up to Rel-16 the DL/UL relied on channel reciprocity. In Rel-17, decoupled DL and UL is possible.

Let’s consider that the DL propagation delay difference is equal to CP, and that the UE is deriving timing from DL-RS associated to TRP 1 and the UE is transmitting to TRP 2, as in Figure 1. Currently, according to timing requirements in TS 38.133, Table 7.1.2-1 the UE shall have capability to follow the frame timing change of the reference cell in connected state and keep the transmission timing error with +-Te. The reference cell is referring to the cell in which the UE is transmitting in UL.

The Te values are a fraction of the CP length for each SCS, as in the table below, considering that the SCS of the SSB and the SCS of the uplink signals are the same:

| SCS [kHz] | CP Length [μs] | Te[μs] | Te/CP [%] |
| --- | --- | --- | --- |
| 15 | 4.7 | 0.39 | 8.3% |
| 30 | 2.3 | 0.26 | 11.3% |
| 120 | 0.59 | 0.11 | 19% |

The UL transmit timing requirements consider that the UE is tracking DL of the reference cell, where the reference cell is the cell where the UE is transmitting, but they do not mention the UE requirements related to track timing from specific TRPs of a cell. Hence, there may be situations where the UE is able to fulfill the transmit timing requirements in TRP1 but not in TRP2.

The UL transmit timing requirement is in place to ensure the network reception. If the UL transmit timing is not as expected UL transmissions from the may not be received by the gNB which of course impacts also the UE and overall performance. The UL transmission is relative to the DL reception in the cell/TRP and if the received DL frame from the cells/TRPs differs one cannot assume using DL frame from one cell/TRP as timing for UL transmission to another cell/TRP. Hence, the DL tracking has to be associated to the cell/TRP where the UE transmit as otherwise the gNB might receive UL signals from UEs using different timing references.

For DL MAC-CE and DCI switching, active TCI state list has been used to trigger TCI switching with data scheduling. If a TCI state is active in DL, the UE is required to conduct time and frequency tracking on the source RS. Such DL MAC-CE active state has been defined in 38.133. We capture below to compare DL and UL switching statements as currently captured in latest Rel-17 version of 38.133.

TS38.133 8.15.3 MAC-CE based downlink TCI state switch delay If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length. … TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise. 8.16.3 MAC-CE based uplink TCI state switch delay For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell, - If target TCI state is known, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms). - If target TCI state is unknown, - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3ms + TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms. NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise

The current specification is not clear related to ‘active’ TCI state for UL. The UE behavior is not clear regarding how a UE manages active UL TCI in a similar manner to active DL TCI. For DL case, if target TCI state is in the active TCI state list for PDSCH/PDCCH, the UE is tracking fine time and frequency synchronization on the resource RS. When DL TCI state change is indicated with a target TCI state in the active TCI state list, the UE can immediately receive and demodulate the PDCCH/PDSCH on the target TCI state based on the synchronization information - as captured below.

In 38.306, DL TCI case clearly specifies maxNumberActiveTCI-PerBWP in tci-statePDSCH, and a UE should track time and frequency sync.

tci-StatePDSCH TS38.306 - maxNumberActiveTCI-PerBWP indicates the maximum number of activated TCI-states per BWP per CC, including control and data. If a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. The UE shall include this field. Note the UE is required to track only the active TCI states.

For UL TCI and UL transmission, RAN4 has mainly discussed path-loss measurement latency. However, we note firstly that fundamental condition for active UL TCI and UL transmission, has been missing. A UE need to acquire and keep time and frequency tracking of the cell and to the source RS associated to the UL TCI for any UL transmission, like the DL reception case. A UE cannot transmit PUCCH/PUSCH/SRS independently from or without DL synchronization to the cell/TRP which the UE transmits to. Putting TA adjustment discussion aside, maxNumberActiveTCI-PerBWP under tci-StatePDSCH is about activated TCI-states per BWP per CC with UE synchronization for both DL and UL.

There is no definition of active TCI state for UL.

A UE need to acquire and keep time and frequency tracking on the DL source RS associated to the UL TCI state to be allowed to transmit in UL.

maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list. Add the time and frequency tracking condition to the active TCI state for UL.

Conclusion

In this contribution we provide our observations and proposals on remaining issues on unified TCI switching requirements.

- In DL TCI state switch delay, the UE is expected to receive as soon as the DL TCI state switch is completed.

For UL TCI state switch, the network is not aware of whether the PL-RS is maintained or not maintained at the UE in case the number of activated TCI states is greater than four.

Our understanding is that the UE can receive in DL when the DL TCI state switching is completed. Independently of the UL TCI state switch status.

- For joint TCI state switch, if the UL TCI state switch delay exceeds the DL TCI state switch delay, the UE is required to receive in DL up to THARQ before it completes UL TCI state switch.

when SSB is indicated as PL-RS in UL TCI state for FR2,

-  The number of sample M will not always be fixed as 5 samples.

-  If a UE performs both L1-RSRP measurements and PL-RS measurements on the same SSB, the number of samples used for L1-RSRP is counted for pathloss measurement.

Up to Rel-16 the DL/UL relied on channel reciprocity. In Rel-17, decoupled DL and UL is possible.

There is no definition of active TCI state for UL.

A UE need to acquire and keep time and frequency tracking on the DL source RS associated to the UL TCI state t

maxNumberActiveTCI-PerBWP under tci-StatePDSCH should be about activated TCI-states with UE synchronization for both DL and UL. The current spec addresses about DL only.

o be allowed to transmit in UL.

Rel-17 active UL TCI state should be under time and frequency tracking. This means that active UL TCI list belongs to active DL TCI state list. Add the time and frequency tracking condition to the active TCI state for UL.

- 4. References

- R4-2214481 WF on FeMIMO RRM impact for unified TCI, RAN4 #104-e

- R4-2211203, WF on FeMIMO RRM impact for unified TCI state, RAN4#103-e

- R4-2214481, WF on FeMIMO RRM impact for unified TCI, Intel

- R4-2214482, WF on FeMIMO RRM requirements for inter-cell beam management, Huawei