# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2216636

# Electronic Meeting, 10th  –19th October, 2022

Source:		Ericsson

Title:			TP for some IoT NTN RF requirement

Agenda item:		7.5.5

Document for:		Approval

# Introduction

In [1], the draft TS 36.102 is agreed and In this paper, we present some of RF requirements based on our companion discussion paper.

# TP proposal

#### 6.5.2.2	Additional Spectrum Emission Mask

6.5.2.2.1		Requirements for network signalling value "NS_xy"

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_xy" is indicated in the cell, the power of any UE emission shall be attenuated below the levels below the mean output power of transmitter specified in Table 6.5.2.2.1-1.

Table 6.5.2.2.1-1: Additional requirements for "NS_xy"

| Spectrum emission limit (dBr) / Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 1.4MHz MHz for NTN Cat-M1 and 200 kHz for NTN NB-IoT | Measurement bandwidth (MBW) |
| ± 0- (BWChannel / 2) | -25 | 4 kHz |
| ± (BWChannel / 2) – 2BWChannel | -35 |  |
| > ± 2BWChannel | -50 for PC3 -53 for PC5 |  |
| NOTE : The measurement bandwidth shall be 4kHz. |  |  |

### 6.4.1A	Frequency error for UE category M1

The NTN UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals after compensation with ideally pre-compensated doppler frequency compared with reference uplink carrier frequency.

[NOTE:	The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift. For the test case, the location of the UE is explicitly provided to the UE from the test equipment.]

Requirement will be verified for at least two cases of which one has zero Doppler conditions

## 7.3.1A	Minimum requirements (QPSK) for UE category M1

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3.1A-1/Table 7.3.1E-1B and Table 7.3.1E-2 for category 0,  Table 7.3.1E-3/Table 7.3.1E-4 for category M1.

Table 7.3.1A-1: Reference sensitivity for FDD and TDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-102.7] | FDD |
| 256 | [-102.2] | FDD |

Table 7.3.1A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-103.5] | HD-FDD |
| 256 | [-103] | HD-FDD |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1A-1/Table 7.3.1A-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1A-3.

NOTE:	Table 7.3.1A-83 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G](informative).

Table 7.3.1A-3: FDD and TDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |

# References

- R4-2215118, Draft spec for TS 36.102, MediaTek Inc