# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2216639

# Electronic Meeting, 10th  –19th October, 2022

Source:		Ericsson

Title:			RF scope discussion in Rel-18 NR Cov-Enh

Agenda item:		6.23.3

Document for:		Approval

# Introduction

The Rel-18 work on further NR coverage enhancements[1] includes study for power domain enhancements of UE transmissions:

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution, we consider RAN4 RF scope of this objective.

# Discussion

## Enhancements to reduce MPR/PAR

A wide variety of schemes to reduce PAPR for a given waveform exist [2].  They can be broadly classified into schemes that are transparent to the receiver, and those that are not.  Transparent schemes reduce the signal peaks by introducing a limited amount of distortion such that the receiver can process the signal assuming it was transmitted in an undistorted way, or they may predistort the signal to compensate for non-linearity in the PA.  Non-transparent schemes modify the signal in some way that should be accounted for in the network, such as transmitting peak reduction signals in additional frequency domain resources, using heavy frequency and/or time domain shaping that the receiver should be aware of to maintain performance, etc.

Because they do not use additional resources, one key feature of transparent MPR reduction schemes is that they can generally be accomplished by UE implementation.  Using UE implementation avoids the need to reconfigure or dynamically control the PAPR reduction scheme, allowing the UE to quickly adapt its transmit power to channel conditions. Perhaps more importantly, transparent MPR reduction can immediately improve UE PA efficiency and/or network coverage as soon as the feature is available in a UE, avoiding the need to upgrade all gNBs in a coverage area in order to benefit from MPR reduction throughout the area.

On the other hand, because non-transparent schemes have more degrees of freedom in their designs, they are more free to modify the signal.  The motivation for such signal modifications is then that there is improved MPR reduction over what is possible with a non-transparent scheme. Transparent MPR reduction schemes therefore can be seen as a baseline from which more advanced schemes can potentially build.

Transparent MPR reduction schemes allow immediate improvements in UE PA efficiency and/or network coverage, rather than waiting for the network to be upgraded to support a non-transparent scheme.

Transparent MPR reduction schemes allow flexible UE implementation, where the UE can dynamically adapt to power requirements and/or channel conditions, without intervention by the network.

Non-transparent schemes are being studied because the extra degrees of freedom in the design as compared to transparent schemes may allow for better MPR reduction.

Transparent MPR reduction schemes are baselines to which non-transparent schemes are compared

Since there are many possible transparent MPR reduction schemes, it is worthwhile to identify a few that should be studied, to allow common understanding of performance among companies.  These schemes should represent a sampling of performance gains vs. complexity tradeoffs. Three possibilities are clipping and filtering, companding, and digital predistortion.  Clipping and filtering is the simplest of the three, but will have the least gains.  Companding may be somewhat more complex, but may have better MPR.  Finally, digital predistortion is substantially more complex, but will have the best performance for MPR reduction via better compliance to unwanted emission requirements, as well as reduced EVM.  There are of course many other schemes beyond these three which can be considered, and so we identify them as candidates for further consideration in order to get the discussion of transparent schemes started,.

Candidate transparent MPR reduction schemes to consider include clipping and filtering, companding, and digital predistortion

MPR reduction schemes aim to reduce the PAPR of the signal input to the PA and thus reduce the PA backoff power while maintaining the PA linearity to the unwanted and in-band emission and signal quality performance (ACLR, SEM, Spur, IBE, EVM, Spectrum flatness). For spectrum shaping, RAN4 has specified the MPR for Pi/2 BPSK with spectrum shaping filter design with the guidance in clause 6.4.2.4 in TS 38.101-1 and TS 38.101-2. Filtering may also be used for tone reservation or baseline schemes such as clipping and filtering. The filter coefficient design is up to UE implementation but for the purpose of the aligning the MPR simulation results, some reference filter coefficients could be useful to align/calibrate the companies simulation result.

The filter coefficient could be one simulation parameter to be discussed and agreed.

Another aspect is the spectrum extension mentioned in objective, when the UE is allocated with more RBs on top of the RBs for data, these RBs could be used to allow different roll-off at the transition band. This could mean also that the IBE will not be applied for these “RBs” reserved for spectrum extension. From the spectrum utilization perspective, these RBs should be limited as it cannot be scheduled by other UE. This parameter could be discussed to be used in simulation.

Percentage and/or number of RBs used for the spectrum extension to be discussed and agreed.

Since MPR reduction schemes may or may not use spectrum extension, for the MPR scheme using the spectrum extension, it is essential that they are compared using a same amount of time-frequency resource and at a same spectral efficiency. The gain from MPR reduction scheme should be evaluated in network level by summing up the MPR reduction gain and the negative gain resulting from the coding rating loss. If there will be distortion introduced in the UE for example spectrum shaping, it is also good the check whether the SNR performance would be impacted with link simulation. Otherwise it is difficult to determine if there is a net benefit of schemes when used in a real system.

Similarly, it is important that the resources allocated by the network if spectrum extension is used are compatible with Rel-15/16/17 UEs.  Therefore, spectrum extension should not allocate resources for spectrum extension outside of the resources allocated using Rel-17 resource allocation mechanisms.

Link simulation would be needed to compare the network gain for MPR reduction with spectrum extension

Compare schemes at the link level using a same amount of time-frequency resource and at a same spectral efficiency, and assuming Rel-17 resource allocation mechanisms.

In MPR table in TS 38.101-1, there is no MPR for inner allocation for QPSK, the study then could focus on the outer RB allocation for QPSK and for higher modulation than 64QAM, EVM requirement is tougher and EVM budget on the spectrum shaping may be further limited. As one study point, the highest modulation order could be supported for improving the MPR could be further investigated.

Investigate if there are modulation scheme limitations for the MPR reduction scheme.

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |

Common MPR simulation assumption for various MPR reduction techniques could be as listed in Table 1 below

Table 1: Common MPR reduction assumptions for RF simulations

| Parameter | Value |
| --- | --- |
| Filter coefficient | TBD (may vary according to MPR reduction scheme) |
| Modulation scheme | QPSK, 16QAM, [64QAM and 256QAM] |
| Waveform | DFT-s-OFDM |
| Carrier frequency and duplex mode | 700MHz (FDD), 4GHz (TDD), 28GHz (TDD) |
| Subcarrier spacing | 700MHz: 15 kHz, 4GHz: 30 kHz, 28GHz: 120 kHz |
| System Bandwidth | 50MHz, 100MHz, 200MHz, 400MHz |
| Number of RBs and starting RB | Sweep different combination |
| Counter-IM3 | 60 dB |

Discuss the simulation assumption parameters in Tables 1.

- The link level setups from the Rel-17 NR coverage enhancement study could be used as a starting point.  The configuration for lower data rate services such as VoIP could be one appealing setup to use, as well as higher data rates and MCS states.

Remaining parameters not given by Tables 1-3 that are needed for the link level simulations can be taken from the Rel-17 NR coverage enhancement TR 38.830, appendices A.1 and A.2.

# Conclusions

In this contribution, we present our view on the remaining RF requirement overview for IoT UE with below observations and proposals:

Observation 1 Transparent MPR reduction schemes allow immediate improvements in UE PA efficiency and/or network coverage, rather than waiting for the network to be upgraded to support a non-transparent scheme.

Observation 2 Transparent MPR reduction schemes allow flexible UE implementation, where the UE can dynamically adapt to power requirements and/or channel conditions, without intervention by the network.

Observation 3 Non-transparent schemes are being studied because the extra degrees of freedom in the design as compared to transparent schemes may allow for better MPR reduction.

Proposal-1:Transparent MPR reduction schemes are baselines to which non-transparent schemes are compared.

Proposal-2:Candidate transparent MPR reduction schemes to consider include clipping and filtering, companding, and digital predistortion.

Proposal-3:The filter coefficient could be one simulation parameter to be discussed and agreed.

Proposal-4:Percentage and/or number of RBs used for the spectrum extension to be discussed and agreed.

Observation 4 Link simulation would be needed to compare the network gain for MPR reduction with spectrum extensionProposal-5:Compare schemes at the link level using a same amount of time-frequency resource and at a same spectral efficiency, and assuming Rel-17 resource allocation mechanisms.

Proposal-6:Investigate if there are modulation scheme limitations for the MPR reduction scheme.

Proposal-7:Discuss the simulation assumption parameters in Tables 1.

Proposal-8:Remaining parameters not given by Tables 1-3 that are needed for the link level simulations can be taken from the Rel-17 NR coverage enhancement TR 38.830, appendices A.1 and A.2.

# References

China Telecom, “RP-221858, Revised WID on Further NR coverage enhancements,” 3GPP TSG RAN meeting #96, Budapest, Hungary, June 6-9, 2022.

Y. Rahmatallah, S. Mohan, “Peak-To-Average Power Ratio Reduction in OFDM Systems: A Survey And Taxonomy”, IEEE Communications Surveys & Tutorials, Vol. 15, No. 4, pp. 1567-1592, Fourth Quarter 2013