3GPP TSG-RAN WG4 Meeting #104b-e              	R4- 2216700

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.7.3.1

Source:	Samsung

Title:	Discussion and simulation results for Enhanced IIOT and URLLC support

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, the performance part of Enhanced IIoT and URLLC WI was under discussion. The test scope and detail test setup of BS demodulation was captured into the WF [1] as

In this contribution, the view on the remaining issue of PUCCH sub slot repetition is provided, and the initial simulation results for PUCCH format 0 is provided.

# 2	PUCCH with sub slot repetition

In this section, the view on remaining issue about test setup for PUCCH requirement with sub-slot repetition is provided.

Test Metric

- Use Prob (DTX to ACK) < 1% - FFS: Prob (ACK miss) < 1% - FFS: Prob (PUCCH NACK to ACK bits) < 0.1%

Regarding the test metric, probability with both ACK miss and NACK to ACK bits were considered in Rel-15 PUCCH requirement definition for format 1 with payload as 2, pending on whether NACK to ACK is the bottleneck of UL requirement. As agreed, PUCCH format 0 requirement will be defined with 1 bit payload, the probability of ACK missed and NACK to ACK should be similar.

Proposal 1: RAN4 only apply the test metric with ACK missing detection for PUCCH 0 sub-slot repetition requirement.

# 3	Simulation results

In this section, the simulation results are provided, based on agreed assumption.

Table 1 simulation assumption for PF0 with sub slot repetition

| Parameter | Test |
| --- | --- |
| SCS/BW | 30KHz SCS/10MHz |
| Number of UCI information bits | 1 |
| SubslotLengthfor PUCCH | 7 |
| Number of repetition | 2 |
| Inter sub-slot hopping | Enabled |
| First PRB prior to frequency hopping | 0 |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of symbols | 2 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 5 for first sub-slot repetition 12 for second sub-slot repetition |

Table 2 Initial simulation results for PF0 with sub-slot repetition

| Number of Tx | Number of Rx | Format | Cyclic Prefix | Channel | Channel bandwidth | Metric | SNR (ideal) | SNR (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 0 | Normal | TDLC-300-100 low | 30KHz 10MHz | ACK missed detection | 0.65 | 2.65 |

# 4	Conclusion

In this contribution, the view on test setup for PUCCH requirement with sub-slot repetition.

Proposal 1: RAN4 only apply the test metric with ACK missing detection for PUCCH 0 sub-slot repetition requirement.

# References

R4-2214396, WF on Enhanced IIoT and URLLC support demodulation and CSI requirements, Nokia