3GPP TSG-RAN WG4 # 104bis-e	R4-2216705

Electronic Meeting, October 10 – October 19, 2022

Agenda item:	4.2.7.3.1

Source:	MediaTek Inc.

Title:	Discussion on PDSCH Requirements for NR-NTN

Document for:	Discussion

- Introduction

In RAN4#104-e, a way forward on NR-NTN PDSCH requirements was agreed [1]. In this contribution, we provide our views for the parameter of NTN TDL-C channel.

- Discussion

In the last meeting, RAN4 discussed whether to consider AoA of the LOS path for the NTN TDL-C channel.

Issue 2-4: AoA of the LOS Path for the NTN TDL-C Channel - Proposals: To further discuss whether to consider the additional parameters for AoA in NTN TDL-C considering 200Hz was agreed as the maximum doppler shift - Option 1: Yes, assume an AoA of Ө = 60 degree for the LOS path in NTN TDL-C channel - Option 2: No, there is no need to define AoA - Agreement: - Agree with Option 2 as starting point. Revisit if any issue is identified.

According to TS38.901 clause 7.7.2, there is a Doppler shift, fs = 0.7 fd, for the first tap in LOS channel model.

“Due to the presence of a LOS path, the first tap in TDL-D and TDL-E follows a Ricean fading distribution. For those taps the Doppler spectrum additionally contains a peak at the Doppler shift fS = 0.7 fD with an amplitude such that the resulting fading distribution has the specified K-factor.”

Therefore, we suggest RAN4 define the Doppler shift for the LOS tap in NTN TDL-C channel, considering either fs = 0.7 fd or fs = cos(Ө) * fd with some specific AoA Ө value.

Proposal 1: RAN4 define the Doppler shift for the LOS tap in NTN TDL-C channel, considering either fs = 0.7 fd or fs = cos(Ө) * fd with some specific AoA Ө value.

- Conclusion

In this contribution, we provide our views the parameter of NTN TDL-C channel and propose that:

Proposal 1: RAN4 define the Doppler shift for the LOS tap in NTN TDL-C channel, considering either fs = 0.7 fd or fs = cos(Ө) * fd with some specific AoA Ө value.

- Reference

- R4-2214386, “WF for NTN demodulation requirements - general and PDSCH”, Qualcomm