3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216766

Electronic meeting, October 10th – 19th 2022

Agenda Item:	8.2.1

Source:	Ericsson

Title:	UL Segmented Transmission for UL synchronization for IoT NTN

Document for:	Discussion

# 1	Introduction

RAN4 received an LS from RAN1related to UL segmented transmission for UL synchronization for IoT NTN [1]. The LS was discussed at last RAN4 meeting with following outcome [2]

#### Sub-topic 3-1: timing adjustment during a UL repetition period

- The following proposals are discussed in this meeting and can be further discussed.

  - Proposal 1: RAN4 shall clarify the IoT NTN behaviour in TS 36.133 for Rel-17 to allow UEs to adjust TA during the ongoing repetition.

  - Proposal 1a: The restriction on UL transmission adjustment shall be updated according to RAN1 LS on per-segment TA pre-compensation.

  - Proposal 2: RAN4 shall specify the IoT NTN behaviour in TS 36.133 to ensure the UE keeps a constant TA value within each segment

  - Proposal 3: Segmented UL transmission can be covered by NTN UE transmit timing requirements, i.e. Te_NTN. FFS whether and how to capture in RAN4

In this contribution, we further discuss and provide our updated view on the above proposals.

# 2	Discussion

Firstly, it is observed that some of the listed proposals are related and not contracting and can therefore be merged. More specifically, our understanding is that proposal 1a and 2 are similar.

Secondly, out of the listed proposal we support option 2 which is in line with the legacy LTE eMTC and NB-IoT timing requirements as quoted below [section 7.20 in TS 36.133]:

“When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.”

Both eMTC and NB-IoT UEs can be configured with large number of repetitions, especially when operating under enhanced coverage. The impact of adjusting the UE uplink timing on eNB DL reception window during an uplink repetition period was discussed extensively in LTE cat-M and NB-IoT requirements which led to the above agreement. Adjusting the TA during an uplink reception period may resulted in phase change of the transmitted signal which causes problem for the eNB to combine the repetitions. This means even if the UE meets the Te requirements, it may still cause problems for the eNB reception window. However, the UE is still allowed to adjust the uplink timing between the segments. For this reason, RAN4 specified that the UE shall note adjust the uplink transmission timing during an ongoing repetition period and our view is that this requirement shall also apply to NTN IoT for eMTC and NB-IoT.

Observation 1	The eMTC and NB-IoT UE in LTE is not allowed to adjust the uplink transmission timing autonomously during an ongoing repetition period.

Observation 2	UE autonomously changing the uplink transmission timing during an ongoing repetition period causes problems for the eNB reception window.

Based on the above observation, we make following proposal:

RAN4 shall specify the IoT NTN behaviour in TS 36.133 to ensure the UE keeps a constant TA value within each segment

- It is also noted that the RAN1 LS is only for information and does not require any action. Therefore we don’t see any need for reply LS.

# Conclusion

In this contribution, we have discussed autonomous uplink timing adjustment during an UL repetition period. Based on the discussions, we make following observations and proposals:

Observation 1	The eMTC and NB-IoT UE in LTE is not allowed to adjust the uplink transmission timing autonomously during an ongoing repetition period.

Observation 2	UE autonomously changing the uplink transmission timing during an ongoing repetition period causes problems for the eNB reception window.

- RAN4 shall specify the IoT NTN behaviour in TS 36.133 to ensure the UE keeps a constant TA value within each segment

# References

R1-2205642, “LS on UL Segmented Transmission for UL synchronization for IoT NTN, RAN1

R4-2214353, “WF on UL Segmented Transmission for UL synchronization for IoT NTN”, MediaTek inc.