3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2216775

Electronic Meeting, 10 – 19 October 2022

Source: 	Huawei, HiSilicon

Title:	Initial consideration on PC1.5 TDD UE RF requirements

Agenda Item:	5.21.2

Document for:	Agreement

# 1	Introduction

The core objectives of the WI are to develop RF requirements that are applicable to PC1.5 UE mobile device and FWA for NR TDD bands.

- Introduction of high power UE (power class 1.5) operation for NR TDD bands

  - Including bands n34, n39, n40 as in table 1. Other bands based on operators request.

- Specify RF characteristics with dual-PA assumption, including following requirements if needed

  - UE maximum output power

  - Tx power tolerance

  - A-MPR, if needed

- Reuse existing SAR mechanism

A new WID [1] on adding PC1.5 support to more TDD bands was established in RAN#97-e. The objectives on core RF requirements are duplicated below:

In the following, we share some initial analysis on the UE RF requirements.

# 2	Discussion

The general UE RF requirements such as ACLR, MPR, etc for PC1.5 are complete in Rel-17 specs for both mobile and FWA devices. Hence only band-specific RF requirements are to be defined for the bands proposed in this WI [1].

The main RF requirement to be updated appears to be the A-MPR for NS_50 for band n39, while band n34 and n40 do not have A-MPR requirements.

For NS_50, the A-MPR requirements have been specified for both PC3 and PC2 in TS 38.101-1 [2]. It’s likely that new A-MPR for PC1.5 will be needed. When comparing the PC1.5 MPR for mobile devices and FWA, the difference is small (i.e. <=1dB). It seems unnecessary to have separate A-MPR evaluations/requirements for the two types of devices.

Proposal 1: Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices.

# 3	Conclusion

In this paper, we have shared our initial analysis on the newly requested PC1.5 TDD bands. And we propose the following as the way forward.

Proposal 1: Evaluate the PC1.5 A-MPR requirements for NS_50 for band n39, using the Rel-17 RF assumptions for mobile devices.

# References

- RP-222351 New WID on High power for FR1 TDD single bands with power class 1.5 UE, China Mobile, RAN#97-e

- 3GPP TS 38.101-1 v17.7.0