3GPP TSG-RAN WG4 Meeting #104bis-e	R4- 2216777

Electronic Meeting, 10 – 19 October 2022

Source: 	Huawei, HiSilicon

Title:	Further discussion on the feasibility of signalling for low MSD

Agenda Item:	6.6.4.2

Document for:	Agreement

# 1	Introduction

A WF [1] on study for lower MSD was agreed in RAN4#104-e for the WI [2].  Regarding the signalling design, five aspects are listed [1], which are duplicated below. In the sequel, we share our views on these issues based on the findings in our companion paper on the feasibility of MSD improvement [3].

## Necessity of signaling improved lower MSD

- Companies can continue discussion on the signaling based MSD improvement method. Companies interested in the direction of necessity of signaling can provide more analysis in next meeting.

## Clarification of the expected behaviour from NW for the potential lower MSD capability

- Basic assumption is that how to schedule UEs with different MSD capabilities is up to NW implementation

- How the lower MSD capability can be utilized by the NW can be further discussed in next meeting

## If lower MSD capability is introduced, should it be an optional capability?

If lower MSD capability is introduced, it should be an optional capability.

## How to indicate the MSD is improved for a band combination

The following aspects related to lower MSD signaling will be further discussed in next meeting.

- Whether the lower MSD capability is a per BC capability

  - how to handle a band combination with different MSD types

  - how to handle the same BC with different victim bands suffered the same MSD type and order

- Whether lower MSD capability for different interference sources could be reported separately

- Whether lower MSD capability means all MSD types for a band combination have been improved

- Whether delta MSD compared to the minimum requirements or directly improved MSD values to be reported

- Whether a single/unique MSD value or MSD threshold(s) for a band combination to be considered

  - If MSD threshold(s), whether a single MSD threshold value or could be multiple intervals? Exact absolute threshold(s) or relative threshold(s)? Different or same threshold(s) for different interference type?

- Relation between MSD reduction and UL power back-off

- Applicability of lower MSD capability for different power class

- Signaling overhead for the lower MSD capability

## Lower MSD capability definition

Lower MSD capability definition is considered, once the group has a clear view on the lower MSD capability.

# 2	Discussion

The MSD specified by 3GPP is an indication of the level of self-interference (caused by harmonic interference, harmonic mixing, cross-band isolation and dual-UL IMD) relative to the reference thermal noise under specific conditions. If those conditions are not met, e.g. a different set of carrier frequencies for the band combination are used, a UE may not suffer from any MSD at all. On the other hand, a UE may optimize the performance of various RF components [3] or employ interference cancellation technique in the baseband, so that the effective level of self-interference can be reduced for some or even all band combinations.

Proposal 1: Low MSD may be defined as an optional UE capability which is used to indicate that the effective level of self-interference at the UE is lower than that specified by 3GPP minimum requirements.

If the low MSD capability is not signaled, the network can simply assume that the UE complies with the minimum requirements by 3GPP. For band combinations that have large MSD values specified by 3GPP, small to moderate improvement that results in large remaining MSD (e.g. >15dB) will probably not change the network’s scheduling decisions.

Proposal 2: Large MSD values (>[15] dB) are not reported. The exact upper bound for reporting can be FFS.

Ideally, the MSD should be improved to 0 or some close value (e.g. <=5 dB). For many band combinations that have MSD<10 dB specified by 3GPP, this is possible by utilizing the techniques discussed in [3]. The network scheduler may treat such UEs as if they’re not affected by self-interference.

Proposal 3: Define MSD <= [5] dB as low MSD. The exact low MSD threshold can be FFS.

After MSD improvement, if the remaining MSD is still between the low MSD threshold and the low MSD upper bound (e.g. 5dB<MSD<=15 dB), it’s not clear how the network scheduler can treat such UEs differently from those exhibiting even large MSD.

Proposal 4: How to report the MSD values between the low MSD threshold and the low MSD upper bound is FFS. One or more intervals may be used depending on the need of the network.

Since version 17.6.0 of TS 38.101-1, up to two test points may be defined for each band combination per source of MSD according to the WF [4]. Some examples are shown in Table 1 below.

Table 1: Example MSD test points for harmonic interference

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | n28 | 5 | 15 | 25 (RBstart=0) | 5 | 37.8 | NOTE 4 | UL1/DL3 |
| n40 | n28 | 20 | 15 | 100 (RBstart=0) | 20 | 30.3 | NOTE 4 | UL1/DL3 |

Since the 1st test point (TP#1) is mandatory and the 2nd test point (TP#2) is optional [4], it seems more reasonable to define the low MSD capability based on TP#1. And the network scheduler needs to extrapolate the MSD value for the actual DL/UL BW in use.

Proposal 5: Define and evaluate the low MSD capability based on the 1st test point for a band combination in the 3GPP spec. How to extrapolate the MSD for the actual DL/UL BW in use is FFS.

For MSD caused by dual-UL IMD, the same victim band may suffer from IMDs of different orders. As shown in Table 2 below, band n3 of CA_n3-n78 has different MSD values specified for IMD2, IMD4 and IMD7. It has been illustrated in [3] that these IMD interferences do not occur simultaneously. It’s reasonable to report the MSD for all IMD orders, and let the network decide which one matters for the current configuration of the carrier frequencies on each band.

Furthermore, for a given band combination there may be different victim bands for the same IMD order. As shown in Table 2, band n78 may suffer from IMD2 of band n1 and n3, and band n3 may suffer from IMD2 of band n1 and n78.

Table 2: Example MSD test points for dual-UL IMD

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3 | n1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | n3 | 1760 | 5 | 25 | 1855 | N/A | TDD | N/A |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| CA_n3-n78 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  |  |  |  |  |  | 28.75 |  |  |
|  | n78 | 3575 | 10 | 25 | 3575 | N/A | TDD | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n78 | 3435 | 10 | 25 | 3435 | N/A | TDD | N/A |
|  | n3 | N/A | 5 | N/A | 1877.5 | 2.2 | FDD | IMD7 |
|  | n78 | 3305 3780 | 10 10 | 1 (RBstart=3) 1 (RBstart=0) | 3305 3780 | N/A | TDD | N/A |

Table 7.3A.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n78 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1750 | 5 | 25 | 1845 | N/A |  | N/A |
|  | n78 | 3700 | 10 | 52 | 3700 | 28.4 | TDD | IMD2 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1770 | 5 | 25 | 1865 | N/A |  | N/A |
|  | n78 | 3360 | 10 | 52 | 3360 | 11.2 | TDD | IMD4 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | 27.9 |  | IMD2 |
|  | n78 | 3780 | 10 | 52 | 3780 | N/A | TDD | N/A |

Proposal 6: When reporting low MSD capability for a given band combination, include the information about the victim band, MSD type (ULn/DLm n=2,3,4,5, m=1,2,3,5, cross-band ISO, or IMDn, n=2,…,7).

For high order band combinations, the MSD requirements are already specified by the lower order fallbacks. There’s no need to report the low MSD capability separately.

Proposal 7: For a band combination of more than 3 bands DL, the low MSD capability is derived based on that of the 2/3 bands DL fallbacks.

Based on the configuration of carrier frequencies for a given deployment, the network can decide whether the UEs in the current cell will suffer from MSD or not. And a given cell may only activate a limited number of band combinations. Hence, it’s unnecessary for a UE to report the low MSD capability for all band combinations that it supports.

Proposal 8: The low MSD capability may be reported by the UE upon network query. The query can be filtered by the set of band combinations, the victim band, or the MSD type.

To improve the signaling efficiency, a single bit low MSD indicator may be used. For a given band combination, this indicator may be set as long as low MSD is achieved for one of the MSD types on one of the victim bands. Upon receiving this indicator, the network can decide whether to further enquire the UE for more details.

Proposal 9: Consider the use of a single-bit low MSD indicator to be reported together with the supported band combinations, which is to assist the network to decide whether to further enquire the low MSD capability.

# 3	Conclusion

Following the feasibility study of MSD improvement in [3], we share our considerations on the potential signaling design. Regarding the issues summarized in the WF [1], the following proposals are made:

Proposal 1: Low MSD may be defined as an optional UE capability which is used to indicate that the effective level of self-interference at the UE is lower than that specified by 3GPP minimum requirements.

Proposal 2: Large MSD values (>[15] dB) are not reported. The exact upper bound for reporting can be FFS.

Proposal 3: Define MSD <= [5] dB as low MSD. The exact low MSD threshold can be FFS.

Proposal 4: How to report the MSD values between the low MSD threshold and the MSD upper bound for signaling is FFS. One or more intervals may be used depending on the need of the network.

Proposal 5: Define and evaluate the low MSD capability based on the 1st test point (TP#1) for a band combination in the 3GPP spec. How to extrapolate the MSD for the actual DL/UL BW in use is FFS.

Proposal 6: When reporting low MSD capability for a given band combination, include the information about the victim band and MSD type (ULn/DLm n=2,3,4,5, m=1,2,3,5, cross-band ISO, or IMDn, n=1,…,7).

Proposal 7: For a band combination consisting of more than 3 bands DL, the low MSD capability is derived based on that of the 2/3 bands DL fallbacks.

Proposal 8: The low MSD capability may be reported by the UE upon network query. The query can be filtered by the set of band combinations, the victim band, or the MSD type.

Proposal 9: Consider the use of a single-bit low MSD indicator, which is reported together with the supported band combinations.

# References

- R4-2214452 WF on study for lower MSD, Huawei, HiSilicon, RAN4#104-e

- RP-220987 New WID Further RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, RAN#95

- R4-2216776 Further discussion on the feasibility of improving MSD, Huawei, HiSilicon, RAN4#104bis-e

- R4-2210565 WF on criteria on Rel-17 enhanced MSD table format, Skyworks, RAN4#103-e