3GPP TSG-RAN WG4 Meeting # 104-Bis-e 	R4-2216788

# Electronic Meeting, Oct. 2022

Source: Qualcomm Incorporated

Title: 	 On UE RF coverage enhancements for Rel-18

Agenda item:	6.23.3

Document for:	Discussion

# Introduction

The objectives of the ‘Further NR coverage enhancements’ WI [1], filtered by relevance to UE RF requirements are twofold:

Study and if necessary specify following power domain enhancements

- Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1)

- Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

In this contribution we offer our views on the scope of the work for the second item. The objectives for the core part of the WI [1] are reproduced in the annex for convenient reference.

# Discussion

There are two broad classes of waveform enhancements to effect MPR/PAR reduction:

- Transparent waveforms: The gNB does not need exact knowledge what technique the UE uses, and it does not need any special consideration, for example to its scheduler. This class can be developed independently by RAN4. An example of this class of waveforms is DFT-s-pi/2 BPSK.

- Non-transparent waveforms: This class comprises new waveforms or techniques defined in RAN1.  These require deliberation and justification in RAN1 before RAN4 can begin any work.

Prior to any guidance from RAN1 on new waveforms or techniques, it is sufficient for RAN4 to focus on transparent  techniques in RAN4.

Proposal 1: RAN4 to focus on transparent waveform enhancements separately from any future support work for RAN1 to evaluate new waveforms or techniques (non-transparent enhancements).

In the context of transparent waveform enhancements for coverage limited cases, it can be assumed that the network has already exercised all its options, including scheduling waveforms with no MPR allowance (‘0 MPR waveforms’). Here we note that the FR1 UE has virtually no room for self-enhancement because of proximity between the UE’s actual UL power and the upper limit on transmitted power for the power class itself, PCMAX_H. A meaningful increase in UL coverage is only possible if this impediment to power increase is removed or loosened.

For FR2, the upper limits on the measured EIRP and measured TRP are driven by regulatory limits and generally have room for a UE to self-enhance.

For the MPR reduction sub-objective, we note that MPR is defined as a back off that a UE can optionally implement to ensure transmit signal quality or emissions compliance. i.e. the UE is not precluded from self-enhancement for waveforms that have non-zero MPR. There is however a limit to the self enhancement possible, which is the same PCMAX_H identified as the upper limit for 0 MPR waveforms. The MPR/PAR reduction objective can therefore focus on increasing PCMAX_H for coverage enhancement.

Proposal 2: RAN4 to focus on enhancing UL power for 0 MPR waveforms for FR1 for the MPR/PAR reduction objective of the WI.

# Conclusions

Proposal 1: RAN4 to focus on transparent waveform enhancements separately from any future support work for RAN1 to evaluate new waveforms or techniques (non-transparent enhancements).

Proposal 2: RAN4 to focus on enhancing UL power for 0 MPR waveforms for FR1 for the MPR/PAR reduction objective of the WI.

# Reference

[1]	RP-221858, ‘Revised WID on Further NR coverage enhancements’, China Telecom, RAN#96, Jun. 2022

# Annex

The objective part of the WID is partially reproduced here for convenience: