3GPP TSG-RAN WG4 Meeting # 104bis-e 	R4-2216799

# Online, 10th – 19th October, 2022

Title:	Remaining issues for system parameters for IoT NTN

Agenda Item:	7.5.2

Source:	MediaTek Inc.

Document for:	Discussion

# Introduction

This document discusses the remaining issues for system parameters for IoT NTN.

# Discussion

## 2.1	Remaining open issues from [1]

Issue 1-3:    Channel spacing

- Agreement:

- Reuse TN specification 36.101/36.104 nominal channel spacing for Cat-M1 and NB-IoT

  - Further discuss the frequency gap for NB-IoT over NTN deployment

## Issue 1-4-2:    M_DL for standalone NB-IoT

Further discussion below options in next meeting:

- Option 1: To follow the same MDL values as TN FDD as baseline

- Option 2: Update the M_DL value to improve spectral efficiency (R4-2213243)

- Option 3: Others are not precluded.

Issue 1-4-3:    EARFCN:

Further discussion in next meeting:

- Start from the highest EARFCN  262143 value and descending manner for B256 and B255

## 2.2	Channel spacing

It seems clear from TS36.104 that the Foffset is only applicable for the first and last NB-IoT carrier operated by the NB-IoT BS, and is not a general requirement on NB-IoT operation.

In TS36.101, while the GSM ACLR requirements only apply with a 100kHz additional gap from the NB-IoT channel edge, the UTRA ACLR requirements apply from the channel edge. Therefore, we believe that the same approach to the applicability of the guardband could apply for NB-IoT over NTN at least from the UE side, unless any issue is identified with coexistence. However, we welcome further feedback from stakeholders on the network deployment side regarding how to handle the Foffset.

Proposal 1: Existing principles for guardband on the UE side should be used at least as the baseline starting point. However, we welcome further feedback from stakeholders on the network deployment side regarding how to handle the Foffset.

## 2.2	Applicable M_DL values

Given that we have agreed to use a 100kHz channel raster for the first 2 frequency bands, we believe that the benefit of applying new M-DL values for non-anchor carriers is even smaller than when discussed at the last meeting. We also believe that this issue is not specific for NTN operation and any unnecessary divergence of NTN operation from NB-IoT TN operation is undesirable.

Proposal 2: Reuse the existing M-DL values defined for FDD (Option 1) as part of this work item also for non-anchor carriers.

## 2.2	EARFCN numbering

Having analysed the proposal at RAN4#104-e on starting the channel numbering from the maximum value (262143) for each band and identifying that this is compatible with existing RRC specification, we support adopting this approach. However, we believe that it makes sense to start with the highest EARFCN at the upper edge of each band and decrementing from there.

We also believe that for future-proofness we should define <1> and <2> as the decrementing framework, for 100kHz and 200kHz raster respectively, similar to NR.

Proposal 3: Adopt the channel numbering approach starting from value 262143 and decrementing, starting from the upper edge for each band. Also adopt a similar framework to 38.101-1 in order to allow either a 100kHz or 200kHz EARFCN channel raster step size for future bands.

# Proposal

Proposal 1: Existing principles for guardband on the UE side should be used at least as the baseline starting point. However, we welcome further feedback from stakeholders on the network deployment side regarding how to handle the Foffset.

Proposal 2: Reuse the existing M-DL values defined for FDD (Option 1) as part of this work item also for non-anchor carriers.

Proposal 3: Adopt the channel numbering approach starting from value 262143 and decrementing, starting from the upper edge for each band. Also adopt a similar framework to 38.101-1 in order to allow either a 100kHz or 200kHz EARFCN channel raster step size for future bands.

# References

[1] R4-2214380, WF on system parameter for IoT over NTN, Ericsson