3GPP TSG RAN WG4 Meeting #104bis-e	R4-2216855

Electronic Meeting, 10-19 October 2022

Agenda Item:	4.6.3.2.1

Source: 	Ericsson

Title:	On offset for cell specific RSRP thresholds for 1Rx Redcap UE

Document for:	Discussion

- Introduction

In the last RAN4 meeting it was agreed that 1Rx Redcap UE will apply a fixed offset value (1 dB magnitude) to cell-specific RSRP thresholds used in procedures in RRC IDLE and RRC INACTIVE states as captured in the LS replied to RAN2 [1]. Such cell-specific RSRP thresholds were also identified in the same reply LS [1]. Furthermore, it was agreed to specify the application of the offset to these cell-specific RSRP thresholds in TS 38.133.

In this paper we analyze how the inclusion of the offset to these cell-specific RSRP thresholds can be specified in TS 36.133.

# Analysis of offset for cell selection and reselection

It was agreed to apply the offset to the following thresholds used for cell selection and cell reselection thresholds and ranking procedures:

- Qrxlevmin and Qqualmin

The UE uses Srxlev and Squal for cell selection and cell reselection procedures. According to clause 5.2.3.2, TS 38.304, Srxlev and Squal are function of Qrxlevmin and Qqualmin respectively:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset)– Pcompensation Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

The 1Rx Redcap UE due to single receiver branch will experience smaller cell size/coverage compared to the 2Rx Redcap UE. The network will signal the values of Qrxlevmin and Qqualmin suitable for the legacy UEs i.e. 2Rx UE. Therefore, the 1Rx RedCap UE may lose the cell coverage or it will be harder to access the cell compared to 2Rx RedCap UE. Therefore, to enable 1Rx Redcap UE to achieve comparable cell selection and reselection performance compared with 2Rx Redcap UE, we suggest that the 1Rx Redcap UE applies negative offset (-1 dB) to the signaled values of Qrxlevmin and Qqualmin.

# Analysis of offset for other procedures

The following cell specific RSRP thresholds are also agreed for which the offset of 1 dB will be applied by the 1Rx RedCap UE in RRC IDLE and RRC INACTIVE states [1]:

- rsrp-ThresholdSSB,

- msgA-RSRP-ThresholdSSB,

- msgA-RSRP-Threshold,

- absThreshSS-BlocksConsolidation,

- sdt-RSRP-Threshold.

- Cell-specific RSRP thresholds used for Rel-16 low mobility and/or not at cell edge conditions, and Rel-17 stationary and not at cell edge conditions for RRC IDLE/INACTIVE state:

  - s-SearchDeltaP-r16,

  - s-SearchDeltaP-Stationary-r17

  - s-SearchThresholdP-r16,

  - s-SearchThresholdQ-r16,

  - s-SearchThresholdP2-r17,

  - s-SearchThresholdQ2-r17.

These are all absolute RSRP thresholds. In these procedures the UE compares the measured RSRP (or value derived based on the measurement RSRP) with the RSRP threshold in a cell to decide whether to perform that procedure or not in that cell. For example, the UE selects 2-step RACH if the SSB based measured RSRP is above msgA-RSRP-Threshold. Otherwise, the UE selects 4-step RACH. Similarly, if the RSRP is higher than sdt-RSRP-Threshold, then the UE initiates the SDT procedure. Otherwise, the UE does not perform the SDT procedure.

The network will signal the values of the cell specific RSRP thresholds which are suitable for the legacy UEs i.e. 2Rx UE. But the RSRP measured by the 1Rx RedCap UE has worse accuracy and therefore will have larger variation compared to the RSRP measured by 2Rx 1Rx RedCap UE. As analzed in previous contribution in [2], the 1Rx UE performing any of the above procedure based on the RSRP threshold signaled for 2Rx UE will lead to unnecessary execution of that procedure due to the overestimation of the RSRP, which is cause due to larger inaccuracy. Therefore, we suggest that the 1Rx RedCap UE applies positive offset value to the signaled value of the RSRP threshold.

# Specification of offset in TS 38.133

It was agreed that that application of offset to the agreed cell specific thresholds for 1Rx RedCap UE will be defined in TS 38.133. One option is to specify this in each relevant section of the specification (TS 38.133) which contains the requirements for the procedure where the corresponding cell specific threshold(s) are used by the UE. Another option is to specify this in one place in the specification in TS 38.133. In our view, the first option is clearer and easy to follow. We therefore prefer to specify the offset in each of the relevant sections of the specification.

# Summary

The following are the proposals related to the application of the offset value (1 dB) by 1Rx RedCap UE to cell-specific RSRP thresholds used in procedures in RRC IDLE and RRC INACTIVE states:

Offset for cell selection and reselection:

- Proposal #1: The 1Rx RedCap UE shall apply -1 dB offset to the signaled value of Qrxlevmin and Qqualmin for cell selection and cell resection procedures:

Offset for other procedures:

- Proposal #2: The 1Rx RedCap UE shall apply +1 dB offset to the signaled value of the following cell-specific RSRP thresholds for executing the corresponding procedures:

  - rsrp-ThresholdSSB,

  - msgA-RSRP-ThresholdSSB,

  - msgA-RSRP-Threshold,

  - absThreshSS-BlocksConsolidation,

  - sdt-RSRP-Threshold.

  - s-SearchDeltaP-r16,

  - s-SearchDeltaP-Stationary-r17

  - s-SearchThresholdP-r16,

  - s-SearchThresholdQ-r16,

  - s-SearchThresholdP2-r17,

  - s-SearchThresholdQ2-r17.

Specification of offset in TS 38.133:

- Proposal #3: The application of the offset to the signaled cell-specific RSRP threshold is specified under each relevant requirement in TS 38.133 defined for the procedure using the the corresponding cell specific threshold(s).

Rel-17 Draft CR to defined the offset to the signaled cell-specific RSRP threshold is provided in [3].

- References

- R4-2214484, “Reply LS on configuring margin for 1 Rx RedCap UEs”, Ericsson.

- R4-2205629, Discussions on RedCap mobility requirements, Ericsson

- R4-2216856, “Draft CR on offset for cell specific RSRP thresholds for 1Rx Redcap UE in 38.133”, Ericsson.