3GPP TSG-RAN WG4 Meeting # 104bis-e					R4-2216868

Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	4.2.6.1

Source: 		Qualcomm Incorporated

Title: 			Open Issues in NTN RRM Test Case Design

Document for:	Discussion

# 1.	Introduction

We present our views on the following four open issues for NTN RRM test cases.

- Numerology

- Acquisition of UE Position

- Reference Propagator Model

- Reference Time Instances for UL Timing Accuracy Requirements

# 2. 	Discussion

Numerology

- Issue 2-3: Test coverage regarding numerology and duplex mode. Agreement - Test configurations for TDD and numerology higher than 15kHz are not considered. - FFS on whether to define UE timing TC for 30KHz SCS. - Test configuration includes both 15KHz SCS and 30KHz SCS cases in the same TC, no separate TC is defined for 30KHz SCS case.

In the current RRM test case configurations, there is neither FDD nor 10/20MHz of channel bandwidth for 30kHz SCS. We do not see a need to introduce a new set of RRC test configurations for 30kHz SCS.

Proposal 1: RAN4 to not define RRM test cases for 30kHz SCS.

Acquisition of UE Position

- Issue 5-2: GNSS signal for NTN UE timing related test cases. Open issues: - Option 1: the test parameter for GNSS signal power levels defined in B.4.1 is reused in NTN test cases. - Option 2: use AT command approach to acquire UE location - AT command approach: Use existing defined AT command: “Update UE Location Information”, defined in TS 38.509 to provide the UE with location coordinates, as used in some existing 3GPP test cases today.

We agree the following aspects pointed out in the last RAN4 meeting:

- It combines a demod test case and an RF/RRM test case, so more complexity.

- The GNSS signal power levels would need to be decided appropriately by RAN4 to acquire the satellite with sufficient accuracy within a certain time.

- Acquisition of the GNSS location would take some time so increase test time.

If NTN UE anyway has to meet those requirements defined in TS 38.171, hence, a separate set of verification tests for that according to TS 38.171, we do not think it needs to be repeated in RRM tests again.

As for the exact value of UE position in the RRM tests, it should be defined in such a way that the smallest elevation angle between the UE and satellite(s) is not smaller than 45 deg. because otherwise an achievable SNR would be too much limited based on the evaluation results presented in the following contribution.

- R4-2119586, “General and RRM requirements impacts,” Qualcomm Incorporated, RAN4 #101-e, Nov. 2021.

Proposal 2: For UE position acquisition in NTN tests, RAN4 to adopt “AT command” based approach. The exact UE position should be defined in such a way that the smallest elevation angle between the UE and satellite(s) is not smaller than 45 deg.

Reference Propagator Model

- Issue 1-1: Margin assumption for evaluating measurement accuracy. Agreement - RAN4 considers propagator model error and timing/frequency error when defining the measurement accuracy. - FFS on the values of propagator model error and timing/frequency error - FFS on the definition of propagator model error - Issue 5-3: Reference time for NTN UE timing related test cases. Agreement - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position. - FFS the following aspects: - For the transmission timing/Doppler shift adjustment and UL reception timing, RAN4 defines a reference propagator model. - The reference propagator model shall be defined in such a way that those UEs using more accurate propagator model than the reference model are not penalized. - The reference model can be determined based on companies’ input. Eckstein Hechler based propagator model can be one of the candidate models.

As there have been concerns from companies on the complexity of sophisticated propagator models to be specified in RRM spec, we have compared the satellite positioning performances based on multiple different models and present one of the simplest one among many. In Figure 1, the performances in terms of satellite position error and velocity error are presented when the estimation is based on a basic Kepler propagation equation, which should be found feasible for most of the device types perhaps even including IoT NTN.

Figure 1. Satellite positioning performances in terms of satellite position error and velocity error when the satellite is on a circular orbit at 600km altitude.

As can be seen from Figure 1, the position error is 3.4m and 6.7m at 5s and 10s, respectively, and the velocity estimation error is 2.6e-5 5 km/s and 5e-5 km/s at 5s and 10s, respectively. Note that SIB-19, which carries satellite ephemeris information, should be broadcasted every 10.24 s (1024 radio frames) because the epoch-time of the ephemeris information is defined with respect to SFN.

epochTime Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris in IE ephemerisInfo and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration. If this field is absent in ntn-Config provided via NTN-NeighCellConfig the UE uses epoch time from the serving satellite ephemeris. In case of handover, this field is based on the timing of the target cell, i.e. the SFN and sub-frame number indicated in this field refers to the SFN and sub-frame of the target cell. This field is excluded when determining changes in system information, i.e. changes of epochTime should neither result in system information change notifications nor in a modification of valueTag in SIB1.

If we compare the Kepler based achievable satellite positioning error (at 10s from the epoch time) to the maximum satellite positioning error assumed for UL transmission timing requirements (shown in Table 1), a basic Kepler equation based satellite positioning method still leave 23m margin, which is almost 77% of the maximum error.

Table 1. UL Transmission Timing Requirements for TN and NTN UEs

Observation 1: A basic Kepler equation-based LEO satellite position estimation can achieve an accuracy of 6.7m error, which is 22.3% of the assumption considered for UE UL transmission timing requirements, when the satellite is on a circular orbit at 600km altitude, and a frequency of ephemeris information update is assumed once every 10.24s. And the estimation error of the satellite velocity is insignificant.

Proposal 3: RAN4 to adopt a Keplerian propagator model as a reference satellite positioning model in NTN RRM test cases. And RAN4 to not consider additional error margin in terms of timing and frequency with the following assumptions:

- The LEO satellite is assumed to be on a circular orbit

- SIB19 is broadcasted once every 10.24s

- During the test, an elevation angle between satellite(s) and UE is not smaller than 45 deg.

Reference Time Instances for UL Timing Accuracy Requirements

- Issue 5-3: Reference time for NTN UE timing related test cases. Agreement - For the test requirement, the reference time should be (NTA + NTA_offset + NTA,common + NTA,UE-specific) ×Tc ± Te_NTN - FFS NTA,UE-specific and NTA,common used by test system do not include UE GNSS estimation error and satellite positioning error from UE calculation. - In test cases for NGSO, a test equipment shall adjust the downlink transmission frame boundary/Doppler shift and UL reception timing according to open-loop TA control related parameters defined in the NTN specific system information and the satellite constellation with respect to UE position.

It was agreed in the last RAN4 meeting that TE shall adjust the downlink transmission frame boundary and UL reception timing. For that, time instances for DL and UL shall be precisely defined in test case descriptions. As presented in our contributions submitted in the past RAN4 meetings (R4-2203856, R4-2207960), NTN propagation delays over forward (DL) and reverse (UL) links are not symmetric (See S3/F3 and S4/F4 in Fig. 2 and Fig. 3) for the same slot index.

Fig 2. Reference system model of timing relation between UE and UL timing synchronization reference point in NTN

Fig 3. Timing Relation between UE DL Reception and UE UL Transmission

Proposal 4: In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted.

- for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

- for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

# 3.	Conclusion

We presented our views on the following four open issues.

Numerology

Proposal 1: RAN4 to not define RRM test cases for 30kHz SCS.

Acquisition of UE Position

Proposal 2: For UE position acquisition in NTN tests, RAN4 to adopt “AT command” based approach. The exact UE position should be defined in such a way that the smallest elevation angle between the UE and satellite(s) is not smaller than 45 deg.

Reference Propagator Model

Observation 1: A basic Kepler equation-based LEO satellite position estimation can achieve an accuracy of 6.7m error, which is 22.3% of the assumption considered for UE UL transmission timing requirements, when the satellite is on a circular orbit at 600km altitude, and a frequency of ephemeris information update is assumed once every 10.24s. And the estimation error of the satellite’s velocity is insignificant.

Proposal 3: RAN4 to adopt a Keplerian propagator model as a reference satellite positioning model in NTN RRM test cases. And RAN4 to not consider additional error margin in terms of timing and frequency with the following assumptions:

- The LEO satellite is assumed to be on a circular orbit

- SIB19 is broadcasted once every 10.24s

- During the test, an elevation angle between the satellite and UE is not smaller than 45 deg.

Reference Time Instances for UL Timing Accuracy Requirements

Proposal 4: In RRM test cases, when a test equipment adjusts downlink transmission frame boundary/Doppler shift and UL reception timing, asymmetric propagation delays on DL and UL for the same slot index shall be taken into account. To model the round trip delay over service link (N_{TA,UE-specific}), the following definitions of reference slot for S3 and S4 (based in Fig. 3) are adopted.

- for S3, the slot when the UL transmission is supposed to arrive at the target satellite based on provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

- for S4, the slot when the DL transmission corresponding to the reference timing of downlink is supposed to arrive at the target satellite based on actual received time of the slot and provided valid ephemeris information (no error in the provided ephemeris information will account for UE error) and a reference propagator model

Fig 3. Timing Relation between UE DL Reception and UE UL Transmission