3GPP TSG-RAN WG4 Meeting RAN4#104-bis-e     				R4-2216879 Online

October 10 – 19, 2022

Agenda Item:	6.6.2

Source:	Lenovo, Motorola Mobility

Title:	EVM Definition for 4x4 UL MIMO

Document for:  Approval

- Introduction

A new work item on furthers RF requirements enhancement for NR frequency range 1 was agreed in [1] and includes the following objectives:

Specify the UE RF requirements to support 4Tx

  - First priority: 4x4 UL MIMO

  - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port

In this contribution, we consider how the EVM requirements are applied for 4x4 UL MIMO.

- EVM for 4x4 UL MIMO

In TS 38.101-1 [2], the EVM for closed loop spatial multiplexing is defined in 6.4D.2.1 as follows:

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in clause 6.4.2.1 apply per layer.

However, the details of how the per-layer EVM should be measured have not been specified.

For consistency with the 2 Tx case, the EVM requirement for 4x4 UL MIMO can be defined on a per layer basis also.  Alternatively, since it has been agreed that the precoding matrix for 4x4 UL is

W=121000010000100001 ,

there is the possibility that EVM could be defined per connector since there is a one-to-one mapping between layers and antenna connectors for this case.  However, a per-connector EVM definition will not work in the case of one-, two-, or three-layer transmission from four antenna ports since each layer will be transmitted from a combination of antennas if full power is to be achieved.

If a per-layer EVM definition is used for 4 Tx, then a MIMO receiver must be defined to separate the layers before EVM is measured.  For the two transmit antenna case, it has been proposed previously in that either the linear zero-forcing MIMO receiver or the linear unbiased MMSE MIMO receiver be used [3].  In [4], the TR 38.884 study on enhanced test methods for FR2 NR UEs, the EVM for rank-2 transmission is measured using the zero-forcing receiver

GZF=H-1

where H is the 2x2 effective channel matrix given by H=HP, H is the 2x2 channel matrix, and P is the 2x2 precoding matrix.

There is a fundamental difference between the EVM measurement for FR1 and FR2 in that for FR2 measurements are radiated while conductive measurements are used for FR1. For radiated tests, the effects of antenna coupling and reverse intermodulation are included in EVM measurement while these aspects are not included in conductive measurements taken at the antenna connectors.  As a result, the conductive EVM measurement may underestimate the magnitude of the transmitter noise and may also reflect a combining gain over the transmitter noise that is not achievable if the transmitter noise is highly correlated.

The EVM definition for 2 Tx transmit diversity explicitly removes the combining gain that results if the transmitter noise is uncorrelated by assuming worst case correlation of the transmitter noise [5].  The same approach has been proposed for 4 Tx transmit diversity in [6].  We now consider if this same approach can be considered when defining EVM for 4x4 UL MIMO as well one-, two-, and three-layer transmission.

For 4x4 UL MIMO, the signal at the antenna connectors is given by

y=H W x+n ,

where x is the 4x1 data vector, W is the 4x4 precoding matrix, H is the 4x4 channel matrix, and n is the 4x1 vector of transmitter noise at the antenna connectors.  If the zero-forcing receiver is used, then the data estimate is given by

x=HW-1H W x+n .

where HW is an estimate of HW based on reference symbols. If HW=HW, then

x=HW-1H W x+n

=x+W-1 n

=x+v ,

where v=W-1 n. The covariance of the per-layer transmitter noise is then given by

Ev vH=W-1En nH W-1H

=WH Σ W .

where we have defined Σ=En nH  and used the fact that for a unitary matrix  W-1=WH.

If the transmitter noise at the antenna connectors is uncorrelated so that the covariance matrix can be represented by the diagonal matrix Σ=diagσ12,σ22,σ32,σ42, then the resulting noise variance for the i-th layer is given by

WH Σ Wi,i=j=14Wj,i2σi2 .

If the transmitter noise is correlated the covariance matrix is not diagonal, then the noise variance for the i-th layer is given by

WH Σ Wi,i=k=14j=14Wj,i*Σj,kWk,i .

In the Appendix, it is shown that

k=14j=14Wj,i*Σj,kWk,i≤j=14Wi,jσi2

where Σi,i,=σi2.  Thus, if coupling between the antennas leads to significant correlation of the transmitter noise, and if the transmitter noise is uncorrelated in the absence of antenna coupling, then the combining gain for the i-th layer can be as large as the ratio

Gi=j=14Wi,jσi2j=14Wi,j2σi2 .

This maximum combining gain represents the maximum increase of the layer signal-to-noise ratio that can result if the conducted transmitter noise is independent while the radiated transmitter noise has worst-case correlation due to antenna coupling.

In order to estimate the covariance of the transmitter noise n, it would be necessary to form the estimate

n=H-1y-W x ,

where H is an estimate of the channel and x is either reference symbols or demodulated data.  Alternatively, if the variance of the transmitter noise at each of the antenna connectors is assumed to be equal so that σi2=σ2, then the maximum combining gain for the i-th layer is given by

Gi=j=14Wi,j2j=14Wi,j2 .

To avoid underestimating the radiated EVM with a conductive measurement, the maximum combining gain can be removed from the EVM measurement. If EVMi denotes the EVM of the i-th layer, then the signal-to-noise ratio of the i-th layer is given by

SNRi=100EVMi2 .

The SNR of the i-th layer with the maximum combining gain removed is then given by

SNRi'=1Gi100EVMi2=100EVMiGi2=100EVMi'2

where EVMi'=EVMiGi.  Thus, the maximum combining gain Gi can be removed from the conductive EVM measurement EVMi by multiplying by the square root of Gi so that

EVMi'=EVMiGi.

Alternatively, since the correlation of the radiated transmitter noise may not be worst case, it may be better to scale the conductive EVM measurement by a fraction f of the maximum combining gain so that

EVMi'=EVMi ∙f ∙Gi,

where f is in the interval (0, 1].

For the case of the 4x4 UL MIMO precoder given by

W=121000010000100001 ,

we have the result that

j=14Wi,j2j=14Wi,j2=1 ,

so that there is no combining gain over the transmitter noise for any of the MIMO layers.  Thus, for this 4-layer precoder there is no combining gain, and at least from this perspective, the conductive EVM measurement should be similar to the radiated EVM measurement.

It can be noted that if the following 4x4 UL precoder were to be used (TPMI 3)

W=1411111-11-111-1-11-1-11 ,

then

Gi=j=14Wi,j2j=14Wi,j2=4 ,

with the result that

EVMi'=EVMiGi=2 EVMi .

- EVM for M-layer transmission from N-antenna ports

In the case that the number of transmission layers is less than the number of antenna ports, the received signal is given by

y=H W x+n ,

where x is the Mx1 data vector, W is the NxM precoding matrix, H is the NxN channel matrix, and n is the Nx1 vector of transmitter noise at the antenna connectors. At least two unbiased linear receivers can be used to separate the layers, and these are the unbiased linear MMSE receiver and the pseudo-inverse receiver. The pseudo-inverse receiver Apseudo is given by

Apseudo=HWHHW-1HWH

When the pseudo-inverse receiver is applied to the M-layer transmission, the result is

Apseudo y=HWHHW-1HWH H W x+n

=x+HWHHW-1HWH H n ,

=x+v ,

where the covariance of the per layer transmitter noise v is given by

Ev vH=B Σ BH  ,

and the MxN matrix B is defined as

B=HWHHW-1HWH H= ApseudoH .

If the transmitter noise is uncorrelated so that Σ=diagσ12,…,σN2, then the noise variance of the i-th layer is given by

BH Σ Bi,i=j=14Bj,i2σi2

If the transmitter noise is correlated the covariance matrix is not diagonal, then the noise variance for the i-th layer is given by

BH Σ Bi,i=k=14j=14Bj,i*Σj,kBk,i .

From the result in the Appendix, it follows that

k=14j=14Bj,i*Σj,kBk,i≤j=14Bi,jσi2.

where Σi,i,=σi2.  Thus, if coupling between the antennas leads to significant correlation of the transmitter noise, and if the transmitter noise is uncorrelated in the absence of antenna coupling, then the maximum combining gain for the i-th layer is given by

Gi=j=14Bi,jσi2j=14Bi,j2σi2 .

To avoid underestimating the radiated EVM with the conductive measurement, the maximum combining gain can be removed from the EVM measurement by multiplying by the square root of Gi so that

EVMi'=EVMiGi

Alternatively, since the correlation of the radiated transmitter noise may not be worst case, it may be better to scale the conductive EVM measurement by a fraction f of the maximum combining gain so that

EVMi'=EVMi ∙f ∙Gi,

where f is in the interval (0, 1].

If we consider the example two-layer precoder given by

W=122111111-1-1

and consider the case that H=I, then

B=HWHHW-1HWH H= WH.

For this example, we have

Gi=j=14Bi,jσi2j=14Bi,j2σi2=4

for all i, and

EVMi'=EVMiGi=2 EVMi .

It can be noted that this same approach can be used with any other linear MIMO receiver.  One possibility would be to use the linear unbiased MMSE receiver.  In this case, the matrix B can be defined as

B=AMMSE, U H .

- EVM adjustment using allowed increase of MPR/A-MPR for multi-antenna transmission

# In some cases, increased MPR or increased A-MPR is allowed for multi-antenna transmission.  These increased values are typically based on PA measurements taken using conductive measurements but with directional couplers used between the PA’s to emulate the coupling that occurs between antennas with limited isolation.  An example can be seen in Table 6.2.2-2 and 6.2D.2-1 below from [2].

Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2D.2-1 Maximum power reduction (MPR) for power class 2 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |

# Instead of using the maximum combining gain to estimate the radiated EVM from the conducted EVM, an alternative is to use the increase in the MPR allowed for multi-antenna transmission to estimate the EVM increase due to radiative coupling.  To adjust the conducted EVM measurement to reflect the increased MPR, there must be a mapping between the need for increased MPR and the increased EVM due to antenna coupling. The EVM that is allowed for each modulation type is shown in the Table 6.4.2.1-1 from [2] below.

Table 6.4.2.1-1: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

# The relationship between EVM and MPR can be seen in the Figure 1 below.

Figure 1: MPR vs. EVM for CP-OFDM Inner Allocation

# From this Figure, it can be observed that as the MPR is increased by 1 dB, the EVM is decreased by approximately 2.  This relationship can be expressed as

# EVM2=EVM1 2 ∆MPR=EVM1 2 ∆MPR2

# where EVM1 and EVM2 are the EVM values before and after the power reduction by ∆MPR. From this observation, the relationship between the conducted EVM measurement and the radiated EVM measurement can be expressed as

# EVMi'=EVM  2 ∆MPR2

# where ∆MPR is the increased MPR allowed for dual Tx for the given modulation type.  In some cases, it may be better to adjust the radiated EVM by only a fraction f of this increase so that

# EVMi'=EVM∙f∙2 ∆MPR2 ,

# where f is in the interval (0, 1].

- Summary

# Based on the discussion and analysis above, we have the following proposals:

# Proposal 1:  Define the EVM for 4 Tx UL MIMO transmission on a per layer basis.

# Proposal 2:  For full-rank transmission, measure the EVM using a zero-forcing MIMO receiver.

# Proposal 3:  For less than full-rank transmission, measure the EVM using a pseudo-inverse receiver.

# Proposal 4: To account for antenna correlation not observed in conductive measurements, increase the conductive EVM measurement by some fraction of the square root of the maximum combining gain so that

# EVMi'=EVMi ∙f ∙Gi

# where f is in the interval (0, 1].

# Proposal 5: Alternatively, in the case that increased MPR is defined for multi-antenna transmission, increase the conductive EVM measurement by

# EVMi'=EVM∙f∙2 ∆MPR2 ,

# where f is in the interval (0, 1].

# References

- RP-220987, “New WID on Further RF requirements enhancement for NR frequency range 1 (FR1),” Huawei, HiSilicon, 3GPP TSG-RAN Meeting RAN#95-e.

- TS 38.101-1, “User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.

- R4-2011520, “On the Transmit EVM Requirement for UL MIMO Transmission,” Lenovo, Motorola Mobility, RAN4#96-e.

- 3GPP TR38.884 Study on enhanced test methods for FR2 NR UEs (Release 17), v1.1.0.

- R4-2107369, “On the EVM Definition for Transmit Diversity and Single Layer Transmission,’ Lenovo, Motorola Mobility, RAN4#98bis-e.

- R4-2214067, “EVM for Transmit Diversity with 4 Tx,” Lenovo, RAN4#104-e.

# Appendix: Maximum Transmitter Noise Variance with Correlated Transmitter Noise

For 4x4 UL MIMO, the variance of the transmitter noise at the output of the zero-forcing receiver is given by

Ev vH=WHEn nHW

=WH Σ W ,

where

Σ=En nH=σ12⋯ε1,N⋮⋱⋮εN,1⋯σN2

and εi,j=Eni nj*.

The noise variance for the i-th layer is given by

WH Σ Wi,i=k=1Nj=1NWj,i*Σj,kWk,i .

Expanding and using the fact that εi,j=εi,j*, this gives

WH Σ Wi,i=k=1Nj=1NWj,i*Wk,i εj,k

=k=1Nj=1k-1Wj,i*Wk,i εj,k+Wk,i*Wj,iεk,j +j=1NWj,i2εj,j

=k=1Nj=1k-12ReWj,i*Wk,i εj,k+j=1NWj,i2σj2 ,

≤k=1Nj=1k-12 Wj,i*Wk,iεj,k+j=1NWj,i2σj2 .

Let ρi,j= εi,jσiσj and note that ρi,j≤1, so that

WH Σ Wi,i≤k=1Nj=1k-12 Wj,i*Wk,iρi,jσiσj+j=1NWj,i2σj2

≤k=1Nj=1k-12 Wj,i*Wk,iσiσj+j=1NWj,i2σj2

=j=1N Wj,iσi2.