3GPP TSG-RAN WG4 Meeting # 104Bis-e												     R4-2216974

Electronic Meeting, October 10 – 19, 2022

Agenda item:			6.18.5

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [104-bis-e][138] FS_NR_pos_UERF

Document for:	Information

# Introduction

This document covers RAN4 discussions on general aspects and accuracy improvement studies of the Rel-18 expanded and improved NR positioning study item.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Deep Shrestha | deep.shrestha@ericsson.com |
| ZTE | Fei Xue | Xue.fei25@zte.com.cn |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |
| LGE | JoongKwan Huh | Joongkwan.huh@lge.com |
| CATT | Qiuge Guo | guoqiuge@catt.cn |
| Huawei | Hu Dan | hudan11@huawei.com |
| Qualcomm | Carlos Cabrera-Mercader | ccmercad@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General and work plan

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216685 Work Plan for Study Item on Expanded and Improved NR Positioning | Intel Corporation, CATT, Ericsson | Paper provides tentative work plans for informational purposes |

## Open issues summary

### Sub-topic 1-1: Work plan

A tentative work plan for RAN1, RAN2, RAN3 and RAN4 tasks is captured in R4-2216655. The work plan for RAN4 has not changed from the one provided in the previous meeting (R4-2212149).

| RAN4 Meetings | Tentative Work Plan |
| --- | --- |
| RAN4#104 August 2022, (RF 0.25 TUs) | - Review initial version of the 3GPP TR 38.859 and input to RAN1 on content, if there is any - Study solutions for accuracy improvement based on NR carrier phase measurements - Focus on RAN4 RF aspects and potential inputs to RAN1, if any - Initial study on potential solutions for PRS/SRS bandwidth aggregation for intra-band carriers considering potential timing errors, phase coherency, frequency errors, power imbalance, etc. |
| RAN4#104bis October 2022, (RF 0.25 TUs RD 0.5 TUs) | - Evaluation work on potential solutions for PRS/SRS bandwidth aggregation for intra-band carriers considering potential timing errors, phase coherency, frequency errors, power imbalance, etc. - Continue study solutions for accuracy improvement based on NR carrier phase measurements - Focus on RAN4 RF aspects and potential inputs to RAN1, if any - Preparation of text proposals for 3GPP TR 38.859, if there are any |
| RAN4#105 November 2022, (RF 0.25 TUs RD 0.5 TUs) | - Finalization of the study on PRS/SRS bandwidth aggregation for intra-band carriers and carrier phase measurements - Preparation of text proposals and study item conclusions on bandwidth aggregation and carrier phase measurements - LS to RAN1 with a request to incorporate RAN4 updates to the 3GPP TR 38.859 |

Issue 1-1: Work plan for RAN4

- Recommended WF

  - Companies are encouraged to review the content of the tentative work plan for RAN4. Please note that only RAN2 and RAN3 plans have updates in R4-2216655. In case there are any comments, please share them in the Open issues section below.

## Companies views’ collection for 1st round

### Open issues

Issue 1-1: RAN4 work plan

| Company | Comments |
| --- | --- |
| ZTE | In general, we are fine with the work plan, however the workplan for RAN4#104e meeting should be removed. In addition, RD part should be discussed and confirmed in RRM session instead of RF session. |
| Nokia | We share ZTE’s view. |
| CATT | Fine with the work plan. We are fine to confirm in RRM session but please note that this work plan discussion is not included in RRM part [220]. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #1-1: Work plan | Recommendations for 2nd round: Add the tentative RAN4 work plan to thread [220] to ensure RRM delegates review its content |

# Topic #2: Accuracy improvement studies

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215430 Discussion on accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers | CATT | Scope of study on PRS/SRS aggregation Proposal 1: RAN4 to focus on a single RF architecture (i.e., single Tx/Rx chain) in SI stage. Proposal 2: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same bandwidth and different bandwidths. Proposal 3: Both TAE requirements and group delay should be considered to verify the performance of PRS/SRS bandwidth aggregation. Baseline assumptions Proposal 4: The FFT assumption should be discussed in RRM part. |
| R4-2215875 Discussion on accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers | LG Electronics | Achievable accuracy gain when TAE is within specified requirement for intra-band contiguous CA Proposal 1: Single chain Tx/Rx architecture is only feasible to achieve PRS BW aggregation gain. Single chain Tx/Rx architecture for Intra-band contiguous CA case Observation 1: If the separation of center frequency between CC0 and CC1 is not an integer multiple of subcarrier spacing, then at least one of CCs needs frequency tuning after IFFT procedure. Proposal 2: To enable simple single chain Tx/Rx architecture, the separation of center frequency between CCs should be studied and specified if necessary (i.e., as an integer multiple of subcarrier spacing). |
| R4-2215883 On accuracy improvement based on bandwidth aggregation for positioning | Ericsson | Observation 1: Type of Rx/Tx RF chain architecture is implementation specific. Observation 2: For intra-band contiguous CA single RF chain assumption is not always true. There can be multiple Tx RF chains to transmit intra-band contiguous carriers where two contiguous carriers are mapped to different Tx RF chains. Observation 3: Impairments and their modeling cannot be generalized to fit in to different types of architectures that depend on the choice of implementation. Proposal 1: Deprioritize RF impairment modeling discussion and no new requirement shall be defined. Proposal 2: Re-use intra-band contiguous CA TAE requirement for bandwidth aggregation for positioning. Proposal 3: Accuracy improvement due to non-coherent aggregation of PRS resources from different carriers/PFLs shall be studied further. Proposal 4: Notification to RAN1 on UE transmit power limitation due to prioritization of PCell over SCell may not be needed at this point of time. |
| R4-2216227 Discussion on NR positioning measurement accuracy improvement based on bandwidth aggregation | Nokia, Nokia Shanghai Bell | PRS/SRS bandwidth aggregation scenario Proposal 1: Prioritize intra-band contiguous CA with simultaneous PRS/SRS transmission for the RF and RRM impacts study. Number of transmitted CCs Proposal 2: CA configurations with 2, 3 and 4 CCs should be investigated and the configuration with 2 CCs should be prioritized over 3 and 4 CCs. FFT processing Proposal 3: RAN4 assumes that legacy FFT processing strategy, that is one FFT processing per CC with standard FFT size is baseline. Hence processing with extended FFT size specifically for high accuracy positioning measurement is not assumed as baseline. Impairment model Proposal 4: RAN4 investigates if RF impairments (i.e., timing errors, phase coherency, frequency errors) appear consistently in aggregated bandwidth. Especially, phase coherency in aggregated bandwidth is important to achieve high accuracy of ToA estimation. Proposal 5: For FR2 gNB/UE, since phase noise is one of RF impairment sources impacting phase coherency, hence its impact needs to be investigated. RF architectures Observation 1: The minimum strict TAE requirement of 260ns in the DL and 130ns in the UL in FR1, which ensures legacy FFT size requirement may lead to positioning uncertainty up to several meters if not handled precisely. This is even more important for the split baseband Tx and separate Tx chain architectures in case of separate PRS/SRS transmission. Observation 2: Even in the scenario where all links are LOS, such as InF-SH, very strict time synchronization in order of sub-nanosecond in FR1 to few picoseconds in FR2 between the carriers is required to see any potential benefit from CA combining gain, which is very difficult to achieve. Observation 3: For single Tx/Rx chain architecture, aggregated carrier bandwidth is limited by PA and analog filter bandwidth. Proposal 6: The architecture with component carriers combined in the baseband being supported by single wideband Tx/Rx chain should be preferred. However, single Tx/Rx chain with legacy IFFT/FFT size may not be able to accommodate the aggregated wideband baseband signal, hence higher IFFT/FFT size for Tx/Rx needs to be considered. Proposal 7: RAN4 should prioritize the single Tx/Rx chain architecture for the RF and RRM impacts investigated in the feasibility study. |
| R4-2216541 Further discussion on CA based positioning enhancement | ZTE | RF architecture for Intra-band contiguous CA case Observation 1: From BS hardware supporting the intra-band contiguous CA case, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source (e.g., frequency error or power imbalance) which might impact positioning accuracy, these contributing factors in intra-band contiguous CA case are negligible. Observation 2: From UE hardware perspective, just similar as BS side, timing difference/phase coherency among the aggregated contiguous carriers would be mainly impacted by group delay response of PA and analog filtering in different frequency ranges. For other error source (e.g., frequency error or power imbalance) which might impact positioning accuracy, these contributing factor in intra-band contiguous CA case are negligible. Performance evaluation for intra-band contiguous CA Proposal 1: RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fulfil the minimum requirements. The minimum requirement for CA based positioning should be much better that the legacy intra-band contiguous CA TAE requirements. Proposal 2: To follow the legacy assumption of the common FFT for intra-band contiguous CA in Rel-18. Measurement results for intra-band contiguous CA TAE Observation 3: for FR1 intra-band contiguous CA scenario, extremely small timing difference (e.g., less than 2ns) in DL/UL between contiguous CCs is feasible. Observation 4: for FR2 intra-band contiguous CA scenario, extremely small timing difference between contiguous CCs due to RF filtering is around 0.2ns. |
| R4-2216725 Study of PRS/SRS bandwidth aggregation – RF aspects | Qualcomm Inc. | Initial conclusion Observation 1: RAN4 has concluded that PRS/SRS bandwidth aggregation for intra-band contiguous carrier is feasible for single-chain Tx/Rx architectures. No adverse effects on measurement accuracy are expected due to relative timing errors, phase errors nor frequency errors between signals/carriers for such architectures. Proposal 1: For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required for single-chain Tx/Rx architectures. Observation 2: RAN4 can further study the impact of group delay errors in the context of PRS/SRS aggregation. Proposal 2: No RAN4 agreements on group delay margin values are expected as part of the Rel-18 study on PRS/SRS bandwidth aggregation. Issues for further study Proposal 3: RAN4 studies modular architectures featuring separate Tx/Rx chains per layer/carrier as part of the Rel-18 study on PRS/SRS bandwidth aggregation. Proposal 4: For modular architectures featuring separate Tx/Rx chains per layer/carrier, RAN4 will study the effects of relative timing errors, phase errors and frequency errors between layers/carriers. Proposal 5: RAN4 should discuss the methodology to evaluate the impact of relative timing errors, phase errors and frequency errors between layers/carriers, including the following: - Scenarios to be simulated - System level simulations vs link level simulations - FR, SCS, RS BW, number of layers/carriers (2, 4?) - Propagation channels - Es/Iot side-condition, number of samples - Measurements: DL-TDOA or Rx-Tx time difference - Metric - Measurement accuracy with coherent processing vs. single layer/carrier - Impairment models - Relative timing error – uniformly distributed between [-t, t] nsec - Relative phase error – uniformly distributed between [-theta, theta] degrees - Frequency error – uniformly distributed between [-d, d] ppm - How to generate errors - Draw a random realization for each error independently per measurement occasion/sample. - Correlated/dependent timing and phase errors, frequency errors independent from timing and phase errors, - Correlated/dependent timing, phase and frequency errors. |
| R4-2215884 On accuracy improvement based on NR carrier phase measurements | Ericsson | Observation 1: RAN1 agreed to study impact of different error sources on carrier phase measurement accuracy. Observation 2: RAN1 agreed on assumptions for modeling of different error sources that have an impact on achievable positioning accuracy based on NR carrier phase measurement. Observation 3: RAN1 has not concluded its evaluation on the impact of different error sources on achievable accuracy based on NR carrier phase measurement. Proposal 1: RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement. |
| R4-2216228 Discussion on NR positioning measurement accuracy improvement based on carrier phase measurement | Nokia, Nokia Shanghai Bell | Observation 1: Minimizing the source of error in carrier phase measurement will result in minimizing the phase measurement error. Observation 2: At FR2, phase noise could potentially hit the positioning signal measurement and degrade the positioning accuracy due to faster phase variation and increased difficulty in estimation. Proposal 1: RAN4 should study how to minimize impact from phase noise in order to reduce phase measurement error. Observation 3: Consolidating phase measurement from multiple TRPs may not be straightforward as the measured phase may differ significantly among multiple TRPs due to many factors as mentioned above. Proposal 2: RAN4 should study different factors influencing phase errors among different TRPs and provide the corresponding feedback to RAN1. Proposal 3: Confirm to RAN1 that the value of X for the initial phase offset between the transmitter and the receiver should be 2pi. Observation 4: The RAN1 practical oscillator-drift model for the UE of [-0.1, 0.1] ppm is aligned with RAN4’s understanding. Observation 5: To handle the carrier phase error and minimize its impact on carrier phase measurement, an error source model needs to be identified. Proposal 4: RAN4 to study error source models based on the error sources identified by RAN1 and provide feedback to RAN1 as needed. Proposal 5: RAN4 should study how to minimize the error at source and how to minimize its impact at the target. Proposal 6: RAN4 should identify methods to minimize measurement errors at both source and target. |

## Open issues summary

### Sub-topic 2-1: Study based on PRS/SRS bandwidth aggregation

Based on discussions in RAN4 #104e and approved WF (R4-2214462), this sub-topic will address the following:

- RF architecture – can we focus on a single RF architecture (i.e., single Tx/Rx chain), align on target architectures

- Studying RF impairment model (timing/group delay/frequency/phase) first to assess performance and accuracy gain with realistic impairments

- Studying achievable accuracy gain when TAE is within specified requirement for intra-band contiguous CA

- Notifying RAN1 of the UE transmit power limitation due to potential prioritization

- Baseline assumption for FFT processing

Issue 2-1a: RF architecture

- Proposals

  - Proposal 1: RAN4 to focus on a single RF architecture (i.e., single Tx/Rx chain) in SI stage.  (CATT, R4-2215430)

  - Proposal 2: Single chain Tx/Rx architecture is only feasible to achieve PRS BW aggregation gain. (LGE, R4-2215875)

  - Proposal 3: To enable simple single chain Tx/Rx architecture, the separation of center frequency between CCs should be studied and specified if necessary (i.e., as an integer multiple of subcarrier spacing). (LGE, R4-2215875)

  - Proposal 4: The architecture with component carriers combined in the baseband being supported by single wideband Tx/Rx chain should be preferred. However, single Tx/Rx chain with legacy IFFT/FFT size may not be able to accommodate the aggregated wideband baseband signal, hence higher IFFT/FFT size for Tx/Rx needs to be considered. (Nokia, R4-2216227)

  - Proposal 5: RAN4 should prioritize the single Tx/Rx chain architecture for the RF and RRM impacts investigated in the feasibility study. (Nokia, R4-2216227)

  - Proposal 6: RAN4 studies modular architectures featuring separate Tx/Rx chains per layer/carrier as part of the Rel-18 study on PRS/SRS bandwidth aggregation. (Qualcomm, R4-2216725)

  - Proposal 7: For modular architectures featuring separate Tx/Rx chains per layer/carrier, RAN4 will study the effects of relative timing errors, phase errors and frequency errors between layers/carriers. (Qualcomm, R4-2216725)

- Recommended WF

  - Companies are encouraged to share their view on the listed proposals. For single RF architecture, beyond prioritization, consider whether we can focus on single RF architecture in SI stage.

  - Additionally, please provide comments on single RF architecture’s aggregation gain (Proposal 2) and whether the separation of center frequency between CCs should be studied and specified if necessary (Proposal 3)

  - For additional RF architectures, discuss whether modular architectures featuring separate Tx/Rx chains per layer/carrier should be part of the PRS/SRS bandwidth aggregation study (Proposal 6, Proposal 7)

Issue 2-1b: PRS/SRS bandwidth aggregation scenario

During RAN4 #104e, intra-band contiguous CA scenario was agreed to be prioritized in the study (R4-2214462). For this scenario, there are two potential configurations to consider:

- Intra-band contiguous CA with simultaneous PRS/SRS transmission

- Intra-band contiguous CA with separate PRS/SRS transmission

- Options

  - Proposal 1: Prioritize intra-band contiguous CA with simultaneous PRS/SRS transmission for the RF and RRM impacts study. (Nokia, R4-2216227)

  - Proposal 2: CA configurations with 2, 3 and 4 CCs should be investigated and the configuration with 2 CCs should be prioritized over 3 and 4 CCs. (Nokia, R4-2216227)

  - Proposal 3: For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required for single-chain Tx/Rx architectures. (Qualcomm, R4-2216725)

- Recommended WF

  - Please provide your views on the configurations and proposals listed above.

Issue 2-1c: RF impairment model and assessment

- Options

  - Proposal 1: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same PRS bandwidth and different bandwidths. (CATT, R4-2215430)

  - Proposal 2: Deprioritize RF impairment modeling discussion and no new requirement shall be defined. (Ericsson, R4-2215883)

Observation: Impairments and their modeling cannot be generalized to fit in to different types of architectures that depend on the choice of implementation.

  - Proposal 3: No RAN4 agreements on group delay margin values are expected as part of the Rel-18 study on PRS/SRS bandwidth aggregation. (Qualcomm, R4-2216725)

Observation: RAN4 can further study the impact of group delay errors in the context of PRS/SRS aggregation.

  - Proposal 4: RAN4 should discuss the methodology to evaluate the impact of relative timing errors, phase errors and frequency errors between layers/carriers, including the following: (Qualcomm, R4-2216725)

    - Scenarios to be simulated

      - System level simulations vs link level simulations

      - FR, SCS, RS BW, number of layers/carriers (2, 4?)

      - Propagation channels

      - Es/Iot side-condition, number of samples

      - Measurements: DL-TDOA or Rx-Tx time difference

    - Metric

      - Measurement accuracy with coherent processing vs. single layer/carrier

    - Impairment models

      - Relative timing error – uniformly distributed between [-t, t] nsec

      - Relative phase error – uniformly distributed between [-theta, theta] degrees

      - Frequency error – uniformly distributed between [-d, d] ppm

      - How to generate errors

        - Draw a random realization for each error independently per measurement occasion/sample.

        - Correlated/dependent timing and phase errors, frequency errors independent from timing and phase errors,

        - Correlated/dependent timing, phase and frequency errors.

- Recommended WF

  - Companies are invited to share their views on the listed proposals and the outlined evaluation methodology

Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA

- Options

  - Proposal 1: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same PRS bandwidth and different bandwidths. (CATT, R4-2215430)

  - Proposal 2: Both TAE requirements and group delay should be considered to verify the performance of PRS/SRS bandwidth aggregation. (CATT, R4-2215430)

  - Proposal 3: Re-use intra-band contiguous CA TAE requirement for bandwidth aggregation for positioning. (Ericsson, R4-2215883)

  - Proposal 4: Accuracy improvement due to non-coherent aggregation of PRS resources from different carriers/PFLs shall be studied further. (Ericsson, R4-2215883)

  - Proposal 5: RAN4 investigates if RF impairments (i.e., timing errors, phase coherency, frequency errors) appear consistently in aggregated bandwidth. Especially, phase coherency in aggregated bandwidth is important to achieve high accuracy of ToA estimation. (Nokia, R4-2216227)

  - Proposal 6: For FR2 gNB/UE, since phase noise is one of RF impairment sources impacting phase coherency, hence its impact needs to be investigated. (Nokia, R4-2216227)

  - Proposal 7: RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fulfil the minimum requirements. The minimum requirement for CA based positioning should be much better that the legacy intra-band contiguous CA TAE requirements. (ZTE, R4-2216541)

- Recommended WF

  - Companies are encouraged to consider which proposals are agreeable and what aspects should be included in the scope of the study

Issue 2-1e: Baseline assumption for FFT processing

- Options

  - Proposal 1: The FFT assumption should be discussed in RRM part. (CATT, R4-2215430)

  - Proposal 2: RAN4 assumes that legacy FFT processing strategy, that is one FFT processing per CC with standard FFT size is baseline. Hence processing with extended FFT size specifically for high accuracy positioning measurement is not assumed as baseline. (Nokia, R4-2216227)

  - Proposal 3: Follow the legacy assumption of the common FFT for intra-band contiguous CA in Rel-18. (ZTE, R4-2216541)

- Recommended WF

  - Companies are invited to provide feedback on the proposals and discuss whether we can agree on the legacy FFT processing as baseline or if this should be addressed in RRM session.

Issue 2-1f: Notifying RAN1 of UE transmit power limitation

- Options

  - Proposal 1: Notification to RAN1 on UE transmit power limitation due to prioritization of PCell over SCell may not be needed at this point of time. (Ericsson, R4-2215883)

- Recommended WF

  - Please share your views on whether notifying RAN1 is needed

### Sub-topic 2-2: Study based on carrier phase measurements

In RAN4 #104e, RAN4 agreed to wait for RAN1 conclusion or LS to start work on this study (R4-2214462).

Issue 2-2a: RAN4 study on RF requirement

- Proposals

  - Proposal 1: RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement. (Ericsson, R4-2215884)

- Recommended WF

  - Proposal is aligned with agreement captured in RAN4 #104e. If needed, companies may provide feedback on the proposal.

Issue 2-2b: Scope of RAN4 study

- Proposals (Nokia, R4-2216228)

  - Proposal 1: RAN4 should study how to minimize impact from phase noise in order to reduce phase measurement error.

  - Proposal 2: RAN4 should study different factors influencing phase errors among different TRPs and provide the corresponding feedback to RAN1.

  - Proposal 3: Confirm to RAN1 that the value of X for the initial phase offset between the transmitter and the receiver should be 2pi.

  - Proposal 4: RAN4 to study error source models based on the error sources identified by RAN1 and provide feedback to RAN1 as needed.

  - Proposal 5: RAN4 should study how to minimize the error at source and how to minimize its impact at the target.

  - Proposal 6: RAN4 should identify methods to minimize measurement errors at both source and target.

- Recommended WF

  - Companies may share their views on the proposals

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1: Study based on PRS/SRS bandwidth aggregation

Issue 2-1a: RF architecture

Issue 2-1b: PRS/SRS bandwidth aggregation scenario

Issue 2-1c: RF impairment model and assessment

Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA

Issue 2-1e: Baseline assumption for FFT processing

Issue 2-1f: Notifying RAN1 of UE transmit power limitation

| Company | Comments |
| --- | --- |
| Ericsson | #Issue 2-1a: P1 and P2: Specifying single RF chain for bandwidth aggregation for positioning means that all the CCs used to transmit PRS resources must be mapped to the same antenna port which may not be supported by the legacy base stations. In our view this will limit the feasibility of using legacy base station for bandwidth aggregation for positioning measurement. However, we understand that the impact of frequency/timing/phase is negligible if the PRS resources from the same RF chain are aggregated for positioning measurement. Rather than specifying single RF architecture for positioning bandwidth aggregation and limiting the implementation at the base station side we would like to propose an alternative solution where UE is notified by the network about the resources that are transmitted by the same RF chain and can be aggregated by the UE for positioning measurements. P3: Comment above also applies to the architecture issue. Fine to study and specify, if needed, separation of center frequency between CCs (i.e., as an integer multiple of subcarrier spacing). We wonder if channel spacing for intra-band contiguous CA can be reused for positioning. P4: Will this proposal not violate the NR numerology? May make sense for the Rx node. P5/P6/P7: Comment to P1 and P2 is also valid for P5/P6/P7. #Issue 2-1b: P1: Fine P2: Number of CC can be discussed in WI phase. P3: Could this proposal be reformulated as: “For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required if the aggregated resources are transmitted/received from the same RF chain.” #Issue 2-1c: P1: Fine. P2: Support this proposal. This is to ensure legacy base station can also support bandwidth aggregation for positioning. P3: Evaluation still needs to be done as proposed in P1. P4: Timing may be a limiting factor. #Issue 2-1d: P1: Discuss this in issue 2-1c. P2: Fine provided no new TAE requirement is defined. This will ensure legacy base station can also support bandwidth aggregation for positioning. P3: support this proposal. P4: We think non-coherent aggregation shall also be investigated. P5: Fine to investigate. P6: This seems to be related to carrier phase measurement for positioning. IN our view, this shall be up to RAN1 to decide. P7: Do not support defining new requirement that will limit feasibility of supporting bandwidth aggregation for positioning by legacy base station. #Issue 2-1e: Legacy FFT can be the baseline. #Issue 2-1f: In our view notification to RAN1 may not be needed at this point of time. |
| ZTE | Issue 2-1a: regarding the RF architecture for intra-band contiguous CA, in general, its implementation should be single chain Tx and Rx, however as mentioned by Ericsson and other companies, it is somehow implementation dependent issue, for separated Tx/Rx chain could be also one way to implement the intra-band contiguous CA. Based on our initial analysis, if the baseband is shared for different Tx/Rx chains, then its timing difference among CCs could be around 5ns-10ns without any further calibration done for it. If necessary, this could be further calibrated down to less than 2ns. In short, we are fine to prioritize the single Tx/Rx chain for intra-band CA based positioning enhancement, however we think that separated Tx/Rx chain is also feasible in some scenarios. Issue 2-1b: firstly, we prefer to focus on the intra-band contiguous CA with simultaneous PRS/SRS transmission firstly, for second scenario, this will introduce lots of other potential errors which will further impact the performance. Regarding two CCs or 3CCs or 4CCs, we think that this depends on the perceived group delay, at least if group delay among 4CCs is limited, this could be also supported in Rel-18. Issue 2-1c: Regarding the group delay as mentioned by Qualcomm, we also expect that it’s difficult to agree any specific value or a range, this is quite implementation dependent, based on our internal product study, this is limited and feasible to support the intra-band CA based positioning. For the proposal 1 from CATT, we are fine to further evaluate its performance, we should also bear in mind that deadline should be next RAN4 meeting to finalize the SI phase. For the proposal 2 from Ericsson on its new RF requirements, we think this should be part of WI discussions. At current phase, we would like to see more achievable performance in practice. For the proposal 4 from Qualcomm, it might be too open and not sure its usage to conclude this topic. Issue 2-1d: First of all, we disagree to reuse the existing TAE requirement which is not relaxed requirement for CA based positioning. For FR2 CA based positioning, we also agree that phase noise model should be considered as well which is different from FR1. Issue 2-1e: For intra-band contiguous CA, the common FFT was assumed in the past, we don’t see any reason to revert its agreement. Issue 2-1f: Agree with proposal 1, Pcell power prioritization should be part of RAN1 spec discussions instead of RAN1. RAN1 should well know its impacts. |
| Nokia | Issue 2-1a: RF architecture We support proposals 4 and 5. RAN4 should prioritize the single Tx/Rx architecture for intra-band contiguous CA and identify the potential aggregation gain. As there is a tight schedule for the study, we believe that we cannot also investigate modular architectures with separate Tx/Rx chains. Issue 2-1b: PRS/SRS bandwidth aggregation scenario We are fine with proposals 1 to 3. Issue 2-1c: RF impairment model and assessment We support proposals 1 and 4. In our view, proposal 2 would result in ideal performance which does not take into account any impairments, not even the least impairments. Proposal 3 is understood to be rather in scope of the WI phase. Proposal 1 should define the scenarios to be investigated and Proposal 4 can serve as baseline for the evaluation methodology. In our view, we should limit the study to link level simulations as done previously for Rel-16 positioning and use a similar evaluation framework. Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA We support proposals 5 and 6. Regarding the timing alignment between intra-frequency contiguous carriers, a range of offsets should be investigated containing still the legacy intra-band contiguous CA TAE requirements as reference case. Issue 2-1e: Baseline assumption for FFT processing We support proposals 1 and 2. Legacy FFT size should form the baseline, but higher FFT sizes should as well be investigated in the RRM session. The investigation should include also common FFT size as considered in 38.817-2 and mentioned in proposal 3. Issue 2-1f: Notifying RAN1 of UE transmit power limitation In our view, there is no harm and even it is appropriate to send such information about Pcell prioritization for Tx power to RAN1 (at this or next meeting), as we have deprioritized the discussion on power imbalance between carriers in case of intra-band contiguous carrier aggregation and hence we will not model this impact in RAN4. |
| LGE | Issue 2-1a: P1/P2/P5/P6/P7: We have similar opinion with ZTE. We are fine to prioritize the single Tx/Rx chain for intra-band CA based positioning enhancement, but if we need to consider separated Tx/Rx chain, we should first review whether it can satisfy the challenging (less than 2ns) TAE requirement. P3: It is important issue to maintain orthogonality. But this has already been reflected in the specification [TS38.101-1]. Therefore, we think there is no need for further discussion on proposal3. ## 5.4A Channel arrangement for CA 5.4A.1 Channel spacing for CA ……. The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of least common multiple of channel raster and sub-carrier spacing less than the nominal channel spacing to optimize performance in a particular deployment scenario. …….. Issue 2-1b: P1/P2/P3: Fine Issue 2-1d: We support P6/P7 Issue 2-1e: We support Proposal3. Issue 2-1f: In our view notification to RAN1 may not be needed at this point of time. |
| CATT | Issue 2-1a: Regarding the RF chain, we propose to focus on single RF chain because we have agreed in last meeting that it is feasible for PRS/SRS BW aggregation. But it doesn’t intend to limit the UE/TRP implementation. It just means to study the aggregation gain and specification impact based on this prioritized architecture due to limited SI timeline. For the separation of center frequency, we are fine to further study. Issue 2-1b: Fine with P1 and P2. For P3, fine with Ericsson’s wording suggestion. Issue 2-1c: Support P1. The aggregated PRS/SRS can be different bandwidths. For P3, we think it should be discussed in WI stage. We are fine not to define exact value of group delay, but the difference between CCs and other impact shall be discussed. For P4, general fine with the methodology to evaluate each impact, but please note that as commented in issue 2-1b, for the case of simultaneous transmission and same RF chain, the impairment models listed in P4 is not needed. Issue 2-1d: Support P1/P2. And also fine to further study P4 and P5. Also we think P1 and P5 have already been discussed in issue 2-1c. And as we commented in issue 2-1c, for the case of simultaneous transmission and same RF chain, the impairments (i.e., timing errors, phase coherency, frequency errors) are not needed. For P7, we think it should be discussed in WI stage. Issue 2-1e: Support P1. We are fine to assume one FFT processing per CC as baseline and share the same view as Nokia that whether to use higher FFT size can be investigated in RRM part. Issue 2-1f: Support P1. |
| Huawei, Hisilicon | For issue 2-1a: We prefer to focus on single RF chain for the SI. Multiple RF chain architecture is a bit complicated for the RF impairments, which should be deprioritized or dropped given the limited timeframe in RAN4. For issue 2-1b: We support proposal 1 and are fine with proposal 2 for the prioritization of 2CC case. For issue 2-1c: We are ok with Proposal 1. RF impairments in terms of implementation capability for TAE, group delay and phase noise should be provided, but how to model the impairments should be further discussed. For issue 2-1d: Ok with proposal 1,2 and 5. For Proposal 3, the intra-band contiguous CA TAE requirement may not be aligned well with the requirement for positioning accuracy, and further evaluation is needed. For Proposal 6, the study for FR2 should be deprioritized. For Proposal 7, RF impairments should be evaluated. But for SI phase, it is not the right time to consider the minimum requirements. For issue 2-1e: We are fine with proposal 1. No need to have assumptions for the FFT processing, which should be an implementation issue. For issue 2-1f: Power imbalance issue has been discussed in last RAN4 meeting, which is related to PCell and SCell power control and could have impact on the positioning accuracy. We think it is necessary to inform RAN1 of the issue, whether to consider it in the SI phase or WI phase is up to RAN1’s discussion and decision. |
| Qualcomm | Issue 2-1a: RF architecture In GTW on 10/11 it was agreed to prioritize single Tx/Rx chain for both UE and BS. It is reasonable to prioritize a single architecture given the short time remaining to complete the study. Regarding IFFT/FFT size, our view is that the (de-)multiplexing of carriers/layers at baseband may be left up to UE/TRP implementation. One option would be to (de-)multiplex the carriers/layers via a single IFFT/FFT as mentioned in Proposal 4. Issue 2-1b: PRS/SRS bandwidth aggregation scenario We support all three proposals. For aggregation of simultaneous PRS/SRS transmissions, we understand that the main impairment to be studied is the impact of group delay calibration over the aggregated bandwidth. Issue 2-1c: RF impairment model and assessment We support Proposal 1 and Proposal 3. Proposal 4 should be considered if RAN4 agrees to study other RF architectures besides single Tx/Rx chain. Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA Note: Proposal 1 was repeated from issue 2-1c. Regarding Proposal 2, our understanding is that TAE would not apply for single Tx/Rx architectures (it would be nominally zero). For other RF architectures we agree that timing alignment errors would have impact. Group delay impact should be studied. Regarding Proposal 3, several companies have provided arguments and evidence showing that the existing TAE requirement would not be sufficient to support PRS aggregation over large bandwidths. Regarding Proposal 4, our understanding is that enabling coherent combining across layers/carriers is the primary goal of the study. That was the case in the Rel-17 study by RAN1. If the scope is expanded to include non-coherent combining, it’s not clear if anything new would need to be specified in RAN1 in Rel-18 to support that use case. Perhaps the proponent of Proposal 4 can clarify. Regarding Proposal 5, if the priority of the study is single Tx/Rx chain (as agreed in GTW) then many of the listed impairments can be avoided. Proposal 6 is OK. Issue 2-1e: Baseline assumption for FFT processing We’re not sure how this FFT assumption would impact the outcome of this study in RAN4. Could the proponents of the three proposals provide their views on this question? If no clear impact is identified, then perhaps this issue does not need to be discussed further in the study phase. Issue 2-1f: Notifying RAN1 of UE transmit power limitation Proposal 1 is OK. |
| Apple | Issue 2-1a: RF architecture Follow GTW’s agreement including RF architecture and FFT assumption Issue 2-1b: PRS/SRS bandwidth aggregation scenario Follow GTW’s agreement on proposal 1 and 2. Proposal 3 needs to further discuss. Issue 2-1c: RF impairment model and assessment We support Proposal 1. Based on the GTW discussion, the RF impairment including group delay, TAE should be studied even for the single Tx/Rx chain cases. Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA OK with proposal 1,3, 5 and 6 On Proposal 3, existing TAE is too large to realise meaningful gain. It is proposed to study how much TAE can be tightened based on single Tx/Rx chain assumption On Proposal 4, we need to clarify if non-coherent aggregation is part of the study. It is unclear how much gain can be achieved by non-coherent aggregation. On Proposal 7, can proponent please clarify what the minimum requirements, other than TAE, refer to? Issue 2-1e: Baseline assumption for FFT processing Proposal 1 is OK. |

Sub topic 2-2:  Study based on carrier phase measurements

Issue 2-2a: RAN4 study on RF requirement

Issue 2-2b: Scope of RAN4 study

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-2a: P1: we support this proposal. Issue 2-2b: P1 and P2: In our view this is up to RAN1. P3: Fine. P4/P5/P6: As proposed in issue 2-2a RAN4 shall wait for at least an initial conclusion from RAN1 and do not perform parallel study on the same topic. |
| Nokia | Issue 2-2a: P1: Although we agree that further progress in RAN1 needs to be awaited to conduct the evaluation in RAN4, RAN4 can start work on evaluation methodology and general modelling assumptions in parallel. This is also needed due to the tight time schedule of the study. For instance, assumptions on PRS BW, SCS, FR, SNR, propagation channels, discussion on modelling of error sources identified by RAN1 as well as on evaluation metrics can take place. Issue 2-2b: All proposals should be considered by RAN4. As stated above, the evaluation methodology including the modelling of error sources is up to RAN4. We propose to align assumptions / evaluation metrics as much as possible to those for PRS/SRS BW aggregation. For P3, in case of consensus, RAN4 should liaise to RAN1. |
| CATT | Issue 2-2a: Support P1. Issue 2-2b: As commented in issue 2-2a, more conclusions from RAN1 are needed. |
| Huawei | Issue 2-2a: RAN4 already have WF in last meeting to wait for the discussion outcome of RAN1. The proposal as mentioned by moderator is aligned with the WF. So we think there is no need to have further discussion of the issue in this meeting. Issue 2-2b: For the moment, we think there is no need to have further discussion in RAN4 on the listed issues as proposed by Nokia. More inputs from RAN1 are needed, otherwise it’s not clear which direction and aspects RAN4 should study and evaluate. |
| Qualcomm | Issue 2-2a: RAN4 study on RF requirement Proposal 1 is consistent with the agreement from the previous RAN4 meeting. Issue 2-2b: Scope of RAN4 study In our view, Proposals 1, 5 and 6 are fairly similar. If there are specific proposals about how to minimize the impact of phase errors, they may be discussed. Proposals 2 and 4 have significant overlap. Proposal 4 would be preferable because it focuses on error sources identified by RAN1. However, at this point RAN1 has not sent any specific request or questions to RAN4. Proposal 3 is OK. Since no issue has been identified, it can be included in the LS to RAN1 at the conclusion of the RAN4 study. |
| Apple | Issue 2-2a: RAN4 study on RF requirement Proposal 1 is OK Issue 2-2b: Scope of RAN4 study Based on the proposal in issue 2-2a, RAN4 can finalize the scope based on RAN1’s conclusions. Issues raised by Nokia are valid but they may be addressed by RAN1. |

## Summary for 1st round

### Open issues

|  | Status summary |
| --- | --- |
| Sub-topic #2-1: Study based on PRS/SRS bandwidth aggregation | Issue 2-1a: RF architecture Agreement (GTW session - October 11th) - Prioritize the single RF chain (Tx/Rx) for BS and UE in the study. Issue 2-1b: PRS/SRS bandwidth aggregation scenario Agreement (GTW session - October 11th) - Prioritize intra-band contiguous CA with simultaneous PRS/SRS transmission for the RF and RRM impacts study. - CA configurations with 2, 3 and 4 CCs should be investigated and the configuration with 2 CCs should be prioritized over 3 and 4 CCs. - The TAE and group delay need be studied. In addition, phase noise needs be studied for FR2. Recommendations for 2nd round: Further discuss Proposal 3: For aggregation of simultaneous PRS/SRS transmissions, no further study of relative timing errors, phase errors nor frequency errors between signals/carriers is required for single-chain Tx/Rx architectures. Issue 2-1c: RF impairment model and assessment Candidate options: - Proposal 1: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same PRS bandwidth and different bandwidths. - Proposal 2: Deprioritize RF impairment modeling discussion and no new requirement shall be defined. Observation: Impairments and their modeling cannot be generalized to fit in to different types of architectures that depend on the choice of implementation. - Proposal 3: No RAN4 agreements on group delay margin values are expected as part of the Rel-18 study on PRS/SRS bandwidth aggregation. Observation: RAN4 can further study the impact of group delay errors in the context of PRS/SRS aggregation. - Proposal 4: RAN4 should discuss the methodology to evaluate the impact of relative timing errors, phase errors and frequency errors between layers/carriers, including the following: - Scenarios to be simulated - System level simulations vs link level simulations - FR, SCS, RS BW, number of layers/carriers (2, 4?) - Propagation channels - Es/Iot side-condition, number of samples - Measurements: DL-TDOA or Rx-Tx time difference - Metric - Measurement accuracy with coherent processing vs. single layer/carrier - Impairment models - Relative timing error – uniformly distributed between [-t, t] nsec - Relative phase error – uniformly distributed between [-theta, theta] degrees - Frequency error – uniformly distributed between [-d, d] ppm - How to generate errors - Draw a random realization for each error independently per measurement occasion/sample. - Correlated/dependent timing and phase errors, frequency errors independent from timing and phase errors, - Correlated/dependent timing, phase and frequency errors. Summary of status: Majority view is that Proposal 1 is agreeable, while Proposal 2 is not. Feedback for Proposal 3 is that evaluation is still needed. Companies are generally ok with Proposal 4. Based on GTW outcome, group delay and TAE will be studied (and phase noise for FR2). Recommendations for 2nd round: Have Proposal 1 as tentative agreement and discuss additional details needed, being mindful of agreed prioritizations Use Proposal 4 as baseline to streamline the evaluation methodology and fine tune the outlined steps. Additionally, discuss how the impairments will be modeled. Issue 2-1d: Achievable accuracy gain study for intra-band contiguous CA Candidate options: - Proposal 1: RAN4 to evaluate the impact of group delay on the performance of PRS/SRS aggregation covering PRS/SRS resources with both same PRS bandwidth and different bandwidths. - Proposal 2: Both TAE requirements and group delay should be considered to verify the performance of PRS/SRS bandwidth aggregation. - Proposal 3: Re-use intra-band contiguous CA TAE requirement for bandwidth aggregation for positioning. - Proposal 4: Accuracy improvement due to non-coherent aggregation of PRS resources from different carriers/PFLs shall be studied further. - Proposal 5: RAN4 investigates if RF impairments (i.e., timing errors, phase coherency, frequency errors) appear consistently in aggregated bandwidth. Especially, phase coherency in aggregated bandwidth is important to achieve high accuracy of ToA estimation. - Proposal 6: For FR2 gNB/UE, since phase noise is one of RF impairment sources impacting phase coherency, hence its impact needs to be investigated. - Proposal 7: RAN4 need to study the minimum requirements to achieve the positioning accuracy improvement for intra-band contiguous CA compared with single component carrier case and its feasibility to fulfil the minimum requirements. The minimum requirement for CA based positioning should be much better that the legacy intra-band contiguous CA TAE requirements. Summary of status: Proposal 1 is agreeable (already included in previous issue). Overall, Proposal 2, Proposal 5, and Proposal 6 are acceptable (some details need confirming/discussion). Proposal 3 is not agreeable, and alternatively, studying how much TAE can be tightened based on single Tx/Rx chain assumption was suggested. While feedback for Proposal 7 (studying min requirements) is that it should be addressed in WI stage. Lastly, Proposal 4 needs further discussion if non-coherent combining is considered and clarification on whether RAN1 work is expected in Rel-18 to support it. Recommendations for 2nd round: Continue discussions focusing on the following - Proposal 2, Proposal 5, Proposal 6 – consider as tentative agreements - Proposal 3 – discuss suggestion to study how much TAE can be tightened based on single Tx/Rx chain assumption - Proposal 4 – further discuss if non-coherent combining is considered and clarify whether RAN1 work is expected in Rel-18 to support it Issue 2-1e: Baseline assumption for FFT processing Agreement (GTW session - October 11th) - The FFT assumption should be discussed in RRM part. Issue 2-1f: Notifying RAN1 of UE transmit power limitation Candidate option: - Proposal 1: Notification to RAN1 on UE transmit power limitation due to prioritization of PCell over SCell may not be needed at this point of time. Majority view is that it is not needed to notify RAN1 at this time. Tentative agreement: Approve Proposal 1 Recommendations for 2nd round: Confirm whether tentative agreement is acceptable |
| Sub-topic #2-2: Study based on carrier phase measurements | Issue 2-2a: RAN4 study on RF requirement Candidate option: - Proposal 1: RAN4 shall wait for conclusion from RAN1 evaluation of impact of different error sources on carrier phase measurement before starting study on RF requirement for NR carrier phase measurement. Most companies support Proposal 1, which is aligned with a previous agreement captured in RAN4 #104e (R4-2214462). Tentative agreement: Approve Proposal 1 Recommendations for 2nd round: Confirm tentative agreement Issue 2-2b: Scope of RAN4 study Candidate options: - Proposal 1: RAN4 should study how to minimize impact from phase noise in order to reduce phase measurement error. - Proposal 2: RAN4 should study different factors influencing phase errors among different TRPs and provide the corresponding feedback to RAN1. - Proposal 3: Confirm to RAN1 that the value of X for the initial phase offset between the transmitter and the receiver should be 2pi. - Proposal 4: RAN4 to study error source models based on the error sources identified by RAN1 and provide feedback to RAN1 as needed. - Proposal 5: RAN4 should study how to minimize the error at source and how to minimize its impact at the target. - Proposal 6: RAN4 should identify methods to minimize measurement errors at both source and target. Overall, companies mainly think more input from RAN1 is needed (aligned with Issue 2-2a). Recommendations for 2nd round: Confirm no further discussion is needed in RAN4 at this time |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on expanded and improved NR positioning – UE RF aspects | Intel Corporation |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215430 |  | Discussion on accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers | CATT | Noted |  |
| R4-2215875 |  | Discussion on accuracy improvement based on PRS/SRS bandwidth aggregation for intra-band carriers | LG Electronics | Noted |  |
| R4-2215883 |  | On accuracy improvement based on bandwidth aggregation for positioning | Ericsson | Noted |  |
| R4-2215884 |  | On accuracy improvement based on NR carrier phase measurements | Ericsson | Noted |  |
| R4-2216227 |  | Discussion on NR positioning measurement accuracy improvement based on bandwidth aggregation | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216228 |  | Discussion on NR positioning measurement accuracy improvement based on carrier phase measurement | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216541 |  | Further discussion on CA based positioning enhancement | ZTE Corporation | Noted |  |
| R4-2216685 |  | Work Plan for Study Item on Expanded and Improved NR Positioning | Intel Corporation, CATT, Ericsson | Noted |  |
| R4-2216725 |  | Study of PRS/SRS bandwidth aggregation – RF aspects | Qualcomm Incorporated | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | WF on expanded and improved NR positioning – UE RF aspects | Intel Corporation |  |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents