3GPP TSG-RAN WG4 Meeting #104-e-bis 												    R4-2216975

Electronic Meeting, October 10 - October 19, 2022

Agenda item:			6.19.4

Source:	Moderator (China Telecom)

Title:	Email discussion summary for [104-bis-e][139] NR_MC_enh_UERF

Document for:	Information

Introduction

This email thread discusses the UE RF aspects for Rel-18 Multi-carrier enhancements WI, including the following topics:

- Topic #1: Tx switching across 3/4 bands with single TAG

- Topic #2: Tx switching with multiple TAGs

List of candidate target of email discussion for 1st round and 2nd round:

- 1st round:

  - Invite comments on the recommended WF under each issue in section 1.2 and 2.2.

  - To help the discussion in Wednesday GTW session, it would be appreciated if companies can add your comments before 22:00 UTC Tuesday.

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| China Telecom | Shan YANG | yangshan@chinatelecom.cn |
| OPPO | Jinqiang | xingjinqiang@oppo.com |
| Huawei | Hu Dan | hudan11@huawei.com |
| Nokia | Hiromasa Umeda | hiromasa.umeda@nokia.com |
| Xiaomi | Yuan Gao | Gaoyuan23@xiaomi.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |
| Sony | Olof Zander | Olof.zander@sony.com |
| Qualcomm | Ville Vintola | vvintola@qti.qualcomm.com |
| vivo | Sanjun Feng | fengsanjun@vivo.com |
| MediaTek | Huanren Fu | huanren.fu@mediatek.com |
| CMCC | Xiaoran ZHANG | zhangxiaoran@chinamobile.com |
| Samsung | Wang, He (Jackson) | h0809.wang@samsung.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: Tx switching across 3/4 bands with single TAG

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215495 | CMCC | Title: Switching for UL Tx switching across 3 or 4 bands Proposal 1: the set of switching periods is the same as in Rel-16/17, and UE is not expected to report a larger switching period per band pair for Rel-18 Tx switching. Proposal 2: Introduce optional UE capability on supporting transmission on the band with the number of Tx chain unchanged during UL switching. |
| R4-2215569 | Qualcomm Incorporated | Title: UL Outage time considerations for 2 TAG Observation 2: It is more evident that UE will need the flexibility to optimise 2 band single TAG switching time and still support 4 band TX switching feature with simultaneous switching on both TX chains. Proposal 2: UE is allowed to optimise TX switching time depending on number of bands in a TX switching band combination |
| R4-2215797 | China Telecom | Title: UL Tx switching across 3/4 bands and Tx switching with 2 TAGs For the length of UL switching period: Proposal 1: For Rel-18 Tx switching, reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching (i.e., option 1). For the impact from switching of one Tx chain on the other Tx chain: Observation 1: If the impact on other band(s) with the number of Tx chain “unchanged” always exists, in the real network, the UL transmission on other band(s) will be interrupted when nothing happens on their own band(s). It will bring system performance degradation as well as network scheduling restriction. Proposal 2: As optional and advanced feature, when one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”), the other Tx chain is expected to be used for transmission on band C during the switching period. For RAN4 RF requirements: Proposal 3: For UL Tx switching across 3 or 4 bands, the basic RF requirements for CA & SUL+CA band combinations with 3 or 4 bands configured and with 1 UL band or 2 UL bands (via UL CA) simultaneous transmission should be satisfied. Proposal 4: It is feasible to define the switching time mask requirements per band pair with 3 or 4 bands configured, and the specification work will be similar to Rel-16/17. Proposal 5: Further discuss whether additional time mask requirements are needed for CA option 2 when the 2 Tx chains are switched between 2 different band pairs across the same time. For the location of switching period: Observation 2: For Rel-18 Tx switching, it would more efficient to discuss the location of switching period in RAN1 together with the switching mechanism. Proposal 6: Inform RAN1 that RAN4 recommends RAN1 to discuss and decide the location of UL switching period for Rel-18 Tx switching. For the applicability of DL interruption: Proposal 7: Reuse the applicability of DL interruption agreed in Rel-16/17: - For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required. - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption. - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching. - For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, DL interruption on the SUL and TDD bands is not required. Proposal 8: For Tx switching across 4 bands, when the status of Tx chain on the 4 bands before and after switching is {1, 1, 0, 0} and {0, 0, 1, 1} respectively, there is ambiguity on the switched band pair of each Tx chain. In RAN4 understanding, RAN1 will discuss and resolve this ambiguity. For UL-MIMO coherence: Proposal 9: For Rel-18 Tx switching across 3 or 4 bands, define new per band per BC capability on UL-MIMO coherence. If the Rel-18 new per band per BC capability on UL-MIMO coherence is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL bands for Tx switching. |
| R4-2215881 | ZTE Wistron Telecom AB | Title: Discussion on Tx switching across 3 or 4 bands Observation 1: Rel-17 Tx switching enhancements are sufficiently able to justify a different switching period for the same band pair. Observation 2: The additional operation of choosing 2 bands out of 3 or 4 bands for the switching in Rel-18 does not have a direct impact on the actual band-pair-wise switching operation, unless such implementation aspects are identified. Proposal: RAN4 to specify the same value of switching period for each band pair in Rel-18 Tx switching as Rel-16/17. |
| R4-2216042 | Xiaomi | Title: Discussion on RF requirements for multi-carrier enhancement Proposal 1: The switching period specified for each band pair specified in Rel-16/17 can be reused for Rel-18 single TAG case. Proposal 1: RAN4 to study the Tx chain impact for Case 3 and Case 4 and to inform RAN1 if any consensus reached. - Case 3: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band A”) to another band (name “band C”) - Case 4: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band C”) to another band (name “band D”) Proposal 2: In case that two Tx chains have different switching periods, UE behavior should be discussed in terms of whether the Tx chain with smaller switching period can allow UL transmission once the switching is completed. |
| R4-2216119 | vivo | Title: Discussion on UE RF requirements for TxSwitching up to 3 or 4 bands In this paper, continue discussion was done for Tx Switching up to 3 or 4 bands for single TAG case. Proposal 1: Either select option 2 (different values can be reported for each band pair for Rel-18 and Rel-16/17) or wait until more information comes from RAN1’s ongoing study. Proposal 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period. Proposal 3: Further discuss whether UL support on the corresponding band pair(s) could be set to a sufficient condition of support of concurrent UL transmission, at least from RAN4’s point of view. Observation 1: RAN4 requirements currently only consider “switched UL” case, and doesn’t consider “dualUL” option which is defined in RAN1. Proposal 4: For Rel-18, RAN4 can keep the REl-16/17 way that only define requirements “switched UL” case. Proposal 5: In case only one band pair would be involved during Tx Switching, the current requirements clauses can be reused with some clarifications. Proposal 6: In case multiple band pairs are involved during the Tx switching process, new requirements clauses may be added, which would also be impacted by RAN1’s undergoing complexity reduction discussion. |
| R4-2216160 | MediaTek Inc. | Title: Continue discussion on UL Tx switching across 3 or 4 bands in Rel-18 Proposal 2: For the band combanation including SUL band, Tx switching can be applied for NUL switched to another UL, NUL switched to corresponding SUL or SUL switched to another UL (non-corresponding NUL). Proposal 3: Switching time of band A/B, B/C, A/C shall be reported separately in UE capability signalling. Proposal 4: Proposal 3 is also applicable for 4-bands Tx switching. |
| R4-2216433 | OPPO | Title: R18 Discussion on Tx switching Observation 1: Different Tx chain could be applied for the same band pair in Rel-18 comparing to Rel-16/17. Proposal 1: Propose to allow UE report different switching period for the same band pair in Rel-18 comparing to Rel-16/17 within the set of switching periods already defined, i.e. {35us, 140us, 210us}. |
| R4-2216653 | Huawei, HiSilicon | Title: Discussion on Multi-carrier enhancements with single TAG Proposal 1: It is proposed that the UE capability of uplinkTxSwitchingPeriod2T2T in Rel-17 is reused in Rel-18 Tx switching among 3 or 4 bands for a band pair. The agreements for Rel-17 2Tx-2Tx swithcing is also valid for Rel-18 Tx switching. Proposal 2：For a certain band pair, the switching period(s) specified in Rel-16/17 can be reused in Rel-18 Tx switching among 3/4 bands. Different switching period values can be reported for different band pairs in Rel-18 Tx switching with 3/4 bands configured. Proposal 3：It is not expected to introduce an advanced UE assumption in Rel-18 to allow the other Tx chain be used for transmission on band C during the switching period. Proposal 4: The requirements for Tx switching Option 1 should be prioritized. |
| R4-2216714 | Samsung | Title: Further Discussion on Switching Time and other RF Aspects for UL Tx Switching Across 3 or 4 Bands Proposal 1: RAN4 adopt Option 2 for the exact value of TX switching period across 3/4 bands: - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured. Proposal 2: For Case 2 (One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).) - Advanced UE implementation can allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption - Applying the baseline or the advanced implementation assumption depends on the particular UE implementation for the given band pair of each band combination. |

Open issues summary

Sub-topic 1-1: Switching period for Tx switching across 3/4 bands

RAN4 Agreements captured in the reply LS R4-2214464:

On the length of switching period:

- For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL.

- The length of switching period is applied per band pair for each band combination.

- For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17.

  - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

- For the same band pair, RAN4 has not concluded on whether the same or a different value can be reported for the specific band pair supporting Tx switching across 3 or 4 bands in Rel-18 compared to Tx switching across 2 bands specified in Rel-16/17.

- Issue 1-1-1: Exact value of Tx switching period

- Options captured in the WF R4-2214463:

  - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18 Tx switching.

  - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 Tx switching with 3/4 bands configured.

- Proposals

  - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18. (CMCC, China Telecom, ZTE, HW, Xiaomi)

    - ZTE: The additional operation of choosing 2 bands out of 3 or 4 bands for the switching in Rel-18 does not have a direct impact on the actual band-pair-wise switching operation, unless such implementation aspects are identified.

    - HW: For a certain band pair, the switching period(s) specified in Rel-16/17 can be reused in Rel-18 Tx switching among 3/4 bands. Different switching period values can be reported for different band pairs in Rel-18 Tx switching with 3/4 bands configured.

  - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands. (QC, vivo, MTK, OPPO, Samsung)

    - QC: It is more evident that UE will need the flexibility to optimise 2 band single TAG switching time and still support 4 band TX switching feature with simultaneous switching on both TX chains.

    - OPPO: Different Tx chain could be applied for the same band pair in Rel-18 comparing to Rel-16/17.

- Recommended WF

  - Further discuss.

| Company | Comments |
| --- | --- |
| China Telecom | We support option 1. For Rel-18, although the configured bands is increased to be 3 or 4, the Tx switching is still conducted between 2 bands for each switching occasion, with the same UE switching behavior as in Rel-16/17. Option 2 will allow negative network performance impact for Rel-18 Tx switching. It seems the main point of option 2 to allow the possibility of different UE implementations. If so, we should also allow different UE implementations in Issue 1-2 (Impact from switching of one Tx chain on the other Tx chain). Considering the tight timeline for this WI, we can compromise to option 2 if option 1 in Issue 1-2 can be agreed. |
| OPPO | Option 2. Because in our understanding the Tx chain used may be different for a band pair when additional candidate switching band comes. The concept is as below figure, for band A and B in Rel-16/17 it uses separate Chain/LO, but in Rel-18 it is possible that UE always support band A+B with one LO and use the other LO for Band C. Then in this scenario the Rel-18 Tx switching time will be different from Rel-16/17 for Tx switching band pair A/B. Figure 1 Tx switching of Band A and B comparison between R16/17 and 18 |
| Huawei | We support option 1. Referring to the UE architecture and analysis in our paper R4-2216653, the behavior of PLL(s) in Rel-18 would be no different from Rel-16/17. Reusing the same switching period is ok. For option 2 ‘a different value can be reported for each band pair’, there are two kinds of understandings: Alt.a. For different band pairs, the switching periods in Rel-18 could be different; Alt.b. For one band pair, the switching period of Rel-18 could be different from that of Rel-16/17 If the correct interpretation of option 2 is Alt.a, we are also ok with it as complementary to option 1. |
| Xiaomi | We support option 1 and share the similar understanding with the proponent. If different switching period is allowed different UE implementation is foreseen. From this perspective, it would be necessary to explicitly study the impact on UE complexity, e.g., share LO or separate LO. The switching period should be derived based on UE architecture. |
| Sony | Option 2 |
| Qualcomm | Option 2. If we consider rel-16 case, and switch is executed from A+C to C+C (2 UL on band C) switch only needs to ramp down TX on band A and ramp up the TX on band C. In Rel-18 case, if the switch is from A+B to B+C. In isolation, the switch is same as previously switch TX from band A to C but since B is involved, and lets also assume band B supports UL MIMO and UE has a PA module that supports band B and C. In the starting condition, TX1 is on band B PA that also supports band C, in same module. Module only has one control bus. So what needs to happen between A+B and B+C states that TX1 is switched from one PA on band B to an other satellite PA on band B so that PA on shared module for band C can be used and transmissions on B continued. The number of control events is much greater in this case even the switch is just from band A to band C and therefore. It should be also noted that the benefit analysis in R1-2204724 provided to RAN4 in an LS attachment concludes that even with longer switching period, there is greter benefits with 3&4 band switching compared to 2 band switching with shorter switching period. So this should not be an issue. |
| vivo | Slightly prefer option 2. The Rel-18 scheme is still more complicated compared to Rel-16/17. The option 1 is a choice for selected for simpler signaling and more alignment between different releases. However, in case actually two different values is needed for a particular implementation, then a larger value would have to be reported for both releases, and that Rel-16/17 scenarios may have to utilized a slightly larger value than it actually needed. |
| MediaTek | We support option 2. Considering R-18 UE would support more CA/DC combos as well as more complex hardware configuration, we think for Tx switching timing that a R-18 UE reports in R-18 network can use same values when the UE operates under R-16/R-17 environment. But this does not mean a R-16/R-17 UE shall apply same Tx switching time for R-18 for Tx switching on 3 or 4 bands. |
| CMCC | We agree with CTC that if we would like to allow flexibility for UE implementation, we should treat issue 1-1-1 together with issue 1-2, which means if issue 1-1-1 agrees on option2, issue 1-2 has to agree with option1. |
| Apple | Option 2 |
| ZTE | Option 1. |

- Summary of round 1 discussion (Oct 12)

  - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18. (CMCC, China Telecom, ZTE, HW, Xiaomi)

    - Note: with the understanding that the switching periods in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in the last meeting.

  - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands. (QC, vivo, MTK, OPPO, Samsung, Sony, Apple)

- Moderator’s recommendation (Oct 12)

  - Option 2 is to allow the possibility of different UE implementations. If so, we should also allow different UE implementations in Issue 1-2 (Impact from switching of one Tx chain on the other Tx chain).

  - As a compromised proposal, it is agreeable to agree option 2 for this issue and agree option 1 in Issue 1-2?

- Discussion after GTW on Oct 12

  - Considering the positions from all companies, suggestions on compromised proposal are encouraged.

| Company | Comments |
| --- | --- |
| ZTE | Normally there should be some UE implementation aspects which can justify the need of this capability, as Rel-17 did. |
| Qualcomm | We can discuss rules what UE is allowed to report, which is different. But one new observation after discussions is that how does UE report supporting these two cases? Does UE Alt 1: declare first 2-band (rel-16/17) TX switching support for band pair (A+B) and tx switching time and then also 3(/4)-band (Rel-18) tx switching case (A+B+C)? Or Alt 2: does UE only report highest band number TX switching case and in it tx switching time for each band pair? In Alt 2, then a separate IE would be needed for tx switching time in case only 2 bands are configured from this larger group but how do we prevent UE from declaring 2-band case separately from 3-band case, i.e. Alt 1. So it to us it seems ran4 has to agree the principle that UE has to report same value, not that vendors have to justify a new capability. To ZTE, the UE implementation aspect is that in 3 band case, the RFFE control is using one RFFE bus (https://www.mipi.org/specifications/rf-front-end), in case switch is from A+B to B+C, the band B may need to be switched to an other band B PA since bands B and C are on same multiband PA module. So simple switch from A->C means that also band B needs to be switched to an other band B PA and the bus has limited capacity to perform switches in time. In simple A->C switch, the bus needs to perform one ramp down and one ramp up and performance maybe 35 usec. But in case A+B ->B+C the bus has to perform four events and then the time exceeds 35 usec and next value is 140 usec. |
| Samsung | We support Moderator’s suggestion for the Option 2 to allow UE implementation flexibility. If no dedicated PA and PLL are prepared and reload processing time is needed, we expect additional switching period would be of necessity since UE complexity is expected to be increased to handle additional bands in RF switches, for TX switching across 3 or 4 bands in comparison to 2 bands. |

- Issue 1-1-2: Issue of two Tx chains switched between two different band pairs

- Proposals

  - Proposal 1: In case that two Tx chains have different switching periods, UE behavior should be discussed in terms of whether the Tx chain with smaller switching period can allow UL transmission once the switching is completed. (Xiaomi)

    - Xiaomi: For example, one of the two Tx chains is to switch from band A to band B with 35us switching period, and the other Tx chain is to switch from band C to band D with 140us switching period. Whether band B on the Tx chain with smaller switching period can perform UL transmission after the switching completion during the followed 105us during which other Tx chain is in process of switching.

- Moderator’s observation

  - Related ambiguity issue: For the example given above, when Tx switching is conducted across 4 bands and the status of Tx chains before and after Tx switching is:

    - Before Tx switching: (band A, band B, band C, band D) = (1, 0, 1, 0)

    - After Tx switching: 	(band A, band B, band C, band D) = (0, 1, 0, 1)

It is possible that the Tx switching can be conducted from band A->B + C->D or band A->D + C->B, and the lengths of switching periods can be different. To moderator’s understanding, this ambiguity issue can be discussed and resolved in RAN1 together with the switching mechanism.

- Recommended WF

  - Encourage feedback on Proposal 1 and the related ambiguity issue.

| Company | Comments |
| --- | --- |
| China Telecom | We agree the Proposal 1 and the related ambiguity issue need to be discussed, and they can be discussed in RAN1 together with the switching mechanism. |
| OPPO | If understand correctly, this relate to whether UE is allowed to transmit in one band while the other band is Tx switching. The answer RAN4 reply to RAN1 in last meeting LS is no. |
| Huawei | For the case in Proposal 1, the larger switching period should be followed. When the Tx chain completes switching with smaller switching period, the UL transmission on band B should wait until the completion of Tx switching on band D, so that the coupling effect between the two PLLs could be avoidable. For the ambiguity issue raised by CTC, it depends on the band pairs that the UE reports. For the band pairs with different switching periods, the longer one should be followed. Clarification is needed when RAN2 designs the signaling. |
| Nokia | Support option 1 and moderator’s observation |
| Xiaomi | In our understanding this is a new switching case involving 4 band and not covered in the reply LS. The switching case captured in the reply LS involves A->B + A->A and A->B + C->C where one Tx chain always remains on one band. From the perspective of UE behavior there is ambiguity issue as mentioned by moderator. Generally we are ok to discuss this in RAN1 together with the switching mechanism. |
| Sony | We support OPPO’s analysis |
| Qualcomm | Switching period is not associated with the TX chain but between two bands. RAN1 should discuss but the finite length of a switching period is something ran4 needs to discuss eventually but again there should not be ambiguity since larger switching should apply, or define outage time as max of switching periods and TA misalignment. |
| Vivo | For proposal 1, we prefer to categorize it into similar scenario that whether UE is allowed to transmit in one band while it did not participate in another band pair’s switching, which is already covered in previous discussion. For the ambiguity issue raised, this may have to be considered by RAN1/2 how to differentiate from signaling point of view. However, it is a bit doubtful whether the actual switching case could be that flexible for actual implementation. If more restrictions for band pairs were set in RAN1’s complexity discussion, this issue may also be some what mitigated in certain cases. |
| NTT DOCOMO | We agree proposal 1, the case should be discussed further. We think this has a relation with the reply LS in last meeting, as OPPO mentioned, and also relation with issue 1-2 in sub topic 1-2, which needs further discussion as agreed in the last meeting WF. The case could be an advanced feature combined with issue 1-2A in sub topic 1-2. |
| MediaTek | Share the view with OPPO. In general, we are fine with the proposal 1. We would like to highlight if the Tx chain A that finish switching earlier or not applying Tx switching, it is possible to transmit data only when the Tx A is not impacted by the other Tx chain B under switching including. The Tx chain A may not be able to transmit if there’s close-in frequency or harmonic frequency pulling by Tx chain B. |
| Apple | For the purpose of the minimum requirement, it is more straightforward to use the worst-case switching period, so that the network can be certain that both uplinks are available to transmit. Similar observation to OPPO: we should not allow one band to transmit while there is a switching operation between two other bands. |
| ZTE | Applying the larger switching period would be a simple solution. We are not sure about the potential gain if introducing a complex scheme to differentiate switching periods for different options. |
| Samsung | Worst case should be assumed for the switching period. |

Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

RAN4 Agreements captured in the reply LS R4-2214464:

RAN1 Question:

RAN WG1 would like to respectfully ask RAN WG4 to provide their feedback on whether following assumption can be considered as baseline UE assumption/behavior even in case of the UL Tx switching across 3 or 4 bands.

- “When one of the two Tx chains is triggered to switch from one band to another band, another Tx chain which is in any of bands is also not expected to be used for transmission during the switching period.”

RAN4 Answer:

RAN4 has discussed the UE assumption/behavior considering two cases:

- Case 1: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on either band A or band B.

For Case 1, RAN4 agreed that neither of Tx chains is expected to be used for transmission during the switching period.

- Case 2: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is maintained on a different band (named “band C”).

For Case 2, RAN4 agreed that, as baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

- Issue 1-2: Impact on the band with the number of Tx chain unchanged due to switching (Case 2)

- Agreement captured in the WF R4-2214463:

  - As baseline UE assumption, neither of Tx chains is expected to be used for transmission on band C during the switching period.

  - RAN4 will further discuss optional advanced features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period as advanced/optional UE assumption.

- Proposals

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching. (CMCC, China Telecom, Samsung)

    - CMCC: The impact of switching between one band pair on another band will cause system performance degradation.

    - China Telecom: If the impact on other band(s) with the number of Tx chain “unchanged” always exists, in the real network, the UL transmission on other band(s) will be interrupted when nothing happens on their own band(s). It will bring system performance degradation as well as network scheduling restriction.

    - Samsung: Applying the baseline or the advanced implementation assumption depends on the particular UE implementation for the given band pair of each band combination.

  - Option 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period. (vivo, HW)

- Recommended WF

  - Further discuss

| Company | Comments |
| --- | --- |
| China Telecom | Support option 1. To the proponents of option 2: UE implementation aspects are analyzed in their tdocs to justify why the other Tx chain cannot be used for transmission for some UEs. However, we cannot conclude that it is the case for all Ues. Option 1 is just to allow the implementation in some advanced Ues, and to avoid system performance degradation. |
| OPPO | Prefer Option 2. For Option 1 we would like to understand better on how UE to indicate the capability and also behavior. Does it mean UE switch between band pair A/B with band A+B+C+D, then UE will report which band can continue transmit when band pair A/B is switching? Is it static per band per band pair per BC capability? |
| Huawei | Option 2. During the Tx switching, the local oscillator would be affected due to load change or phase adjustment of additional routing to the second Tx chain. It is not expected to introduce an optional UE assumption in Rel-18 to allow the other Tx chain be used for transmission on band C during the switching period. Furthermore, for the optional assumption, it is more complicated for CA option 2 dualUL, with the issue of timing that the transmission on band C and band B could not start at the same time. |
| Nokia | Option 1 |
| Sony | Prefer Option 2 |
| Qualcomm | This capability would need to be to indicate for each band pair indicate a band that is not affected. At this stage, we prefer not to define additional optional capabilities. And as we have seen, this kind of capability wil lead to a basket work item in Rel-19 the declaration of that capability is denied for same band combinations therefore effective making it mandatory. Option 2 is our preference. |
| Vivo | Prefer option 2. It is doubtful whether this “advanced” feature is feasible or not, and how much gain could be observed. |
| NTT DOCOMO | Our preference is option 1, and have a similar view with China Telecom. And we think this could be could be an advanced feature combined with issue 1-1-2 in sub topic 1-1. |
| MediaTek | The concept of the proposal is good. However considering implementation complexity as other companies mentioned and we have to consider multi-TAG operation together in the R-18 WI also, we tend to support option 2. But we are open to discuss introducing the capability in R-19. |
| CMCC | Option1. This is different from the DL interruption capability which some band combinations are mandated to support no interruption. Since we already agreed the baseline UE assumption. For this advanced UE implementation, this is optional UE capability. |
| Apple | We do not see a strong motivation to introduce the proposed optional capability and associated UE behavior. If a network deployment envisions not switching one band at all, then either the Rel-16 Tx switching behavior or a simple inter-band UL CA configuration can be used instead. Thus, Option 2 is our preference. |
| ZTE | Without a justification on the capability, we don’t see the need at this moment. We can further discuss if any aspect is identified. |

- Summary of round 1 discussion (Oct 12)

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching. (CMCC, China Telecom, Samsung, Nokia, NTT DCM)

    - OPPO: Is it per band per band pair per BC capability?

    - QC: This capability would need to be to indicate for each band pair indicate a band that is not affected.

    - MTK: Multi-TAG case is more complicated. Can introduce the capability in Rel-19.

    - CMCC: The baseline agreed in the last meeting is still the baseline. With no intention to mandate this advanced UE feature in later release.

  - Option 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period. (vivo, HW, OPPO, Sony, QC, vivo, MTK, Apple)

- Moderator’s recommendation (Oct 12)

  - Agree Option 1 if option 2 in Issue 1-1 is agreed?

    - The capability is reported separately for 1-TAG and 2-TAG cases.

    - Not intend to mandate this advanced UE feature in later release.

Discussions in GTW:

CMCC: support the moderator compromised solution. We should treat the issues in the same level.

Ericsson: it is not clear regarding the difference between single TAG and dual TAG. We would like to know that multiple-TAG can be indicated by the existing signaling.

ZTE: We need the capability in Rel-17. In Rel-18 the real switching happens between two bands. We do not see the need to introduce the capability. If the difference is justified, we can agree with recommendation. We would like to have Rel-17 as default if the signaling is not reported.

Qualcomm: why are they to be compromise? The capability is set to mandatory as the next step.

Nokia: We have similar comment as Ericsson. We are talking about the capability if the capability is reported separately.

Apple: first, we do not see the strong motivation to introduce the capability. I do not see how UE behavior is different for two-TAG from UE RF.

Samsung: Echo moderator proposal here to allow UE implementation in both issues. We can list these are optional features to address Qualcomm concern.

NTT DOCOMO: support moderator recommendation. Then we can accommodate the different UE implementation.

VIVO: we do not think the optional features are necessary. It is easily as mandatory. We can simplify the options. We can accept the proposal for the first one to have the same values. UE may report more relaxed one if it needs the different values for different releases.

Huawei: two issues are not directly related. Based on the analysis, the same period as Rel-17 should be reused. For the first issue, we do not see the reason to use different values. For the second one, the optional features should not be introduced.

Mediatek: for issue 1-1, we consider Rel-18 UE supports more CA and DC combination and more hardware configuration. We support option 2. For capability of single TAG and multiple TAG, we agree that we can use the same value.

CMCC: for the second issue we do not understand how it can be easily mandated. If so, it seems that UE does not need interruption. The proposal is clear. There is no conclusion to mandate anything for the time being.

China Telecom: to not to report the advanced capability, we can remove the bullet. For switching period, even if the new capability is allowed and not reported, it is valuable to use Rel-17 values and we can reword. In rel-16 when we introduce downlink interruption, the baseline is that there is no interruption. Now the baseline is that UE cannot transmit. The situation is different. Regarding Apple comment on the scenario, if there are 3 bands configured, the low band is anchor band, maybe there is no switching on the anchor band and the switching will happen between middle and high bands and will impact the low band, which has impact on the deployment.

Qualcomm: we are talking about issue 1-2. It only matters to dual UL switching. Why are these issues coupled with each other?

Moderator: companies’ views do not change. The same principle behind applies.

Xiaomi: the switching period should be derived based on UE implementation. It seems necessary to study the impact of release18 whether the same or different UE architectures should be used. For first issue, we can compromise to Option 2. For the second issue, we agree not to introduce the capability.

OPPO: for the first issue, we are talking about UE to support Rel-16/Rel-17/Rel-18 features. If the UE design is not changed, the values can be the same. When there are changes, the switching time would be different. It is up to implementation. We would like to keep flexibility to UE. For the second one, we share the concern from other vendors.

ZTE: to OPPO, Tx switching for rel-16/17/18 features are dependent. The same principle and design apply for Rel-18. About the wording not intent to mandate the advanced feature in the future release, which makes no sense.

Apple: if we have low band as anchor band, this proposal of different capabilities, there will be period of time when low band there is transmit and the high/middle band are switched. Do we not need consider this scenario at all if the 0 is signaled?

- Discussion after GTW on Oct 12

  - Considering the positions from all companies, suggestions on compromised proposal are encouraged.

| Company | Comments |
| --- | --- |
| ZTE | For the compromised proposal, more inputs are needed at this moment to justify the introduction of such capability. |
| Qualcomm | Agree, the WI is so complex that additional options should be well justified. It also seems that the agreement on RTRC configuring sw period on one of the bands creates the situation when one band will never transmit regardless it is switching or not if one band is switching. Ref agreement in issue 1-4 |

- Issue 1-2A: Understanding of the scenarios in Issue 1-2

- Proposals

  - The following case a and case b need also to be studied in RAN4. (Xiaomi)

    - Case a: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band A”) to another band (name “band C”)

    - Case b: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band C”) to another band (name “band D”)

- Moderator’s observation

  - In moderator’s understanding, the difference of Case 1 and Case 2 in the previous RAN4 LS is that whether there is a band with the number of Tx chain unchanged due to switching. With this, the Case a and Case b mentioned by Xiaomi belong to case 1.

- Recommended WF

  - In Case a and Case b, the numbers of Tx chains in all the related bands are changed due to switching. Thus Case a and Case b belongs to case 1, and neither of Tx chains is expected to be used for transmission during the switching period.

| Company | Comments |
| --- | --- |
| China Telecom | Agree with the recommended WF. |
| OPPO | For Case a and b there is no Tx chain keep unchanged which is different from the Case1 and 2 in the RAN4 to RAN1 LS where one chain keep the band unchanged no matter it is one of the switching band or another band. Therefore, no need to further consider this in the discussion of Tx switching impact to the other chain. |
| Huawei | Agree with the recommended WF |
| Xiaomi | We think case a and case b are different case from case 1 and 2 mentioned in reply LS as explained by Oppo. The related UE behavior can be discussed together with Issue 1-1-2. |
| Sony | Agree with OPPO |
| Qualcomm | Agree with the recommended WF. |
| vivo | Agree with the recommended WF |
| MediaTek | Agree with OPPO |
| CMCC | Agree with recommended WF. |
| Apple | Would like to suggest adding the following to the recommended WF: “In order to conclude the work item, RAN4 will identify an example band combination for each Case a and Case b.” |
| ZTE | To be clear, we list the case definitions: Case 1: one Tx chain A->B, the other Tx chain maintain A or B Case 2: one Tx chain A-> B, the other Tx chain maintain C (a third band) Case a: one Tx chain A->B, the other Tx chain A->C Case b: one Tx chain A->B, the other Tx chain C->D It can be seen clearly that in Case 1 or Case 2, there is always a Tx chain maintaining a band, and the difference is that in Case 1 the maintained Tx chain is for one of the band in one band pair switched at the other Tx chain, and in Case 2, the maintained Tx chain is for the third band. In this sense, Case a or b are not covered by Case 1 or 2. However, if we look back the origin of Tx switching in Rel-16, the belief is that there is a band in UL-MIMO (2T) either before or after the switching, and Case a or b obviously does not have such a band. While having the same belief in minds, we don’t think Case a and b are the intended cases for Tx switching. |
| Samsung | Agree with the recommended WF. |

Sub-topic 1-3: RAN4 RF requirements

- RAN4 Agreements captured in the reply LS R4-2214464:

  - For concurrent UL transmission on 2 bands:

For UL Tx switching across 3 and 4 bands, the support of concurrent UL transmission on 2 (out of 3 or 4) bands at least requires UL CA support on the corresponding band pair(s) by the UE.

- Agreements captured in the WF R4-2214463:

  - No need to define RF requirements for UL CA with UL simultaneous transmission on 3 and 4 bands in the WI.

  - For the next meeting, encourage analysis/identification of the RAN4 RF requirements needed for this WI.

- Proposals

  - Proposals on generic time mask requirements

    - China Telecom: It is feasible to define the switching time mask requirements per band pair with 3 or 4 bands configured, and the specification work will be similar to Rel-16/17.

    - vivo: For Rel-18, RAN4 can keep the REl-16/17 way that only define requirements “switched UL” case.

    - RAN4 Rel-16/17 requirements currently only consider “switched UL” case, and doesn’t consider “dualUL” option which is defined in RAN1.

    - In case only one band pair would be involved during Tx Switching, the current requirements clauses can be reused with some clarifications.

    - In case multiple band pairs are involved during the Tx switching process, new requirements clauses may be added, which would also be impacted by RAN1’s undergoing complexity reduction discussion.

    - HW: The requirements for Tx switching Option 1 (“switched UL”) should be prioritized.

  - Proposals on concurrent UL transmission on 2 bands

    - vivo: Further discuss whether UL support on the corresponding band pair(s) could be set to a sufficient condition of support of concurrent UL transmission, at least from RAN4’s point of view.

  - Proposals on the need of additional requirements for CA option 2

    - China Telecom: Further discuss whether additional time mask requirements are needed for CA option 2 when the 2 Tx chains are switched between 2 different band pairs across the same time.

- Recommended WF

  - Encourage feedback on the above proposals.

| Company | Comments |
| --- | --- |
|  | - Proposals on generic time mask requirements - Proposals on concurrent UL transmission on 2 bands - Proposals on the need of additional requirements for CA option 2 |
| China Telecom | - Proposals on generic time mask requirements In Rel-16, the switching time mask is only defined for switchedUL case, considering that: 1) the UE RF behavior of switching one of the two Tx chains is similar for dualUL and switchedUL cases, 2) to simplify the requirements definition. This Rel-16 approach is then reused for Rel-17. For Rel-18, as starting point, we can also define the switching time mask for switchedUL case, to avoid a lot number of new switching cases and requirements in RAN4 spec. However, this does not imply only switchedUL is supported from RAN4 perspective. - Proposals on concurrent UL transmission on 2 bands - Proposals on the need of additional requirements for CA option 2 For CA option 2, when the 2 Tx chains are switched between 2 different band pairs across the same time, this UE behaviour cannot be covered and verified by the switching time mask for switchedUL. So, perhaps additional requirements are needed. |
| OPPO | - Proposals on generic time mask requirements Our understanding is current time mask can be applied to both switched UL or dual UL case even though the time mask looks like only specify the switched UL. Because, for the dual UL case, one of the UL will keep unchanged and the other UL switches, so the requirement only needs to be specified for the switched UL chain. - Proposals on concurrent UL transmission on 2 bands Not quite understand the proposal here. - Proposals on the need of additional requirements for CA option 2 In our view the Rel-16/17 time mask can be reused for single TAG. |
| Huawei | - Proposals on generic time mask requirements Considering the Tx switching is discussed per band pair as agreed in the last meeting. The time mask of band pair with 2 bands is ok. The RF requirements of 3/4 band need further consideration. - Proposals on concurrent UL transmission on 2 bands The proposal of concurrent transmission needs further clarification. Whether the proposal is related to other WGs. - Proposals on the need of additional requirements for CA option 2 |
| Nokia | Before jumping into conclusions, it is better to list all the possible switching cases and which cases cannot be covered by the existing requirements. |
| Xiaomi | - Proposals on generic time mask requirements We are generally ok with these two proposals. For Rel-18 switching time mask, the similar way as Rel-16/17 can be followed for switched UL case. But for dual UL case, not sure of what kind of new requirements should be added. - Proposals on concurrent UL transmission on 2 bands - Proposals on the need of additional requirements for CA option 2 Not sure of what kind of new requirements can be added for dual UL case. Currently our understanding is that the Rel-16/17 time mask can be reused for Rel-18. |
| Ericsson | We agree with Nokia. - Proposals on the need of additional requirements for CA option 2 A specific time mask for this case would be needed to verify functionality for option 2. The impact on the non-switched band may also have to be considered if closer in frequency to the switching band pair for example. |
| Qualcomm | When band = TX chains, the switching cases are simple and dual UL only means simultaneous transmiossions and switched UL requirements can cover the case when one TX is switched. Case with > bands involved than TX chain creates little more complexity and in requirements cases with dualUL should be considered. Especially different tx switching time case for example A+A -> B+C when sw period between A and B is not the same as A and C. Regarding the switched UL, the benefit analysis in R1-2204724 was done only for switched UL vs switching period. If dual UL would be used with a proper 26 dBm PA to enable 26+26 dBm dual UL cases and not need to settle for 2Tx based 26 dBm cases, the benefit would be even bigger and issues with longer or unequal switching times would not be there. |
| vivo | - Proposals on generic time mask requirements Tends to have similar understanding with China Telecom, that current requirements are based on switched UL case, and this can be Rel-18 requirements’ starting point. It is also helpful to first align this basic understanding which seems also in line with Noka’s proposal. |
| MediaTek | - Proposals on generic time mask requirements If the band pair of Tx switching is same with R-16/R-17, the requirements can be re-used. If the band pair of Tx switching for case 1 and case 2 are different with that in R-16/R-17, new requirements and UE capability signaling shall be introduced in R-18. - Proposals on concurrent UL transmission on 2 bands If the proposal means for UE supporting CA option 2, we are fine. Or the proposal needs further elaboration - Proposals on the need of additional requirements for CA option 2 The operation scenarios case a & b in Issue 1-2A is new in R-18. New requirements and UE capability signaling shall be introduced |
| CMCC | - Proposals on generic time mask requirements The switching time mask requirements apply to both switched UL and dual UL. - Proposals on the need of additional requirements for CA option 2 Some clarifications on requirements may be needed to cover this case, but we don’t think new time mask requirements are needed. |
| Apple | As a general observation, RAN4 should define the CA configurations corresponding to 3 and 4 band UL scenarios envisioned in this work item. These configurations should also include the restriction that simultaneous operation is supported in 2 bands at a time. We will anyway need to define requirements for at least one example band combination per scenario in order to complete the work item. – Proposals on generic time mask requirements We would prefer to prioritize UL MIMO for dual UL switching scenarios. For example, switching between 3, 4 bands with UL MIMO configured on each and switching between a two-band UL CA configuration and an UL MIMO configuration on a different (third) band. Having these scenarios defined as a baseline would help to develop TxD aspects as a second priority (and only if we have enough time in the release). – Proposals on concurrent UL transmission on 2 bands In our understanding, a UE with 2 Tx should support concurrent operation on 2 bands, provided only 1 Tx is needed per band. – Proposals on the need of additional requirements for CA option 2 We are open to discuss how to structure time mask requirements for this feature. |

- Status of Rel-16/17 requirements:

  - In Rel-16/17, common requirements on switching time mask are defined for “switched UL” case and “dual UL” case, and no additional specific requirements are defined for “dual UL” case.

    - Note: For the dual UL case, when one Tx chains keep unchanged and the other Tx chain switches, the requirement only needs to be specified for the switched Tx chain.

- Summary of round 1 discussion

  - Proposals on generic time mask requirements

    - Option 1: Follow Rel-16/17 approach, and define common switching time mask requirements for “switched UL” case and “dual UL” case (China Telecom, OPPO, HW, [Nokia], Xiaomi, [E///], vivo, MTK, CMCC)

      - Nokia, E///, vivo: On top of the existing/common requirements, better to list all the possible switching cases and which cases cannot be covered by the existing requirements

  - Proposals on the need of additional requirements for CA option 2

    - Option 1: May be needed when the 2Tx chains are switched between 2 different band pairs, and further discuss by taking the switching cases discussed in RAN1 into account (CTC, Nokia, E///, QC, MTK)

    - Option 2: not needed (OPPO, Xiaomi, CMCC)

- Moderator’s recommendation on Rel-18 RF requirements

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

GTW Agreement

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

Sub-topic 1-4: Location of switching period

- Related Rel-17 Agreements in RAN4 (in R4-2103234, LS on Rel-17 Tx switching enhancements):

  - In RAN4 #97e, the follow agreements have been reached for 2Tx-2Tx switching between two uplink carriers for SUL and UL CA:

    - Location of switching period

    - Reuse Rel-16 agreement for UL CA and SUL, i.e., semi-statically configure the switching period on one of the two uplink carriers

  - In RAN4 #97e and RAN4 #98e, the follow agreements have been reached for 1Tx-2Tx and 2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA:

    - Switching time mask related requirements

    - For the location of switching period, it is semi-statically configured on one of the two uplink bands.

- Proposals

  - Proposal 1: Inform RAN1 that RAN4 recommends RAN1 to discuss and decide the location of UL switching period for Rel-18 Tx switching. (China Telecom)

    - In Rel-18, the number of switching cases is increased with the number of configured bands, and also it would be possible that the two Tx chains can be switched between two different band pairs.

    - For Rel-18 Tx switching, it would more efficient to discuss the location of switching period in RAN1 together with the switching mechanism.

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
| China Telecom | In general, we think the Rel-16/17 approach of semi-static configuration of switching period on one of the band for each switching band pair should be followed. We can recommend RAN1 to reuse the Rel-16/17 approach and discuss further details for Rel-18 Tx switching scenario. |
| OPPO | Agree with CT comment. Rel-16/17 approach can be reused. |
| Huawei | Some companies have already raised the issue in RAN1, referring to R1-2208427, R1-2209455, R1-2210001 and R1-2210035. We may not have to inform RAN1 of it. We can wait for RAN1 discussion. |
| Nokia | Support the proposal 1. |
| Xiaomi | Ok to wait for RAN1 agreements given ongoing discussion in RAN1. |
| Qualcomm | Support the general idea of asking ran1 to discuss this but also the case of time misaligned bands and switching between multiple band with different timing should be included in the part of point of discussion. |
| vivo | Agree that basic semi-static configuration similar to Rel-16/17 can be used as baseline, and detailed adaptive solution can be discussed in RAN1. |
| CMCC | Agree to discuss in RAN1. |
| Apple | Coordination with RAN1 is important to avoid any misunderstanding about the location of the switching period. Also agree with Qualcomm that the case of different bands with different timing needs to be considered by RAN1 in their design. Perhaps an LS to this effect can be useful to send this meeting. |

- Summary of round 1 discussion (Oct 12)

  - Option 1: Inform RAN1 that RAN4 recommends RAN1 to discuss and decide the location of UL switching period for Rel-18 Tx switching. (China Telecom, OPPO, Nokia, QC, vivo, CMCC)

    - China Telecom, OPPO, vivo: Recommend to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario.

    - QC: also cover the case of time misaligned bands

  - Option 2: May not have to inform RAN1 of it, and can wait for RAN1 discussion. (Huawei, Xiaomi)

- Moderator’s recommendation (Oct 12)

  - Inform RAN1 that: RAN4 recommends to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

Huawei: we do not need to inform RAN1.

Qualcomm: it includes the note of multi-TAG cases. There may be two switching period. We need inform RAN1. I do not know why Huawei said RAN1 is discussing this and do not need send LS.

China Telecom: in the previous release, the location is decided in RAN4. It is clear to let Ran1 know RAN4 conclusions. To Qualcomm, this agreement applies to single TAG case. If there is anything else, we can inform RAN1 additionally.

Qualcomm: one LS to cover both case single-TAG and multi-TAG. I do not think single-TAG and multi-TAG are different thing. We should not separate the two things.

Apple: we agree to coordination with RAN1 is important. We should cover both cases. Informing RAN1 is important.

Huawei: we can wait for RAN1 progress.

Ericsson: we would like to clarify why there are two switching for multi-TAG and how it can work. What occurs? UE is not allowed to transmit on any band during switching. We could clarify the relation to the existing mask.

Qualcomm: to Ericsson, it is unclear how it can work. For A->B and C->D, do we allocate the switching period on B? Do you place the switching period on the band which does not transmit?

Ericsson: Nokia makes good proposal. It could be good to get understanding by collecting the scenario in some form.

Moderator: if we could not send LS to RAN1, shall we assume that this issue will be discussed in RAN4?

Moderator: add one more issue for two-TAG in the summary.

Ericsson: for multi-TAG case with 3/4 bands, our understanding is either to do switching between bands with the same TAG or do switching between bands with different TAGs. Network needs to know when UE needs not transmit on any carrier, which should be clear from specification. Dual-TAG should be more flexible.

GTW Agreement:

  - For single-TAG case, RAN4 agrees to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

Sub-topic 1-5: Applicability of DL interruption

- Related Rel-17 Agreements in RAN4 RF session (in R4-2103234, LS on Rel-17 Tx switching enhancements):

  - In RAN4 #97e, the follow agreements have been reached for 2Tx-2Tx switching between two uplink carriers for SUL and UL CA:

    - Applicability of DL interruption

    - Reuse the Rel-16 agreement:

    - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

    - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

  - In RAN4 #97e and RAN4 #98e, the follow agreements have been reached for 	1Tx-2Tx and 2Tx-2Tx switching between 1 carrier on band A and 2 contiguous aggregated carriers on band B for SUL and UL CA:

    - Applicability of DL interruption

    - The same agreements are applied for the scenarios with either one carrier or two contiguous aggregated carriers on band B

    - For SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

    - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

- Proposals

  - Proposal 1: Reuse the applicability of DL interruption agreed in Rel-16/17, with the blue text added considering the Rel-18 Tx switching scenario. (China Telecom)

    - For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

    - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

    - For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, DL interruption on the SUL and TDD bands is not required.

    - For Tx switching across 4 bands, when the status of Tx chain on the 4 bands before and after switching is {1, 0, 1, 0} and {0, 1, 0, 1} respectively, there is ambiguity on the switched band pair of each Tx chain. In RAN4 understanding, RAN1 will discuss and resolve this ambiguity.

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
| China Telecom | Agree with Proposal 1. |
| OPPO | Not quite understand the intention of last bullet. Does it mean RAN4 ask RAN1 to clarify the mapping between the two pair of bands like {band A, band B} -> {band C, band D}, is it A->C, B->D or A->D, B->C? The mapping of Tx chain to each band is UE implementation dependent, not sure the necessity of this mapping. If want to decide the location of Tx switching period, maybe can restrict the location on bands either before or after switching. |
| Huawei | We are ok with the first and second bullets in Proposal 1. The last bullet is up to the outcome of Issue 1-1-2. |
| Nokia | Support the proposal 1 |
| Qualcomm | Seems ok but what really is the proposal, adding the blue text? |
| vivo | Ok with the proposal. |
| MediaTek | Not agree with the sub-bullet: For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, DL interruption on the SUL and TDD bands is not required. The proposal is only applicable for SUL+TDD bands with single-TAG operation with same UL/DL configuration. The proposal may not be applicable for the case involving FDD bands. |
| CMCC | Ok with the added blue bullets. |
| Apple | With reference to the last bullet in blue text, Tx to band mapping is always up to UE implementation, so it is not clear what RAN1 needs to resolve. Would it be possible to update the wording to make the potential request to RAN1 more clear? |
| Samsung | Support P1. |

Sub-topic 1-6: UL-MIMO coherence

- Related Rel-17 Agreements in RAN4 (in R4-2120039, LS on UL-MIMO coherence for Rel-17 Tx switching):

  - Regarding the UL-MIMO coherence for the above Rel-17 Tx switching scenarios, RAN4 reached the following agreements:

    - For 3CC (within 2 bands) 1Tx-2Tx switching

    - The Rel-16 per BC UE capability for 2CC 1Tx-2Tx switching can be applied, i.e., the same capability applies to both Rel-16 and Rel-17 1Tx-2Tx switching.

    - If the per BC UL-MIMO capability for 1Tx-2Tx switching is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable bands for 1Tx-2Tx switching.

    - For 2CC and 3CC 2Tx-2Tx (within 2 bands) switching

    - Introduce new per band per BC UL-MIMO coherence capability, and the same capability applies to both 2CC and 3CC (within 2 bands) 2Tx-2Tx switching.

    - If the per band per BC UL-MIMO coherence capability is absent, the per BC UL-MIMO coherence capability for 1Tx-2Tx switching is applicable to 2Tx-2Tx switching.

    - If both the per band per BC UL-MIMO coherence capability and per BC UL-MIMO coherence are absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the bands for 2Tx-2Tx switching.

    - The above UL-MIMO coherence capability for Tx switching applies when UE is configured with uplink switching, and uplink switching is triggered by the switching mechanisms specified in RAN1 between last transmitted SRS and scheduled PUSCH transmission.

- Proposals

  - Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, define new per band per BC capability on UL-MIMO coherence. If the Rel-18 new per band per BC capability on UL-MIMO coherence is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL bands for Tx switching. (China Telecom)

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
| China Telecom | Agree with Proposal 1. |
| OPPO | Ok with proposal. Does Rel-17 per band per BC capability can be applied to this Rel-18 band combination? If it is, then no new capability is needed. |
| Huawei | Agree with the Proposal. |
| vivo | Basically Agree, still can have further discussion on the commonality with Rel-17 capability. |
| Apple | OK with the proposal |
| ZTE | Fine with the proposal. |
| Samsung | Okay with the proposal. |

Companies views’ collection for 1st round

Open issues

Provided under each issue in section 1.2

Summary for 1st round

- Sub-topic 1-1: Switching period for Tx switching across 3/4 bands

- Issue 1-1-1: Exact value of Tx switching period

Summary of round 1 discussion:

  - Option 1: Reuse the same switching period for each band pair as UE reported in Rel-16/17, i.e., UE does not need to report new or larger switching period per band pair for Rel-18. (CMCC, China Telecom, ZTE, HW, Xiaomi)

    - Note: with the understanding that the switching periods in Rel-18 could be different for different band pairs, according to the granularity of per band pair per BC agreed in the last meeting.

  - Option 2: Although the set of switching periods is the same as in Rel-16/17, a different value can be reported for each band pair in Rel-18 band combination with 3/4 bands. (QC, vivo, MTK, OPPO, Samsung, Sony, Apple)

  - QC question: how does UE report supporting these two cases? Does UE

Alt 1: declare first 2-band (rel-16/17) TX switching support for band pair (A+B) and tx switching time and then also 3(/4)-band (Rel-18) tx switching case (A+B+C)?

Or

Alt 2: does UE only report highest band number TX switching case and in it tx switching time for each band pair?

In Alt 2, then a separate IE would be needed for tx switching time in case only 2 bands are configured from this larger group but how do we prevent UE from declaring 2-band case separately from 3-band case, i.e. Alt 1. So it to us it seems ran4 has to agree the principle that UE has to report same value, not that vendors have to justify a new capability.

Recommendations for 2nd round:

  - Considering the positions from all companies, suggestions on compromised proposal are encouraged.

- Issue 1-1-2: Issue of two Tx chains switched between two different band pairs

Summary of round 1 discussion:

As commented by several companies, this issue can be discussed together with Issue 1-2A.

- Sub-topic 1-2: Impact from switching of one Tx chain on the other Tx chain

- Issue 1-2: Impact on the band with the number of Tx chain unchanged due to switching (Case 2)

Summary of round 1 discussion:

  - Option 1: Introduce optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged (i.e., one Tx chain is maintained on the band) during UL switching. (CMCC, China Telecom, Samsung, Nokia, NTT DCM)

  - Option 2: Do not define other optional features to allow the other Tx chain can be expected to be used for transmission on band C during the switching period. (vivo, HW, OPPO, Sony, QC, MTK, Apple)

Recommendations for 2nd round:

  - Considering the positions from all companies, suggestions on compromised proposal are encouraged.

- Issue 1-2A: Understanding of the scenarios in Issue 1-2

Recommendations for 2nd round:

Check if the following proposal is agreeable:

- When two Tx chains are switched between two different band pairs with different lengths of switching periods, as baseline UE assumption, neither of the two Tx chains is expected to be used for transmission during the larger one of the two switching periods.

- Whether advanced optional UE ability will be considered is related to the discussion in Issue 1-2.

- In round 2 or the next meeting, to help companies better understand the scenarios, operators are encouraged to provide an example band combination for Case a and Case b respectively.

  - Case a: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band A”) to another band (name “band C”)

  - Case b: One of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), and the other Tx chain is triggered to switch from one band (named “band C”) to another band (name “band D”)

  - NOTE: With no implication that the example band combination will be captured in RAN4 spec.

- Sub-topic 1-3: RAN4 RF requirements

GTW Agreement

  - Following Rel-16/17 approach, common switching time mask requirements for “switched UL” case and “dual UL” case are to be defined.

  - Further discuss whether to define additional requirements for CA option 2, by considering the switching cases to be introduced in RAN1.

- Sub-topic 1-4: Location of switching period

GTW Agreement:

  - For single-TAG case, RAN4 agrees to reuse the Rel-16/17 approach (i.e., semi-static configuration of switching period on one of the band for each switching band pair) and discuss further details for Rel-18 Tx switching scenario in RAN1.

- Sub-topic 1-5: Applicability of DL interruption

Summary of round 1 discussion:

The following updated proposal 1 is ok to companies, while the text in purple requires further discussion.

  - Updated Proposal 1: Reuse the applicability of DL interruption agreed in Rel-16/17, with the purple text added to be discussed further.

    - For combinations of SUL+TDD and TDD+TDD CA with the same UL-DL pattern, DL interruption is not required.

    - For the other duplex mode combinations, define different capabilities for UEs with and without DL interruption.

    - UE capability is defined as per band per band combination for each band pair supporting UL Tx switching.

    - For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, DL interruption on the SUL and TDD bands is not required.

    - For Tx switching across 4 bands, when the status of Tx chain on the 4 bands before and after switching is {1, 0, 1, 0} and {0, 1, 0, 1} respectively, there is ambiguity on the switched band pair of each Tx chain. In RAN4 understanding, RAN1 will discuss and resolve this ambiguity.

Summary of views on the newly added purple text:

    - For SUL+TDD+FDD combinations, for Tx switching between SUL and TDD bands, DL interruption on the SUL and TDD bands is not required.

OK with this sub-bullet: China Telecom, OPPO, Huawei, Nokia, Qualcomm, vivo, CMCC, Apple, Samsung

Not agree with this sub-bullet: MTK (The proposal may not be applicable for the band combination involving FDD bands)

    - For Tx switching across 4 bands, when the status of Tx chain on the 4 bands before and after switching is {1, 0, 1, 0} and {0, 1, 0, 1} respectively, there is ambiguity on the switched band pair of each Tx chain. In RAN4 understanding, RAN1 will discuss and resolve this ambiguity.

OK with this bullet: China Telecom, Nokia, Qualcomm, vivo, CMCC, Samsung

Not agree with the sub-bullet: OPPO, HW, Apple (The mapping of Tx chain to each band is UE implementation dependent.)

Recommendations for 2nd round:

- Further discuss the purple text.

- Sub-topic 1-6: UL-MIMO coherence

Summary of round 1 discussion:

All companies are ok with the proposal 1, and 2 companies suggest to further discuss the commonality with Rel-17 capability.

  - Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, define new per band per BC capability on UL-MIMO coherence. If the Rel-18 new per band per BC capability on UL-MIMO coherence is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL bands for Tx switching. (China Telecom, OPPO, HW, vivo, Apple, ZTE, Samsung)

    - OPPO, vivo: Does Rel-17 per band per BC capability can be applied to this Rel-18 band combination? To have further discussion on the commonality with Rel-17 capability.

Tentative agreements:

  - Agree on updated Proposal 1: For Rel-18 Tx switching across 3 or 4 bands, apply define new per band per BC capability on UL-MIMO coherence. If the Rel-18 new per band per BC capability on UL-MIMO coherence is absent, the existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL bands for Tx switching.

  - Further discuss whether the Rel-17 per band per BC capability can be applied to this Rel-18 band combination.

Recommendations for 2nd round:

- Further discuss.

Discussion on 2nd round

Topic #2: Tx switching with multiple TAGs

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215569 | Qualcomm Incorporated | Title: UL Outage time considerations for 2 TAG We made following observations: Observation 1: True Outage time can accommodate transient periods in some cases but not necessary all. Observation 2: It is more evident that UE will need the flexibility to optimise 2 band single TAG switching time and still support 4 band TX switching feature with simultaneous switching on both TX chains. Observation 3: With 3 and 4 band switching feature two switching period handling need to be discussed - When TX1 is switched from band A to band B and TX2 is switched from band C to D - TX1 is switched from band A to B and TX2 is switched from band A to C (Band A had UL MIMO) for example two switching periods on two band pairs that are not in same TAG And made the following proposals: Proposal 1: Refine the agreement that “band C is not expected to be available for transmissions for the duration of the switching time + any partial symbol on any band overlapping with switching time. Proposal 2: UE is allowed to optimise TX switching time depending on number of bands in a TX switching band combination Proposal 3: UE requirements are written in such way that network defines UE behaviour. |
| R4-2215953 | Ericsson, Sony | Title: Uplink TX switching with dual-TAG across two bands For switching across two bands with dual-TAG Observation 1: the UL switching time is the same for single TAG and dual-TAG cases, the existing uplinkTxSwitchingPeriod-r16, the TxSwitchingOption-r16 and the switching-period location capabilities can be reused for the dual-TAG scenario. Observation 2: for uplink switching, the network must configure a slot offset K2 to accommodate the Toffset required (the UE processing procedure time defined for the uplink transmission triggering the switch) including any effect of timing differences between the switched carriers and the switching period. This is no different from the general case for PUSCH preparation with multiple component carriers configured and timing differences up to those specified in 38.133 for non-collocation. Observation 3: supportedNumberTAG (optional) can also be reported for band combinations with switching. Proposal 1: modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs. For the three-band case with dual-TAG: Observation 4: the “three-band” case of an FDD-TDD combination with intra-band non-contiguous CA in the TDD band, two band entries within the TDD, could also be considered; the carriers in the FDD and TDD band belonging to separate TAGs. |
| R4-2215954 | Ericsson | Title: Introduction of ON/OFF time mask for TX switching across two bands with dual-TAG |
| R4-2215797 | China Telecom | Title: UL Tx switching across 3/4 bands and Tx switching with 2 TAGs We had the following observations and proposals for Tx switching with two TAGs: For the UL switching time: Observation 3: With the time difference excluded from the switching time, the UL switching time is decided by UE implementation of switching from RF perspective. Proposal 10: The UL switching time is the same for single TAG and 2 TAGs. Proposal 11: Reuse the existing wording in TS 38.214, and agree the following bullet: - UE may omit the uplink transmissions corresponding to any TAG during the UE switching time. For the UL outage time: Observation 4: For the 2 bands with 2-TAG, the UL transmission timing difference at UE is half of the TA difference. Observation 5: The MTTD of 34.6 us for inter-band NR CA is more than 10 times of the actual propagation delay difference of UE to two BSs in typical deployment scenario. Proposal 12: For deriving the UL outage time, use half of the difference between the actual TAs on the two TAGs. Proposal 13: For the timing and measurement error, 3 aspects need to be considered, including: a) BS synchronization accuracy, b) UE transmit timing error, c) TA quantization error. Proposal 14: For BS synchronization accuracy for synchronized network, the BS synchronization accuracy requirement of 3us defined in clause 7.4.2 of TS 38.133 can be used. Proposal 15: For UE transmit timing error, the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs. Proposal 16: For TA quantization error, as defined in TS 38.213, it can be up to 5.2 us for 15kHz SCS. For PUSCH preparation time: Proposal 17: The timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed. If needed, RAN4 can further check with RAN1 whether this understanding is correct. |
| R4-2216042 | Xiaomi | Title: Discussion on RF requirements for multi-carrier enhancement Proposal 3: The switching time for UL Tx switching should be the same for single TAG and 2 TAG. - The switching time mask should not capture the timing difference cause by two TAGs. The similar issue was also discussed in Rel-17 NR SL. |
| R4-2216160 | MediaTek Inc. | Title: Continue discussion on UL Tx switching across 3 or 4 bands in Rel-18 Observation 1: NR DC was not considered in R-16/R-17 Tx switching Proposal 1: Since NR DC was not considered in R-16/R-17 Tx switching, it is not considered for multi-TAG operation for R-18 Multi-carrier enhancements |
| R4-2216433 | OPPO | Title: R18 Discussion on Tx switching Observation 2: TA difference is NW configuration dependent and is not UE hardware requirement. In Rel-17 V2X the time mask for switching between uu and SL only contains the switching period and no TA difference included. Proposal 2: Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. Observation 3: MTTD, Timing and measurement errors are all RRM parameters which are to be considered in the outage time, should be discussed in RRM session. Proposal 3: The “Outage time” discussion should be moved to RRM session considering the parameters other than switching time are all RRM parameters like MTTD, Timing and measurement errors. Observation 4: It is unclear when multi-TAG UL Tx switching is configured to UE whether TA difference will be considered in NW scheduling. Proposal 4: When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling. |
| R4-2216654 | Huawei, HiSilicon | Title: Discussion on Multi-carrier enhancements with multiple TAG Proposal 1：UL switching time is the same for single TAG and multiple TAGs. Proposal 2：The principle of intra-band V2X con-current operation time mask can be followed for UL Tx switching with multiple TAGs. Proposal 3：The impact of Tx switching with multiple TAG can be considered as scheduling restriction in RAN4 specification. There is no impact on RAN1 specification is foreseen from RF perspective. Proposal 4：For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers. |
| R4-2216655 | Huawei, HiSilicon | Title: Draft CR for 38.101-1 to clarify the time mask for switching with multiple TAGs |

Open issues summary

Sub-topic 2-1: UL switching time and outage time

- Issue 2-1-1: UL switching time

- Agreement captured in the WF R4-2215163:

  - Agreement:

    - RAN4 further discuss if the UL switching time is the same for single TAG and 2 TAGs

    - UL switching time should not include timing difference up to MTTD between two TAGs.

  - FFS:

    - UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.

- Proposals on UL switching time

  - Option 1: The UL switching time is the same for single TAG and dual-TAG cases (E///, China Telecom, Xiaomi, HW)

    - E///: the existing uplinkTxSwitchingPeriod-r16, the TxSwitchingOption-r16 and the switching-period location capabilities can be reused for the dual-TAG scenario. supportedNumberTAG (optional) can be reported separately for band combinations with switching.

    - China Telecom: With the time difference excluded from the switching time, the UL switching time is decided by UE implementation of switching from RF perspective.

    - Xiaomi: The switching time mask should not capture the timing difference cause by two TAGs. The similar issue was also discussed in Rel-17 NR SL.

- Proposals on the wording in the FFS bullet

  - Proposal 1: UE requirements are written in such way that network defines UE behaviour. (QC)

  - Proposal 2: Reuse the existing wording in TS 38.214, and agree the following bullet (China Telecom)

    - UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.

  - Proposal 3: When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling. (OPPO)

  - Proposal 4: For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers. (HW)

- Recommended WF

  - For UL switching time, can we agree the option 1?

  - For the wording in the FFS bullet, further feedback is encouraged.

| Company | Comments |
| --- | --- |
|  | - For UL switching time, can we agree the option 1? - For the wording in the FFS bullet, further feedback is encouraged. |
| China Telecom | - For UL switching time, can we agree the option 1? Support option 1. - For the wording in the FFS bullet, further feedback is encouraged. It would be easier to reach agreement if we reuse the wording the existing spec, as proposed in Proposal 2. We also agree with the technical points in Proposal 1/3/4. |
| OPPO | - For UL switching time, can we agree the option 1? Ok with Option 1. - For the wording in the FFS bullet, further feedback is encouraged. Ok with proposal 2/3/4, regarding proposal 1 it is vague, but looking at the paper it actually means “specify UE behaviour such that UE respects UL grants in all situations and requirements for UE are written such that network assigns grants according to the defined UE behaviour”, we are ok with this part. |
| Huawei | - For UL switching time, can we agree the option 1? We are ok with option 1. - For the wording in the FFS bullet, further feedback is encouraged. It is up to gNB scheduling restriction. We are open to Proposal 1,3 and 4 For Proposal 2, does it mean that TS 38.214 is to be revised? Our preference is clarification on RAN4 spec for scheduling restriction. |
| Nokia | - For UL switching time, can we agree the option 1? Support option 1 - For the wording in the FFS bullet, further feedback is encouraged. We tend to agree with Proposal 2 at least in terms of “Reuse the existing wording in TS 38.214” However, we are afraid that the captured text in the last meeting WF looks odd. It’s not understandable specifically for a part of “corresponding to any TAG”. Pure RF UL switching period itself is irrelevant from TAGs. We don’t see the reason to include that part… For proposal 1, 3 and 4, they are a kind of options that network may take, but they must not be mixed with UE RF requirements. |
| Ericsson | - For UL switching time, can we agree the option 1? We support Option 1. We also agree with the technical points raised in the proposals. Proposal 1: the network should make sure that the UE can omit transmission on any carrier during the switching period preceding the time T_0 (38.214) when the UE is expected to transmit on a carrier after switching. Proposal 3: the network has to account for timing differences between the carriers/cells also in the non-switched case. Proposal 4: this is consistent with Proposal 2. - For the wording in the FFS bullet, further feedback is encouraged. We support Proposal 2, the same wording as 38.214 that governs UE behavior. |
| Xiaomi | - For UL switching time, can we agree the option 1? Support option 1. |
| Sony | For UL switching time, can we agree the option 1? Yes For the wording in the FFS bullet? Support Option 2 |
| Qualcomm | Swtime option 1 is ok For FFS text: we would prefer to put the scheduling on network responsibility. Ericsson has a good language in their paper R4-2215953 “symbols at the trailing edge of the subslots/slots on carrier 2 must be blanked by the gNB during the switching period prior in the switch back to carrier 1.” Even it is not in the changes to TS they propose in the same document. But we should talk what kind of CR is needed, I see many different discussions and single Tag and multi Tag spec changed may not be different. |
| vivo | For UL switching time, can we agree the option 1? OK |
| MediaTek | - For UL switching time, can we agree the option 1? We are OK with option 1 - For the wording in the FFS bullet, further feedback is encouraged. The proposals are not controversial. We share QCM’s view that we also prefer to put the scheduling on network responsibility |
| CMCC | For UL switching time: OK with option1. |
| Apple | Option 1 is fine for us. For dual TAG scenarios, we need to make sure that the network will take care of the timing difference between the bands by its scheduling. The specification should capture this as a side condition on network behavior. As Qualcomm commented, the following text from Ericsson’s contribution can be a good starting point: “Since the uplink subslots/slots on carrier 1 are ‘early’ compared to those of carrier 2, this means that symbols at the trailing edge of the subslots/slots on carrier 2 must be blanked by the gNB during the switching period prior in the switch back to carrier 1. This is possible e.g. for PUSCH transmissions using mapping A since the gNB is aware of the timing advance of the different TAG that are configured by the gNB.” |

- Summary of round 1 discussion (Oct 12)

  - Option 1: The UL switching time is the same for single TAG and dual-TAG cases (E///, China Telecom, Xiaomi, HW, OPPO, Nokia, Sony, QC, vivo, MTK, CMCC, Apple)

- Moderator’s recommendation (Oct 12)

  - Agree option 1

GTW Agreement:

- The UL switching time is the same for single TAG and dual-TAG cases

- Issue 2-1-2: UL outage time and RAN4 CR text

- Agreement captured in the WF R4-2215163:

  - Factors for UL outage time discussed in RF session, to be further checked in RRM session

    - UL switching time (UE capability)

    - The difference between the TA on the two TAGs, up to MTTD

    - Timing and measurement error

- Proposals on RAN4 CR text

  - Option 1: Modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs. (E///, see CR in R4-2215954)

  - Option 2: Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. (OPPO, Huawei - see CR in R4-2216655)

    - OPPO: TA difference is NW configuration dependent and is not UE hardware requirement. In Rel-17 V2X the time mask for switching between uu and SL only contains the switching period and no TA difference included.

    - HW: The impact of Tx switching with multiple TAG can be considered as scheduling restriction in RAN4 specification. There is no impact on RAN1 specification is foreseen from RF perspective.

- Proposals on UL outage time

  - Proposal 1 (China Telecom)

    - For deriving the UL outage time, use half of the difference between the actual TAs on the two TAGs.

    - For the timing and measurement error, 3 aspects need to be considered, including: a) BS synchronization accuracy, b) UE transmit timing error, c) TA quantization error.

    - For BS synchronization accuracy for synchronized network, the BS synchronization accuracy requirement of 3us defined in clause 7.4.2 of TS 38.133 can be used.

    - For UE transmit timing error, the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs.

    - For TA quantization error, as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS.

  - Proposal 2: The “Outage time” discussion should be moved to RRM session considering the parameters other than switching time are all RRM parameters like MTTD, Timing and measurement errors. (OPPO)

  - Proposal 3: Refine the agreement that “band C is not expected to be available for transmissions for the duration of the switching time + any partial symbol on any band overlapping with switching time. (QC)

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
|  | - Proposals on RAN4 CR text - Proposals on UL outage time |
| China Telecom | - Proposals on RAN4 CR text Open for discussion. - Proposals on UL outage time For Proposal 3 from QC, when the UL outage time is in the unit of OFDM symbols, we agree with the proposal 3. |
| OPPO | - Proposals on RAN4 CR text Option 2 is preferred. - Proposals on UL outage time Proposal 2 is preferred. This is RRM expertise instead of RF. |
| Huawei | - Proposals on RAN4 CR text Option 2. - Proposals on UL outage time For UL outage time related factors that affect scheduling, how it should be reflected in the spec needs further discussion. We prefer TS38.101-1 referring to CR R4-2216655. As mentioned by OPPO, the parameters are all RRM related. We are also open that some factors related to scheduling restriction are reflected in RRM spec. For Proposal 3, in general, we are ok with the consideration. A question to be clarified, which part of spec would be impact for the proposal. |
| Nokia | - Proposals on RAN4 CR text Need more discussion. E.g., why can a UE keep transmitting carrier 1 in a period enclosed in blue rectangle? - Proposals on UL outage time We support option 2 |
| Ericsson | - Proposals on RAN4 CR text Option 1 as proponent. The mask should be specified so that the UE functionality can be properly tested with timing differences between the uplink carriers. The mask should not preclude any other scheduling constraint for operation in the field for avoiding transmission in the switching period (the gNB should make sure the UE does not have to transmit in the switching period and account for the timing difference). We are considering a modification of the proposed masks in R4-2215954 to align further with the single-TAG case. - Proposals on UL outage time We agree with Proposal 1 but note that all aspects are already included in MTTD specified for inter-band in 38.133: MTTD = DTprop (30 us) + BS TAE (3 us) + Te,2UL where the worst-case timing error between two active UL for SCS = 15k on both is Te,2UL = 2 (TARES + TAeUE + Te) = 2 (512 + 256 + 768) Tc = 3072 Tc ≈ 1.6 us with TARES the TA resolution error (from 38.213, the resolution defined as 16*64*Tc/2µ that yields 1024Tc (+/- 512Tc) TAeUE the timing advance adjustment accuracy setting error (38.133 clause 7.3.2.2) Te the UE timing error limit (38.133, Table 7.1.2-1) Hence MTTD = 30 + 3 + 1.6 us = 34.6 us as specified in 38.133, which is slightly shorter than one symbol duration at SCS = 30k. The outage time with dual-TAG is therefore up to a switching period + MTTD rounded upwards to a number of symbols in the relevant numerology. In practice the time T0 at which the UE is expected to transmit after a switch coincides with symbol boundaries. The outage time is not the absolute TA (twice the DTprop) but related to the timing difference between the carriers at the UE. We also note that the maximum DTprop = 30 us in the MTTD corresponds to 9 km path difference between the sites. Proposal 3 is reasonable particularly for ‘smaller’ frequency separations to the switching band pair (dual-TAG with more than two bands). |
| Xiaomi | - Proposals on RAN4 CR text Prefer option 2 - Proposals on UL outage time Prefer proposal 2. |
| Sony | Proposals on RAN4 CR text Option 1 |
| MediaTek | - Proposals on RAN4 CR text Prefer option 2 - Proposals on UL outage time Proposal 2 |
| Apple | Agree to discuss Option 1 further. One advantage we see with this proposal is that the specification on Tx mask would define a side condition which specifies network behavior: if the network accommodates the TA difference by scheduling, then the UE can fulfill the corresponding Tx mask requirement. To align this with the RRM discussion currently ongoing, our understanding is that the time mask for carriers involved in the switching is captured in the RF specification, while impact on other carriers is captured in the RRM specification. |
| Samsung | Prefer Option 1 for RAN4 CR text. |

- Issue 2-1-2A: RAN4 CR text

- Summary of round 1 discussion (Oct 12)

  - Option 1: Modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs. (E/// - see CR in R4-2215954, Sony)

    - The mask should be specified so that the UE functionality can be properly tested with timing differences between the uplink carriers.

  - Option 2: Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. (OPPO, Huawei - see CR in R4-2216655, Xiaomi, MTK)

    - The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

GTW Discussion:

Apple: for dual-TAG case, UE depends much on network scheduling. It is very valuable to capture in the spec as the side condition. Option 1 is the good way forward for the discussion.

Ericsson: we also support Option 1. We would need some kind of verification. We have the long discussion on the outage time. It would be better to capture it in the spec.

Mediatek: we do not oppose to have further discussion on the time mask. To capture all of these in the spec, that may easily make readers confusing.

Qualcomm: for option 2, why do we use the same time mask? How can we handle this case?

Ericsson: to MTK, from gNB perspective, in the single-TAG case, the switching period proceed the time of T_0 and that is the location for switching. For mutli-TAG, network must maintain two different DL timing. The additional blanking is needed to be made. We should make sure that the blanking should be taken on one carrier and the relation of the switching period location to T_0.

Huawei: the switching period of multi-TAG can be the same as single TAG case. TA can be considered in gNB schedule restriction.

Mediatek: we think if there is time mask blanking. We wonder if it should be captured in technique report rather in the spec.

- Discussion after GTW on Oct 12

| Company | Comments |
| --- | --- |

Sub-topic 2-2: PUSCH preparation time

- Observation captured in the WF R4-2215163:

  - The PUSCH preparation time includes the effect of TA and difference between multiple active component carriers in addition to the switching period (Rel-16)

- Proposal

  - Proposal 1: The timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed. (E///, China Telecom)

    - E///: For uplink switching, the network must configure a slot offset K2 to accommodate the Toffset required (the UE processing procedure time defined for the uplink transmission triggering the switch) including any effect of timing differences between the switched carriers and the switching period. This is no different from the general case for PUSCH preparation with multiple component carriers configured and timing differences up to those specified in 38.133 for non-collocation.

    - China Telecom: If needed, RAN4 can further check with RAN1 whether this understanding is correct.

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
| China Telecom | Proposal 1. |
| OPPO | Proposal 1 is ok. |
| Huawei | We agree with E/// and CTC that RAN1 specification update is not needed. |
| Nokia | Agree with proposal 1 |
| Ericsson | Proposal 1 |
| Xiaomi | Agree with proposal 1 |
| Sony | Proposal 1 |
| vivo | OK with Proposal 1 |
| Apple | It is important to ensure that the understanding between RAN1 and RAN4 are aligned on PUSCH preparation time. |

- Summary of round 1 discussion (Oct 12)

  - Proposal 1: The timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed. (E///, China Telecom, OPPO, HW, Nokia, Xiaomi, Sony, vivo)

- Moderator’s recommendation (Oct 12)

  - Agree proposal 1.

GTW Agreement:

- The timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed.

Sub-topic 2-3: Sceanrio

- Proposals

  - E///: the “three-band” case of an FDD-TDD combination with intra-band non-contiguous CA in the TDD band, two band entries within the TDD, could also be considered; the carriers in the FDD and TDD band belonging to separate TAGs.

  - MTK: Since NR DC was not considered in R-16/R-17 Tx switching, it is not considered for multi-TAG operation for R-18 Multi-carrier enhancements.

- Recommended WF

  - Encourage feedback

| Company | Comments |
| --- | --- |
| China Telecom | For E///’s proposal, in our understanding, intra-band non-contiguous CA is not covered in the Tx switching scenario. For MTK proposal, we agree not to consider NR-DC. |
| OPPO | According to WID, seems CA is not part of Rel-18 enhancement, and DC neither. |
| Huawei | We share the similar view with China Telecom and OPPO for E///’s proposal. Agree with MTK’s proposal. |
| Ericsson | The intra-band non-contiguous CA in the TDD band is two carriers with UL-MIMO on both carriers that could be widely separated e.g. in n77. However, the actual performance gain achieved with intra-band TDD switching (2T-2T) in a FDD-TDD configuration is uncertain. We agree with MTK. |
| Xiaomi | For E///’s proposal we share the similar view with CTC. For MTK’s proposal, we agree. |
| vivo | Agree with MTK. |
| Apple | We don’t recommend extending the work item scope any further to include intra-band non-contiguous CA and NR DC. |

Sub-topic 2-4: Location of switching period

- Recommended WF

  - Encourage proposals on the location of switching period for Tx switching across 3 or 4 bands with 2-TAG.

| Company | Comments |
| --- | --- |

Companies views’ collection for 1st round

Open issues

Provided under each issue in section 2.2

Summary for 1st round

- Sub-topic 2-1: UL switching time and outage time

- Issue 2-1-1: UL switching time

GTW Agreement:

- The UL switching time is the same for single TAG and dual-TAG cases

- Issue 2-1-1A: on the wording in the FFS bullet

Background (captured in the WF R4-2215163):

  - FFS:

    - UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.

Summary of round 1 discussion

  - Proposal 1: UE requirements are written in such way that network defines UE behaviour (i.e., specify UE behaviour such that UE respects UL grants in all situations and requirements for UE are written such that network assigns grants according to the defined UE behavior). (QC)

    - CTC, E///, OPPO: Agree with technical point.

  - Proposal 2: Reuse the existing wording in TS 38.214, i.e., “UE may omit the uplink transmissions corresponding to any TAG during the UE switching time.” (China Telecom, OPPO, E///, Sony)

    - Nokia: We tend to agree with Proposal 2 at least in terms of “Reuse the existing wording in TS 38.214” However, we are afraid that the captured text in the last meeting WF looks odd. It’s not understandable specifically for a part of “corresponding to any TAG”.  Pure RF UL switching period itself is irrelevant from TAGs. We don’t see the reason to include that part… For proposal 1, 3 and 4, they are a kind of options that network may take, but they must not be mixed with UE RF requirements.

  - Proposal 3: When multi-TAG UL Tx switching is configured to UE the precondition should be NW will take the TA differences into account in the UL transmission scheduling. (OPPO)

    - CTC, E///: Agree with technical point.

  - Proposal 4: For the case of Tx switching with multi-TAG for the two uplink carriers, the UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols that overlaps with the switching period on both the carriers. (HW)

    - CTC, E///: Agree with technical point.

  - Proposal 5: symbols at the trailing edge of the subslots/slots on carrier 2 must be blanked by the gNB during the switching period prior in the switch back to carrier 1. (QC, Apple)

    - Apple: For dual TAG scenarios, we need to make sure that the network will take care of the timing difference between the bands by its scheduling.  The specification should capture this as a side condition on network behaviour.

Recommendations for 2nd round:

- Further discuss the possible wording for the WF and the CR.

- Issue 2-1-2: UL outage time and RAN4 CR text

- Issue 2-1-2A: RAN4 CR text

Summary of round 1 discussion

  - Option 1: Modify the time mask for TX switching to include the case of dual TAG with different timing advance on the two TAGs. (E/// - see CR in R4-2215954, Sony, Samsung, Apple - to discuss further based on option 1, QC - agree to introduce new time mask but the E/// CR needs to be modified)

    - E///: The mask should be specified so that the UE functionality can be properly tested with timing differences between the uplink carriers.

    - Apple: To define a side condition which specifies network behaviour. The time mask for carriers involved in the switching is captured in the RF specification, while impact on other carriers is captured in the RRM specification.

  - Option 2: Apply same approach as in R17 V2X that the time mask only contains the UE hardware requirement (switching period), and no TA difference included. (OPPO, Huawei - see CR in R4-2216655, Xiaomi, MTK)

    - The impact of Tx switching with multiple TAGs can be considered as scheduling restriction.

Recommendations for 2nd round:

- Further discuss.

- Issue 2-1-2B: UL outage time

Summary of round 1 discussion

  - Proposal 1 (China Telecom)

    - For deriving the UL outage time, use half of the difference between the actual TAs on the two TAGs.

    - For the timing and measurement error, 3 aspects need to be considered, including: a) BS synchronization accuracy, b) UE transmit timing error, c) TA quantization error.

    - For BS synchronization accuracy for synchronized network, the BS synchronization accuracy requirement of 3us defined in clause 7.4.2 of TS 38.133 can be used.

    - For UE transmit timing error, the requirement defined in clause 7.1 of TS 38.133 can be seen an upper bound, and the sum of maximum UE transmit timing error is 1.56 us for the carriers with 2 TAGs.

    - For TA quantization error, as defined in TS 38.213, it can be up to 5.2 us for 15 kHz SCS.

E/// comment: We agree with Proposal 1 but note that all aspects are already included in MTTD specified for inter-band in 38.133.

  - Proposal 2: The “Outage time” discussion should be moved to RRM session considering the parameters other than switching time are all RRM parameters like MTTD, Timing and measurement errors. (OPPO, Nokia, Xiaomi, MTK)

  - Proposal 3: Refine the agreement that “band C is not expected to be available for transmissions for the duration of the switching time + any partial symbol on any band overlapping with switching time. (QC)

    - China Telecom, Huawei, E///: Agree with the technical point when the UL outage time is in the unit of OFDM symbols.

  - Proposal 4: No need to capture the exact UL outage time in the spec, since it affect BS scheduling only according to our CR in R4-2216655. (HW)

Recommendations for 2nd round:

- Further discuss.

- Sub-topic 2-2: PUSCH preparation time

GTW Agreement:

- The timing difference effect and switching period have already been considered in the PUSCH preparation time in the current specification, and further specification update is not needed.

- Sub-topic 2-3: Sceanrio

Summary of round 1 discussion

  - E///: the “three-band” case of an FDD-TDD combination with intra-band non-contiguous CA in the TDD band, two band entries within the TDD, could also be considered; the carriers in the FDD and TDD band belonging to separate TAGs.

    - CTC, OPPO, HW, Xiaomi, Apple: intra-band non-contiguous CA is not covered in the Tx switching scenario.

  - MTK: Since NR DC was not considered in R-16/R-17 Tx switching, it is not considered for multi-TAG operation for R-18 Multi-carrier enhancements.

    - CTC, OPPO, HW, MTK, Xiaomi, vivo, Apple: Support

Tentative agreements:

- In RAN4 understanding:

  - The band combinations containing intra-band non-contiguous CA is not considered in the WI.

  - NR-DC with multi-TAG operation is not considered in the WI.

- Sub-topic 2-4: Location of switching period

Recommendations for 2nd round:

  - Encourage proposals on the location of switching period for Tx switching across 3 or 4 bands with 2-TAG.

Discussion on 2nd round

- Recommendations for Tdocs

  - 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on … | YYY |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |
|  | WF on UL Tx switching across 3/4 bands with single TAG | China Telecom |  |
|  | LS on Rel-18 UL Tx switching | China Telecom | To: RAN1, RAN2 Note: 2-TAG specific issues can also be covered if agreeable. |
|  | WF on UL Tx switching with multiple TAGs | Ericsson |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215495 |  | Switching for UL Tx switching across 3 or 4 bands | CMCC | Noted |  |
| R4-2215569 |  | UL Outage time considerations for 2 TAG | Qualcomm Incorporated | Noted |  |
| R4-2215613 |  | On R18 CA enhancement - RRM | Apple | Noted |  |
| R4-2215797 |  | UL Tx switching across 3/4 bands and Tx switching with 2 TAGs | China Telecom | Noted |  |
| R4-2215881 |  | Discussion on Tx switching across 3 or 4 bands | ZTE Wistron Telecom AB | Noted |  |
| R4-2215953 |  | Uplink TX switching with dual-TAG across two bands | Ericsson, Sony | Noted |  |
| R4-2215954 |  | Introduction of ON/OFF time mask for TX switching across two bands with dual-TAG | Ericsson | Postponed |  |
| R4-2216042 |  | Discussion on RF requirements for multi-carrier enhancement | Xiaomi | Noted |  |
| R4-2216119 |  | Discussion on UE RF requirements for TxSwitching up to 3 or 4 bands | vivo | Noted |  |
| R4-2216160 |  | Continue discussion on UL Tx switching across 3 or 4 bands in Rel-18 | MediaTek Inc. | Noted |  |
| R4-2216433 |  | R18 Discussion on Tx switching | OPPO | Noted |  |
| R4-2216653 |  | Discussion on Multi-carrier enhancements with single TAG | Huawei, HiSilicon | Noted |  |
| R4-2216654 |  | Discussion on Multi-carrier enhancements with multiple TAG | Huawei, HiSilicon | Noted |  |
| R4-2216655 |  | Draft CR for 38.101-1 to clarify the time mask for switching with multiple TAGs | Huawei, HiSilicon | Postponed |  |
| R4-2216714 |  | Further Discussion on Switching Time and other RF Aspects for UL Tx Switching Across 3 or 4 Bands | Samsung | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

  - 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents