3GPP TSG-RAN WG4 Meeting #104-bis-e                                                    	R4-2217065

Online, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 37.718-11-11: DC_28_n20

Agenda item:			5.3.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-11-11 to include DC_28_n20.

The proposed TIB,c, RIB and MSD values were derived from those present in 38.101-3 for DC_20_n28.

# 2. Reference

# 3. Text Proposal

<Start of Text Proposal>

### 6.1.X	DC_28_n20

#### 6.1.X.1	Configuration for DC

Table 6.1.X.1-1: Inter-band DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

#### 6.1.1.2	Maximum output power for DC

Table 6.1.X.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n20A |  |  |  |  | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |

#### 6.1.1.3	Spurious emission band UE co-existence for DC

The requirements for spurious emission band UE coexistence that exist for DC_20_n28 in 38.101-3 can be re-used for DC_28_n20 as per table 6.1.X.3-1.

Table 6.1.X.3-1: 2UL Band 28 + Band n20 Spurious Emission Coexistence Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n20 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |

#### 6.1.X.4	MSD analysis for DC

Table 6.1.X.4-1 lists the Band 28A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.X.4-1: Band 28 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fn_low | fn_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 |  | 832 | 862 |  |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high |  | 2* fn_low | 2* fn_high |  |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  |  | 1664 – 1724 |  |  |
| 3rd harmonics frequency limits | 3*fx_low |  | 3*fx_high | 3* fn_low |  | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  |  | 2496 – 2586 |  |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ |  | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 84 – 159 |  |  | 1535 – 1610 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ |  | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 544 – 664 |  |  | 916 – 1021 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ |  | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2238 – 2358 |  |  | 2367 – 2472 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) |  | (fx_high + max BW fn) | (fn_low – max BW fx) |  | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 768 |  |  | 812 – 882 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ |  | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ |  | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1412 |  |  | 1748 – 1883 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ |  | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ |  | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 168 – 318 |  |  | 3070 – 3220 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ |  | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ |  | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3106 |  |  | 3199 – 3334 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ |  | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ |  | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2580 – 2745 |  |  | 1950 – 2160 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ |  | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ |  | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1000 – 1180 |  |  | 385 – 580 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ |  | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ |  | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4196 |  |  | 3644 – 3854 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ |  | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ |  | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4082 |  |  | 3773 – 3968 |  |  |

Based on Table 6.1.X.4-1,

-	2nd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66.

-	3rd order harmonics may fall into Rx frequencies of band 11, 21, 32, 38, 41, 45, 50, 51, 69, 74, 75, 76, 90, 91, 92, 93 and 94.

-	2nd order IMD may fall into Rx frequencies of band 24.

-	3rd order IMD may fall into Rx frequencies of bands 8, 30, 40 and 71.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 35, 39, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 1, 2, 4, 7, 10, 25, 31, 34, 36, 38, 41, 43, 48, 49, 65, 66, 69, 70, 72, 73, 77, 78, 87, 88 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 6.1.X.4-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.X.4-2: 2UL Band 28 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

#### 6.1.X.5	∆TIB and ∆RIB values

Table 6.1.X.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28_n20 | 28 | 0.5 |
|  | n20 | 0.5 |

Table 6.1.X.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_28_n20 | 28 | 0 |
|  | n20 | 0 |

<End of Text Proposal>