3GPP TSG-RAN WG4 Meeting # 104-bis-e                                                      R4-2217147

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:			6.9.4

Source:	Moderator (Apple)

Title:	Email discussion summary for [104-e][234] NR_RRM_enh3

Document for:	Information

# Introduction

This email discussion summary includes general and work plan (6.9.1), RRM core requirements for FR2 SCell activation delay reduction (6.9.2), 6.9.2.1	L3 part enhancement for FR2 SCell activation (6.9.2.1), 6.9.2.2	L1 part enhancement for FR2 SCell activation (6.9.2.2) and 6.9.2.3	Other potential enhancement for FR2 SCell activation (6.9.2.3).

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Apple | Jie Cui | Jie_cui@apple.com |
| Qualcomm | Hyunwoo Cho | hyuncho@qti.qualcomm.com |
| LGE | Jin-Yup Hwang | jinyup.hwang@lge.com |
| CMCC | Jingjing Chen | chenjingjing@chinamobile.com |
| Xiaomi | Xuhua Tao | taoxuhua@xiaomi.com |
| Huawei, HiSilicon | Zhongyi Shen | shenzhongyi3@huawei.com |
| OPPO | Roy Hu | hurongyi@oppo.com |
| Nokia | Lei Du | lei.du@nokia-sbell.com |
| vivo | Yanliang SUN | yanliang.sun@vivo.com |
| ZTE | Chenchen Zhang | zhang.chenchen@zte.com.cn |
| China Telecom | Lu Yang | yangl75@chinatelecom.cn |
| MTK | Ogeen Toma | Ogeenhanna.toma@mediatek.com |
| NTT DOCOMO, INC. | Hiroki Yamada | hiroki.yamada.pc@nttdocomo.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: General and work plan (6.9.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215599 | Apple, OPPO | The updated work plan is proposed to reflect the phased work for FR2 SCell activation enhancement. |

  - Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

    - Sub-topic 1-1: Work plan for R18 eFeRRM

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: work plan for R18 eFeRRM

- Proposals (Apple, OPPO): the updated work plan for R18 eFeRRM in R4-2215599 is agreeable

- Recommended WF

  - Agree on the proposal

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Agree on the proposal. |
| Qualcomm | Agree on the proposal |
| LGE | Agree on the proposal |
| Intel | Agree on the proposal |
| OPPO | Agree on the proposal |
| Nokia | Agree on the proposal |
| China Telecom | Agree on the proposal |
| MTK | Agree on the proposal |

  - Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 1-1: Work plan for R18 eFeRRM

|  | Status summary |
| --- | --- |
| Issue 1-1: work plan for R18 eFeRRM | Agreements: The updated work plan for R18 eFeRRM in R4-2215599 is agreeable. Candidate options: Recommendations for 2nd round: This issue is closed, and no discussion in 2nd round. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

# Topic #2: L3 part enhancement for FR2 SCell activation (6.9.2.1)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215456 | Xiaomi | Proposal 1: RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation. Proposal 2: RAN4 to introduce the state of not performing full Rx beam sweeping for L3 measurement during FR2 SCell activation. - When the measured RSRP is higher than a threshold, the UE enters in the state of not performing full Rx beam sweeping. The UE can perform the measurement with a sub-set of Rx beam or non-Rx beam sweeping during this state. - When RSRP variance is larger than a threshold, the UE exits the non-changing Rx beam state and perform the full Rx beam sweeping. Proposal 3: The A-TRS can be used for L3 measurement for FR2 unknown SCell activation if the QCL information of the A-TRS is provided to UE. Proposal 4: If the triggered A-TRS is QCL-ed with the SSB of inter-band SpCell or one of inter-band active serving cell, A-TRS is configured for AGC adjustment, cell search and fine timing tracking for FR2 unknown SCell activation. |
| R4-2215801 | LG Electronics Inc. | - Proposal 1: Introduce enhanced Rx beam sweeping for L1 and L3 part in FR2 SCell activation with following • P1-1: Perform L3 (cell synchronization, measurement,…) with reduced Rx beam sweeping (e.g., rough Rx beam), and then perform L1-RSRP with reduced Rx beam sweeping (e.g., narrow beam sweeping corresponding to selected rough Rx beam) • P1-2: Perform L3 (cell synchronization, measurement,…) with full Rx beam sweeping (e.g., narrow Rx beam), and then skip the L1-RSRP procedure - Proposal 2: Consider the further evaluation of cell synchronization and measurement in the L3 part with less sample than existing requirements |
| R4-2215807 | NTT DOCOMO, INC. | Proposal 1: Consider to reduce sample number and skip Tfinetiming in unknown FR2 by reusing from the L3 measurement results and further should discuss that kind of redundant parameter. Proposal 3: Wait for FR2 multi-Rx chain DL reception evaluation then select whether or how to leverage conclusions. |
| R4-2216744 | Qualcomm Incorporated | Proposal 1: Unknown SCell in FR2 needs to split in two categories for FR2 SCell activation delay reduction purpose. 1) completely unknown SCell, 2) semi-unknown SCell. Proposal 2: SCell activation delay reduction should be applied for semi-unknown SCell scenario where SCell is activated from deactivated state. |
| R4-2215356 | Intel Corporation | Proposal 1: A new UE capability can be introduced to further reduce the FR2 RX beam sweeping factor to be less than 8. Proposal 2: Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase. Proposal 3: For cell search, 1*8 samples will be reduced to M samples(M<Rx beam sweeping factor) where M is fixed number and M is independent of Rx beam sweeping factor. |
| R4-2215530 | China Telecom | Proposal 1: In unknown FR2 SCell activation scenario, UE is able to obtain timing/frequency/beam information of SCell based on the A-TRS provided that the A-TRS has same QCL information with reference signals of any other active serving cell in another band. Proposal 2: The extension of A-TRS based unknown FR1 SCell activation into inter-band scenario can also be introduced. |
| R4-2215600 | Apple | Proposal 1: introduce the UE capability to indicate Rx beam sweeping factor (≤8) for L3 cell synchronization and measurement of FR2 unknown SCell activation enhancement. Proposal 2: RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case. Observation: for legacy requirement, TFirstSSB_MAX + 15*TSMTC_MAX is used for AGC and PSS/SSS detection, and 8*Trs is used for cell measurement, T/F tracking and SSB index reading. Proposal 3: reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 SCell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX. Proposal 4: for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement. Proposal 5: AP-CSI-RS and/or A-TRS based fast SCell activation is not apply to L3 part for unknown FR2 SCell activation enhancement. |
| R4-2215719 | CMCC | Proposal 1: for L3 enhancement for FR2 SCell activation, it is proposed to perform enhancement on RX beam sweeping factor. Proposal 2: for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline. Proposal 3: For the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), it is proposed to reduce the number of samples (i.e. from 2 samples reduced to 1 sample). Proposal 4: if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed. Proposal 5: it is proposed to discuss how to apply the conclusions of the measurement delay reduction in multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI after the multi-Rx chain DL reception WI has corresponding conclusions. |
| R4-2215785 | Nokia, Nokia Shanghai Bell | Proposal 1: The availability of a valid L3 measurement result at the time of SCell activation shall be considered to reduce the SCell activation delay. Proposal 2: The UE indication on the up-to-date L3 measurement status of the to-be-activated SCell is introduced to reduce the FR2 unknown SCell activation delay. Proposal 3: The UE can apply a reduced beam sweeping factor based on the latest cell/measurement status at the time of SCell activation. Proposal 4: The reduced Rx beam sweeping factor can be assumed for cell search step (X*Trs) if the UE is able to identify the Rx beam setting after AGC settling. Proposal 5: RAN4 should align the understanding of cell measurement/synchronization/AGC/T/F tracking in the existing FR2 unknown SCell activation delay requirements. Proposal 6: RAN4 to discuss the potential of simplified AGC settling with 1 sample. Proposal 7: RAN4 to study the solution to reduce the unknown SCell activation delay based on the A-TRS. Proposal 8: A-TRS can be triggered based on the latest measurement status at UE. |
| R4-2215809 | OPPO | Proposal 1: Introduce a set of candidate values less than 8 based on UE capability for FR2 Rx beam sweep factors. Proposal 2: For L3 measurement, consider to reduce measured RS samples (e.g., 1 sample) under -2dB SINR. Proposal 3: A-TRS can be considered to reduce FR2 SCell activation delay, e.g., the time period of cell search, acquiring timing/frequency of target cell. Proposal 4: RAN4 should firstly conclude baseline L3 enhancement in multi-Rx WI and then discuss how to leverage conclusions to FR2 SCell activation enhancement. |
| R4-2215865 | vivo | Observation 1: Rel-17 positioning WI has introduced the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for FR2. Proposal 1: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 SCell activation. Observation 2: For cell search, 1 sample has been considered where the number can’t be reduced. And for AGC settling, 1 sample is for coarse AGC and 1 sample is for fine AGC where the number can’t be also reduced. |
| R4-2216272 | Huawei, HiSilicon | Proposal 1: UE can obtain coarse timing information via QCL typeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped. Proposal 2: UE can obtain beam information via QCL typeD to inter-band serving cell subject to gNB configuration. Proposal 3: For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms. |
| R4-2216480 | ZTE Corporation | Proposal 1: So as to reduce the latency of unknown FR2 SCell activation, for L3 part enhancement, the sample of AGC adjustment can be reduced from 2 sample to 1 sample since of higher SNR side condition. Proposal 2: Two possible methods to reduce Rx beam number during L3 part can be considered: 1) Directly reduce the Rx beam number from 8 to [x] for each component in L3 part, but the corresponding performance loss would be verified to be acceptable. 2) UE performs one non-reduced Rx beam sweeping in AGC adjustment, and then UE performs reduced Rx beam sweeping during cell search with the assumption of [4] Rx beam numbers based on the beam information obtained in AGC adjustment. Proposal 3: For the UE capable of 2 panels, the conclusion on the measurement delay reduction in WI of FR2 multi-panel Rx can be applied to the L3 procedure in FR2 SCell activation. Since the WI of FR2 multi-panle Rx is also discussed in parallel, so we can first discuss the SCell activation without considering multi-panel Rx. |
| R4-2216758 | MediaTek inc. | Proposal 1: Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay. Proposal 2: Wait for the corresponding conclusions on the measurement delay reduction in multi-Rx chain DL reception WI before discussing the corresponding enhancement in SCell activation. Proposal 3: The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown SCell activation scenario. Proposal 4: Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the SCell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources. |
| R4-2216828 | Ericsson | - RAN4 to study the use of SSB periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation. - RAN4 to agree that some UEs with higher capability shall be able to achieve RX beam sweeping reduction. The new factor is N1 and N1 is FFS. - RAN4 to discuss and specify SCell activation for following two cases. - Scenario1: SCell is unknown due to the fact that UE did not sent measurement report to gNB in last X seconds. - SCell is unknown due to the fact that UE may be measuring it for first time. - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX. - RAN4 to agree that UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before. - RAN4 to agree on either of the following as SCell activation delay framework for L3 part. - TFirstSSB_MAX + (N1-1) *TSMTC_MAX + 0.5N1*Trs - TFirstSSB_MAX + (N2-1) *TSMTC_MAX + 0.5N2*TSMTC_MAX + 0.25N2Trs - For Scenario2, RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X ms]. - If UE performed a full RX beam sweeping for a procedure, next procedures or steps which fall within RX beam constant time do not need to perform RX beam sweeping. - RAN4 to agree to apply relevant agreements of multi-RX chain to SCell activation delay too, if the UE supports this capability. - RAN4 to agree that UE sends a beam information report to during or at the end of L3 part of the procedure. Contents of the beam information report are FFS. |

  - Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

    - Sub-topic 2-1 Scenarios for FR2 SCell activation enhancement

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-1-1: Scenarios/status/categories for FR2 SCell activation enhancement

- Proposals

  - Option 1 (Qualcomm):

    - Unknown SCell in FR2 needs to split in two categories for FR2 SCell activation delay reduction purpose. 1) completely unknown SCell, 2) semi-unknown SCell.

    - SCell activation delay reduction should be applied for semi-unknown SCell scenario where SCell is activated from deactivated state.

    - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario.

  - Option 2 (Nokia):

    - The availability of a valid L3 measurement result at the time of SCell activation shall be considered to reduce the SCell activation delay.

    - The UE indication on the up-to-date L3 measurement status of the to-be-activated SCell is introduced to reduce the FR2 unknown SCell activation delay.

  - Option 3 (Ericsson):

    - RAN4 to discuss and specify SCell activation for following two cases.

      - Scenario1: SCell is unknown due to the fact that UE did not sent measurement report to gNB in last X seconds.

      - Scenario2: SCell is unknown due to the fact that UE may be measuring it for first time.

  - Option 4 (last meeting agreement): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

    - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

    - The extension of the enhancement solutions to FR1 can also be discussed.

- Recommended WF

  - Please companies discuss whether the FR2 SCell activation enhancement shall be limited to certain UE scenario/status/category as option 1/2/3 or shall be for generic FR2 unknown Scell without intra-band serving cell?

  - Or it shall be discussed case by case (e.g., sample number reduction may consider such scenario/status/category but Rx beam sweeping factor reduction may not. This “e.g.” is just an example, but not a proposal)?

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 4 and we think the enhancement is discussed for the unknown FR2 SCell but not a middle status between unknown and known. However, we are also open to discuss the SCell scenario/status/category if majority companies want so, and therefore we can also accept to discuss the SCell scenario/status/category in a case by case way for different enhancement parts. |
| Qualcomm | We support option 1 and scenario 1 at option 3. We also support option2 if UE can report L3 RSRP report after receiving SCell activation command. For the second bullet in option 4, it can discuss when the first bullet is finalized. So, we prefer to prioritize FR2 case. The optional capability in option1 is separate topic. We can discuss it separately at 2-2-1. For purely unknown Scell where UE measure the target Scell for the first time, we think only limited enhancement is applicable. On the other hand, target Scell is also unknown for UE who measured but not reported, the fast SCell activation schemes are applicable. |
| LGE | We slightly prefer option 4 as the last meeting agreements. If two category for unknown UE as option 1 or separate scenarios as option 3 are considered, do we need to define separate test case for those and how to distinguish those UEs? |
| Intel | Option 1/2/3 are trying to further classify the unknown condition into two cases. Case 1 is lack of recent L3 report during a time limitation. Case 2 is that SCell is never measured before. For case 1, since UE didn’t send L3 report timely, the old L3 coarse timing and measurement seems to be out of date as well. UE anyway need to perform new L3 measurement. For case 2, since UE has not measurement L3 before, new L3 procedure is needed. Enhancement on legacy L3 measurement can be further studied. Therefore, it seems that for both case, UE needs to perform new L3 measurement. We prefer don’t split the cases and further discuss how to enhance legacy L3 part, which can apply for both cases. |
| Xiaomi | Support option 4, we prefer to discuss the potential solution to reduce the delay for generic FR2 unknown SCell without intra-band serving cell in first phase, if RAN4 has concluded on solution of delay reduction for each component, then RAN4 can define the specific requirement case by case, e.g. the case of FR2 unknown SCell activation with cross-carrier QCL information and the case of blind FR2 unknown SCell activation. |
| Huawei | We support option 4 which is the generic guidance. For option 1-3, we believe it is better to be discussed in related issues. E.g. the known conditions. |
| Ericsson | We think Rel-15 requirements are mentioned for absolute worst-case scenario assuming that UE will always start the SCell activation from scratch. In reality it may not be true. UE may use some prior known information. We think option 1 and option 3 are somewhat similar. We therefore agree with QC to introduce semi-unknown state in addition to the existing known and unknown states. One option is to reduce the SCell activation delay when the SCell is in semi-unknown state. We can define some conditions under which the SCell is in semi-unknown state. For example, if the UE has sent the measurement report for the SCell during the last T1 seconds then the SCell can be considered in semi-unknown state. T1 must be longer than time for being in known state and T1 can be FFS. If the conditions for the semi-unknown state are met, then SCell activation requirements can be speed up i.e., delay can be shorter compared to the case when the SCell is unknown but longer when the SCell is known. Having said that complete unknown SCell activation may also needs to be looked into. Enhancement feasible in different cases may be different. |
| OPPO | Support option 4 in generally. We also understand the motivation of other options on introducing different scenarios or different type of UEs. Actually, UE may use some prior known information to reduce the Scell activation delay. We can further discuss such limitations. But they may be decoupled with the known or unknown conditions which can be kept as the legacy. |
| Nokia | Support Option 1,2,3. We understood Option 1,2,3 are addressing the same issue i.e. different scenarios of unknown SCells. As existing “unknown” condition assumes the worst case without considering the latest SCell status, it would be helpful to clarify these scenarios and discuss the potential for activation delay reduction accordingly. We don’t think the intention is to limit the enhancement to certain scenario. But it is more identifying the difference to facilitate the discussion. There is probably more room for delay reduction in semi-unknown scenario but this does not prevent the optimization for blind detection case. What is the purpose to list Option 4? Option 1,2,3 is sub-category of unknown SCell hence not against Option 4. |
| vivo | In our understanding, option 1/2/3 are quite similar, except some details. We do not think this is out-of-scope of what was agreed in option 4. The enhancements are still mainly for ‘FR2 unknown Scell without intra-band serving cell’ We are ok to work in the direction of option 1/2/3, although some issues are not clear to us. For example, how does network/TE distinguish UE between the two cases, as mentioned in option 1/3. Without UE reporting, network/TE may not know the actual UE unknown type, if unknown type is further classified. Since in this WI, RAN2 work can be triggered, we are open to further discuss how to solve above issues. Therefore, we support to work in the direction of option 1/2/3. Regarding the wording, we prefer the 1st and 3rd bullet of option 1. The exact definition of ‘semi-unknown’ can be further discussed. |
| ZTE | We prefer Option 1, 2 3 since we believe the classification makes sense. Even though we classify into two categories, such as completely unknown and semi-unknown, which does not means not any reduction is necessary for the totally unknown case, just means we can decide the reduction case by case. |
| China Telecom | Support option 4. We prefer to focus on the generic FR2 unknown SCell without intra-band serving cell scenario and the extension of enhancement solutions to FR1 scenario. As for the case mentioned in option 1-3, RAN4 can discuss them after the generic unknown scenario has conclusions. |
| MTK | Support Option 4. We are also fine to further discuss Option 1,2,3. These options try to differentiate between the different scenarios for unknown SCell. |

    - Sub-topic 2-2 Beam related enhancement for L3 part

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- Proposals

  - Option 1 (Xiaomi, Intel, Apple, CMCC, Ericsson, OPPO, vivo): RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation.

    - Option 1a (CMCC): for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline.

    - Option 1b (vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 SCell activation.

  - Option 2 (Xiaomi): RAN4 to introduce the state of not performing full Rx beam sweeping for L3 measurement during FR2 SCell activation.

    - When the measured RSRP is higher than a threshold, the UE enters in the state of not performing full Rx beam sweeping. The UE can perform the measurement with a sub-set of Rx beam or non-Rx beam sweeping during this state.

    - When RSRP variance is larger than a threshold, the UE exits the non-changing Rx beam state and perform the full Rx beam sweeping.

  - Option 3 (LGE):

    - P1-1: Perform L3 (cell synchronization, measurement,…) with reduced Rx beam sweeping (e.g., rough Rx beam), and then perform L1-RSRP with reduced Rx beam sweeping (e.g., narrow beam sweeping corresponding to selected rough Rx beam)

    - P1-2: Perform L3 (cell synchronization, measurement,…) with full Rx beam sweeping (e.g., narrow Rx beam), and then skip the L1-RSRP procedure

  - Option 4 (Nokia): based on option 2 in issue 2-1-1:

    - The UE can apply a reduced beam sweeping factor based on the latest cell/measurement status at the time of SCell activation.

    - The reduced Rx beam sweeping factor can be assumed for cell search step (X*Trs) if the UE is able to identify the Rx beam setting after AGC settling.

  - Option 5 (ZTE):

    - Directly reduce the Rx beam number from 8 to [x] for each component in L3 part, but the corresponding performance loss would be verified to be acceptable.

  - Option 6 (ZTE):

    - UE performs one non-reduced Rx beam sweeping in AGC adjustment, and then UE performs reduced Rx beam sweeping during cell search with the assumption of [4] Rx beam numbers based on the beam information obtained in AGC adjustment.

  - Option 7 (MediaTek):

    - Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay.

  - Option 8 (Ericsson): based on option 3 in issue 2-1-1

    - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX.

      - RAN4 to agree that UE can speed up the remaining steps in SCell activation with a shorter beam scaling factor based on prior Rx beam information from the step before.

      - RAN4 to agree on either of the following as SCell activation delay framework for L3 part.

        - TFirstSSB_MAX + (N1-1) *TSMTC_MAX + 0.5N1*Trs

        - TFirstSSB_MAX + (N2-1) *TSMTC_MAX + 0.5N2*TSMTC_MAX + 0.25N2Trs

    - For Scenario2, RAN4 to define RX beam constant time. Where, RX beam constant time is a time duration or window within which the RX beams are assumed to be constant or non changing. Rx beam constant time to be agreed as [X ms].

      - If UE performed a full RX beam sweeping for a procedure, next procedures or steps which fall within RX beam constant time do not need to perform RX beam sweeping.

  - Option 9 (Qualcomm): from on option1 in Issue 2-1-1,

    - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1/1a/1b. For option 2, such criteria is based on the FR2 RSRP measurement, that means UE needs to do the fully Rx beam sweeping first to check the criteria, and then UE can determine the state of Rx beamforming, and we are wondering if it still has benefit to reduce the measurement time (measurement is already done at this stage in our view). For option 3, P1-1 is similar as option 1 and P1-2 can be discussed in L1-RSRP part For option 4, it’s up to the discussion in issue 2-1-1. For option 5, we are fine with it, but it can be merged to option 1 (the capability is needed in our view since this R18 feature is optional). For option 6, we think the AGC and cell search is performed together, the same Rx beam sweeping factor shall be considered for L3. For option 7, it’s up to the discussion in issue 2-1-1. |
| Qualcomm | It depends on outcome of Issue 2-1-1. For purely unknown scenario: we agree with option 7 and don’t support all other options. For Semi unknown scenario: We support option1b, option 9. We support UE capability to reduce either number of beam sweeping factor or number of SSBs to be measured. We think It does not necessarily beam sweeping related, UE can indicate a required number of SSBs to complete AGC and cell search. |
| LGE | We think that L1 and L3 Rx beam sweeping enhancement should be discussed together as option 3. And we are fine with option 1 and option 5. |
| Intel | Support option1/1a/1b . |
| CMCC | Support option 1/1a/1b. option 1a and option 1b are for the detailed value of reduced RX beam sweeping factor, can be used as baseline for further discussion. |
| Xiaomi | Option 1 and option 2, for option 2, it is feasible and beneficial to introduce the conditions to allow UE enters the state of not performing full Rx beam sweeping. When the measured RSRP is good enough, it is assumed that the UE does not need to do the Rx beam sweeping and the UE is in the state of not performing full Rx beam sweeping. The UE can perform the measurement with a sub-set of Rx beam or non-Rx beam sweeping during this state. Upon the RSRP variant is larger than a threshold, the UE should exit the non-changing Rx beam state and perform the full Rx beam sweeping. To Apple comment, the criteria checking can be done by measurement sample level, e.g., UE can perform the measurement with full set of Rx beam at first measurement sample, and if the criteria is fulfilled, then, UE can perform the measurement with sub-set of Rx beam in next measurement sample. |
| Huawei | As analyzed in our paper, it is no well justified how UE can reduce beam sweeping number from implementation perspective as also mentioned in option 7. The procedures involved in SCell activation are common RRM activations (e.g. L1 and L3 measurement), so it is not clear how UE can reduce the beam sweeping for SCell activation. Before claiming that UE can do so and introducing capabilities, more clarifications on feasibilities are needed. |
| Ericsson | We think we should discuss requirements for case by case as the reduction can be possible only in particular scenarios. We are generally support option 1. |
| OPPO | Support option 1 in general. UE capability could be a good compromise. And for the UE supporting faster beam sweeping or less measured SSB samples, reduced L3 delay requirements can be defined at least. |
| Nokia | This issue seems including two sub-issues: 1) if a reduced beam sweeping factor can be used and 2) the condition when a reduced factor can be used. For 1), we agree a reduced beam sweeping factor can be defined and corresponding UE capability is needed, hence we are fine with Option 1 in general. But we may not have to start from HST or positioning as there are some special deployment assumptions behind. Instead, the discussion should consider SCell activation procedure and identify the potential of reduced beam sweeping. In addition, we should discuss the applicability of reduced beam sweeping factor as in Option 4, 8. This is also relevant to issue 2-1-1. If the UE has measured the SCell with SSB index (i.e. semi-unknown case), there is no need for the UE to do AGC, cell search and L1-RSRP, so beam sweeping can be removed from activation procedure. Otherwise, the reduced beam sweeping factor may be used for AGC, cell search and/or L1-RSRP measurement steps. The possibilities and conditions need to be further discussed. One more issue is the performance loss due to reduced beam sweeping factor. We would expect the reduced beam sweeping factor is used only if there is no performance impact on SCell activation. Otherwise, the legacy beam sweeping factor can be used. |
| vivo | We support option 1/1a/1b. 1st bullet of option 3 is also related to it. Firstly, solutions in option 2 and 2nd bullet of option 8 are not preferred in our view. We do not want to mix the enhanced requirements with whether UE is stable or not. Secondly, we think the enhancement here could be considered for UE in certain status, e.g. related to the intermediate status between known and unknown. We agree with QC that UE capability would be needed. This is the similar situation as positioning, which also consider special procedure of the UE. Therefore, the concern from option 7 can be solved in our understanding. Option 4 is also aligned to this in our understanding. However, we do not want to have different beam sweeping factor for AGC adjustment and cell search. |
| ZTE | In fact multiple options can be combined. Regarding to whether an additional UE capability needed, we are open to further discuss. Regarding to the beam sweeping factor reduction, we support, and if the classification in Issue 2-1-1 is allowed, we can discuss the beam sweeping factor case by case. |
| China Telecom | Fine with option 5. The reduced Rx beam sweeping factor can be considered for the delay enhancement in L3 part for AGC and cell search, however, it is considerable to discuss whether the performance loss of reduced Rx beam sweeping factor can be acceptable. |
| MTK | We support option 7. For unknown SCell, L3 part is the first procedure after RF retuning, for which we don’t think reducing Rx beam sweeping factor should happen at this stage as it will have significant impact on the performance of AGC and cell search procedure. This reduction can be considered in the next stage i.e. L1-RSRP measurements. Also as commented by HW, the feasibility of this reduction should be justified considering its impact on the performance. |
| NTT DOCOMO, INC. | We support option 1/1a/1b. |

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

- Proposals

  - Option 1 (NTT DoCoMo): Wait for FR2 multi-Rx chain DL reception evaluation then select whether or how to leverage conclusions.

  - Option 2 (Intel): Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase.

  - Option 3 (Apple, CMCC, OPPO, MediaTek): RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case.

  - Option 4 (ZTE): For the UE capable of 2 panels, the conclusion on the measurement delay reduction in WI of FR2 multi-panel Rx can be applied to the L3 procedure in FR2 Scell activation. Since the WI of FR2 multi-panel Rx is also discussed in parallel, so we can first discuss the Scell activation without considering multi-panel Rx.

  - Option 5 (Ericsson): RAN4 to agree to apply relevant agreements of multi-RX chain to Scell activation delay too, if the UE supports this capability.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 3 (all the options are not quite different). In multi-Rx chain DL reception WID, only single carrier case is considered for enhancement, and it would be very difficult to maintain a CR for FR2 Scell activation enhancement if it was also discussed and specified in multi-Rx chain DL reception WI, i.e., there will be 2 CRs from two Wis for the same enhancement. Thus, our view is to use the conclusion of single FR2 carrier enhancement from multi-Rx chain DL reception WI to the R18 eFeRRM WI, but the FR2 Scell activation enhancement shall be discussed in R18 eFeRRM WI. |
| Qualcomm | We support option2. We think the target scenario is not aligned between two WI. |
| LGE | We support option 2. We prefer not to consider multi-Rx chain DL reception in the first phase. |
| Intel | Support option 2, also fine with option 3. From our understanding, for FR2 multi-RX chain DL reception, it’s FFS whether L3 measurement will be enhanced. Besides, if beam sweeping factor reduction will be discussed, it will mainly focus on simultaneous multi-RX chain reception on the same carrier from m-TRP. For single Scell activation, it seems that the scenario is a little different. We are also fine to further discuss the issue when there are some conclusions from multi-RX chain WI. |
| CMCC | Option 3. The main consideration is not to mix two parallel ongoing WI. |
| Xiaomi | Option 3 |
| Huawei | Support option 2. R18 Multi-RX is for single component apparently not related with Scell activation. Besides, the bundled discussion between two ongoing R18 WI shall be avoided. |
| Ericsson | Support option 5 and ok with option 3 also. |
| OPPO | Option 3. |
| Nokia | The options are not far away. We prefer Option 3 terminology. It is noted the discussion on FR2 SCell activation delay reduction is NOT dependent on FR2 multi-Rx chain UEs. But for the UE supporting multi-Rx chain capability, the conclusion on the measurement delay reduction for such UEs can be considered to reduce the SCell activation delay. It would be good to clarify in which WI this will be discussed to avoid duplicated discussion. |
| vivo | Option 2. We should not consider concurrent discussion of different WI. We think option 2 is fair since we only need to clarify this for the 1st phase. For option 3, we are not sure what dose the condition after ‘if’ means. The concurrent discussion already happens. What if different conclusions are eventually achieved for different WI? ‘No conclusion’ would be one kind of conclusions. |
| ZTE | Option 3 and 4. Since of two WIs are discussed in parallel, so we can first discuss SCell activation all alone and at the same time wait for the outcome of multi-panel Rx. |
| China Telecom | Support option 3. |
| MTK | Option 3. |
| NTT DOCOMO, INC. | Option 1,3. |

    - Sub-topic 2-3 AGC/Cell measurement/synchronization sample number related enhancement for L3 part

Issue 2-3-1: Align the understanding of cell measurement/synchronization/AGC in the existing FR2 unknown SCell activation delay requirement

- Proposal:

  - Option 1(Nokia): RAN4 should align the understanding of cell measurement/synchronization/AGC/T/F tracking in the existing FR2 unknown SCell activation delay requirements

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Our preference is no need to discuss the option 1, because even in the existing FR2 unknown SCell activation the UE behavior for cell measurement/synchronization/AGC/T/F tracking is up to UE implementation, e.g., some UE vendor may have separated steps for AGC and cell searching but some may have same steps. We recommend to directly discuss the enhancement part. |
| Qualcomm | Our understanding is current spec define 2 samples with 8 beam sweeping for AGC and 1 sample with 8 beam sweeping for cell search is required for FR2 unknown Scell. And agree with some UE vendor may have different steps. |
| Intel | Fine with option 1. It seems that company have different understanding regarding to AGC and cell search time. |
| CMCC | OK with option 1. |
| Xiaomi | Agree with Apple, different UE may have different steps for AGC and cell search. |
| Huawei | For the specification perspective, we share similar views QC on the understanding of each components, and also agree that there may be difference among different implementations. |
| Ericsson | We agree with proposal 1. Currently it is not very clear, enhancement possible may depends on the steps. |
| OPPO | Fine with option 1. A generic model as mentioned by QC can be taken as baseline or worst case for further discussion regarding different UE implementations. |
| Nokia | We’d like to clarify the UE behaviour within TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs time period. We understood this corresponds to PSS/SSS detection time i.e. 24*Trs. Is it so PSS/SSS detection comprises AGC and cell search/T/F sync steps? But companies seem using different terminology for the steps: - TFirstSSB_MAX + 15*TSMTC_MAX: AGC (1 sample for coarse AGC and 1 sample for fine AGC) - 8*Trs: cell search, time/frequency synchronization, time/frequency tracking It would be good to align the understanding to facilitate the discussion on potential delay reduction. |
| vivo | Not sure how this discussion will help the progress, especially if no convergence can achieve. Our understanding is the same as QC. Trs should be for cell search in our understanding. |
| ZTE | Agree with Option 1. And we have the following understanding for current L3 procedure: 2 samples AGC and 1 sample cell search are needed, always 8 beam sweeping factor is assumed for AGC and cell search. |
| China Telecom | In our understanding, there are 2 samples for AGC and 1 sample for cell search in the existing FR2 unknown SCell activation procedure, and each sample needs 8 Rx beam sweeping. |
| MTK | We have similar understanding as QC, i.e., 2 samples are used for AGC settling and 1 sample is used for Cell search for FR2 unknown SCell case. |
| NTT DOCOMO, INC. | Support with option 1. |

Issue 2-3-2: enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part of current FR2 unknown SCell activation delay

- Proposals

  - Option 1 (Apple, CMCC, OPPO, ZTE): reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 Scell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX.

  - Option 2 (Nokia): RAN4 to discuss the potential of simplified AGC settling with 1 sample.

  - Option 3 (MediaTek): The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown Scell activation scenario.

  - Option 4 (Ericsson): based on option 3 in issue 2-1-1

    - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Support option 1. We did some simulation for cell searching and found that the sample number of PSS/SSS detection can be reduced to 1 with -2dB SNR, and we also think with such good SNR condition AGC settling can work well with 1 sample |
| Qualcomm | We don’t support all options for now. This is based on outcome of 2-2-1. |
| LGE | We support option 1, and further evaluation could be considered if needed |
| Intel | Depends on 2-3-1. It’s better to first needs to clarify whether the 16 samples are AGC time or cell search time. From our understanding, it refers to AGC time. For high capability UE, it’s possible to further reduce the sample. We are open to further discuss. |
| CMCC | Support Option 1. For the component of (TFirstSSB_MAX + 15*TSMTC_MAX ), 2 samples are assumed in existing requirements. Since the side condition is -2dB for Scell activation, which is higher than other mobility related requirements, the reduction on the sample number can be considerd. |
| Xiaomi | In our understanding, the component of “TFirstSSB_MAX + 15*TSMTC_MAX” is for AGC (2 sample for AGC), and we are open to reduce the AGC adjustment from 2 sample to 1 sample. In addition, we also think the delay for AGC setting can be reduced by considering the scaling factor reduction, e.g. 8 for 1st sample, and 2 or 4 for 2nd sample. |
| Huawei | We kind of support option 3. As also commented in issue 2-3-1, the actual usage of each samples may depends on UE implementations. For R15 UE, it is possible to use less samples to complete AGC/detection. For some conservative UE, it may use more samples as allowed in the requirements. We don’t see the necessity to specify such sample reduction as it more replies on UE implementations. |
| Ericsson | We think option 1, 2 and 4 are similar. In some cases, like semi-known state or scenario 1, UE may need only 1 sample for AGC and cell search. Beam sweeping factor can be discussed separately. For scenario 2, UE could reduce the RX beam sweeping based on previous step. |
| OPPO | Support option 1. Beam sweeping factor reduction can be discussed separately. |
| Nokia | We are fine with Option 1,2 and 4. As commented in Issue 2-3-1, we’d like to understand why 2 samples are needed for AGC. Is there any difference on the measurement of 1st and 2nd sample so that the 2nd sample can be removed? |
| vivo | Need more discussion. For the band of SCell without intra-band active serving cell, do RAN4 still need to consider case when there is more than one deactivated SCells in this band? Can L3 measurements performance scarified, for example, to prioritize the SCell activation? With above consideration, we support option 3. Rx Beam sweeping factor can be a separate issue. |
| ZTE | Support Option 1, 2, 4. Furthermore, if the classification in Issue 2-1-1 is allowed, we can decide whether such AGC sample reduction is possible for both two cases. |
| China Telecom | We are open to discuss whether sample number for AGC can be reduced to 1. Meanwhile, we need to discuss whether the performance loss of reduced sample number can be acceptable. |
| MTK | Support Option 3. For Option 1, we don’t think the side condition of -2dB SINR can be related to AGC. AGC is related to the total received power, not to the SNR. For example, if the total received power at the UE is very strong, the first sample could be saturated and therefore UE may still need for the second sample to settle the Rx AGC. |

Issue 2-3-3: enhancement of “8*Trs” part of current FR2 unknown SCell activation delay

- Proposals

  - Option 1 (Apple): for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement.

  - Option 2 (CMCC): if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed.

  - Option 3 (MediaTek): The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown SCell activation scenario.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1, we think this part(8*Trs) is also dependent to how much we enhanced on the previous part (TFirstSSB_MAX + 15*TSMTC_MAX). But we can also compromise to option 2 if majority companies support it. |
| Qualcomm | For semi-unknown scenario, we support option 1. |
| Intel | Depends on 2-3-1. First needs to clarify whether the 8 samples are AGC time or cell search time. From our understanding, it refers to cell search for coarse timing. Rough timing corresponding to different RX beam may not differ quite a lot. Therefore, maybe less than 8 samples can be used. |
| CMCC | As for whether to remove the component of 8*Trs, according to the discussion in last meeting, some companies think 8*Trs is for T/F tracking used for L1 measurement, and considering coarse timing is aquired in the component of (TFirstSSB_MAX + 15*TSMTC_MAX ), UE may be able to acquire the sufficient timing information for the L1-RSRP measurement. From this point of view, 8*Trs can be removed. However, other companies think (TFirstSSB_MAX + 15*TSMTC_MAX ) is only for AGC, 8*Trs is the time for cell search/detection to have the coarse timing. From this point of view, 8*Trs can not be removed. It seems it is necessary to firstly allign the understanding, if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed. That is the consideration we propose option 2, we are also open for further discussion. |
| Xiaomi | In our understanding, the component of “8*Trs” is for cell search (1 sample for cell search), and we think this procedure cannot be removed. However, it can be reduced by reducing the scaling factor of Rx beam sweeping. |
| Huawei | Similar comments as issue 2-3-2. |
| Ericsson | For scenario 1, UE could perform cell search and AGC at the same time. For scenario 2, UE could perform reduced RX beam sweeping for cell search. |
| OPPO | Fine with option 1 and 2. Option 1 and 2 are not contradictory. This issue is related to the assumption of AGC and cell search. If ACG and cell timing information can be acquired by at most 2 samples (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), then 8*Trs can be removed. |
| Nokia | Fine with Option 1. Before the 8*Trs, the UE has been monitoring SSBs for the time period TFirstSSB_MAX + 15*TSMTC_MAX. Hence the UE should be able to identify the DL timing of the SCell. What additional timing does the expect with 8*Trs? |
| vivo | Option 3. Option 1 or 2 can be discussed if RAN4 agrees to differential UE behavior by UE capability on this ‘8*Trs’, i.e. whether it is used for cell search or used for time-frequency sync. Currently we do not see strong motivation for this differentiation. In FR2, TCI should be activated afterward, and the understanding of ‘time-frequency sync’ here is vague. |
| ZTE | Fine with Option 1 and 2. If UE can acquire coarse timing of the target SCell within TFirstSSB_MAX + 15*TSMTC_MAX, 8*Trs can be removed. |
| MTK | Support Option 3. The first 2 samples of L3 are used for settling the AGC, not for acquiring the timing. The third sample of L3 (8*Trs) is used for cell search, and it is already based on one sample, which cannot be removed as it is the first time when the UE acquire the timing information for the unknown SCell. |

    - Sub-topic 2-4 RS related enhancement for L3 part

Issue 2-4-1: RS related enhancement of FR2 unknown SCell activation

- Proposals

  - Option 1 (Xiaomi, China Telecom, OPPO): The A-TRS can be used for L3 measurement for FR2 unknown Scell activation if the QCL information of the A-TRS is provided to UE.

    - Option 1a (Xiaomi, China Telecom): If the triggered A-TRS is QCL-ed with the SSB of inter-band SpCell or one of inter-band active serving cell, A-TRS is configured for AGC adjustment, cell search and fine timing tracking for FR2 unknown Scell activation.

    - Option 1b (China Telecom): The extension of A-TRS based unknown FR1 Scell activation into inter-band scenario can also be introduced.

  - Option 2 (Apple): AP-CSI-RS and/or A-TRS based fast Scell activation is not apply to L3 part for unknown FR2 Scell activation enhancement.

  - Option 3 (Nokia): based on option 2 in issue 2-1-1:

    - RAN4 to study the solution to reduce the unknown Scell activation delay based on the A-TRS.

    - A-TRS can be triggered based on the latest measurement status at UE.

  - Option 4 (Huawei):

    - UE can obtain coarse timing information via QCL typeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped.

    - UE can obtain beam information via QCL typeD to inter-band serving cell subject to gNB configuration.

    - For FR2 unknown Scell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

  - Option 5 (MediaTek): Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the Scell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 2. Since stage 1 is for enhancement on FR2 unknown Scell activation, the PSS/SSS detection and DL synchronization is needed to identify the target Scell. The AP-CSI-RS and A-TRS is not supposed to be used for cell synchronization or coarse timing acquisition (time correlation based on CSI-RS is not even considered in CSI-RS L3). If the coarse timing of AP-CSI-RS and A-TRS is known to UE, then it could be directly used for L1 measurement rather than L3 UE behavior on the Scell. In order to narrow down the possible solutions for unknown FR2 Scell activation enhancement, we think RAN4 can preclude to use AP-CSI-RS and/or A-TRS for L3 part in unknown FR2 Scell activation enhancement. Moreover, we are not sure if the inter-band FR2 serving cell timing information and beam information can be reused for target unknown FR2 Scell. The inter-band serving cell and target Scell will have inter-band MRTD and TAE is 3us, which is larger than CP, we don’t think the QCL type C can directly lead UE to reuse the timing from this inter-band serving cell. Regarding the beam information, CDM is not introduced/specified yet, the QCL type D information from inter-band FR2 serving cell will still need UE do perform the Rx beam sweeping for target Scell(inter-band serving cell and target Scell still need to do IBM). |
| Qualcomm | We support option 2. Regardless of purely unknown or semi unknown, it does not make sense UE to receive QCLed A-TRS before UE report RSRP with SSB index. |
| LGE | We support option 2. If AP-CSI-RS or A-TRS is available for L3 part, we think that RAN4 needs to discuss how to get feasible QCL source for AP-CSI-RS or A-TRS first. |
| Intel | Legacy A-TRS only apply when there is active serving cell in the same band or the Scell is known. The difficulty lies in how to obtain the coarse timing. We understand that it may be possible that timing information may be obtained from some other way, however, it seems that the applicable scenarios will be limited. Therefore, we prefer option 2. |
| Xiaomi | Option 1/1a and option 4, if the QCL information can be provided to UE, it is beneficial to configure A-TRS to speed-up the FR2 unknown Scell activation procedure under certain condition. If A-TRS is QCL type-C to SSB of inter-band SpCell or one of inter-band active serving cell, the cell search may not be needed, and if A-TRS is QCL type-C to SSB of inter-band SpCell or one of inter-band active serving cell, the scaling factor of Rx beam sweeping is not needed during AGC and cell search procedure. |
| Huawei | We support option 4 which basically share the same logic as option 1. To Apple’s comments: We would like to clarify that the intention is not to perform cell detection using A-TRS, which we also believe is not support yet. Instead, it means UE can obtain the timing and beam information via inter-band CC. Then the inapplicable unknown case in Rel-17 can be extended to “known” case, where UE can use A-TRS for AGC and time tracking. Regarding the TAE, it is the minimum requirements for BS. Under some conditions/deployment, if gNB can have better timing alignment, then it can make such configurations, which is also supported by existing RAN1/2 spec. To Apple’s comments: It does not mean it will apply to all inter-band cases. We are open to further investigate necessary conditions. |
| Ericsson | We agree with option 2. At the end of option 2, UE could send beam information report and based that A-TRS or AP-RS can be scheduled. |
| OPPO | Option 1. We are open to discuss the feasiblity of using A-TRS for L3 part and the corresponding conditions, e.g., A-TRS is QCL typeC to serving cell SSB. |
| Nokia | Option 3. There seems to be common understanding A-TRS can be used only if network knows about the beam information. We could discuss in what conditions A-TRS can be used for unknown SCell activation. This is also relevant to the scenarios in Issue 2-1-1. If the UE has measured the SCell, it may inform the beam information to network triggering the A-TRS. In addition, some QCL relation proposed in Option1 can be discussed, but the question is if it is possible to configure QCL for inter-band cells. One more issue is if A-TRS can be used for purposes other than AGC and time/freq sync (which was defined in R17). If the usage is extended, we may need check with RAN1 on the feasibility. |
| vivo | Need more discussion. If our understanding is correct, the basic logic for option 1/4 is that, in case there is network configuration, UE may assume QCL-C and QCL-D to the inter-band active serving cell for the SCell. We understand that the configuration is ok from RAN1/2 point of view, but the spec impact for RRM would be slightly large, and probably cannot be handled in this WI. We agree with Apple regarding the aspects of MRTD and CBM/IBM for the inter-band case. These issues are controversial since R15 and without any conclusion yet, for FR2. If RAN4 can somehow agree 1. to relax UE requirements for MRTD/MTTD based on network configuration of QCL-C for the inter-band case, and 2. to confirm that at least some UEs can support to use the Rx beam information from another band, then we are OK to further discuss this enhancement. Therefore, we are open to FFS option 1/4. |
| ZTE | Support Option 1a, 1b and 4. We understand Apple’s concern is whether timing and beam info can be acquired by UE in inter-band case. We agree with Huawei’s view, if QCL relation between the to-be-activated SCell and one inter-band active serving cell can be guaranteed, the timing and beam info can be acquired by UE. Regarding to the timing, current TAE requirement for inter-band CA is the worst case, and in fact in practice the TAE can be much better than current 3us under some deployment, such as co-located deployment. We provided some analysis in our tdoc R4-2216478. Regarding to the applicability of AP CSI-RS/AP TRS, we are open to discuss. We believe the altimate target is that once the known-like condition can be guaranteed, only 3ms latency needed for the unknown SCell activation. Of course we are open to discuss the detailed known-like condition. |
| China Telecom | Support option 1/1a/1b/4. In inter-band unknown FR2 SCell activation scenario, as mentioned in our paper, we think A-TRS is able to be used for both L3 and L1 part to reduce the whole procedure delay. In our views, the use of A-TRS depends on network’s configuration, that is, if network provide UE with A-TRS transmission time, QCL information, etc., UE is able to use A-TRS to complete activation process without the process of AGC tuning, cell search, L1-RSRP measurement and report, which brings significant delay reduction. The A-TRS based solution may be used under some conditions, we are open to further study and discuss the conditions. |
| MTK | We can support Option 2. We have same view as Apple. |

    - Sub-topic 2-5 Others for L3 part

Issue 2-5-1: Other enhancement for L3 part in FR2 unknown SCell activation

- Proposals

  - Proposal 1 (Ericsson): RAN4 to study the use of SSB periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation.

  - Proposal 2 (Ericsson): RAN4 to agree that UE sends a beam information report to during or at the end of L3 part of the procedure. Contents of the beam information report are FFS.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | For proposal 1, the cell searching is based on SMTC periodicity, so we think we can use SMTC to replace SMTC_MAX. For proposal 2: is that a L3 reporting or L1 reporting? after this beam reporting does UE still need L3 measurement? Need FFS. |
| Qualcomm | We support Proposal 1 for semi-unknown cases and when UE has new capability to indicate required number of SSBs to be measured. |
| Xiaomi | P#1: SMTC is configured for RRM measurement purpose, if SSB periodicity is used in SCell activation procedure, the UE measurement behavior is confused for other RRM measurements. P#2: it can be discussed under issue 3-1-1 and issue 3-1-3. |
| Huawei | For proposal 1, is it targeting the case when there is no serving cell in the same band? We think it is already stated in existing spec: in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. For proposal 2, it needs FFS. It seems talking about reporting beam via L3 measurement, which is discussed in issue 3-1-3. |
| Ericsson | We support proposal 1 and 2. To Apple: It can be based on the L3. Contents of the report can be FFS. To Huawei and Apple: we would like point that it can be SSB periodicity instead of SMTC periodicity. |
| Nokia | For P1, according to definition below, TSMTC_MAX refers to the SMTC of SCell for inter-band CA. This sounds to be the same as the proposal? - In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. Fine with P2 at least for the blind SCell activation case. For the unknown SCell which the UE has measured, it may send the beam information report even earlier e.g. in response to the SCell activation. |
| vivo | We think P1 would be similar to 2-3-2. We have similar discussion there. We think P2 would be similar to 2-1-1 discussion. In option 2 of 2-1-1, there is similar proposal. We are open to discuss this. |
| ZTE | Fine with the motivation of Proposal 1 but as pointed by Huawei and Nokia, it seems has been supported in current spec. |
| MTK | For P1 this is already captured in the specs as mentioned by HW, and since it is L3 measurements it should be based on SMTC periodicity. |

  - Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 2-1 Scenarios for FR2 SCell activation enhancement

|  | Status summary |
| --- | --- |
| Issue 2-1-1: Scenarios/status/categories for FR2 SCell activation enhancement | Discussion status: 4 companies support option 1, 2 companies support option 2, 4 companies support option 3 and 7 companies support option 4. Companies also mentioned that option 1/2/3 are further categorization from option 4, and they are not conflicted with option 4(last meeting agreement). Candidate options: - Option 1 (Qualcomm, Nokia, vivo(1st and 3nd bullet), ZTE): - Unknown SCell in FR2 needs to split in two categories for FR2 SCell activation delay reduction purpose. 1) completely unknown SCell, 2) semi-unknown SCell. - SCell activation delay reduction should be applied for semi-unknown SCell scenario where SCell is activated from deactivated state. - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario. - Option 2 (Nokia, ZTE): - The availability of a valid L3 measurement result at the time of SCell activation shall be considered to reduce the SCell activation delay. - The UE indication on the up-to-date L3 measurement status of the to-be-activated SCell is introduced to reduce the FR2 unknown SCell activation delay. - Option 3 (Ericsson, QC(scenario 1), Nokia, ZTE): - RAN4 to discuss and specify SCell activation for following two cases. - Scenario1: SCell is unknown due to the fact that UE did not sent measurement report to gNB in last X seconds. - Scenario2: SCell is unknown due to the fact that UE may be measuring it for first time. - Option 4 (last meeting agreement) (Apple, LGE, Xiaomi, HW, OPPO, CTC, MTK): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed. Recommendations for 2nd round: Moderator: In order to find a middle ground for discussion, we try to keep option 4 (existing agreement) as baseline and also to make sure companies can discuss the further categorization in a case-by-case way. In this issue 2-1-1, moderator encourage proponents of option 1/2/3 to consolidate the UE category in this meeting, and we can use this new UE category to discuss in the following enhancement issues to determine whether and how to use this new category for enhancement. Based on the above analysis, moderator propose a new option 5: - Option 5 (Moderator): - RAN4 to use option 4 as baseline to discuss the enhancement, and the new UE categorization based on option 1/2/3 can be discuss case-by-case in enhancement issues to determine whether and how to use such new category for enhancement. Recommend to discuss it in GTW and 2nd round, agreements will be captured in the WF. |

Sub-topic 2-2 Beam related enhancement for L3 part

|  | Status summary |
| --- | --- |
| Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) | Discussion status: Majority companies supported option 1 in general, and some companies mentioned that the detailed enhancement can be FFS. Option 7 is the option opposite to option 1, and the views on other options are diverse. Candidate options: - Option 1 (Apple, LGE, Intel, CMCC, Xiaomi, Ericsson, OPPO, Nokia, vivo): RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation. - Option 1a (Apple, Intel, CMCC, vivo): for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline. - Option 1b (Apple, QC(for semi-unknown case), Intel, CMCC, vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 SCell activation. - Option 2 (Xiaomi): RAN4 to introduce the state of not performing full Rx beam sweeping for L3 measurement during FR2 SCell activation. - When the measured RSRP is higher than a threshold, the UE enters in the state of not performing full Rx beam sweeping. The UE can perform the measurement with a sub-set of Rx beam or non-Rx beam sweeping during this state. - When RSRP variance is larger than a threshold, the UE exits the non-changing Rx beam state and perform the full Rx beam sweeping. - Option 3 (LGE, vivo(1st bullet)): - P1-1: Perform L3 (cell synchronization, measurement,…) with reduced Rx beam sweeping (e.g., rough Rx beam), and then perform L1-RSRP with reduced Rx beam sweeping (e.g., narrow beam sweeping corresponding to selected rough Rx beam) - P1-2: Perform L3 (cell synchronization, measurement,…) with full Rx beam sweeping (e.g., narrow Rx beam), and then skip the L1-RSRP procedure - Option 5 (LGE, CTC): - Directly reduce the Rx beam number from 8 to [x] for each component in L3 part, but the corresponding performance loss would be verified to be acceptable. - Option 7 (QC(for pure unknown case), HW, MTK): - Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay. - Option 9 (QC): from on option1 in Issue 2-1-1, - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario. Recommendations for 2nd round: Recommend to discuss it in GTW and 2nd round. To facilitate the 2nd round discussion, I only kept the options explicitly supported by companies in 1st round, but companies can still retrieve the 1st round options if they want in 2nd round. Agreement will be captured in the WF. For proponents of option 5, please check if you can compromise to option 1, since the FR2 SCell activation enhancement itself can be an optional feature in moderator’s view and then option 5 can be treated as a sub-option under option 1. As commented by other companies, this issue is also related with issue 2-1-1 (sub-category of pure-unknown or semi-unknown), moderator suggestion is we can also discuss those sub-category in this issue to see if Rx beam reduction enhancement rely on such sub-category or not. |
| Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception | Discussion status: Majority companies supported option 2 (5 companies) and option 3 (10 companies). Candidate options: - Option 2 (QC, LGE, Intel, HW, vivo): Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase. - Option 3 (Apple, Intel, CMCC, Xiaomi, Ericsson, OPPO, Nokia, ZTE, CTC, MTK): RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case. - Option 4 (ZTE): For the UE capable of 2 panels, the conclusion on the measurement delay reduction in WI of FR2 multi-panel Rx can be applied to the L3 procedure in FR2 Scell activation. Since the WI of FR2 multi-panel Rx is also discussed in parallel, so we can first discuss the Scell activation without considering multi-panel Rx. - Option 5 (Ericsson): RAN4 to agree to apply relevant agreements of multi-RX chain to Scell activation delay too, if the UE supports this capability. Recommendations for 2nd round: Based on the 1st round discussion, moderator proposed an option 6 to merge the option 2 and 3. - Option 6 (moderator): - In 1st phase, RAN4 not to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI. - In 2nd phase, if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case, RAN4 to discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI. Recommend to discuss it in GTW and 2nd round. To facilitate the 2nd round discussion, I only kept the options explicitly supported by companies in 1st round, but companies can still retrieve the 1st round options if they want in 2nd round. Agreement will be captured in the WF. |

Sub-topic 2-3 AGC/Cell measurement/synchronization sample number related enhancement for L3 part

|  | Status summary |
| --- | --- |
| Issue 2-3-1: Align the understanding of cell measurement /synchronization /AGC in the existing FR2 unknown SCell activation delay requirement | Discussion status: Majority companies supported option 1 (6 companies) and option 1a (8 companies). Candidate options: - Option 1(Intel, CMCC, Ericsson, OPPO, Nokia, ZTE): RAN4 should align the understanding of cell measurement/synchronization/AGC/T/F tracking in the existing FR2 unknown SCell activation delay requirements - Option 1a (QC, HW, OPPO, Nokia, vivo, ZTE, CTC, MTK): in exisiting FR2 unknown SCell activation case: - TFirstSSB_MAX + 15*TSMTC_MAX: AGC (1 sample for coarse AGC and 1 sample for fine AGC) - 8*Trs: cell search, time/frequency synchronization, time/frequency tracking - Option 2 (Apple, Xiaomi): Option 1 is not needed. Recommendations for 2nd round: Recommend to further discuss options in 2nd round and how option 1/1a can help to facilitate the enhancement discussion in the following issues. Please companies also check if the UE implementation can be directly discussed/considered in the following individual enhancement issues. Agreement will be captured in the WF. |
| Issue 2-3-2: enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part of current FR2 unknown SCell activation delay | Discussion status: 7 companies supported option 1, 3 companies supported option 2, 3 companies supported option 3, and 3 companies supported option 4. Candidate options: - Option 1 (Apple, LGE, CMCC, Ericsson(for semi-unknown), OPPO, Nokia, ZTE): reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 Scell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX. - Option 2 (Ericsson(for semi-unknown), Nokia, ZTE): RAN4 to discuss the potential of simplified AGC settling with 1 sample. - Option 3 (HW, vivo, MTK): The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown Scell activation scenario. - Option 4 (Ericsson(for semi-unknown), Nokia, ZTE): based on option 3 in issue 2-1-1 - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX. Recommendations for 2nd round: Recommend to further discuss options in 2nd round. Agreement will be captured in the WF. |
| Issue 2-3-3: enhancement of “8*Trs” part of current FR2 unknown SCell activation delay | Discussion status: Views are quite diverse among companies on option 1/2/3. Candidate options: - Option 1 (Apple, QC (for semi-unknown), OPPO, Nokia, ZTE): for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement. - Option 2 (CMCC, OPPO, ZTE): if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed. - Option 3 (vivo, MTK) The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown SCell activation scenario. Recommendations for 2nd round: Recommend to further discuss options in 2nd round. Since different UE may have different implementation of using this 8*Trs part, please companies double check if the option 2 can be used as a middle ground for compromise. Agreement will be captured in the WF. |

Sub-topic 2-4 RS related enhancement for L3 part

|  | Status summary |
| --- | --- |
| Issue 2-4-1: RS related enhancement of FR2 unknown SCell activation | Discussion status: Views are quite diverse among companies on option 1/2/3/4. Candidate options: - Option 1 (Xiaomi, HW, OPPO, CTC): The A-TRS can be used for L3 measurement for FR2 unknown Scell activation if the QCL information of the A-TRS is provided to UE. - Option 1a (Xiaomi, ZTE, CTC): If the triggered A-TRS is QCL-ed with the SSB of inter-band SpCell or one of inter-band active serving cell, A-TRS is configured for AGC adjustment, cell search and fine timing tracking for FR2 unknown Scell activation. - Option 1b (ZTE, CTC): The extension of A-TRS based unknown FR1 Scell activation into inter-band scenario can also be introduced. - Option 2 (Apple, QC, LGE, Intel, Ericsson, MTK): AP-CSI-RS and/or A-TRS based fast Scell activation is not apply to L3 part for unknown FR2 Scell activation enhancement. - Option 3 (Nokia): based on option 2 in issue 2-1-1: - RAN4 to study the solution to reduce the unknown Scell activation delay based on the A-TRS. - A-TRS can be triggered based on the latest measurement status at UE. - Option 4 (Xiaomi, HW, ZTE, CTC): - UE can obtain coarse timing information via QCL typeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped. - UE can obtain beam information via QCL typeD to inter-band serving cell subject to gNB configuration. - For FR2 unknown Scell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms. Recommendations for 2nd round: Recommend to further discuss options in 2nd round. Agreement will be captured in the WF. Based on the 1st round discussion, encourage companies to further consider the MRTD/MTTD/CBM related issues when using QCL information from inter-band serving CC. |

Sub-topic 2-5 Others for L3 part

|  | Status summary |
| --- | --- |
| Issue 2-5-1: Other enhancement for L3 part in FR2 unknown SCell activation | Discussion status: Views are quite diverse among companies. Candidate options: - Proposal 1 (Ericsson, QC(for semi-unknown)): RAN4 to study the use of SSB periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation. - Proposal 1a (Apple): RAN4 to study the use of SMTC periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation. - Proposal 1b (HW, Nokia, MTK):the proposal 1 is already in existing spec. - Proposal 2 (Ericsson): RAN4 to agree that UE sends a beam information report to during or at the end of L3 part of the procedure. Contents of the beam information report are FFS. Recommendations for 2nd round: Recommend to further discuss options in 2nd round. Agreement will be captured in the WF. For P1, please note the periodicity in original P1 is SSB periodicity but not SMTC periodicity. P2 is related with other issues, e.g., issue 2-1-1, 3-1-1 and 3-1-3, please companies check if we can discuss P2 in those issues and remove P2 from this issue 2-5-1. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

### Sub-topic 2-1 Scenarios for FR2 SCell activation enhancement

Issue 2-1-1: Scenarios/status/categories for FR2 SCell activation enhancement

- Option 1 (Qualcomm, Nokia, vivo(1st and 3nd bullet), ZTE):

  - Unknown SCell in FR2 needs to split in two categories for FR2 SCell activation delay reduction purpose. 1) completely unknown SCell, 2) semi-unknown SCell.

  - SCell activation delay reduction should be applied for semi-unknown SCell scenario where SCell is activated from deactivated state.

  - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario.

- Option 2 (Nokia, ZTE):

  - The availability of a valid L3 measurement result at the time of SCell activation shall be considered to reduce the SCell activation delay.

  - The UE indication on the up-to-date L3 measurement status of the to-be-activated SCell is introduced to reduce the FR2 unknown SCell activation delay.

- Option 3 (Ericsson, QC(scenario 1), Nokia, ZTE):

  - RAN4 to discuss and specify SCell activation for following two cases.

    - Scenario1: SCell is unknown due to the fact that UE did not sent measurement report to gNB in last X seconds.

    - Scenario2: SCell is unknown due to the fact that UE may be measuring it for first time.

- Option 4 (last meeting agreement) (Apple, LGE, Xiaomi, HW, OPPO, CTC, MTK): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

  - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

  - The extension of the enhancement solutions to FR1 can also be discussed.

- Option 5 (Moderator):

  - RAN4 to use option 4 as baseline to discuss the enhancement, and the new UE categorization based on option 1/2/3 can be discuss case-by-case in enhancement issues to determine whether and how to use such new category for enhancement.

GTW (Oct-14) agreement:

  - RAN4 to use option 4 as baseline to discuss the enhancement, and further unknown SCell scenario categorization can be discussed case-by-case in enhancement issues to determine whether and how to use such new category for enhancement.

  - Option 4 (last meeting agreement): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

    - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

    - The extension of the enhancement solutions to FR1 can also be discussed.

Recommendations for 2nd round:

Moderator:

Based on GTW discussion, the general direction is clear for this issue. Please companies continue discussion on details of further unknown SCell scenario categorization.

Recommend to discuss it in GTW and 2nd round, agreements will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | Thanks moderator for the good organization. On top of GTW agreement, we would like to make sure we are not proposing new state of unknown. Semi-unknown is just wording to help better understanding the scenario. Also we have one suggestion to moderator. If companies are fine, could you please consider having a new sub-topic regarding L3 measurement report during L3 procedure (legacy or enhanced way) from Qualcomm/Nokia/Ericsson proposals for next meeting? E.g) how L3 report can be configured/allowed/triggered during L3, what UE can report and how frequently, what is UE/NW behavior after report (keep doing L1 (enhanced way) or skip to TCI activation command, etc). we think this is necessary step before talking about case by case. We also put more comments about the framework at 3-5-1. Thanks! |
| Xiaomi | According to the existing known condition for FR2 SCell, the further unknown SCell scenario can be categorized as follows: Scenario 1: The target SCell has never been measured to the UE; Scenario 2: UE has not sent a valid L3-RSRP measurement report with SSB index during the period equal to 4s 3s/4s before receiving SCell activation command; Scenario 3: During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes is not detectable |
| Moderator | Thanks Qualcomm for the suggestion! In general it makes sense to me, and it’s also in line with my plan to have a new sub-topic for discussing how to consolidating this condition/categorization/classification (no matter whatever we call it, to me it’s just a condition to help or determine enhancement). But whether or how to use such new condition for enhancement can be discussed in each individual issue. The new sub-topic in my mind could be: - Sub-topic x: Consolidating the further unknown SCell scenario categorization (e.g., L3 measurement/reporting condition for Scell activation enhancement) Note: the condition/scenario categorization here does not necessarily means RAN4 will have new definition for unknown SCell. It’s up to the conclusion from discussion. - Option 1: e.g., QC’s proposal - Option 2: e.g., Nokia’s proposal - Option 3: e.g., Ericsson’s proposal. - Option 4: others Please note that: the above tentative plan for sub-topic is for NEXT meeting. In this meeting, please continue discussion based on the options under current issue 2-1-1. |
| Ericsson | We think introducing a new state between known and unknown may be helpful for further organize the discussion and also clear UE and NW behaviour. Definition of new state can be further discussed. |
| LGE | Fine with the GTW agreement. We are still unclear which UE can apply semi-unknown procedure without any definition of condition or state for completely unknown and semi-unknown. |
| Intel | Generally, we are fine to further discuss the detail case how L3 part can be further reduced based on some prior information. |
| OPPO | Prefer not to change current definition of known/unknown state. We are fine with the suggestion by moderator and open to further discuss how to use new condition for enhancement in next meeting. |
| Nokia | We agree to QC’s proposal and the sub-topic approach from moderator. As discussed in GTW, it seems companies are fine to discuss different conditions of unknown SCell, wherein if the SCell has been measured i.e. the measurement status is well recognized. It would be helpful to list this measurement relevant condition explicitly as a separate sub-topic in the WF. Other conditions are not excluded and can be further discussed. |
| China Telecom | Support option 4/5. We support to use option 4 as baseline to discuss the enhancement, and we are fine to further discuss case-by-case about the scenario categorization. |
| Huawei | We are fine with GTW agreements, and still suggest to discuss the “further divided known/unknown state” under the related delay component. As it may not work for all delay reduction solutions, e.g. it may only work for detection delay reduction or only for L1-RSRP reduction. |
| ZTE | Fine with the GTW agreements. Further more, how to differentiate between complete unknown and semi unknown, which should be identified to avoid misunderstanding between UE and NW if different reduction applied for the two categories. |
| vivo | Support GTW agreements. For the scenario categorization, we think the following aspects need to be considered: 1. The L3 measurement status of the UE, before L3 measurement report, which is mostly event-trigger based, is sent to gNB. 2. The indication of network_synchronization assumption which may relax UE requirements on MRTD. 3. The UE capability in FR2, which may reduce Rx beam or shorten legacy procedures, by the help of the information that obtained in 1 or 2 above. |
| MTK | Support the GTW agreement. We are also open to discuss the new-sub topic in the next meeting. |
| Moderator | Agreement for final check: - RAN4 to use option 4 as baseline to discuss the enhancement, and further unknown SCell scenario categorization can be discussed case-by-case in enhancement issues to determine whether and how to use such new category for enhancement. - Option 4 (last meeting agreement): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI - FR2 unknown Scell without intra-band serving cell is considered for 1st phase. - The extension of the enhancement solutions to FR1 can also be discussed. - FFS on option 1/2/3 and other options are not precluded. |

### Sub-topic 2-2 Beam related enhancement for L3 part

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- Option 1 (Apple, LGE, Intel, CMCC, Xiaomi, Ericsson, OPPO, Nokia, vivo): RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation.

  - Option 1a (Apple, Intel, CMCC, vivo): for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline.

  - Option 1b (Apple, QC(for semi-unknown case), Intel, CMCC, vivo): Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 Scell activation.

- Option 2 (Xiaomi): RAN4 to introduce the state of not performing full Rx beam sweeping for L3 measurement during FR2 SCell activation.

  - When the measured RSRP is higher than a threshold, the UE enters in the state of not performing full Rx beam sweeping. The UE can perform the measurement with a sub-set of Rx beam or non-Rx beam sweeping during this state.

  - When RSRP variance is larger than a threshold, the UE exits the non-changing Rx beam state and perform the full Rx beam sweeping.

- Option 3 (LGE, vivo(1st bullet)):

  - P1-1: Perform L3 (cell synchronization, measurement,…) with reduced Rx beam sweeping (e.g., rough Rx beam), and then perform L1-RSRP with reduced Rx beam sweeping (e.g., narrow beam sweeping corresponding to selected rough Rx beam)

  - P1-2: Perform L3 (cell synchronization, measurement,…) with full Rx beam sweeping (e.g., narrow Rx beam), and then skip the L1-RSRP procedure

- Option 5 (LGE, CTC):

  - Directly reduce the Rx beam number from 8 to [x] for each component in L3 part, but the corresponding performance loss would be verified to be acceptable.

- Option 7 (QC(for pure unknown case), HW, MTK):

  - Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay.

- Option 9 (QC): from on option1 in Issue 2-1-1,

  - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario.

Recommendations for 2nd round:

Recommend to discuss it in GTW and 2nd round. To facilitate the 2nd round discussion, I only kept the options explicitly supported by companies in 1st round, but companies can still retrieve the 1st round options if they want in 2nd round. Agreement will be captured in the WF.

For proponents of option 5, please check if you can compromise to option 1, since the FR2 SCell activation enhancement itself can be an optional feature in moderator’s view and then option 5 can be treated as a sub-option under option 1.

As commented by other companies, this issue is also related with issue 2-1-1 (sub-category of pure-unknown or semi-unknown), moderator suggestion is we can also discuss those sub-category in this issue to see if Rx beam reduction enhancement rely on such sub-category or not.

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with option1. We also remove the semi unknown wording to avoid confusion because regardless of UE’s measurement status it is unknown in NW side. |
| Xiaomi | Support option 1/1a, however, we think the side condition to apply this UE capability is needed, in my understanding, the Rx beam is less than 8 may not apply to all the SCell activation scenarios, e.g. blinding SCell activation, 8 Rx beam should be assumed even UE support this UE capability. |
| Apple | We are fine with option 1. And option 1a/1b can be discussed together with the new condition in issue 2-1-1 in next meeting. |
| Ericsson | We support option 1. |
| LGE | We are fine with option 1/1b. But we think we don’t need FR2 HST or Rel-17 positioning as baseline in option 1a. |
| Intel | Fine with Option 1/1a/1b for further discussion. |
| CMCC | Option 1/1a/1b |
| OPPO | Support option 1. |
| Nokia | We support Option 1. In addition, we need discuss in what condition or which step can this reduced beam sweeping factor be used. This could be relevant to Issue 2-1-1 and other options. |
| China Telecom | We are fine with the recommendations from moderator. It is considerable to discuss whether the performance loss of reduced Rx beam sweeping factor can be acceptable. |
| Huawei | We are still not sure whether we can directly introduce the capability without analyzing the conditions and impact first. To us, introducing the capability is how to reflect the conclusions in the spec and it should be the last step of the discussion. However, it seems like we are just skipping the discussion on the conditions/scenarios when the beam sweeping can be reduced and decide to introduce the capability at the very first step. From our understanding, it is more reasonable to analyze the conditions/scenarios first as option 2/3. If the applicable scenario is confirmed, then we can introduce the capability. But for now, it even not clear whether the capability only work for SCell activation or also work for normal L3 measurement. So we propose following compromise solutions: RAN4 to consider introducing the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 Scell activation which is subject to the discussion on feasible conditions and scenarios. |
| ZTE | Support Option 1/1a/1b. |
| vivo | Support option 1 |
| MTK | Option 7. We still think Rx beam sweeping factor should not be reduced at this stage especially if we are considering to skip L1-RSRP measurement. It means this will be the only stage where UE can obtain proper information about the beams. We appreciate companies want to enhance the delay but reducing Rx beam sweeping factor during AGC/cell search could have very bad impact on the performance or even link failure, e.g., when the UE is moving or rotating. We think the reduction is not justified. |
| Moderator | Narrow down the options for next meeting discussion. Agreement for final check: FFS: - Option 1: RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation. - Option 1a: for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline. - Option 1b: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 Scell activation. - Option 2: - Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay. |

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

- Option 2 (QC, LGE, Intel, HW, vivo): Don’t need to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI at least in 1st phase.

- Option 3 (Apple, Intel, CMCC, Xiaomi, Ericsson, OPPO, Nokia, ZTE, CTC, MTK): RAN4 will discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case.

- Option 4 (ZTE): For the UE capable of 2 panels, the conclusion on the measurement delay reduction in WI of FR2 multi-panel Rx can be applied to the L3 procedure in FR2 Scell activation. Since the WI of FR2 multi-panel Rx is also discussed in parallel, so we can first discuss the Scell activation without considering multi-panel Rx.

- Option 5 (Ericsson): RAN4 to agree to apply relevant agreements of multi-RX chain to Scell activation delay too, if the UE supports this capability.

- Option 6 (moderator):

  - In 1st phase, RAN4 not to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI.

  - In 2nd phase, if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case, RAN4 to discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI.

Recommendations for 2nd round:

Based on the 1st round discussion, moderator proposed an option 6 to merge the option 2 and 3.

Recommend to discuss it in GTW and 2nd round. To facilitate the 2nd round discussion, I only kept the options explicitly supported by companies in 1st round, but companies can still retrieve the 1st round options if they want in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We are okay with moderators’ recommendations (option 6) |
| Xiaomi | Support option 3, and we are also fine with option 6 from moderator |
| Apple | Support Option 6. |
| Ericsson | We think currently scoping discussion is ongoing in multi-RX discussion. We could revisit this once the scope is clear in multi-RX discussion. |
| LGE | We are fine with the moderator suggestion as option 6. Just for question of “1st phase and 2nd phase”, is there timeline for those? |
| Intel | Fine with Option 6. |
| CMCC | Both option 3 and option 6 are OK for us. |
| OPPO | Fine with Option 6. |
| Apple2 | To LGE: 1st phase and 2nd phase has been defined in the agreed work plan (R4-2215599) in this meeting. |
| Nokia | We are fine with Option 6. |
| China Telecom | Support option 6 about the recommendations for the 1st and 2nd phase from the moderator. |
| Huawei | Fine with option 6. |
| ZTE | Fine with Option 6. |
| MTK | Fine with option 6. |
| Moderator | Please Ericsson check if can compromise to option 6. Agreement for final check: - In 1st phase, RAN4 not to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI. - In 2nd phase, if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case, RAN4 to discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI. |

### Sub-topic 2-3 AGC/Cell measurement/synchronization sample number related enhancement for L3 part

Issue 2-3-1: Align the understanding of cell measurement /synchronization /AGC in the existing FR2 unknown SCell activation delay requirement

- Option 1(Intel, CMCC, Ericsson, OPPO, Nokia, ZTE): RAN4 should align the understanding of cell measurement/synchronization/AGC/T/F tracking in the existing FR2 unknown SCell activation delay requirements

  - Option 1a (QC, HW, OPPO, Nokia, vivo, ZTE, CTC, MTK): in existing FR2 unknown SCell activation case:

    - TFirstSSB_MAX + 15*TSMTC_MAX: AGC (1 sample for coarse AGC and 1 sample for fine AGC)

    - 8*Trs: cell search, time/frequency synchronization, time/frequency tracking

- Option 2 (Apple, Xiaomi): Option 1 is not needed.

Recommendations for 2nd round:

Recommend to further discuss options in 2nd round and how option 1/1a can help to facilitate the enhancement discussion in the following issues. Please companies also check if the UE implementation can be directly discussed/considered in the following individual enhancement issues. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We support option1 just from sharing basic understanding perspective. but it does not mean the enhancement is subject to each individual component (e.g coarse AGC / fine AGC/ cell search). |
| Xiaomi | Fine option 1a |
| Apple | We still cannot agree on option 1, and we think discussion can continue directly in the following issue for activation enhancement; the reason is the option 1a is purely up to UE implementation. We also have question to companies, if cell search is not performed together with AGC, how UE can judge if the AGC settling is good or not? In our view, the RSSI cannot help during coarse AGC settling. But only when UE really detects something by using certain AGC, UE can tell that this AGC is OK to use. |
| Ericsson | We think if the cell search or SSB detection is not performed during AGC, we are not sure how UE can store the AGC? If SSB detection is performed during coarse and fine AGC, we are not what is UE operation in cell search phase. Clear common understanding is helpful in achieving delay reduction possible. |
| LGE | Similar view with QC. We think option 1 can be just basic understanding for L3 part for the group. |
| Intel | Support option 1 for clarify common understanding regarding L3 part. |
| CMCC | Support option 1. Allign the understanding could help the discussion. As for Option 1a, if this is companies’ common understanding, we are OK with it. |
| OPPO | Support option 1. It is helpful to further analysis with this basic understanding. Option 1a could be one kind of possibile UE implementation or assumption. |
| Nokia | We can understand the different UE implementation but it would be good clarify the UE behaviour as we are discussing the potential of optimizing each of the steps. The controversial part seems to be “cell search”, and if AGC is included in cell search step or in parallel. Reading from RAN1 spec (as below), it includes time/freq sync based on PSS/SSS which sounds to be PSS/SSS detection time which is 24*Trs in RAN4. Could companies clarify the mapping of these terminologies: cell search, PSS/SSS detection, T/F sync and AGC? Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. |
| China Telecom | Support option 1a about the basic understanding. |
| Huawei | For option 1a, we think it is the assumption from spec perspective. But we also agree with Apple that it is up to UE implementation. So we are not sure how to use the conclusion of the issue if any? |
| ZTE | Support Option 1 and 1a. Aligned understanding is helpful to the further discussion of reduction. |
| vivo | Same comment as 1st round. |
| MTK | Option 1/1a. It can be helpful to align the basic understanding regarding L3 part. |
| Moderator | FFS on all options in next meeting. |

Issue 2-3-2: enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part of current FR2 unknown SCell activation delay

- Option 1 (Apple, LGE, CMCC, Ericsson(for semi-unknown), OPPO, Nokia, ZTE): reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 Scell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX.

- Option 2 (Ericsson(for semi-unknown), Nokia, ZTE): RAN4 to discuss the potential of simplified AGC settling with 1 sample.

- Option 3 (HW, vivo, MTK): The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown Scell activation scenario.

- Option 4 (Ericsson(for semi-unknown), Nokia, ZTE): based on option 3 in issue 2-1-1

  - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX.

Recommendations for 2nd round:

Recommend to further discuss options in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We have question for option1, it assumes there is UE capability to support indication of beam sweeping factor or number of SSBs from issue 2-2-1? Without UE capability, we support option3. |
| Xiaomi | From sample number reduction point of view, we support option 3. And in my understanding, option 1 is not correct, since the delay of TFirstSSB_MAX + 15*TSMTC_MAX is for AGC, as clarified in option 1a in issue 2-3-1. |
| Apple | We support option 1. To QC: we assume UE capability is needed. We think the enhancements in this R18 WI shall be all optional features. |
| Ericsson | We support option 4. Further we need to discuss case by case like semi-unknown state or completely unknown state. |
| LGE | We support option 1. We think that further study is needed to define which conditions option 1 can be available depending on the candidate solutions of other issues. |
| Intel | Further discuss these number for detailed case whether some prior information is known or not. |
| CMCC | Support Option 1. As for whether to dicuss this issue case by case, we are open for discussion. |
| Nokia | We are fine with Option 1/2/4, and share E///’s view. To discuss the possibility of delay reduction, probably we need understand the difference of the UE measurement on the 3 samples. Could some company give an example? This may also help the discussion in Issue 2-3-2. |
| China Telecom | Fine with option 3. We are not sure if the reduced sample number has influence on the performance. |
| Huawei | Currently we support option 3 based on the understanding that the beam sweeping reduction is another issue. We think further study is needed on the conditions. To Apple about UE capability: The capability is for enhanced Scell activation or sample reduction itself? |
| ZTE | Support Option 1. Regarding to the discussion case by case, we are open for further discuss. |
| vivo | Option 3. |
| MTK | Support Option 3. As we commented in the first round for Option 1, we don’t think the side condition of -2dB SINR can be related to AGC. AGC is related to the total received power, not to the SNR. For example, if the total received power at the UE is very strong, the first sample could be saturated and therefore UE may still need for the second sample to settle the Rx AGC. |
| Moderator | FFS on all options in next meeting. |

Issue 2-3-3: enhancement of “8*Trs” part of current FR2 unknown SCell activation delay

- Option 1 (Apple, QC (for semi-unknown), OPPO, Nokia, ZTE): for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement.

- Option 2 (CMCC, OPPO, ZTE): if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed.

- Option 3 (vivo, MTK) The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown SCell activation scenario.

Recommendations for 2nd round:

Recommend to further discuss options in 2nd round. Since different UE may have different implementation of using this 8*Trs  part, please companies double check if the option 2 can be used as a middle ground for compromise. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We are okay option2 with changes “8*Trs can be reduced or removed”. |
| Xiaomi | From sample number reduction point of view, we support option 3. And in my understanding, option 1 is not correct, since the delay of 8 Trs is for cell search, as clarified in option 1a in issue 2-3-1. |
| Apple | We support option 1. As discussed in issue 2-3-1, the 8*Trs does not have to be cell searching time, but it’s up to UE implementation, and we think cell search has already contained in the TFirstSSB_MAX + 15*TSMTC_MAX part. We can also agree with option 2 with QC’s suggestion, that is: - Option 2a: if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be reduced or removed. |
| Ericsson | We think we need further discussion when the 8Trs can be removed. But in general, under certain conditions it is possible. |
| Intel | Same comments as 2-3-2. |
| CMCC | OK with option 2. |
| OPPO | OK with the updated option 2a. In this case, option 1 and 2a are similar. |
| Nokia | We are fine with Option 1/2 in general. It is a bit difficult to agree on the wording without aligning the understanding on each of the step. In Option 2/2a, it seems assuming AGC is included in cell sync? |
| Huawei | Based on the assumption the 8Trs is for cell detection, the conditions needs further discussion. And we think it is also related to issue 2-4-1. We propose to add following option: Option 2c: If timing information can be acquired from QCL-ed inter-band serving cell, 8*Trs can be removed. |
| ZTE | Fine with Option 1 and 2a. |
| vivo | Option 3 |
| MTK | Support Option 3. To Apple since this is up to UE implementation and some companies think cell search cannot be performed during AGC. Therefore, we cannot accept Option 1. Also, some companies want to reduce the number of samples for AGC and want to remove cell search and L1-RSRP as well; and on top of that they want to reduce Rx beam sweeping factor. This means we will end up having only one sample for L3/L1 part. I wonder how UE supposed to retune AGC, do cell search and obtain info on beams for the unknown SCell in FR2 with ONLY one sample! |
| Moderator | FFS on all options in next meeting. |

### Sub-topic 2-4 RS related enhancement for L3 part

Issue 2-4-1: RS related enhancement of FR2 unknown SCell activation

- Option 1 (Xiaomi, HW, OPPO, CTC): The A-TRS can be used for L3 measurement for FR2 unknown Scell activation if the QCL information of the A-TRS is provided to UE.

  - Option 1a (Xiaomi, ZTE, CTC): If the triggered A-TRS is QCL-ed with the SSB of inter-band SpCell or one of inter-band active serving cell, A-TRS is configured for AGC adjustment, cell search and fine timing tracking for FR2 unknown Scell activation.

  - Option 1b (ZTE, CTC): The extension of A-TRS based unknown FR1 Scell activation into inter-band scenario can also be introduced.

- Option 2 (Apple, QC, LGE, Intel, Ericsson, MTK): AP-CSI-RS and/or A-TRS based fast Scell activation is not apply to L3 part for unknown FR2 Scell activation enhancement.

- Option 3 (Nokia): based on option 2 in issue 2-1-1:

  - RAN4 to study the solution to reduce the unknown Scell activation delay based on the A-TRS.

  - A-TRS can be triggered based on the latest measurement status at UE.

- Option 4 (Xiaomi, HW, ZTE, CTC):

  - UE can obtain coarse timing information via QCL typeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped.

  - UE can obtain beam information via QCL yped to inter-band serving cell subject to gNB configuration.

  - For FR2 unknown Scell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

Recommendations for 2nd round:

Recommend to further discuss options in 2nd round. Agreement will be captured in the WF.

Based on the 1st round discussion, encourage companies to further consider the MRTD/MTTD/CBM related issues when using QCL information from inter-band serving CC.

| Company | Comments |
| --- | --- |
| Qualcomm | we support option2. We don’t think CBM is applicable for FR2 inter-band but IBM is used. We are not clear how UE get QCLed information before reporting. |
| Xiaomi | Support option 1/4, and we are open to study the MRTD/CBM related issues when using QCL information from inter-band serving CC. |
| Apple | Option 2. Same view as QC for CBM part. Regarding the timing part, in 1st round network vendor thought network can be aware if the TAE is quite small or not, but we are not sure when beam cannot be aligned, how much timing difference UE can see at DL reception side even the TAE is small. If UE directly reuse the timing from inter-band serving cell but the real RTD is larger than CP, then it may result into a failed activation. We would like to understand more on how network can assume such small RTD at UE. |
| Ericsson | Agree with option 2. |
| LGE | We support option 2. We are not clear how UE can get QCL assumption to A-TRP under unknown SCell case, and also not clear that QCL with the SSB of inter-band can be used. |
| Intel | Prefer option 2. Similar view as Qualcomm and Apple. |
| Nokia | We prefer Option 3. We think this also depends on the conditions of unknown. In case of semi-unknown case, and if the UE is able to send L3 reporting, it in practice turns the SCell into known state hence A-TRS can be applied. Probably this is not exactly A-TRS for L3 part as L3 part is not needed. Anyway, this scenario needs to be discussed. |
| China Telecom | Support option 1/1a/1b/4. We think the use of A-TRS is up to NW configuration, if NW has better time alignment, NW can configure information for UE to use A-TRS to obtain timing and beam information from inter-band active serving cell. In practical deployment scenario, network is able to achieve better time alignment under some conditions, and the RTD can be reduced to a smaller value compared with legacy requirement. We are open to discuss the conditions of practical deployment. |
| Huawei | We support option 1/4. Actually, we believe these options are discussion different aspects. For option 2, the right understanding is that UE can not perform L3 measurement based on A-TRS for FR2 unknown SCell as A-TRS can not be used for cell searching. But option 1 and option 4 are about UE can obtain coarse timing and beam information via inter-band QCL instead of performing measurement based on A-TRS. For the timing part, we agree with Apple that is should be under certain conditions when the RTD is small. It doesn’t not means it can apply to all inter-band case. Thus, we are open to further discuss the conditions. For the beam information, we believe such inter-band QCL type D indication may depend on UE implementations. The beam mapping between inter-band may be not that accurate. We are open to further discuss the feasibility and conditions. We would like to rephrase option 4 as follows: - UE can obtain coarse timing information via QCL typeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped. - The conditions are FFS (e.g. RTD is smaller than X) - UE can obtain beam information via QCL yped to inter-band serving cell subject to gNB configuration. - The feasibility and conditions are FFS. |
| ZTE | Support Option 1/1a/1b and 4. Regarding to the concern raised by Qualcomm, to our understanding, the assumption of IBM for inter-band CA means independent beam, rather than different beam assumption between inter-band serving cells. For the case of co-located inter-band CA, why need to apply different QCL-TypeD between inter-band serving cells? Regarding to the concern on TAE, in practice much smaller value can be expected from the perspective of inter-band CA BS acchitecture. Figure 1. The general illustration of inter-band CA BS architecture |
| vivo | FFS. Agree that UE capability should be considered. |
| MTK | Support Option 2. |
| Moderator | FFS on all options in next meeting. |

### Sub-topic 2-5 Others for L3 part

Issue 2-5-1: Other enhancement for L3 part in FR2 unknown SCell activation

- Proposal 1 (Ericsson, QC(for semi-unknown)): RAN4 to study the use of SSB periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation.

- Proposal 1a (Apple): RAN4 to study the use of SMTC periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation.

- Proposal 1b (HW, Nokia, MTK):the proposal 1 is already in existing spec.

- Proposal 2 (Ericsson): RAN4 to agree that UE sends a beam information report to during or at the end of L3 part of the procedure. Contents of the beam information report are FFS.

Recommendations for 2nd round:

Recommend to further discuss options in 2nd round. Agreement will be captured in the WF.

For P1, please note the periodicity in original P1 is SSB periodicity but not SMTC periodicity.

P2 is related with other issues, e.g., issue 2-1-1, 3-1-1 and 3-1-3, please companies check if we can discuss P2 in those issues and remove P2 from this issue 2-5-1.

| Company | Comments |
| --- | --- |
| Qualcomm | We are okay with discussing P2 in 2-1-1, 3-1-3. As we suggested in 2-1-1, Could we consider having a new sub-topic regarding L3 measurement report during L3 part for next meeting? |
| Xiaomi | P#1: prefer to use SMTC periodicity, if SSB periodicity is used, the impact on other measurements should be considered, e,g, L1-RSRP measurements. P#2: prefer to discuss P2 with other issues together, e,g, issue 2-1-1, 3-1-1 and 3-1-3. |
| Apple | We support proposal 1a and agree this 1a is already covered in current spec. |
| Ericsson | Support Proposal 1 and proposal 2. |
| Intel | For P1, we prefer not to use SSB periodicity for L3 measurement and still use SMTC. it will have impact on L1 measurement since it’s FR2. We are fine with 1a. For P2, we are fine to discuss it in issue 2-1-1, 3-1-3. |
| Nokia | We are fine to further discuss P1 and P2. |
| Huawei | For P1, we share similar view as Xiaomi. For P1a, yes if there is no other serving cell in the same band. |
| vivo | FFS |
| ZTE | Prefer to use SMTC periodicity rather than SSB periodicity since here we discuss L3 part. So prefer 1a. |
| MTK | For P1 we have same view as Xiaomi. For P1a, in our understanding, this is already in existing spec. |
| Moderator | FFS on all proposals in next meeting. |

# Topic #3:	L1 part enhancement for FR2 SCell activation (6.9.2.2)

Main technical topic overview. The structure can be done based on sub-agenda basis.

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215456 | Xiaomi | Proposal 5: For the beam sweeping factor enhancement for L1-RSRP measurement, the conclusions on beam sweeping factor enhancement for L3 measurement can be reused. Proposal 6: It is feasible to consider reporting the SSB indexes during the cell search procedure to reduce L1-RSRP measurement delay. Proposal 7: A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index. |
| R4-2215801 | LG Electronics Inc. | Proposal 3: Deprioritize prioritization enhancement for L1-RSRP that may affect other measurement behaviors |
| R4-2215807 | NTT DOCOMO, INC. | Observation 1: When UE having capability for L1-RSRP measurement for a cell with different PCI serving cell have been finished L1-RSRP measurements, there would be no need treat that as unknown cell. The other FR2 SCell can be treated as the cell with at least one active serving cell on that FR2 band. Proposal 2: FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell. |
| R4-2216744 | Qualcomm Incorporated | Proposal 3: When Scell is semi-unknown to UE, NW can trigger UE to report RSRP before L1 RSRP measurement and report. The framework can be illustrated as following steps while UE perform the legacy FR2 Scell activation procedure. Step 1. NW configure either periodic or aperiodic (event-based) report config before SCell activation command. Step 2. NW trigger preconfigured report config. (e.g MAC-CE can trigger the report config) Step 3: UE report latest RSRP measurement for target SCell Step 4: UE can skip L1-RSRP measurement and report when UE reports RSRP measurement with associated SSB index during step3. Step 5: Either A-TRS based or SSB-based for fine time tracking. TCI activation command and CSI-RS recourse activation for CQI reporting. Proposal 4: optional UE capability can be used to specify the number of SSBs to be measured before RSRP reporting or the number of A-TRS. |
| R4-2215357 | Intel Corporation | Observation 1: For known case, L3-RSRP measurement will be reported with SSB index and TCI state is selected based on L3 report. Proposal 1: L1-RSRP measurement can be skipped and L3 measurement result can be used for measurement reporting with SSB index. Proposal 2: TCI activation may be skipped. UE can assume to use the reported best beam assumption for the following PDCCH and CQI measurement. Proposal 3: Fine timing tracking is still needed no matter whether TCI activation is skipped or not. Proposal 4: Semi-persistent CSI-RS activation or RRC based CSI configuration command can be sent with SCell activation command together. |
| R4-2215601 | Apple | Proposal 1: for unknown R18 FR2 SCell activation enhancement, following L1 part enhancement alternatives can be considered: - skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs - UE can also report the beam information based on cell search and skip the L1-RSRP procedure - Introduce UE capability to support beam sweeping factor reduction for SSB based L1 measurement Proposal 2: the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown SCell activation requirement. Proposal 3: L1-RSRP measurement during FR2 unknown SCell activation has higher priority over L3 measurement and it’s performed in non-DRX mode even DRX is configured. Proposal 4: for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused. Proposal 5: for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE. Proposal 6: for unknown R18 FR2 SCell activation enhancement, following enhancements based on AP CSI-RS and/or A-TRS are considered: - Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement - Use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command |
| R4-2215718 | CMCC | Proposal 1: the L1-RSRP measurement (TL1-RSRP, measure) could be skipped under some conditions (e.g.. the RS configured for L1-RSRP measurement has been measured during L3 synchronization or is QCLed type D with the RS for L3 procedures). Proposal 2: if L1-RSRP measurement (TL1-RSRP, measure) need to be kept for some cases, it is proposed to consider reduction on RX beam sweeping factor. Proposal 3: for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline. Proposal 4: if TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report, delay on uncertainty of TCI configuration/activation can be saved. Otherwise, delay on uncertainty of TCI configuration/activation is kept. |
| R4-2215786 | Nokia, Nokia Shanghai Bell | Proposal 1: For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection. Proposal 2: RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell. Proposal 3: Do not consider the sample number reduction for L1-RSRP measurement as 1 sample has been assumed for SCell activation. Proposal 4: The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance. |
| R4-2215810 | OPPO | Proposal 1: Consider beam sweeping factor reduction for L1 measurement during unknown FR2 SCell activation. Proposal 2: Define the same UE capability of Rx beam sweeping factor in FR2 for L1 and L3 measurement. Proposal 3: Consider the feasibility of skipping L1-RSRP measurement, and study its conditions, e.g., L1/L3 measurement configuration at least including RSs type/RS BW/QCLed RS for L1 and L3 measurement. Proposal 4: When TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report, the uncertainty of TCI configuration/activation can be saved. Observation 1: A-TRS can be configured to conduct fine timing tracking for TCI after the TCI activation command to reduce the waiting time for next SSB/CSI-RS. Proposal 5: Use A-TRS corresponding to the TCI state for fine timing tracking after TCI activation command. |
| R4-2215866 | vivo | Proposal 1: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for L1-RSRP measurement during unknown FR2 SCell activation. Proposal 2: When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8. Proposal 3: During the SCell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can be reduced accordingly. |
| R4-2216273 | Huawei, HiSilicon | Observation 1: The feasibility on beam sweeping factor reduction (not related with FR2 multi-Rx) is not well justified. Observation 2: The coupled discussion among WI discussed in parallel shall be avoided. Observation 3: A-TRS can be used for AGC and time/frequency tracking in Rel-17 fast SCell activation. Observation 4: Fast SCell activation can be used when UE can obtain coarse timing and beam information in FR2. Observation 5: For FR2 unknown SCell activation without intra-band serving cell, UE can obtain the coarse timing and beam information via QCL relation when the QCL source is the SSB of inter-band serving cell. Observation 6: The above case may depend on NW deployments (e.g. co-located). If gNB provides such configurations (inter-band QCL relation), it means UE can utilize the information and active the SCell based on A-TRS. Proposal 1: For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms. Proposal 2: L1-RSRP measurement during SCell activation shall have higher priority over L3 measurement and it should be perform in non-DRX mode even DRX is configured. Proposal 3: For unknown case, the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. |
| R4-2216479 | ZTE Corporation | Proposal 1: Once the conditions that same SSBs used for both L1-RSRP measurement and L3 part or the CSI-RS resources used for L1-RSRP is QCLed type D with the SSBs used for L3 part can be satisfied, the L1-RSRP measurement can be ignored and directly reporting the SSB-RSRP obtained from L3 part is fine. Proposal 2: Considering the RS resource configuration for L3 part and L1 measurement is independent, so more RS resources configured for L1 measurement than L3 part is possible. In such case, UE still needs to perform L1-RSRP measurement based on the RS resources not detected during L3 part and reports the suitable RSRP to NW. Proposal 3: Reduction of Rx beam sweeping number in L1-RSRP measurement is another solution to reduce the L1-RSRP measurement latency, but the corresponding performance loss should be verified to be acceptable. Proposal 4: L1-RSRP measurement can be performed in non-DRX mode even DRX is configured. |
| R4-2216759 | MediaTek inc. | Observation 1: L1-RSRP measurements delay can be reduced through P and N parameters. Proposal 1: L1-RSRP measurements delay can be reduced by considering the non-DRX configuration. Proposal 2: L1-RSRP measurements delay can be reduced by deactivating the MGs during L1 measurements for unknown SCell activation in FR2. Proposal 3: L1-RSRP measurements delay can be reduced by introducing a UE capability for reduced Rx beam sweeping factor during L1 measurements for unknown SCell activation in FR2. Proposal 4: Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing. Proposal 5: The uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. Proposal 6: Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the SCell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources. |
| R4-2216829 | Ericsson | Proposal 1: RAN4 to agree that some UEs with higher capability shall be able to achieve RX beam sweeping factor reduction for L1-RSRP measurement. Proposal 2: RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement. Proposal 3: RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS. Proposal 4: Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping. Proposal 5: RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements. Proposal 6: RAN4 shall not prioritise L1-RSRP measurements over L3 measurements during SCell activation. In other words, RAN4 shall not introduce any new behaviour for prioritisation. Proposal 7: If RAN4 to discuss cell search measurements to be used as beam measurement report, accuracy requirements need to be defined. Proposal 8: L1-RSRP report cannot be skipped during SCell activation if the accuracy requirements are not specified for beam measurement report based on cell search. |

  - Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

    - Sub-topic 3-1 Enhancement for L1-RSRP

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 3-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- Proposals

  - Option 1(Xiaomi, Apple, Nokia, CMCC, OPPO, vivo, ZTE, MediaTek): RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP

    - Option 1a (Xiaomi):

      - For the beam sweeping factor enhancement for L1-RSRP measurement, the conclusions on beam sweeping factor enhancement for L3 measurement can be reused.

      - It is feasible to consider reporting the SSB indexes during the cell search procedure to reduce L1-RSRP measurement delay.

    - Option 1b (OPPO, vivo, MediaTek, Ericsson): Introduce UE capability to support beam sweeping factor reduction for L1 measurement

    - Option 1c (Apple): Introduce UE capability to support beam sweeping factor reduction for SSB based L1 measurement

    - Option 1d (Nokia): For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection.

    - Option 1e (CMCC):

      - if L1-RSRP measurement (TL1-RSRP, measure) need to be kept for some cases, it is proposed to consider reduction on RX beam sweeping factor.

      - for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline.

    - Option 1f (ZTE):

      - Reduction of Rx beam sweeping number in L1-RSRP measurement is another solution to reduce the L1-RSRP measurement latency, but the corresponding performance loss should be verified to be acceptable.

    - Option 1g(vivo):

      - When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8.

- Recommended WF

  - Moderator: Option 1 was proposed by majority companies, and the detailed condition/solutions can be discussed in this meeting, i.e., option 1a/1b/1c/1d/1e/1f/1g. Please note that the discrepancy between option 1b and 1c are ‘L1 measurement’ or ‘SSB based L1 measurement’.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Support option 1/1c. In our view, if CSI-RS is used for L1-RSRP, it has little room to do enhancement with beam sweeping factor reduction, as the CSI-RS based L1-RSRP measurement delay is using the beam sweeping factor of {resource number each resource set}/UE Rx number. |
| Qualcomm | It depends on outcome of Issue 3-1-3. We think UE can skip L1 RSRP measurement and report when UE report L3 RSRP measurement with SSB index. |
| LGE | To reduce Rx beam sweeping factor in L1 part, we think that it should be related L3 part measurement. So we prefer to further discuss based on option 1/1a or 1d. Further discussion on the UE capability is needed. |
| Intel | For unknown case, for cell detection and AGC steps, Rx beam sweeping will still be applied and it’s possible to derive L3 measurement result and find out the best Rx beam. We propose that L1-RSRP measurement can be skipped and L3 measurement will be reported with SSB index. |
| CMCC | We are OK with option 1/1e. We would like to provide more consideration on option 1e. As mentioned by companies in last meeting, under some conditions, it is possible that UE could get the beam information without performing L1-RSRP measurement. The beam information can be acquired in previous L3 related procedures (e.g. cell synchronization). For example, if the RS configured for L1-RSRP measurement has been measured during L3 synchronization or is QCLed type D with the RS for L3 procedures, L1-RSRP measurement could be skipped. If L1-RSRP cannot be skipped for some cases, the delay could be reduced. In existing requrements, one of the contributors of the long delay is the scaling factor of N. N is about the RX beam sweeping, and value of N is settled as 8, which will result in long measurement delay on L1 measurement. It is necessary to consider enhancement on beam sweeping factor. |
| Xiaomi | Option 1a, we think the conclusion on beam sweeping factor enhancement for L3 part can be reused, e.g. introduce UE capability and the state of not performing full Rx beam sweeping to reduce the L1-RSRP measurement delay. In addition, we also think it is feasible to consider the UE to report the SSB index during the cell search procedure to reduce or skip L1-RSRP measurement delay. |
| Huawei | Similar comments as issue 2-2-1. Before introducing the capability, the feasibility how UE can directly use less beam sweeping number needs justified. |
| Ericsson | We support option 1b. It should be applicable for both SSB and CSI-RS based finer beams. These finer beams UE RX beam sweeps within the wider beam. It is tradeoff between how narrow beams UE could use at the end of SCell activation vs how fast data transmission can start. |
| OPPO | The capability of Rx beam sweeping can be shared for L3 and L1 part. Whether it should be only for SSB or L1-RSRP measurement can be skipped, need more discussion. |
| Nokia | Support Option 1b and 1d. In our views, it is more justified to reduce the beam sweeping in L1 measurement as the UE has experienced beam sweeping during L3 part behaviour. It may reuse or at least refer to the Rx beam pattern derived during AGC and cell detection procedure to simplify the L1-RSRP measurement. For capability indication, it may be sufficient to indicate the reduced beam sweeping for L1 measurement. Is there any reason to restrict to SSB-based L1 measurement? Regarding to the performance loss, we prefer not introducing any performance loss due to reduced beam sweeping factor. Otherwise if any performance loss is foreseen, the UE should not indicate the support of the capability. |
| vivo | Do not see the difference between 1b and 1c. In legacy CSI-RS requirements, there is already capability reporting and no enhancement is needed. What we meant is only about SSB-based solution. Support option 1b, 1c, 1e, 1g. |
| ZTE | Support Option 1d, 1f. We support the beam sweeping factor reduction, but when determining the detailed reduced number, the tradeoff between performance and latency should be considered. |
| China Telecom | The reduced Rx beam sweeping factor can be considered for the delay enhancement in L1 part, but it is considerable to discuss whether the performance loss of reduced Rx beam sweeping factor can be acceptable. |
| MTK | Fine with Option 1. It is fine to consider reduced Rx beam sweeping factor for L1-RSRP than for L3 part as the UE will have some information about the beams during L3 part, thus they can be reduced to enhance L1-RSRP measurement time. |
| NTT DOCOMO | OK, We support with option 1. |

Issue 3-1-2: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

- Proposals

  - Option 1 (Apple, Nokia): if L1-RSRP measurement is needed, the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown Scell activation requirement.

- Recommended WF

  - Agree on option 1.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Option 1. |
| Qualcomm | Option 1. |
| LGE | Support recommended WF |
| CMCC | We are OK with that the sample number of PHY filtering cannot be reduced if M=1 is used for L1-RSRP measurement. But M can also be 3 for L1-RSRP measurement. According to spec (clause 9.5.4 in TS 38.133), M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. We would like to know whether reduction is considered when M=3? |
| Xiaomi | Fine with option 1. |
| Huawei | Option 1. |
| Ericsson | Agree with option 1. |
| OPPO | OK with option 1. |
| Nokia | Option 1. |
| vivo | Support option 1. |
| ZTE | Fine with Option 1. |
| China Telecom | Support option 1. |
| MTK | Fine with option1. |
| NTT DOCOMO | Support option 1. |

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation?

- Proposals

  - Option 1 (Nokia, Intel, Apple, CMCC, OPPO, ZTE, Xiaomi): RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

    - Option 1a (Intel, Apple): L1-RSRP measurement can be skipped and L3 measurement result can be used for measurement reporting with SSB index.

    - Option 1b (Apple, CMCC, OPPO, ZTE): skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs

  - Option 2 (Ericsson):

    - If RAN4 to discuss cell search measurements to be used as beam measurement report, accuracy requirements need to be defined.

    - L1-RSRP report cannot be skipped during SCell activation if the accuracy requirements are not specified for beam measurement report based on cell search.

  - Option 3 (ZTE):

    - Considering the RS resource configuration for L3 part and L1 measurement is independent, so more RS resources configured for L1 measurement than L3 part is possible. In such case, UE still needs to perform L1-RSRP measurement based on the RS resources not detected during L3 part and reports the suitable RSRP to NW.

- Recommended WF

  - TBA.

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Support option 1/1a/1b. If SSBs are configured as L1-RSRP/beam measurement RS and this SSB has been measured during L3 synchronization or measurement, UE may skip L1-RSRP/beam measurement, and directly report SSB-RSRP to network based on L3 measurement. If CSI-RS is configured as L1-RSRP/beam measurement RS and this CSI-RS is QCLed type D with the SSB measured during L3 synchronization or measurement, UE may skip L1-RSRP/beam measurement, and also directly report SSB-RSRP to network based on L3 measurement. Another alternative is that UE can also report the beam information based on cell search and skip the L1-RSRP procedure, and then network can also use the reported rough beam information to determine the TCI. For option 2, we understand the motivation, but currently we don’t have such accuracy requirement for the L1-RSRP measurement during the SCell activation. |
| Qualcomm | We support Option 1a. we think UE can further skip L1 report if we agree option 2 in issue 2-1-1. UE can report L3 RSRP |
| LGE | Support option 1, and we think that both option 1a and 1b could be considered to skip L1-RSRP measurement. |
| Intel | Support option 1a. In legacy known case, TCI state is selected based on L3 report. It seems that L1-RSRP measurement is not necessary step. For option 1b, the limitation that L3 measurement and L1 measurement are using same RS or QCLed type D RSs seems not to be necessary. |
| CMCC | Support option 1/1a/1b |
| Xiaomi | Option 1, if the UE is required to report the SSB index during the cell search procedure, the L1-RSRP measurement procedure can be skipped. |
| Huawei | Generally fine with option 1. But some details needs further discussion as mentioned by Apple. E.g. legacy L1 report or L3 report. And UE is still performing L1 RSRP measurement, it means there could be measurement results based on L3 measurement and results based on L1measurent which may have different absolute values using different beams (rough/fine), |
| Ericsson | We support option 2. By skipping the L1-RSRP reporting UE may only be measuring wider beams and may not be measuring finer beams during L3 stage even if they are based on SSB. That means at the end of SCell activation, only UE is scheduled on wider beams. One example could be, SSB0 and SSB1 can be wider beams and SSB 2 to 7 may be finer beams. SMTC may be configured only to cover SSB0 and 1. Since UE will be measuring only in SMTC, it may not have measured finer beams. SSB0; SSB1; SSB2; SSB3; SSB4; SSB5; SSB6; SSB7. |
| OPPO | Support option 1/1b. L1 measurement results could rely on L3 measurement and the condition of using same RS or QCLed type D RSs can ensure the confidence of results. |
| Nokia | Option 1 and Option 1a. This issue again depends on the scenarios of unknown SCells. - If the UE has measured the SCell and acquired a valid L3 measurement, this can be used as the beam information as in known case. Then neighbor AGC/cell search and L1-RSRP measurement is not needed. The activation can follow the procedure for activating a known SCell. - If the UE has not acquired a valid L3 measurement, it may use the L3 measurement during AGC/cell search phase and L1-RSRP measurement can be skipped. On Option 1b, we’d like to understand the difference between L3 and L1 measurement on the same RS. As beam sweeping is considered in both L3 and L1 part, what is the difference inbetween? |
| vivo | Support option 1/1a. Detail would be FFS, e.g. how to trigger the L3 reporting. We do not think QCL-D is needed. We prefer to re-use the scheme that similar to legacy which is specified in known condition. If it can be skipped, there is no need to consider which RS is used for L1 measurement. |
| ZTE | Support Option 1/1b. Our concern in Option 3 is derived from the independent RS configuration for L1 measurement and L3 measurement. L3 measurement aims to identify rough beam while L1 measurement aims to obtain fine beam, E/// provide a good example, only 2 SSB indexes with rough beam assumption configured for L3 measurement, but 5 SSB indexes with fine beam assumption configured for L1 measurement, under such situation, if skip L1 measurement, fine beam gain would accordingly be lost. |
| China Telecom | OK with option 1/1a/1b. |
| MTK | Fine with Option 1 to skip L1-RSRP measurement if the UE can perform full Rx beam sweeping at L3 part since it will be the only chance for the UE to obtain beam information for the unknown SCell in FR2. |
| NTT DOCOMO | Support Option 1/1a/1b. |

Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

- Proposals

  - Option 1 (LGE): Deprioritize prioritization enhancement for L1-RSRP that may affect other measurement behaviors

  - Option 2 (Nokia): The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance.

  - Option 3 (Ericsson): RAN4 shall not prioritise L1-RSRP measurements over L3 measurements during SCell activation. In other words, RAN4 shall not introduce any new behaviour for prioritisation.

  - Option 4 (Apple, Huawei): L1-RSRP measurement during FR2 unknown SCell activation has higher priority over L3 measurement

  - Option 5 (Apple, Huawei, ZTE, MediaTek): L1-RSRP measurement is performed in non-DRX mode even DRX is configured.

  - Option 6 (MediaTek): L1-RSRP measurements delay can be reduced by deactivating the MGs during L1 measurements for unknown SCell activation in FR2.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 4 and 5, but can compromise to option 1/3 because of the mentioned impact the L3 mobility performance. Option 6 will also seriously impact the existing L3 mobility based on MG. |
| Qualcomm | We support Option3. We don’t support different rule for sharing factor P only for FR2 Scell activation. About option5, does legacy UE already perform non-DRX even DRX is configured as UE keeps monitoring PDCCH and UE does not go DRX state. |
| LGE | We prefer not to introduce prioritization enhancement for L1-RSRP. It could be affect other measurement behavior. So, prefer deprioritize the issue as option 1 and option 3. |
| Xiaomi | Fine with option 5, and regarding the prioritization of L1-RSRP, we have concern on it, since it will have impact on UE measurement behavior for other RRM measurements, which is not desired for us. |
| Huawei | Support option 4 and 5. Indeed there will be impact on mobility measurement. But SCell activation is not very frequency or periodic procedure, and the L1-RSRP for beam will only occupy 8 samples, which will not have significant impact on mobility measurement. Besides, in legacy requirements, we already have such priority rules where the cell detection for SCell activation is prioritized over other procedure as follows: “ Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.” Thus, it is not a problem to also prioritize L1 during SCell activation. To QC about DRX: Yes, at least in the test cases, DRX is assumed in the test requirements when DRX is configured. |
| Ericsson | We do not agree prioritizing L1 measurement over L3 measurement. In our view, if HO performance is affected, faster SCell activation is of no use. |
| OPPO | Support Option 3 with no prioritization. Option 5 is generally ok as well. |
| Nokia | Option 2 and 3. We share same view with Ericsson. |
| vivo | We are OK to option 4, provided the clarification from Huawei. For option 5, we think non-DRX can be considered for enhanced UE. |
| ZTE | Fine with Option 5. We believe the occurence of FR2 unknown SCell activation would not very frequent, so only prioritizing L1 measurement than L3 measurement during SCell activation happening, which would not lead to obvious impact to RRM measurement. |
| China Telecom | The impact of prioritization enhancement on other measurement performance need to be considered. We are open to further discuss. |
| MTK | Support Option 5. Given the concern on Option 6 on the mobility, we are fine not to consider it. This issue also depends on the conclusion of the previous issue for skipping L1-RSRP. |

    - Sub-topic 3-2 TCI related enhancement for L1 part

Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation

- Proposals

  - Option 1 (Intel): Fine timing tracking is still needed no matter whether TCI activation is skipped or not.

  - Option 2 (MediaTek): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing.

  - Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements.

  - Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 4 but can compromise to not remove fine timing tracking part (option 1/2) if majority companies have concern on it. |
| Qualcomm | We support option1 and option 2. If UE can report fast L3 RSRP report, then it is beneficial UE to perform fine time tracking to compensate accuracy error. Furthermore, skipping fine time tracking does not reduce latency significantly. |
| Intel | We support option 1/2. Since we recommend that L1 measurement can be skipped, L3 part only provide coarse timing. Therefore, Fine time tracking is needed for following CSI measurement and data reception. |
| Xiaomi | Option 2, prefer not to skip fine timing tracking, instead, the A-TRS can be configured for fine timing tracking which can reduce the delay. |
| Huawei | We support option 1and 2. Fine timing tracking is still needed after UE obtain coarse timing and it only contributes little part of overall delay. |
| Ericsson | Option 3. As mentioned in our paper, we are not sure about the timing requirement of UE when taking measurement. Can UE vendors confirm this? |
| OPPO | Fine with option 2(not to skip). But open to discuss how to further reduce fine time tracking delay. |
| Nokia | We understood the fine time tracking is performed after receiving the TCI activation command. Since the UE has acquired the SSB timing during L3 and L1 part behaviour, and TCI activation is also based on L3 or L1 measurement, what additional does the UE need via fine time tracking? Is it needed only if TCI activation command indicates a different beam? |
| vivo | We support option 2. Not sure what is ‘skip TCI activation’ in option 1. |
| ZTE | Prefer Option 1/2. To our understanding, the fine time tracking is accompanied with TCI state switching, when target TCI state is not maintained in the active TCI state list, UE needs 1*Trs to perform fine time tracking. For the case of unknown SCell activation, since UE has no prior info about the TCI state, so UE needs to perform such fine time tracking for the TCI state. |
| MTK | Support Option 1 and Option 2. Same view as QC, Intel, Xiaomi and HW. |

Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation

- Proposals

  - Option 1 (Intel, CMCC, OPPO, Huawei, MediaTek): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

    - Option 1a(Apple): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE.

  - Option 2 (vivo): During the SCell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can be reduced accordingly.

  - Option 3 (Ericsson): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1 and 1a. The reason for 1a is we shall still keep the case that: if network wants to explicitly indicate the TCI to UE, UE shall follow network command; otherwise, UE will use by-default assumption to determine this active TCI. |
| Qualcomm | We support otpion3. |
| Intel | Support option 1 and 1a. we think option 1 didn’t preclude the option that TCI activation can be configured from NW. |
| CMCC | Support option 1. |
| Xiaomi | Option 1. |
| Huawei | Support option 1/1a. |
| Ericsson | Option 3. We think gain in delay reduction is quite small compared to the change it brings to both UE and NW and hence we do not support. |
| OPPO | Option 1. |
| Nokia | Option 3. Although the TCI indication is based on the received L3/L1 measurement reporting, it is up to network which TCI to indicate. TCI activation command gives a clear indication on which beam the UE is monitoring. This can avoid misunderstanding of the selected beam and ensure the following channel measurement and data Tx. |
| vivo | Support option 2 but also ok with option 3. For option 1, we have the same understanding as option 3. |
| ZTE | Prefer Option 3. Which TCI state to indicate to UE, it is up to NW, perhaps not the best TCI state UE supposed since NW would consider the scheduling comprehensively. To avoid the misunderstanding between NW and UE, TCI state indication should be kept. |
| China Telecom | Fine with option1/1a. |
| MTK | Option 1. |

    - Sub-topic 3-3 Aperiodic RS related enhancement for L1 part

Issue 3-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation

- Proposals:

  - Option 1 (Xiaomi, Apple, OPPO): A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index.

  - Option 2 (Qualcomm): Optional UE capability to specify number of A-TRS for fine time tracking after L3 RSRP reporting w/ SSB index.

  - Option 3 (MediaTek): Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the Scell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1. The fine timing tracking is after L1-RSRP reporting and TCI activation, and therefore the timeline to schedule A-TRS is clear to network; i.e., after UE reports the L1-RSRP to network for TCI determination, network can choose to schedule A-TRS for UE to perform fine timing tracking for TCI after the TCI activation command. |
| Qualcomm | We support option1 and option2. |
| LGE | We support option1 and option 2 can be considered |
| Xiaomi | Option 1, once NW determine the best Tx beam according the reported SSB indexes, A-TRS can be used for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index. |
| Huawei | We agree that A-TRS can be used for fine timing for unknown case. But currently we are not supporting replace SSB by A-TRS for the fine timing. The problem is that gNB is not aware of when UE is ready for A-TRS if UE still needs to perforce other L3/L1 procedure. Companies proposed to introduce an indication of completion to NW. considering the delay reduction on comes from the time uncertainty for the next SSB. But with the indication, it means additional UL/DL transmission is needed before schedule the A-TRS. Considering the processing time from both UE and gNB side and the communication delay, we wondering whether it can indeed bring much delay reduction? |
| Ericsson | We support option 1. A-TRS can be scheduled based on beam measurement report at the end of L3 part. |
| OPPO | Support option 1. A-TRS is beneficial to reduce the waiting time for next periodic RS. |
| Nokia | We are open to discuss the A-TRS based solution, but would like to first discuss the feasibility of using A-TRS for fine timing tracking. A-TRS has been defined in R17 for AGC and T/F synchronization. The feasibility of using this for other purposes e.g. fine time tracking and/or L1-RSRP needs to be studied/confirmed by RAN1. We need send LS to RAN1 asking for the feasibility. |
| vivo | Generally OK to option 1 and 2. But we think this should be discussed after 3-1-3 is concluded. Without details of L3 reporting, it is difficult to discuss timeline. |
| ZTE | Support Option 1. |
| China Telecom | In our views, A-TRS is able to be used for both L3 and L1 part to reduce the whole procedure delay in unknown case. NW can configure information of A-TRS to make UE complete activation process. |
| MTK | We have similar view as HW. Also, since fine timing tracking is based on 1*Trs, which is already small, the benefit of having A-TRS to reduce (1*Trs) will not be much. Therefore, our preference is Option 3. |

Issue 3-3-2: Aperiodic RS for L1-RSRP measurement during FR2 unknown SCell activation

- Proposals:

  - Option 1 (Apple): Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement.

  - Option 2 (Ericsson):

    - RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement.

    - RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support option 1. But we think option 2 raised a good point, our view is we can use AP CSI-RS for L1-RSRP after UE indicates the L3 stage completion, but we are also open to check if A-TRS can be used for L1-RSRP. |
| Qualcomm | Currently we do not support all options. It depends on the outcome of L1 RSRP measurement skip. |
| LGE | We support option 2. Depending on the conclusion of Issue 3-1-3, this issue could be skipped. |
| Huawei | Same comments as 3-3-1. |
| Ericsson | We support option 2. |
| Nokia | Option 2 could be the starting point. A-TRS has been defined in R17 for AGC and T/F synchronization. The feasibility of using this for other purposes e.g. fine time tracking and/or L1-RSRP needs to be confirmed by RAN1. We also agree to send LS to RAN1 asking for the feasibility. |
| vivo | FFS. It depends on conclusions of other issues. |
| ZTE | When to trigger A-TRS by NW, this should be firstly identified. If additional interaction between NW and UE is needed accordingly, whether latency reduction would be realized? |
| China Telecom | Same comment as issue 3-3-1. |
| MTK | We don’t support the above options. The issue also depends on the conclusion of Issue 3-1-3. In addition, if the UE has to indicate to the NW the completion of L3 stage (to trigger AP CSI-RS for L1-RSRP), it means another delay will be introduced for this communication between the UE and the NW, which will restrict the benefit of using AP CSI-RS for L1-RSRP. |

    - Sub-topic 3-4 SP CSI-RS and periodic CSI-RS related enhancement for L1 part

Issue 3-4-1: SP CSI-RS and periodic CSI-RS related enhancement during FR2 unknown SCell activation

- Proposals:

  - Proposal 1 (Intel): Semi-persistent CSI-RS activation or RRC based CSI configuration command can be sent with SCell activation command together.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We are not very much sure about the feasibility of proposal 1, since when UE receive SCell activation command, both UE and network has no idea which TCI will be used/chosen, then how can network decides to choose specific CSI-RS for CQI? It needs more discussion. |
| Intel | The comment of Apple make sense. It’s hard to choose specific CSI-RS before L1/L3 report. We will further think about it. |
| Nokia | We understood SP/P-CSI-RS for channel measurement is triggered only if TCI state is known. It may be sent with TCI state activation to save MAC uncertainty time. But at the time of SCell activation, network has no beam information of the unknown SCell, on which beam would SP/P-CSI-RS be sent? |
| ZTE | We understand the motivation of Proposal 1, but we concern NW maybe not able to identify the suitable beam for CSI measurement when triggering the SCell activation, so NW can not decide which SP CSI-RS resouce should be activated. |

    - Sub-topic 3-5 Other enhancement for L1 part

Issue 3-5-1: Other enhancement for L1 part

- Proposals:

  - Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell.

  - Proposal 2 (Qualcomm): When Scell is semi-unknown to UE, NW can trigger UE to report RSRP before L1 RSRP measurement and report. The framework can be illustrated as following steps while UE perform the legacy FR2 Scell activation procedure.

    - Step 1. NW configure either periodic or aperiodic (event-based) report config before SCell activation command.

    - Step 2. NW trigger preconfigured report config. (e.g MAC-CE can trigger the report config)

    - Step 3: UE report latest RSRP measurement for target SCell

    - Step 4: UE can skip L1-RSRP measurement and report when UE reports RSRP measurement with associated SSB index during step3.

    - Step 5: Either A-TRS based or SSB-based for fine time tracking. TCI activation command and CSI-RS recourse activation for CQI reporting.

  - Proposal 3 (Huawei): For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | For proposal 1, in our view, it’s related with non-serving cell L1-RSRP in R17 FeMIMO. We think R17 FeMIMO feature for FR2 SCell activation enhancement shall be discussed later after we conclude on baseline enhancement. Technically, we don’t understand very well on how non serving cell L1 measurement can help in this case, because in FeMIMO, our understanding is UE is performing non-serving cell L1 after UE has already performed L3 measurement of neighbor cell; but in this SCell activation, unknown SCell means UE has not perform L3 measurement before the activation command. When we discuss this point in 2nd phase, we need more elaboration from proponents. For proposal 2, we can wait until issue 2-1-1 concluded. For proposal 3, same comments as to issue 2-4-1. |
| Qualcomm | We support Proposal2. As we discuss in the current meeting, we have very good idea in each category to reduce the delay requirement. However, those are disconnected from each other issues. For example, we are discussing L1 measurement enhancement by reducing beam sweeping. On the other hand, we are discussing L1 measurement skipping. L1 report skipped from L3 measurement report. All good ideas from companies cannot be combined as those are disconnected. Thus, we propose a framework for fast FR2 unknown Scell activation when UE report RSRP during L3 part. Details are FFS. |
| Xiaomi | Fine with proposal 3. |
| Huawei | We share similar understanding as QC about the whole discussion. We have various solution targeting certain component of the SCell activation procedure, at the same time some of them can impact each other’s. We think the joint/unified framework for delay reduction is even more important. For the above proposals, we support proposal 3 which is an overall framework involving both L3 and L1 delay reduction. Besides, the spec impact is minor as such inter-band QCL indication is already supported. |
| Ericsson | We do not understand the proposal 1 clearly. How it impacts the SCell activation. For proposal 2: Even today, NW can trigger aperiodic measurement reports before SCell activation. Mandating NW scheduling behaviour for SCell activation cannot be standardized in our view. Considering that, we think this WI shall focus on methods for SCell activation enhancement after SCell activation is triggered. For proposal 3: this solution may be applicable for only co-location scenarios. If the serving cell and to be activated SCell are non-co-located we do not understand how this works. Even for colocation, sometimes, NW may not be able to provide QCL info w.r.t active serving cell. |
| OPPO | OK to further discuss what is the unified framework for delay reduction. |
| Nokia | We are fine with the approach in Proposal 2. The details can be further discussed. This is also relevant to the scenario discussion in Issue 2-1-1. We can further discuss Proposal 3. We would need check if it is possible to configure QCL for inter-band cells from network point of view. For proposal 1, does “different PCI” mean the SCell is operating on the same carrier but with different PCI? We could discuss the scenario but should not limit the discussion to this specific scenario. |
| vivo | Support proposal 2. Open to FFS proposal 3. The questions are provided in issue 2-4-1. For proposal 1, R17 CDP L1-RSRP measurements can only be performed for the intra-frequency case. Proposal 1, in our understanding, is focusing on the inter-frequency case and should be discussed in R18 feMob WI. |
| ZTE | Support Proposal 3. For co-located deployment, with the help of an active inter-band serving cell, UE can acquire gain factor, timing and beam info so that both L3 and L1 component can be ignored. We are open to discuss the detailed conditions. |
| China Telecom | Support proposal 3. We are open to discuss the unified framework for delay reduction and consider delay reduction solutions of the whole procedure including both L3 and L1 part. |
| MTK | Proposal 2 may need work in RAN2 and we are not sure if this is within the scope. |
| NTT DOCOMO | We support option 1/2/3. Our proposal, UE having capability for L1-RSRP measurement from a cell with different PCI use measurement result from L3 stage for L1-RSRP reporting or be able to skip L1-RSRP measurement for a serving cell. IF UE having capability for L1-RSRP measurement from a cell with different PCI have been finished L3-stage to L1-RSRP measurement report, UE could being activate the SCell belongs to FR2 and there is at least one active serving cell on that FR2 band. By expanding the condition of “there is at least one active serving cell on that FR2 band”, cells that were unknown until now can be regarded as known. sorry if the discussion is in the wrong place. |

  - Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 3-1 Enhancement for L1-RSRP

|  | Status summary |
| --- | --- |
| Issue 3-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception) | Discussion status: Views are quite diverse among companies on all options. Candidate options: - Option 1(Apple, CMCC, OPPO, MTK): RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP - Option 1a (Xiaomi): - For the beam sweeping factor enhancement for L1-RSRP measurement, the conclusions on beam sweeping factor enhancement for L3 measurement can be reused. - It is feasible to consider reporting the SSB indexes during the cell search procedure to reduce L1-RSRP measurement delay. - Option 1b (Ericsson, Nokia, vivo): Introduce UE capability to support beam sweeping factor reduction for L1 measurement - Option 1c (Apple, vivo): Introduce UE capability to support beam sweeping factor reduction for SSB based L1 measurement - Option 1d (Nokia, ZTE): For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection. - Option 1e (CMCC, vivo): - if L1-RSRP measurement (TL1-RSRP, measure) need to be kept for some cases, it is proposed to consider reduction on RX beam sweeping factor. - for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline. - Option 1f (ZTE): - Reduction of Rx beam sweeping number in L1-RSRP measurement is another solution to reduce the L1-RSRP measurement latency, but the corresponding performance loss should be verified to be acceptable. - Option 1g(vivo): - When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8. Recommendations for 2nd round: Continue discussion in 2nd round. Agreement will be captured in the WF. Encourage companies to check if option 1 can be used as a general agreement and detailed solutions can be FFS. |
| Issue 3-1-2: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation | Agreements: - If L1-RSRP measurement is needed, the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown Scell activation requirement. Candidate options: Recommendations for 2nd round: This issue is closed and no discussion in 2nd round. |
| Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation? | Discussion status: Majority companies supported option 1, and some companies mentioned that detailed solutions can be FFS. Tentative agreements: RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell. Candidate options: - Option 1 (Apple, LGE, CMCC, Xiaomi, HW, OPPO, Nokia, vivo, ZTE, CTC, MTK): RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell. - Option 1a (Apple, QC, LGE, Intel, CMCC, Nokia, CTC): L1-RSRP measurement can be skipped and L3 measurement result can be used for measurement reporting with SSB index. - Option 1b (Apple, LGE, CMCC, OPPO, vivo, ZTE, CTC): skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs - Option 2 (Ericsson): - If RAN4 to discuss cell search measurements to be used as beam measurement report, accuracy requirements need to be defined. - L1-RSRP report cannot be skipped during SCell activation if the accuracy requirements are not specified for beam measurement report based on cell search. Recommendations for 2nd round: Option 2 may also be considered as one of the conditions to apply or not apply such enhancement of skipping L1-RSRP. Thus, put option 1 as tentative agreement and companies can double check if this option 1 is agreeable in 2nd round. Please Ericsson check if tentative agreement is acceptable or not. Continue discussion for the detailed solutions in 2nd round. Agreement will be captured in the WF. |
| Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation | Discussion status: There were different views between option 3 and option 4, we can FFS on whether prioritization is allowed between L1/L3. There was no explicit objection to option 5. QC had question on option 5 and HW clarified for that question. Tentative agreements: - L1-RSRP measurement is performed in non-DRX mode even DRX is configured. - FFS on whether prioritization is allowed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation. Candidate options: - Option 1 (Apple, LGE): Deprioritize prioritization enhancement for L1-RSRP that may affect other measurement behaviors - Option 2 (Nokia): The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance. - Option 3 (Apple, QC, LGE, Ericsson, OPPO, Nokia): RAN4 shall not prioritise L1-RSRP measurements over L3 measurements during SCell activation. In other words, RAN4 shall not introduce any new behaviour for prioritization. - Option 4 (Apple, HW, vivo): L1-RSRP measurement during FR2 unknown SCell activation has higher priority over L3 measurement - Option 5 (Apple, Xiaomi, HW, OPPO, vivo, ZTE, MTK): L1-RSRP measurement is performed in non-DRX mode even DRX is configured. Recommendations for 2nd round: Please companies check if tentative agreement is acceptable or not. Continue discussion for the FFS part in 2nd round. Agreement will be captured in the WF. |

Sub-topic 3-2 TCI related enhancement for L1 part

|  | Status summary |
| --- | --- |
| Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation | Discussion status: Majority companies support option 2 (9 companies). Candidate options: - Option 1 (Apple, QC, Intel, HW, ZTE, MTK): Fine timing tracking is still needed no matter whether TCI activation is skipped or not. - Option 2 (Apple, QC, Intel, Xiaomi, HW, OPPO, vivo, ZTE, MTK): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing. - Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements. - Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused. Recommendations for 2nd round: Continue discussion in 2nd round. Please companies check if option 2 is acceptable or not. Agreement will be captured in the WF. |
| Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation | Discussion status: Diverse views on this issues, especially for option 1/1a/3. Candidate options: - Option 1 (Apple, Intel, CMCC, Xiaomi, HW, OPPO, CTC, MTK): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. - Option 1a(Apple, Intel, HW, CTC): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE. - Option 2 (vivo): During the SCell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the SCell activation delay can be reduced accordingly. - Option 3 (QC, Ericsson, Nokia, vivo, ZTE): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping. Recommendations for 2nd round: Continue discussion in 2nd round. Agreement will be captured in the WF. |

Sub-topic 3-3 Aperiodic RS related enhancement for L1 part

|  | Status summary |
| --- | --- |
| Issue 3-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation | Discussion status: Majority companies supported option 1 (8 companies). Candidate options: - Option 1 (Apple, QC, LGE, Xiaomi, Ericsson, OPPO, vivo, ZTE): A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index. - Option 2 (QC, vivo): Optional UE capability to specify number of A-TRS for fine time tracking after L3 RSRP reporting w/ SSB index. - Option 3 (MTK): Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the Scell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources. Recommendations for 2nd round: Continue discussion in 2nd round. Agreement will be captured in the WF. |
| Issue 3-3-2: Aperiodic RS for L1-RSRP measurement during FR2 unknown SCell activation | Discussion status: This issue also depends on other issues 3-1-3 and 3-3-1. Candidate options: - Option 1 (Apple): Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement. - Option 2 (Ericsson, LGE, Nokia): - RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement. - RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS. Recommendations for 2nd round: Continue discussion for this issue in 2nd round and check if LS is needed. |

Sub-topic 3-4 SP CSI-RS and periodic CSI-RS related enhancement for L1 part

|  | Status summary |
| --- | --- |
| Issue 3-4-1: SP CSI-RS and periodic CSI-RS related enhancement during FR2 unknown SCell activation | Discussion status: The proposal may need more justification based on the discussion, and so far it’s not agreeable. Candidate options: - Proposal 1 (Intel): Semi-persistent CSI-RS activation or RRC based CSI configuration command can be sent with SCell activation command together. Recommendations for 2nd round: No discussion for this issue in 2nd round. This proposal will not be captured in the WF for FFS unless Intel still wants to keep it for next meeting. |

Sub-topic 3-5 Other enhancement for L1 part

|  | Status summary |
| --- | --- |
| Issue 3-5-1: Other enhancement for L1 part | Discussion status: The views are quite diverse. And some companies mentioned to discuss the enhancement as joint/unified framework, like in proposal 2. Candidate options: - Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell. - Proposal 2 (QC, Nokia, vivo): When Scell is semi-unknown to UE, NW can trigger UE to report RSRP before L1 RSRP measurement and report. The framework can be illustrated as following steps while UE perform the legacy FR2 Scell activation procedure. - Step 1. NW configure either periodic or aperiodic (event-based) report config before SCell activation command. - Step 2. NW trigger preconfigured report config. (e.g MAC-CE can trigger the report config) - Step 3: UE report latest RSRP measurement for target SCell - Step 4: UE can skip L1-RSRP measurement and report when UE reports RSRP measurement with associated SSB index during step3. - Step 5: Either A-TRS based or SSB-based for fine time tracking. TCI activation command and CSI-RS recourse activation for CQI reporting. - Proposal 3 (Xiaomi, HW, ZTE, CTC): For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms. Recommendations for 2nd round: Continue discussion in 2nd round. Agreement will be captured in the WF. Please companies share your view on how to discuss each enhancement part in a joint/unified framework, and I really appreciate if we can find a better way to organize the discussion. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

### Sub-topic 3-1 Enhancement for L1-RSRP

Issue 3-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

- Option 1(Apple, CMCC, OPPO, MTK): RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP

  - Option 1a (Xiaomi):

    - For the beam sweeping factor enhancement for L1-RSRP measurement, the conclusions on beam sweeping factor enhancement for L3 measurement can be reused.

    - It is feasible to consider reporting the SSB indexes during the cell search procedure to reduce L1-RSRP measurement delay.

  - Option 1b (Ericsson, Nokia, vivo): Introduce UE capability to support beam sweeping factor reduction for L1 measurement

  - Option 1c (Apple, vivo): Introduce UE capability to support beam sweeping factor reduction for SSB based L1 measurement

  - Option 1d (Nokia, ZTE): For FR2 unknown SCell activation, a smaller Rx beam sweeping factor is assumed for L1-RSRP measurement based on the measurement during cell detection.

  - Option 1e (CMCC, vivo):

    - if L1-RSRP measurement (TL1-RSRP, measure) need to be kept for some cases, it is proposed to consider reduction on RX beam sweeping factor.

    - for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline.

  - Option 1f (ZTE):

    - Reduction of Rx beam sweeping number in L1-RSRP measurement is another solution to reduce the L1-RSRP measurement latency, but the corresponding performance loss should be verified to be acceptable.

  - Option 1g(vivo):

    - When the L3 part including cell search and AGC settling has been completed, if the SSB is used to execute L1-RSRP measurement, the Rx beam sweeping factor of L1-RSRP measurement can be less than 8.

Recommendations for 2nd round:

Continue discussion in 2nd round. Agreement will be captured in the WF.

Encourage companies to check if option 1 can be used as a general agreement and detailed solutions can be FFS.

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with option1 as a general agreement and details can be FFS. |
| Xiaomi | Support option 1/1a, and we think one UE capability on beam sweeping factor reduction should be applied to both L3 measurement and L1 measurement. |
| Apple | Support option 1 and details can be FFS. |
| Ericsson | Support option 1. |
| LGE | We are fine with the option 1. |
| Intel | Depends on Issue 3-1-3. |
| CMCC | OK with moderator’s suggestion, agree option 1 as a general agreement and detailed solutions can be FFS. |
| OPPO | Support option 1. |
| Nokia | Support Option 1 in general and 1b/1d. |
| China Telecom | Fine with option 1. It is considerable to discuss whether the performance loss of reduced Rx beam sweeping factor can be acceptable. |
| Huawei | Regarding the capability, same comments as 2-2-1. We are fine to further discuss the details about beam Rx beam sweeping factor reduction for L1-RSRP. |
| ZTE | Support Option 1. |
| vivo | Support option 1. |
| MTK | This depends on Issue 3-1-3 conclusion. |
| Moderator | Since Intel and MTK mentioned the issue 3-1-3, we can check if L1-RSRP measurement of FR2 unknown SCell activation is not skipped, then option 1 in this issue can be agreed. Please companies check the following tentative agreement. Agreement for final check: - If L1-RSRP measurement of FR2 unknown SCell activation is not skipped, RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP - Details are FFS |

Issue 3-1-2: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

Agreements:

- If L1-RSRP measurement is needed, the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown Scell activation requirement.

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation?

Tentative agreements:

RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Candidate options:

- Option 1 (Apple, LGE, CMCC, Xiaomi, HW, OPPO, Nokia, vivo, ZTE, CTC, MTK): RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

  - Option 1a (Apple, QC, LGE, Intel, CMCC, Nokia, CTC): L1-RSRP measurement can be skipped and L3 measurement result can be used for measurement reporting with SSB index.

  - Option 1b (Apple, LGE, CMCC, OPPO, vivo, ZTE, CTC): skip L1-RSRP measurement and use measurement result from L3 stage for L1-RSRP reporting, if L3 measurement and L1 measurement are using same RS or QCLed type D RSs

- Option 2 (Ericsson):

  - If RAN4 to discuss cell search measurements to be used as beam measurement report, accuracy requirements need to be defined.

  - L1-RSRP report cannot be skipped during SCell activation if the accuracy requirements are not specified for beam measurement report based on cell search.

Recommendations for 2nd round:

Option 2 may also be considered as one of the conditions to apply or not apply such enhancement of skipping L1-RSRP. Thus, put option 1 as tentative agreement and companies can double check if this option 1 is agreeable in 2nd round.

Please Ericsson check if tentative agreement is acceptable or not. Continue discussion for the detailed solutions in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine with Tentative agreement. |
| Xiaomi | Fine with the tentative agreement. |
| Apple | Fine with tentative agreement. |
| Ericsson | We are fine with tentative agreement. |
| LGE | Support the tentative agreement |
| Intel | Fine with the tentative agreement. |
| CMCC | OK with the tentative agreement. |
| OPPO | Support the tentative agreement |
| Nokia | Fine with the tentative agreement. |
| China Telecom | Fine with the tentative agreements. |
| Huawei | Fine with the tentative agreements. |
| ZTE | Fine with the tentative agreement. |
| MTK | Fine with the tentative agreements. |
| Moderator | Agreement for final check: - RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell. |

Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

Tentative agreements:

- L1-RSRP measurement is performed in non-DRX mode even DRX is configured.

- FFS on whether prioritization is allowed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation.

Candidate options:

- Option 1 (Apple, LGE): Deprioritize prioritization enhancement for L1-RSRP that may affect other measurement behaviors

- Option 2 (Nokia): The prioritization of L1-RSRP measurement needs to be well justified taking into account the negative impact on mobility performance.

- Option 3 (Apple, QC, LGE, Ericsson, OPPO, Nokia): RAN4 shall not prioritise L1-RSRP measurements over L3 measurements during SCell activation. In other words, RAN4 shall not introduce any new behaviour for prioritization.

- Option 4 (Apple, HW, vivo): L1-RSRP measurement during FR2 unknown SCell activation has higher priority over L3 measurement

- Option 5 (Apple, Xiaomi, HW, OPPO, vivo, ZTE, MTK): L1-RSRP measurement is performed in non-DRX mode even DRX is configured.

Recommendations for 2nd round:

Please companies check if tentative agreement is acceptable or not. Continue discussion for the FFS part in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We are okay the Tentative agreements. |
| Xiaomi | Fine with the tentative agreement |
| Apple | Fine with tentative agreement. |
| Ericsson | First part of the tentative agreement looks fine to us. We do not agree with second part. L3 measurements are more important than SCell activation. |
| LGE | Support the tentative agreement. |
| Intel | OK with the tentative agreement. |
| CMCC | OK with the tentative agreement. |
| OPPO | OK with the tentative agreement. |
| Nokia | Agree with Ericsson’s comment. We already have prioritization rule between L1 and L3 measurement. Here the discussion would be more “if prioritization needs to be changed” instead of “prioritization is allowed”? FFS on whether prioritization needs to be changed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation. |
| China Telecom | Fine with the tentative agreements. |
| Huawei | OK with the tentative agreement. |
| ZTE | OK with the tentative agreement. |
| vivo | OK with the tentative agreement. |
| MTK | Fine with the tentative agreements. |
| Moderator | Revised 2nd bullet based on Nokia and Ericsson’s comments. Agreement for final check: - L1-RSRP measurement is performed in non-DRX mode even DRX is configured. - FFS on whether prioritization needs to be changed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation |

### Sub-topic 3-2 TCI related enhancement for L1 part

Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation

- Option 1 (Apple, QC, Intel, HW, ZTE, MTK): Fine timing tracking is still needed no matter whether TCI activation is skipped or not.

- Option 2 (Apple, QC, Intel, Xiaomi, HW, OPPO, vivo, ZTE, MTK): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing.

- Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements.

- Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused.

Recommendations for 2nd round:

- Continue discussion in 2nd round. Please companies check if option 2 is acceptable or not. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We support option 2. |
| Xiaomi | Option 2 |
| Apple | We can compromise to option 2. |
| Ericsson | Though we do not disagree with option 2, we need to confirm what is the timing accuracy assumption during L3 phase. |
| Intel | Support Option 2. |
| OPPO | Support Option 2. |
| Nokia | We can support Option 4. If there is no difference on the timing, probably the fine timing tracking can be skipped. We can further discuss if this is a reasonable condition e.g. there is any gap between the timing acquired from L1 and timing after fine time tracking. |
| Huawei | Could we know what is the essential difference between option 1 and option 2. To us, option 2 is more like the reason of option 1. Is it more reasonable to agree on option 1? |
| ZTE | Support Option 2. |
| vivo | Option 2 |
| MTK | Support Option 2. |
| Moderator | FFS on all proposals in next meeting. |

Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation

- Option 1 (Apple, Intel, CMCC, Xiaomi, HW, OPPO, CTC, MTK): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

  - Option 1a(Apple, Intel, HW, CTC): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE.

- Option 2 (vivo): During the Scell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the Scell activation delay can be reduced accordingly.

- Option 3 (QC, Ericsson, Nokia, vivo, ZTE): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping.

Recommendations for 2nd round:

- Continue discussion in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We support option3 and we can compromise to option1a. Option1a does not mean TCI state is skipped as it is up to NW. |
| Xiaomi | Option 1 |
| Apple | We support option 1a, to including two possible case: network gives TCI or network doesn’t give TCI. |
| Ericsson | We support option 3. We do not think option 1a will work. When to provide TCI state info is up to NW scheduling. That is why there is an uncertainty component is there in delay equation. If option 1a has to work, we have to introduce some timing restriction before which NW has to provide TCI. We do not think introducing such ehavior is needed. |
| Intel | Support option 1 and 1a. it’s left to NW whether to send TCI activation command. |
| OPPO | Support Option 1. |
| Nokia | We support Option 3. It is not only about gain but more the negative impact due to uncertainty of the TCI state determination. If the L1-RSRP report does not reach the network, there would be mis-aligned understanding on the following activation steps, which is not desired. |
| Huawei | Support option 1a. |
| ZTE | Support Option 3. |
| vivo | Option 2 is preferred. Also OK to option 3. |
| MTK | Support Option 1. |
| Moderator | FFS on all proposals in next meeting. |

### Sub-topic 3-3 Aperiodic RS related enhancement for L1 part

Issue 3-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation

- Option 1 (Apple, QC, LGE, Xiaomi, Ericsson, OPPO, vivo, ZTE): A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index.

- Option 2 (QC, vivo): Optional UE capability to specify number of A-TRS for fine time tracking after L3 RSRP reporting w/ SSB index.

- Option 3 (MTK): Since AP RS is only one-shot resource and network has no idea when to trigger it, because the exact UE processing during the Scell activation is unknown to the NW, UE may still perform faster with the pre-configured (periodic) resources.

Recommendations for 2nd round:

- Continue discussion in 2nd round. Agreement will be captured in the WF.

| Company | Comments |
| --- | --- |
| Qualcomm | Support option 1. Option2 is technical part of option1 how number of A-TRS can be requested to NW as there is no A-TRS configuration in Scell activation command. |
| Xiaomi | Option 1, once NW determine the best Tx beam according the reported SSB indexes, A-TRS can be scheduled for fine timing tracking |
| Apple | Support option 1. |
| Ericsson | Support option 1 |
| LGE | We support option 1 |
| Intel | Fine with option 1. |
| OPPO | Support Option 1. |
| Nokia | We think the usage of A-TRS also needs to be checked by RAN1. We can ask for the usage for both fine time tracking and L1-RSRP measurement in one LS. |
| Huawei | Fine with option 1 |
| ZTE | Support Option 1. |
| vivo | Support option 1. |
| MTK | We are fine to compromise to Option 1, however, we think the gain of this reduction using A-TRS is small given the 1*Trs is used for fine timing tracking |
| Moderator | Since majority companies think A-TRS can be used for fine timing checking, please Nokia check if can compromise to option 1. Agreement for final check: - A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index |

Issue 3-3-2: Aperiodic RS for L1-RSRP measurement during FR2 unknown SCell activation

- Option 1 (Apple): Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement.

- Option 2 (Ericsson, LGE, Nokia):

  - RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement.

  - RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS.

Recommendations for 2nd round:

Continue discussion for this issue in 2nd round and check if LS is needed.

| Company | Comments |
| --- | --- |
| Qualcomm | We are fine to send LS to make clarification. One question is are we agree that UE should report L3 measurement report to get QCLed RS before L1 RSRP measurement with A-TRS or CSI-RS. |
| Apple | We are not sure if only AP-TRS shall be considered here, why not use AP CSI-RS to do such L1-RSRP? If AP-CSI-RS is used here, we may not need to check with RAN1. |
| Ericsson | We think both AP CSI-RS and A-TRS are under study. Since it is starting phase of the study, it is better to check with RAN1 so that we can be clear on which RS can be used and therefore can make informed decision during next meetings. To QC: We think beam information report is needed during the or at the end of L3 part. This can be L3 measurement report too. We can further study this. To Apple: If A-TRS can be used for L1-RSRP, then NW can configure single configuration of A-TRS for L1-RSRP and fine timing. Otherwise we need to configure AP-RS for L1-RSRP and A-TRS for fine timing. Before making such agreements, we think it is better to check with RAN1. |
| LGE | We support option 2 and fine to send LS |
| OPPO | We support RAN4 to further study AP-RS and A-TRS for L1-RSRP measurement. Also fine with LS for clarification. |
| Nokia | Option 2. In addition, we can ask the usage for fine time tracking and L1-RSRP measurement separately and in combination. |
| Huawei | We are open to future discuss the feasibility and the impact to RAN1. |
| ZTE | Fine with further study the usage of AP-RS/A-TRS for L1-RSRP measurement. |
| MTK | Depends on Issue 3-1-3 conclusion. |
| Moderator | FFS on all option 1 in next meeting. Proponents of LS can bring in draft LS in next meeting. |

### Sub-topic 3-4 SP CSI-RS and periodic CSI-RS related enhancement for L1 part

Issue 3-4-1: SP CSI-RS and periodic CSI-RS related enhancement during FR2 unknown SCell activation

- Proposal 1 (Intel): Semi-persistent CSI-RS activation or RRC based CSI configuration command can be sent with SCell activation command together.

Recommendations for 2nd round:

No discussion for this issue in 2nd round. This proposal will not be captured in the WF for FFS unless Intel still wants to keep it for next meeting.

### Sub-topic 3-5 Other enhancement for L1 part

Issue 3-5-1: Other enhancement for L1 part

- Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell.

- Proposal 2 (QC, Nokia, vivo): When Scell is semi-unknown to UE, NW can trigger UE to report RSRP before L1 RSRP measurement and report. The framework can be illustrated as following steps while UE perform the legacy FR2 Scell activation procedure.

  - Step 1. NW configure either periodic or aperiodic (event-based) report config before Scell activation command.

  - Step 2. NW trigger preconfigured report config. (e.g MAC-CE can trigger the report config)

  - Step 3: UE report latest RSRP measurement for target Scell

  - Step 4: UE can skip L1-RSRP measurement and report when UE reports RSRP measurement with associated SSB index during step3.

  - Step 5: Either A-TRS based or SSB-based for fine time tracking. TCI activation command and CSI-RS recourse activation for CQI reporting.

- Proposal 3 (Xiaomi, HW, ZTE, CTC): For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

Recommendations for 2nd round:

Continue discussion in 2nd round. Agreement will be captured in the WF.

Please companies share your view on how to discuss each enhancement part in a joint/unified framework, and I really appreciate if we can find a better way to organize the discussion.

| Company | Comments |
| --- | --- |
| Qualcomm | Thanks Moderator for hard work. We understand it is difficult to merge from diverse area. We comment more explanation for on proposal2. UE does not need to indicate whether this is semi-unknown or unknown. Anyhow both are unknown. We think whether UE can report RSRP report fast is important factor in this WI. Thus, our framework is starting from L3 measurement report and we want to share the whole flow of the proposed framework as followings - L3 measurement and reporting: - What kinds of enhancement can be applied for L3 part (e.g lower beam sweeping e.t.c) - Can NW allow UE to report L3 RSRP measurement during L3 measurement? - If so how report can be configured and triggered? And how frequently and when UE can report. - If UE report L3 RSRP measurement during L3 part, then what would be the next UE and NW behavior. FFS 2-1) Can UE skip L1 part (measurement and report) and expect to receive TCI activation command? 2-2) UE does not skip L1 part, what kinds of enhancement can be considered for L1 part a) beam sweeping reduction b) AP-RS or A-TRS based measurement e.t.c Note : We think using L3 RSRP measurement for L1 RSRP measurement report is effectively same as 1-2) if it is allowed. We hope this can help for better understanding about our proposal and help to merge into joint framework to smooth progress. Thanks. |
| Ericsson | We think we need further discussion. We can come back in next meeting. |
| LGE | We are generally similar understanding with QC. The L3 part enhancement is related to L1 part enhancement. In other words, L1-RSRP enhancement would depends on L3 part. But, we think that it can be also one way to find a enhancement solution for each part and then check under which conditions the overall enhancements for FR2 SCell activation delay reduction can be made. |
| Nokia | We support Proposal 2. The detailed step can be further discussed after we conclude on previous issues. |
| China Telecom | Support proposal 3. In our understanding, the joint/unified framework needs to consider the whole process including L3 and L1 part. For example, both L3 and L1 part involve Rx beam sweeping procedure, whether and how to reduce Rx beam sweeping factor should consider from the whole process. Besides, using A-TRS for UE to obtain timing/beam information from inter-band active serving cell adopts the idea of the joint/unified framework, which considers the whole process of SCell activation. |
| Huawei | Thanks Moderator for leading the discussion with diverse solutions. For option 2, our understanding is that it is for the case when NW configures the measurement but UE does not report the results (e.g. event not triggered). Form our understanding, the typical implementation could be that, NW configure and measurement And periodic reporting when NW want to active the SCell. Then the gain may come from RRC report configure –MAC CE triggered report. We support option 3 as discussed in other issues. We believe the common issue is discuss the feasibility and conditions when UE can obtaining and beam from inter-band active serving cell. |
| ZTE | Fine with Proposal 3. |
| vivo | FFS. Same comment as 1st round. |
| MTK | We are open to further study the proposals. However, we wonder the impact of the proposed framework in P2 could have on RAN1/RAN2 work. |
| Moderator | FFS on all proposals in next meeting. |

# Topic #4:	Other enhancements for FR2 SCell activation (6.9.2.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

  - Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215801 | LG Electronics Inc. | Proposal 4: Take further discussion on the other possible enhancements solution after stabilizing L1 and L3 part enhancement. |
| R4-2215531 | China Telecom | Proposal 1: The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions. Proposal 2: The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced. |
| R4-2215787 | Nokia, Nokia Shanghai Bell | Observation 1: The known condition is based on a time constraint of 3/4s of L3-RSRP reporting. Observation 2: The latest L3-RSRP reporting is still valid as long as the remaining conditions are fulfilled i.e. reported SSBs with indexes remains detectable, and the TCI state is selected based on one of the latest reported SSB indexes. Proposal 1: The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown. Observation 3: The UE is able to monitor the PDCCH of SCell after downlink synchronization and thus is ready for data transmission before the end of SCell activation. Proposal 2: The potential to enable earlier data transmission within the activation period shall be studied. Proposal 3: The UE shall be scheduled by the network immediately after L1-RSRP reporting. Proposal 4: The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell. |
| R4-2216274 | Huawei, HiSilicon | Observation 1: Extending the time duration in known conditions implies that gNB can configure long report interval in the case that the SCell can remain detectable for a long time which may have power saving gain. Proposal 1: RAN4 to further discuss the known condition extension to facilitate SCell activation. Proposal 2: Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). |
| R4-2216478 | ZTE Corporation | Observation 1: The inter-band CA TAE performance in FR2 could be between 10ns and 65ns and these values could be further optimized with pre-compensation algorithm and higher granularity sampling clock in the practice. Observation 2: The inter-band CA TAE performance in FR1 could be between 10ns and 65ns and these values could be further optimized with pre-compensation algorithm and higher granularity sampling clock in the practice. Proposal 1: Similar as the enhancement for fast SCell activation in Re-17, for unknown FR2 target SCell case, if the timing and beam can be obtained from an active serving cell in another band, then the cell search and L1-RSRP can be ignored. Proposal 2: SCell activation delay requirement in FR2 inter-band CA could be optimized down to 3ms via ScellwithoutSSB. Proposal 3: SCell activation delay requirement in FR1 inter-band CA could be optimized down to 3ms via ScellwithoutSSB. |
| R4-2216760 | MediaTek inc. | Proposal 1: Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell. |
| R4-2216830 | Ericsson | Proposal 1: RAN4 not to consider mandatory aperiodic reports before SCell activation. Proposal 2: RAN4 to study relaxation of SCell known condition time. Proposal 3: RAN4 not to consider CBRA for SCell activation |

  - Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

    - Sub-topic 4-1 Other possible enhancement for FR2 SCell activation

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 4-1-1: Timeline to discuss other possible enhancement of FR2 unknown SCell activation (the enhancement not included in topic #2 and #3)

- Proposals

  - Proposal 1 (LG): Take further discussion on the other possible enhancements solution after stabilizing L1 and L3 part enhancement.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support proposal 1. We also prefer to concentrate on the above L3/L1 enhancement first. |
| Qualcomm | The proposal is not clear. any possible enhancement should be proposed and discussed before stabilizing phase 1. |
| LGE | Support proposal 1. We prefer to focus on L1 and L3 enhancement first. |
| Huawei | We do not support proposal 1. The discussion is organized into L3/L1 aspects though companies are not perfectly aligned about the definition of L3 and L1 components. But it does not mean a solution only for L3 or L1 is more important or promising than the one that can achieve better delay reduction on overall delay. |
| Ericsson | Same view as QC. |
| Nokia | Agree with QC. Some proposals are categorized into other enhancements as it does not fall into L3/L1 enhancement, but they may be relevant to the L3/L1 part proposals. There is no need to discuss those in sequence. |
| ZTE | Same view as QC. |

Issue 4-1-2: SCell without SSB for inter-band FR2 unknown SCell activation

- Proposals

  - Proposal 1 (China Telecom): The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions.

    - Proposal 1a (ZTE): SCell activation delay requirement in FR2 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

  - Proposal 2 (China Telecom): The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced.

    - Proposal 2a (ZTE): SCell activation delay requirement in FR1 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | The SCell without SSB means the target SCell needs to get timing and beam information from another inter-band FR2 serving SCell. However, as we commented in other above issue, the inter-band serving cell and target SCell will have inter-band MRTD and TAE is 3us, which is larger than CP, we don’t think the QCL type C can directly lead UE to reuse the timing from this inter-band serving cell. Regarding the beam information, CDM is not introduced/specified yet, the QCL type D information from inter-band FR2 serving cell will still need UE do perform the Rx beam sweeping for target SCell(inter-band serving cell and target SCell still need to do IBM). |
| Qualcomm | We do not support all proposals. This is out-of-scope. In this R18 WI, the objective is enhancement from the legacy FR2 SCell activation scenario. |
| Huawei | We can see the benefits and open to further discuss. |
| Ericsson | We do not understand yet how UE can derive time and freq sync from inter-band serving cells for non-colocation. |
| Nokia | We have same concern with Ericsson. Are we discussing non-colocated inter-band scenario? The feasibility of configuring QCL for non-colocated cells need to be discussed. |
| vivo | We are open to further discuss. RAN2 impact for this would be expected. In current 38.306, the following is captured. scellWithoutSSB Defines whether the UE supports configuration of SCell that does not transmit SS/PBCH block. This is conditionally mandatory with capability signalling for intra-band CA but not supported for inter-band CA. |
| ZTE | Support all options. To Apple, as we interpreted in Issue 2-4-1, current TAE requirement for inter-band CA is the worst case, and in fact in practice the TAE can be much better than current 3us under some deployment, such as co-located deployment, the practical TAE between different bands can be optimized to 10ns-65ns, much less than CP. So timing info can be acquired via an inter-band active serving cell. Regarding to beam info, for co-located deployment, it also can be obtained by UE via the inter-band active serving cell. To Ericsson and Nokia, for non-colocated inter-band scenario, we are open to discuss whether timing and beam info can be acquired. But for co-located inter-band scenario, it is feasible. To vivo, yes, the current scellWithoutSSB in only allowed for intra-band CA, which is closely related with the conservative TAE requirement under inter-band CA currently. But in fact the actual TAE can be much more optimized, so applying scellWithoutSSB under inter-band CA is feasible and we can discuss the condition in details. We believe applying the scellWithoutSSB capability into co-located inter-band scenario, which can bring ultimate latency reduction up to only 3ms, |
| China Telecom | Support Proposal 1/1a/2/2a. The legacy requirement has applied to FR2 intra-band scenario, and we think it is able to extend into FR2 inter-band scenario to obtain significant delay reduction. Meanwhile, the extension into FR1 inter-band scenario can also be considered to avoid duplicate discussion. In our understanding, if the two bands are close together, the timing difference between two bands should be reduced smaller enough in the practical deployment, therefore, timing and beam information are able to be reused. Meanwhile, the feasibility and performance of wideband RF is investigated in mmWave multi-band BS (for example, n258 + n261) scenario. We agree to discuss further about this issue in details. |

Issue 4-1-3: known/unknown condition enhancement for FR2 SCell activation

- Proposals

  - Proposal 1 (Nokia): The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown.

  - Proposal 2 (Huawei, Ericsson):RAN4 to further discuss the known condition extension to facilitate SCell activation.

  - Proposal 3 (Ericsson): RAN4 not to consider mandatory aperiodic reports before SCell activation

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We support proposal 2, and open to discuss if the 3/4s constraint can be extended or not. |
| Qualcomm | We do not support Proposal 1, Proposal 2. There is no technical reason to have different unknown condition only for FR2 Scell. We support proposal 3. We would like to ask periodic or aperiodic report for L3 measurement report can be considered after Scell receiving activation command. |
| LGE | We can further discuss proposal 1 and proposal 2. But we prefer to focus on L1/L3 enhancement in unknown case first. |
| Huawei | We support proposal 2. More clarification is needed for proposal 3. |
| Ericsson | We agree with the proposals 2 and 3. We don’t see any need to remove the L3-RSRP reporting which is used to determine known and unknown conditions of the SCell. If this condition is removed, then network cannot know whether the SCell is known or unknown. Current known condition may not be strictly reflecting the actual scenario. We think we could introduce a new state (between current known and unknown state) where UE channel did not change much though UE did not send measurement report for last 3 sec or 5 sec. Rather the UE has sent the report during the period which is much longer than 3 or 4 seconds. Please also check our reply under issue 2-1-1. To Huawei, we mean starting point for enhancement is after SCell activation command is received. |
| Nokia | Fine with discuss all the proposals. P1 and P2 seems addressing the same known/unknown condition. If the known condition can be extended, some unknown cases can be avoided hence the long activation delay can be minimized. We’d like to understand more on P3. When the UE is in deactivated SCell, it keeps L3 measurement measurement and reporting, where the configuration is up to network. It seems P3 is already the case now? To QC’s question, the 3s/4s issue is not restricted to FR2 only. This time constraint only defines how long the UE has sent the reporting, it does not mean the UE has not measured the SCell. Removing the time constraint is also extending the scope of known condition. |
| vivo | We think the discussion of P1/P2 should be de-prioritized. De-activated SCell measurement is intra-frequency measurement and 3/4s is already enough for known condition. For P3, we are not sure what is the ‘aperiodic report’ here. Does it mean L3 MR reporting? Without this, how to determine the TCI without L1 reporting? We can not support P3. |
| ZTE | Support Proposal 2. |

Issue 4-1-4: Applying the timing/beam information of active serving cell on FR2 inter-band

- Proposals

  - Proposal 1 (Huawei): Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

  - Proposal 2 (ZTE): Similar as the enhancement for fast SCell activation in Re-17, for unknown FR2 target SCell case, if the timing and beam can be obtained from an active serving cell in another band, then the cell search and L1-RSRP can be ignored.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | We have concern on both proposal 1 and 2, same comments as for issue 2-4-1 and 4-1-2. |
| Qualcomm | We do not support all proposals. Inter-band CA uses IBM. There is no coordination between two independent RX chains. It is unclear how NW knows beam info for QCLed RS to target Scell. |
| LGE | it is not clear how to apply timing and beam from active serving cell in another band for unknown target SCell. |
| Huawei | We support proposal 1 and proposal 2. The conditions can be further discuss in terms of the timing difference, etc. |
| Ericsson | Cannot support them yet. |
| Nokia | Probably we should start from if there is any scenario where the same timing/beam information is applied for an FR2 inter-band cell. Then we can discuss the condition and the activation procedure. |
| vivo | We are OK to further discuss option 1 and 2. Same comment as issue 4-1-2 |
| ZTE | Support both of options. To Qualcomm, the TCI state indicated to the target SCell can be associated with a QCL-TypeD source RS which is transmitted in the inter-band active serving cell. Based on RAN1/2 signalling, QCL-TypeD/C can be cross cell referenced. We are open to further discuss the detailed conditions for inter-band case. |
| China Telecom | Support proposal 1 and proposal 2. We think timing/beam information of active serving cell on FR2 inter-band can be used for FR2 SCell activation under some conditions, we are open to further discuss the conditions. |

Issue 4-1-5: Scheduling enhancement for FR2 unknown SCell activation

- Proposals

  - Proposal 1 (Nokia): The potential to enable earlier data transmission within the activation period shall be studied.

  - Proposal 2 (Nokia): The UE shall be scheduled by the network immediately after L1-RSRP reporting.

  - Proposal 3 (Nokia): The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | For proposal 1/2/3, we didn’t see any scheduling limitation in current spec, that is, in existing requirement, network can also schedule UE earlier but UE is allowed to not execute the scheduling data if it doesn’t complete activation. Proposal 1/2/3 is up to network implementation and may not need to capture in the spec. |
| Qualcomm | We have similar view as Apple. We do not support all proposals. Without fine time tracking and CQI report, UE decoding performance may be poor. |
| Huawei | Similar views as Apple. |
| Nokia | P1,2,3. The intention of the proposals is to enable early data transmission before the UE sends valid CSI reporting. As the scheduling is possible once getting the DL timing, it is a waste of system capability to wait until the end of SCell activation. The UE may continue refining the beam by L1-RSRP, channel measurement, but they could be performed in parallel with data transmission to minimize the negative impact due to SCell activation. To Apple’s comment, is there any reason that UE does not respond to the scheduling if it has acquired the DL timing and rough/refined beam? The UE can also benefit from early data transmission. |

Issue 4-1-6: CBRA for FR2 unknown SCell activation

- Proposals

  - Option 1 (MediaTek): Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

  - Option 2 (Ericsson): RAN4 not to consider CBRA for SCell activation.

- Recommended WF

  - TBA

- 1st round Comment collection:

| Company | Comments |
| --- | --- |
| Apple | Support option 2. CBRA is L3 procedure and SCell activation is L2, we don’t want to change the foundation of the legacy RACH for this activation case; it will cause a lot RAN2 work in our view. |
| Qualcomm | We support Option2. |
| LGE | Prefer option 2. |
| Ericsson | We think CBRA may introduce interruption on other active serving cells. In worst case interruptions may impact the purpose of SCell activation itself. |
| Nokia | As CBRA would also introduce interruption and may take long time, we wonder if CBRA could reduce the SCell activation delay. It would be good if some delay estimation could be provided to justify the approach. |
| vivo | Prefer option 2. For the SCell activation, PDCCH order can be used. |
| MTK | Support Option 1. We appreciate the concerns from companies regarding the interruption on the serving cell, however, with the UE capability parallelTxPRACH-SRS-PUCCH-PUSCH defined in 38.306, UE can be able to perform CBRA on the SCell without causing interruption on the serving cell. Therefore, we think CBRA approach in Option 1 can still bring enhancement to the SCell activation delay. |

  - Companies views’ collection for 1st round

    - Open issues

One of the two formats, i.e. either example 1 or 2 can be used by moderators.

    - CRs/TPs comments collection

For close-to-finalize WIs and maintenance work, comments collections can be arranged for TPs and CRs. For ongoing WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

  - Summary for 1st round

    - Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

Sub-topic 4-1 Other possible enhancement for FR2 SCell activation

|  | Status summary |
| --- | --- |
| Issue 4-1-1: Timeline to discuss other possible enhancement of FR2 unknown SCell activation (the enhancement not included in topic #2 and #3) | Discussion status: QC/Ericsson/Nokia/ZTE commented that any possible enhancement should be proposed and discussed before stabilizing phase 1. Candidate options: - Proposal 1 (Apple, LGE): Take further discussion on the other possible enhancements solution after stabilizing L1 and L3 part enhancement. Recommendations for 2nd round: Many companies would like to not limit the discussion to certain parts at the beginning stage of this WI, Moderator think it makes sense. No discussion for this issue in 2nd round. This proposal will not be captured in the WF for FFS unless LGE still wants to keep it for next meeting. |
| Issue 4-1-2: SCell without SSB for inter-band FR2 unknown SCell activation | Discussion status: ZTE and CTC supported these proposals but other companies had objection/comment/question to the proposals. Candidate options: - Proposal 1 (ZTE, CTC): The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions. - Proposal 1a (ZTE, CTC): SCell activation delay requirement in FR2 inter-band CA could be optimized down to 3ms via ScellwithoutSSB. - Proposal 2 (ZTE, CTC): The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced. - Proposal 2a (ZTE, CTC): SCell activation delay requirement in FR1 inter-band CA could be optimized down to 3ms via ScellwithoutSSB. Recommendations for 2nd round: Continue discussion in 2nd round. Agreement will be captured in WF. |
| Issue 4-1-3: known/unknown condition enhancement for FR2 SCell activation | Discussion status: 5 companies suggested to FFS on the known condition. Candidate options: - Proposal 1 (Nokia): The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown. - Proposal 2 (Apple, HW, Ericsson, ZTE, Nokia):RAN4 to further discuss the known condition extension to facilitate SCell activation. - Proposal 3 (QC, Ericsson): RAN4 not to consider mandatory aperiodic reports before SCell activation Recommendations for 2nd round: More discussion is needed, and recommended to discuss them in next meeting. No discussion in 2nd round. Those candidate options will be captured for FFS in WF. |
| Issue 4-1-4: Applying the timing/beam information of active serving cell on FR2 inter-band | Discussion status: The views on proposal 1 and 2 are diverse. Candidate options: - Proposal 1 (HW, ZTE, CTC): Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms). - Proposal 2 (HW, ZTE, CTC): Similar as the enhancement for fast SCell activation in Re-17, for unknown FR2 target SCell case, if the timing and beam can be obtained from an active serving cell in another band, then the cell search and L1-RSRP can be ignored. Recommendations for 2nd round: More discussion is needed, and this issue is also related with issue 2-4-1 and 4-1-2. Recommended to discuss them in next meeting. No discussion in 2nd round. Those candidate options will be captured for FFS in WF. |
| Issue 4-1-5: Scheduling enhancement for FR2 unknown SCell activation | Discussion status: 3 companies thought this scheduling enhancement is covered in existing requirement. Candidate options: - Proposal 1 (Nokia): The potential to enable earlier data transmission within the activation period shall be studied. - Proposal 2 (Nokia): The UE shall be scheduled by the network immediately after L1-RSRP reporting. - Proposal 3 (Nokia): The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell. Recommendations for 2nd round: Continue discussion in 2nd round and answer Nokia’s question. Agreement will be captured in WF. |
| Issue 4-1-6: CBRA for FR2 unknown SCell activation | Discussion status: 5 companies had comment and concern on option 1. Candidate options: - Option 1 (MTK): Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell. - Option 2 (Apple, QC, LGE, Ericsson, vivo): RAN4 not to consider CBRA for SCell activation. Recommendations for 2nd round: Continue discussion in 2nd round and please MTK check if they can compromise to option 2. Agreement will be captured in WF. |

    - CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

  - Discussion on 2nd round (if applicable)

### Sub-topic 4-1 Other possible enhancement for FR2 SCell activation

Issue 4-1-1: Timeline to discuss other possible enhancement of FR2 unknown SCell activation (the enhancement not included in topic #2 and #3)

- Proposal 1 (Apple, LGE): Take further discussion on the other possible enhancements solution after stabilizing L1 and L3 part enhancement.

Recommendations for 2nd round:

Many companies would like to not limit the discussion to certain parts at the beginning stage of this WI, Moderator think it makes sense.

No discussion for this issue in 2nd round. This proposal will not be captured in the WF for FFS unless LGE still wants to keep it for next meeting.

Issue 4-1-2: SCell without SSB for inter-band FR2 unknown SCell activation

- Proposal 1 (ZTE, CTC): The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions.

  - Proposal 1a (ZTE, CTC): SCell activation delay requirement in FR2 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

- Proposal 2 (ZTE, CTC): The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced.

  - Proposal 2a (ZTE, CTC): SCell activation delay requirement in FR1 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

Recommendations for 2nd round:

Continue discussion in 2nd round. Agreement will be captured in WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t have strong view at this point. We are open to discuss about its applicability. |
| Apple | The feasibility is still our concern, regarding the beam information and timing information from an inter-band CC. Need more discussion. |
| Ericsson | We share same concern as Apple. We can further discuss. |
| Nokia | We’d like to firstly understand the scenario for the discussion. More discussion is needed. |
| China Telecom | Support Proposal 1/1a/2/2a. In our views, if two bands are close, RTD can be reduced to a smaller value compared with legacy requirement in practical deployment. The QCL relation can be indicated between inter-band cells so as to obtain timing/beam information from inter-band serving cell. In the FR2 multi-band BS study item, the wideband RF between some band combination is investigated. If the wideband RF is feasible, the timing/beam information can be reused from inter-band active serving cell without any SSB/SMTC for the target SCell. We are open to discuss further in details. |
| Huawei | We are fine to further discuss with the benefits observed. |
| ZTE | Fine with all Proposals. Regarding to how to address the concerns on timing and beam info under inter-band CA case, same view as in Issue 2-4-1. |
| vivo | FFS. Same comment as 1st round. |
| Moderator | FFS on all proposals in next meeting. |

Issue 4-1-3: known/unknown condition enhancement for FR2 SCell activation

- Proposal 1 (Nokia): The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown.

- Proposal 2 (Apple, HW, Ericsson, ZTE, Nokia):RAN4 to further discuss the known condition extension to facilitate SCell activation.

- Proposal 3 (QC, Ericsson): RAN4 not to consider mandatory aperiodic reports before SCell activation

Recommendations for 2nd round:

More discussion is needed, and recommended to discuss them in next meeting. No discussion in 2nd round. Those candidate options will be captured for FFS in WF.

Issue 4-1-4: Applying the timing/beam information of active serving cell on FR2 inter-band

- Proposal 1 (HW, ZTE, CTC): Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

- Proposal 2 (HW, ZTE, CTC): Similar as the enhancement for fast SCell activation in Re-17, for unknown FR2 target SCell case, if the timing and beam can be obtained from an active serving cell in another band, then the cell search and L1-RSRP can be ignored.

Recommendations for 2nd round:

More discussion is needed, and this issue is also related with issue 2-4-1 and 4-1-2. Recommended to discuss them in next meeting. No discussion in 2nd round. Those candidate options will be captured for FFS in WF.

Issue 4-1-5: Scheduling enhancement for FR2 unknown SCell activation

- Proposal 1 (Nokia): The potential to enable earlier data transmission within the activation period shall be studied.

- Proposal 2 (Nokia): The UE shall be scheduled by the network immediately after L1-RSRP reporting.

- Proposal 3 (Nokia): The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

Recommendations for 2nd round:

Continue discussion in 2nd round and answer Nokia’s question. Agreement will be captured in WF.

| Company | Comments |
| --- | --- |
| Qualcomm | We are open to discuss about its applicability. More study is needed. |
| Apple | We think all proposals are not forbidden in current spec. To Nokia’s question in 1st round, we think when the UE has DL timing and beam/TCI information, UE is able to be scheduled, and this early scheduling is not precluded in current spec. That means, before valid CQI reporting, if network wants to schedule this UE, it’s possible and allowed, but whether or not this UE will response or react correctly to this scheduling is up to the status at UE; and we believe even in R15, if network schedules this UE before valid CQI reporting, as long as UE is ready on timing and beam, UE is happy to receive such scheduled data, to increase the throughput and reduce latency. |
| Ericsson | Maybe we can further discuss this after getting clarity on the solutions we are going to agree as part of this WI. |
| Nokia | Thanks for Apple’s response. We can bring some detailed discussion next meeting. |
| vivo | FFS. Same comment as 1st round. |
| Moderator | FFS on all proposals in next meeting. |

Issue 4-1-6: CBRA for FR2 unknown SCell activation

- Option 1 (MTK): Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

- Option 2 (Apple, QC, LGE, Ericsson, vivo): RAN4 not to consider CBRA for SCell activation.

Recommendations for 2nd round:

Continue discussion in 2nd round and please MTK check if they can compromise to option 2. Agreement will be captured in WF.

| Company | Comments |
| --- | --- |
| Apple | Option 2, but fine to FFS. |
| Ericsson | Option 2. We also agree with Apple comment in the first round. Since there is lot of impact on RAN2, it should be discussed in RAN2 first. |
| Nokia | Option 2. We wonder how much benefit there could be. |
| Huawei | We are open to further discuss and evaluate the benefits and impacts to RAN1/2 spec. |
| vivo | FFS. Same comment as 1st round. |
| MTK | We are fine to FFS. We understand that this could have RAN1/2 impact. The idea here is kind of similar to the framework suggested by QC in Issue 3-5-1 to report L3 measurements, but here we apply RACH procedure instead which can be faster than L3 measurement report. In our understanding, both this approach and the framework in Issue 3-5-1 can have RAN1/2 impact. |
| Moderator | FFS on all proposals in next meeting. |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on R18 eFeRRM - FR2 SCell activation enhancement | Apple |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215599 |  | Updated work plan for R18 eFeRRM | Apple, OPPO | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2217249 |  | WF on R18 eFeRRM - FR2 SCell activation enhancement | Apple | Agreeable |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents