3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217177

# Electronic Meeting, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2589 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test cases for Intra- and inter-frequency CHO for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Perf |  |  |  |  |  | Date: |  |  | 2022-9-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test cases for Intra- and inter-frequency SAN time-based conditional handover from FR1 to FR1 and for Intra- and inter-frequency SAN distance-based conditional handover from FR1 to FR1 have been introduced. Following modification are need based on agreements in last meeting: - Section number of the four TCs are incorrect. - Capturing the TCs for GSO and NGSO in the same clauses. - Satellite configuration will be defined in a dedicated clause. - For location condition handover, D1-1 and D1-2 is not appropriate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following modifications are made: - Section number of the four TCs are modified to A.14.2.1.3 ~ A.14.2.1.6. - Capturing the TCs for GSO and NGSO in the same clauses. - Satellite configuration refers to the corresponding clause, and Koffset for NGSO is modified to [4]ms from 8ms. - Regarding test cases for CHO with location condition, D1-1 and D1-2 condition is modified fulfilled at T2. - Modefy D1-1 and D1-2 based on TS38.331, modify UE moving speed, and modify the test requirements for location based conditional handover accodingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for Intra- and inter-frequency SAN conditinal handover will not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.14.2.1.3, A.14.2.1.4, A.14.2.1.5, A.14.2.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2215394 |  |  |  |  |  |  |  |  |

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1

A.14.2.1.3.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN time-based conditional handover from FR1 to FR1 specified in clause 6.1C.2.

A.14.2.1.3.2	Test Parameters

The test scenario comprises of 1 NR FDD carrier and 2 cells as given in table A.14.2.1.3.2-1, A.14.2.1.3.2-2, and A.14.2.1.3.2-3. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure intra-frequency neighbour cell. The RRC message implying time-based handover to cell 2 with Event CondEvent T1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1 and time condition event t1-Threshold-r17 is fulfilled.

Table A.14.2.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.2.1.3.2-2: General test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | RMC in [A.x] | For GSO satellite configuration |
|  | Config 2 |  | RMC in [A.x] | For NGSO satellite configuration |
| UE position (N,S, H) |  |  | [(0, 0, 0)] | Set by AT command |
| t1-Threshold-r17.condEventT1-r17 |  | s | T2 | Entering condition |
| duration-r17.condEventT1-r17 |  | slot | 1000 | Give 1s search duration |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.14.2.1.3.2-3: Cell specific test parameters for Intra-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration |  |  | Unit |  |  |  | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | T1 |  |  | T2 |  |  |  | T1 |  | T2 |  |
| NR RF Channel Number |  |  | Config 1,2 |  |  |  |  | 1 |  |  |  |  | 1 |  |  |  |
| BWchannel |  |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |  |
| BWP BW |  |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  |  | 10: NRB,c = 52 |  |  |  |
| TACommon |  | Config 1,2 | s | 0 |  | 0 |  |
| TACommonDrift |  |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  |  | s | 0 |  | 0 |  |
| Koffset |  |  | Config 1 |  |  | ms |  | [239] |  |  |  |  | [239] |  |  |  |
|  |  |  | Config 2 |  |  | ms |  | [4] |  |  |  |  | [4] |  |  |  |
| Kmac |  |  | Config 1,2 |  |  | ms |  | 0 |  |  |  |  | 0 |  |  |  |
| DRX Cycle |  |  |  |  |  | ms |  | Not Applicable |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2 |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated DL BWP |  |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
|  | Initial UL BWP |  |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated UL BWP |  |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2 |  |  | dB |  | 0 |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1,2 |  |  | dBm/ 15kHz |  | -98 |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  |  | dBm/ SCS |  | -98 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | dB |  | 8 |  |  | -3.3 |  | -Infinity |  | 2.36 |  |
|  |  |  |  |  |  | dB |  | 8 |  |  | 8 |  | -Infinity |  | 11 |  |
| SSB_RP |  |  |  |  |  | dBm/ SCS |  | -90 |  |  | -90 |  | -Infinity |  | -87 |  |
| IoNote3 |  |  |  |  |  | dBm/ 9.36MHz |  | -61.41 |  |  | -57.06 |  | -61.41 |  | -57.06 |  |
| Propagation condition |  |  |  |  |  | - |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.14.2.1.3.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 872 ms.

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1

A.14.2.1.4.1	Test Purpose and Environment

This test is to verify the requirement for inter -frequency SAN time-based conditional handover from FR1 to FR1 specified in clause 6.1C.2.

A.14.2.1.4.2	Test Parameters

The test scenario comprises of 2 NR FDD carrier and one cell on each carrier as given in table A.14.2.1.4.2-1, A.14.2.1.4.2-2, and A.14.2.1.4.2-3. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure inter-frequency neighbour cell and Gap pattern ID gp0. The RRC message implying time-based handover to cell 2 with Event CondEvent T1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1 and time condition event t1-Threshold-r17 is fulfilled.

Table A.14.2.1.4.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.2.1.4.2-2: General test parameters for Inter-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | RMC in [A.x] | For GSO satellite configuration |
|  | Config 2 |  | RMC in [A.x] | For NGSO satellite configuration |
| UE position (N,S, H) |  |  | [(0, 0, 0)] | Set by AT command |
| t1-Threshold-r17.condEventT1-r17 |  | s | T2 | Entering condition |
| duration-r17.condEventT1-r17 |  | slot | 1000 | Give 1s search duration |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.14.2.1.4.2-3: Cell specific test parameters for Inter-frequency SAN time-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration |  |  | Unit |  | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | T1 | T2 |  | T1 | T2 |  |
| NR RF Channel Number |  |  | Config 1, 2 |  |  |  | 1 |  |  | 2 |  |  |
| BWchannel |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
| BWP BW |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
| TACommon |  | Config 1, 2 | s | 0 |  | 0 |  |
| TACommonDrift |  |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  |  | s | 0 |  | 0 |  |
| Koffset |  |  | Config 1 |  | ms |  | [239] |  |  | [239] |  |  |
|  |  |  | Config 2 |  | ms |  | [4] |  |  | [4] |  |  |
| Kmac |  |  | Config 1, 2 |  | ms |  | 0 |  |  | 0 |  |  |
| DRX Cycle |  |  |  |  | ms |  | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |
| CORESET Reference Channel |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |
| TRS configuration |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1, 2 |  | dB |  | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Config 1, 2 |  | dBm/ 15kHz |  | -98 |  |  |  |  |  |
| Note2 |  |  |  |  | dBm/ SCS |  | -98 |  |  |  |  |  |
|  |  |  |  |  | dB |  | 4 | 4 |  | -Infinity | 9 |  |
|  |  |  |  |  | dB |  | 4 | 4 |  | -Infinity | 9 |  |
| SSB_RP |  |  |  |  | dBm/ SCS |  | -94 | -94 |  | -Infinity | -89 |  |
| IoNote3 |  |  |  |  | dBm/ 9.36MHz |  | -64.59 | -64.59 |  | -70.05 | -60.53 |  |
| Propagation condition |  |  |  |  | - |  | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |

A.14.2.1.4.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 872 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

Tmeasure = 600 + 200 ms; Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 872 ms.

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1

A.14.2.1.5.1	Test Purpose and Environment

This test is to verify the requirement for intra-frequency SAN distance-based conditional handover from FR1 to FR1 specified in clause 6.1C.2.

A.14.2.1.5.2	Test Parameters

The test scenario comprises of 1 NR FDD carrier and 2 cells as given in table A.14.2.1.5.2-1, A.14.2.1.5.2-2, and A.14.2.1.5.2-3. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure intra-frequency neighbour cell. The RRC message implying distance-based handover to cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1 and location condition event condEventD1-r17 is fulfilled.

Table A.14.2.1.5.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.2.1.5.2-2: General test parameters for Intra-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1 | One NR NTN satellite RF channel |
| Initial conditions | Active cell |  | Cell 1 | FDD duplex mode cell |
|  | Neighbouring cell |  | Cell 2 | FDD duplex mode cell |
| Final condition | Active cell |  | Cell 2 |  |
| Satellite configuration | Config 1 |  | RMC in [A.x] | For GSO satellite configuration |
|  | Config 2 |  | RMC in [A.x] | For NGSO satellite configuration |
| UE position (N,S, H) at T1 start |  |  | [(0, 0, 0)] | Set by AT command |
| UE moving speed |  | km/h | [(108, 0, 0)] | Set by AT command |
| referenceLocation1-r17.condEventD1-r17 |  | m | [(-700, 0, 0)] | Reference location for serving cell |
| referenceLocation2-r17.condEventD1-r17 |  | m | [(1300, 0, 0)] | Reference location for target cell |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m | [20] | D1-1 Location condition is fulfilled at T2 |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m | [20] | D1-2 Location condition is fulfilled at T2 |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 10 |  |
| T2 |  | s | 6 |  |

Table A.14.2.1.5.2-3: Cell specific test parameters for Intra-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration |  |  | Unit |  | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |  |
| NR RF Channel Number |  |  | Config 1, 2 |  |  |  | 1 |  |  |  | 1 |  |  |  |
| BWchannel |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |
| BWP BW |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |
| TACommon |  | Config 1, 2 | s | 0 |  | 0 |  |
| TACommonDrift |  |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  |  | s | 0 |  | 0 |  |
| Koffset |  |  | Config 1 |  | ms |  | [239] |  |  |  | [239] |  |  |  |
|  |  |  | Config 2 |  | ms |  | [4] |  |  |  | [4] |  |  |  |
| Kmac |  |  | Config 1, 2 |  | ms |  | 0 |  |  |  | 0 |  |  |  |
| DRX Cycle |  |  |  |  | ms |  | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB Configuration |  |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1, 2 |  | dB |  | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | Config 1, 2 | dBm/ 15kHz | -98 |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/ SCS | -98 |  |  |  |
|  |  |  | dB | 8 | -3.3 | -Infinity | 2.36 |
|  |  |  | dB | 8 | 8 | -Infinity | 11 |
| SSB_RP |  |  | dBm/ SCS | -90 | -90 | -Infinity | -87 |
| IoNote3 |  |  | dBm/ 9.36MHz | -61.41 | -57.06 | -61.41 | -57.06 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.14.2.1.5.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [872]  ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

50m UE position error is allowed.

D1-1 time = (50m*6) / (108km/h*1000/3600) = 10s from T1 start.

Tmeasure = max(600 + 200 ms, 10s – 10s) = 800 ms;

Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 800ms + 62ms + 10ms = 872 ms.

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1

A.14.2.1.6.1	Test Purpose and Environment

This test is to verify the requirement for inter -frequency SAN distance-based conditional handover from FR1 to FR1 specified in clause 6.1C.2.

A.14.2.1.6.2	Test Parameters

The test scenario comprises of 2 NR FDD carrier and one cell on each carrier as given in table A.14.2.1.6.2-1, A.14.2.1.6.2-2, and A.14.2.1.6.2-3. Both handover delay and interruption length are tested.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2. During T1, the UE is configured to measure inter-frequency neighbour cell and Gap pattern ID gp0. The RRC message implying distance-based handover to cell 2 with Event D1 shall be sent to UE, at a time earlier than TRRC (10ms) before the beginning of T2.

Starting T2, cell 2 becomes detectable and offset better than cell 1 and location condition event condEventD1-r17 is fulfilled.

Table A.14.2.1.6.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| 2 | NGSO, NR FDD, 15kHz SSB SCS, 10 MHz BW |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Table A.14.2.1.6.2-2: General test parameters for Inter -frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Unit |  | Value |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RF Channel Number |  |  |  | 1, 2 |  | Two NR NTN satellite RF channel |  |
| Initial conditions | Active cell |  |  | Cell 1 |  | FDD duplex mode cell |  |
|  | Neighbouring cell |  |  | Cell 2 |  | FDD duplex mode cell |  |
| Final condition | Active cell |  |  | Cell 2 |  |  |  |
| Satellite configuration | Config 1 |  |  | RMC in [A.x] |  | For GSO satellite configuration |  |
|  | Config 2 |  |  | RMC in [A.x] |  | For NGSO satellite configuration |  |
| UE position (N,S, H) at T1 start |  |  |  | [(0, 0, 0)] |  | Set by AT command |  |
| UE moving speed |  | km/h |  | [(108, 0, 0)] |  | Set by AT command |  |
| referenceLocation1-r17.condEventD1-r17 |  | m |  | [(-700, 0, 0)] |  | Reference location for serving cell |  |
| referenceLocation2-r17.condEventD1-r17 |  | m |  | [(1300, 0, 0)] |  | Reference location for target cell |  |
| distanceThreshFromReference1-r17.condEventD1-r17 |  | 50m |  | [20] |  | D1-1 Location condition is fulfilled at T2 |  |
| distanceThreshFromReference2-r17.condEventD1-r17 |  | 50m |  | [20] |  | D1-2 Location condition is fulfilled at T2 |  |
| hysteresis-r17.condEventD1-r17 |  | 10m | 0 |  |
| timeToTrigger-r17.condEventD1-r17 |  | s | 0 |  |
| A3-Offset in condition |  | dB |  | 0 |  |  |  |
| Hysteresis |  | dB |  | 0 |  |  |  |
| Time To Trigger |  | s |  | 0 |  |  |  |
| Filter coefficient |  |  |  | 0 |  | L3 filtering is not used |  |
| Access Barring Information |  | - |  | Not Sent |  | No additional delays in random access procedure. |  |
| Time offset between cells |  |  |  | 3 s |  | Synchronous cells |  |
| T1 |  | s |  | 10 |  |  |  |
| T2 |  | s |  | 6 |  |  |  |

Table A.14.2.1.6.2-3: Cell specific test parameters for Inter-frequency SAN distance-based conditional handover from FR1 to FR1

| Parameter |  | Test configuration |  |  |  | Unit |  |  | Cell 1 |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | T1 |  |  | T2 |  | T1 |  |  | T2 |  |
| NR RF Channel Number |  |  | Config 1, 2 |  |  |  | 1 |  |  |  | 2 |  |  |  |  |
| BWchannel |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |  |
| BWP BW |  |  |  |  | MHz |  | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |  |  |
| TACommon |  | Config 1, 2 | s | 0 |  | 0 |  |
| TACommonDrift |  |  | s | 0 |  | 0 |  |
| TACommonDriftVariation |  |  | s | 0 |  | 0 |  |
| Koffset |  |  | Config 1 |  |  | ms |  |  | [239] |  |  |  |  | [239] |  |  |  |  |
|  |  |  | Config 2 |  |  | ms |  |  | [4] |  |  |  |  | [4] |  |  |  |  |
| Kmac |  |  | Config 1, 2 |  | ms |  | 0 |  |  |  | 0 |  |  |  |  |
| DRX Cycle |  |  |  |  | ms |  | Not Applicable |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  |  |  |  |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  |  |  |  |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |
| TRS configuration |  |  |  |  |  |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  |  |  | kHz |  | 15 kHz |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | Config 1, 2 |  | dB |  | 0 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  |  | Config 1, 2 |  | dBm/ 15kHz |  | -98 |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/ SCS | -98 |  |  |  |
|  |  |  | dB | 4 | 4 | -Infinity | 9 |
|  |  |  | dB | 4 | 4 | -Infinity | 9 |
| SSB_RP |  |  | dBm/ SCS | -94 | -94 | -Infinity | -89 |
| IoNote3 |  |  | dBm/ 9.36MHz | -64.59 | -64.59 | -70.05 | -60.53 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

A.14.2.1.6.3 Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than [872]  ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay is defined in clause 6.1C.2, can be expressed as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

where:

RRC procedure delay TRRC = 10 ms and is specified in clause 12 in TS 38.331 [2].

TEvent_DU = start of T2

50m UE position error is allowed.

D1-1 time = (50m*6) / (108km/h*1000/3600) = 10s from T1 start.

Tmeasure = max(600 + 200 ms, 10s – 10s) = 800ms;

Tinterrupt = 62ms; TCHO_execution = 10ms.

This gives a total of 800ms + 62ms + 10ms = 872 ms.