3GPP TSG-RAN WG4 Meeting # 104e	R4-2216023

# Electronic Meeting, Aug.15th –26th ,2022

Title: 	Discussions and simulation results on FR2-2 PUCCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.3.2

Document for:	Discussion

Background

As per [1], RAN 4 agreed to introduce FR2-2 PUCCH performance requirements. In this paper we provide our initial simulation results based on our proposed simulation assumptions.

Simulation results

Simulation results for PF0

Table 2-1: Simulation assumptions for FR2-2 PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |

Table 2-2: Simulation results for FR2-2 PF0

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 5.4 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -10.3 |
|  |  | TDLA10-650 Low | 1 | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 5.8 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -7.4 |

Simulation results for PF1

Table 2-3: Simulation assumptions for FR2-2 PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 and 16 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 2-4: Simulation results for FR2-2 PF1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Target(dB) SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -4.8 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -6.4 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -14.6 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<0.1% | -16.4 |
|  |  | TDLA10-650 Low | 2 | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -5.0 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -6.6 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -12.4 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% | -14.3 |

Simulation results for PF2

Table 2-5: Simulation assumptions for FR2-2 PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs-1) |
| Number of PRBs | 4 and 9 |
| Number of symbols | 1 and 2 |
| The number of UCI information bits | 4 and 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

Table 2-6: Simulation results for FR2-2 PF2

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 120 | 100 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% | 1.8 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% | -1.2 |
|  |  | TDLA10-650 Low | 480 | 400 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% | 1.4 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% | -1.3 |

Simulation results for PF3

Table 2-7: Simulation assumptions for FR2-2 PF3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

Table 2-8: Simulation results for FR2-2 PF3

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| \1 | 2 | QPSK | TDLA30-650 Low | 120 | 100 | 14 | 1 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -2.2 |
|  |  |  |  |  |  | 14 | 1 | 16 | Enabled | 1+1 | Prob(UCI BLER)<1% | -2.6 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -0.5 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+1 | Prob(UCI BLER)<1% | -0.6 |
|  |  |  | TDLA10-650 Low | 480 | 400 | 14 | 1 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -2.5 |
|  |  |  |  |  |  | 14 | 1 | 16 | Enabled | 1+1 | 14 | -3.0 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -0.4 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+1 | Prob(UCI BLER)<1% | -0.4 |

Simulation results for PF4

Table 2-9: Simulation assumptions for FR2-2 PF4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 1 and 16 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Table 2-10: Simulation results for FR2-2 PF4

| Number of TX antennas | Number of demodulation Branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of PRB | Additional DMRS configuration | Test metric | Target SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 120 | 100 | 1 | 1+0 | Prob(UCI BLER)<1% | 1.4 |
|  |  |  |  |  | 1 | 1+1 | Prob(UCI BLER)<1% | 1.3 |
|  |  |  |  |  | 16 | 1+0 | Prob(UCI BLER)<1% | -10.8 |
|  |  |  |  |  | 16 | 1+1 | Prob(UCI BLER)<1% | -11.1 |
|  |  | TDLA10-650 Low | 480 | 400 | 1 | 1+0 | Prob(UCI BLER)<1% | 2.5 |
|  |  |  |  |  | 1 | 1+1 | Prob(UCI BLER)<1% | 2.0 |
|  |  |  |  |  | 16 | 1+0 | Prob(UCI BLER)<1% | -10.6 |
|  |  |  |  |  | 16 | 1+1 | Prob(UCI BLER)<1% | -11.0 |

Discussions

One issue is whether to define the PUCCH requirements for 960kHz SCS

For subcarrier spacing and CBW for PUCCH requirement: - Option 1: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz and 480kHz/400MHz. - Option 2: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz. - Option 3: Consider the following SCS and CBW combination for FR2-2 PUCCH requirements definition: - 120kHz/100MHz, 480kHz/400MHz, and 960kHz/400MHz*. - Note: 960 kHz SCS is declared as optional.

Based on our understanding, 960kHz SCS leads to very low testable SNR which is not feasible for testing. Therefore we propose to not consider 960kHz SCS and focus on 120/480kHz SCS.

Proposal 1: Not consider 960kHz SCS for PUCCH requirements definition.

Conclusion

In this paper we provide our initial simulation results for FR2-2 PUCCH.

Reference

[1]   R4-2210664 WF on general and BS aspects for FR2-2 demodulation requirements. Intel Corporation