3GPP TSG-RAN WG4 Meeting # 104-bit-e	R4-2216013

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results for FR2-2 PDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.1

Document for:	Discussion

Background

As per [1], RAN 4 defined many cases for FR2-2 PDSCH simulation work. In this paper, we provide our simulation results.

Simulation results

The common simulation assumptions are captured in Table 2-1 and simulation results are captured in Table 2-2

Table 2-1: Common simulation assumptions for FR2-2 PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Bandwidth |  | MHz | 100,400 |
| SCS |  | kHz | 120,480 |
| PDSCH configuration | PDSCH mapping type |  | Type A |
|  | Start symbol |  | 2 |
|  | Symbol length |  | 12 |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | dmrs-AdditionalPosition |  | pos1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 8 |
| HARQ transmissions |  |  | 4 |
| HARQ transmission sequence |  |  | {0,2,3,1} |
| Phase noise model |  |  | Set 2 specified in TR 38.808 |
| Phase noise compensation |  |  | CPE only |

Table 2-2: Simulation results for FR2-2 PDSCH requirements

| Test | CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | SNR(dB)@70% of max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | 100/120 | MCS 4/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | -2.4 |
| 1-2 | 100/120 | MCS13/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | 6.0 |
| 1-3 | 100/120 | MCS13/Rank1 | TDLD30-200 | 2x2 ULA Low | 66 | 5.2 |
| 1-4 | 100/120 | MCS17/Rank1 | TDLD30-200 | 2x2 ULA Low | 30 | 9.0 |
| 2-1 | 400/480 | MCS 4/Rank1 | TDLA10-200 | 2x2 ULA Low | 66 | -2.8 |
| 2-1 | 400/480 | MCS 4/Rank1 | TDLD10-200 | 2x2 ULA Low | 66 | -3.4 |
| 2-2 | 400/480 | MCS13/Rank1 | TDLD10-200 | 2x2 ULA Low | 16 | 5.4 |

Conclusion

In this paper, we provide our simulation results for FR2-2 PDSCH.

Reference

[1]   R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated