3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216015

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results on PDCCH and PBCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.2

Document for:	Discussion

Background

As per [1], RAN4 listed some test cases for PDCCH and PBCH simulation results alignment and interested companies can bring simulation results in this meeting. In this paper, we provide our initial simulation results for PDCCH and PBCH.

Simulation results

PDCCH requirements

The summary of PDCCH simulation results are captured in Table 2-1:

Table 2-1: Summary of PDCCH simulation results

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | SNR (dB)@70% 1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-200 | 1_0 | 40 | 1x2 Low | 2.8 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA30-200 | 1_1 | 56 | 1x2 Low | -0.3 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA30-200 | 1_1 | 56 | 2x2 Low | -3.5 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA30-200 | 1_0 | 40 | 2x2 Low | -6.5 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 0.0 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low | -2.9 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -6.6 |

PBCH requirements

The summary of PBCH simulation results are captured in Table 2-2:

Table 2-2: Summary of PBCH simulation results

| Test number | Bandwidth (MHz) / Subcarrier spacing (kHz) | Propagation condition | Antenna configuration and correlation matrix | SSB index | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNRBB (dB) |
| 2 | 100 / 120 | TDLA30-200 | 1 x 2 Low | Unknown | 1 | -9.0 |
| 3 | 400 / 480 | TDLA10-200 | 1 x 2 Low | Unknown | 1 | -7.2 |

Conclusion

In this paper, we provide the simulation results for PDCCH and PBCH.

Reference

[1]   R4-2210665 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated