3GPP TSG-RAN WG4 Meeting #104b-e              	 R4-2217415

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.4.2.1

Source:	Samsung

Title:	Simulation results for PUSCH demodulation requirement for Rel-17 coverage enhancement

Document for:	Discussion

# 1	Introduction

In the previous RAN4 meeting, it was agreed to introduce PUSCH demodulation requirement with TBoMS and JCE. The detail simulation assumptions for each requirement were discussed to facilitate the requirement definition. The related agreement was captured into the WF [1] as

In this contribution, the view on the remaining issue of test setup of PUSCH requirement for Rel-17 coverage enhancement was provided. Meanwhile, the updated ideal and impairment results are provided

# 2	PUSCH enhancements

## 2.1 PUSCH with Joint Channel Estimation

During the last meeting, the remaining issue for the number of HARQ process is under discussion. Different number of HARQ process will have some impact on the delay of re-transmission(s) for each process. Considering the agreed channel model for JCE simulation is low speed scenario. Then, the impact on PUSCH requirement with different number of HARQ process is minor. We are fine to not specify the specific value of HARQ process in the specification, the value is up to BS implementation and scheduling.

Proposal 1: No limitation for number of HARQ process for PUSCH JCE requirement

# 3	Simulation results

In this section, based on the agreed simulation assumption in the last meeting, the updated simulation results are provided for requirement derivation.

Table 1. Ideal simulation results for PUSCH with JCE for 1T2R

| Case | SCS | slot | BW | Mapping type | Tx/Rx | DMRS | MCS | Channel | Length | SNR(70%) (ideal) | SNR(70%) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T2R | 1 +0 (2) | MCS4 | TDLA30-10 | 14 | -11.2 | -9.2 |
| 2 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T2R | 1 +1 (2, 11) | MCS4 | TDLA30-10 | 14 | -11.7 | -9.7 |
| 3 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T2R | 1+0 (0) | MCS4 | TDLA30-10 | 14 | -11.2 | -9.2 |
| 4 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -11.6 | -9.6 |
| 5 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -11.6 | -9.6 |
| 6 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T2R | 1 +1 (2, 11 ) | MCS4 | TDLA30-10 | 14 | -12.0 | -10 |
| 7 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -11.6 | -9.6 |
| 8 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -12.0 | -10 |
| 9 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -5.8 | -3.8 |
| 10 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T2R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -6.2 | -4.2 |
| 11 | 30KHz SCS TDD(7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -5.8 | -3.8 |
| 12 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -6.2 | -4.2 |
| 13 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -5.7 | -3.7 |
| 14 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T2R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -6.1 | -4.1 |
| 15 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -5.7 | -3.7 |
| 16 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -6.1 | -4.1 |
| 17 | 60KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 50MHz | Type B | 1T2R | 1 +1 (0,10) | MCS4 | TDLA30-75 | 14 | -6.3 | -4.3 |
| 18 | 120KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 50MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-75 | 14 | -6.1 | -4.1 |

Table 2. Ideal simulation results for PUSCH with JCE for 1T4R

| Case | SCS | slot | BW | Mapping type | Tx/Rx | DMRS | MCS | Channel | Length | SNR (70%) | SNR (70%) impairment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T2R | 1 +0 (2) | MCS4 | TDLA30-10 | 14 | -14.5 | -12.5 |
| 2 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T2R | 1 +1 (2, 11) | MCS4 | TDLA30-10 | 14 | -15.0 | -13.0 |
| 3 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T2R | 1+0 (0) | MCS4 | TDLA30-10 | 14 | -14.5 | -12.5 |
| 4 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -15.0 | -13.0 |
| 5 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -14.7 | -12.7 |
| 6 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T2R | 1 +1 (2, 11 ) | MCS4 | TDLA30-10 | 14 | -15.0 | -13.0 |
| 7 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -14.7 | -12.7 |
| 8 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -15.0 | -13.0 |
| 9 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -9.0 | -7.0 |
| 10 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T2R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -9.3 | -7.3 |
| 11 | 30KHz SCS TDD(7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -9.0 | -7.0 |
| 12 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -9.3 | -7.3 |
| 13 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T2R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -9.0 | -7.0 |
| 14 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T2R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -9.3 | -7.3 |
| 15 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T2R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -9.0 | -7.0 |
| 16 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T2R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -9.3 | -7.3 |

Table 3. Ideal simulation results for PUSCH with JCE for 1T8R

| Case | SCS | slot | BW | Mapping type | Tx/Rx | DMRS | MCS | Channel | Length | SNR(70%) (ideal) | SNR(70%) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T8R | 1 +0 (2) | MCS4 | TDLA30-10 | 14 | -17.3 | -15.3 |
| 2 | 15KHz SCS FDD | 8 slots | 5MHz | Type A | 1T8R | 1 +1 (2, 11) | MCS4 | TDLA30-10 | 14 | -18.1 | -16.1 |
| 3 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T8R | 1+0 (0) | MCS4 | TDLA30-10 | 14 | -17.3 | -15.3 |
| 4 | 15KHz SCS FDD | 8 slots | 5MHz | Type B | 1T8R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -18.1 | -16.1 |
| 5 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T8R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -17.4 | -15.4 |
| 6 | 30KHz SCS FDD | 8 slots | 10MHz | Type A | 1T8R | 1 +1 (2, 11 ) | MCS4 | TDLA30-10 | 14 | -18.1 | -16.1 |
| 7 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T8R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -17.4 | -15.4 |
| 8 | 30KHz SCS FDD | 8 slots | 10MHz | Type B | 1T8R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -18.1 | -16.1 |
| 9 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T8R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -11.8 | -9.8 |
| 10 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type A | 1T8R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -12.4 | -10.4 |
| 11 | 30KHz SCS TDD(7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T8R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -11.8 | -9.8 |
| 12 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 10MHz | Type B | 1T8R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -12.4 | -10.4 |
| 13 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T8R | 1 +0 (2,) | MCS4 | TDLA30-10 | 14 | -11.8 | -9.8 |
| 14 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type A | 1T8R | 1 +1 (2,11 ) | MCS4 | TDLA30-10 | 14 | -12.5 | -12.1 |
| 15 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T8R | 1+0 (0,) | MCS4 | TDLA30-10 | 14 | -11.8 | -9.8 |
| 16 | 15KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5MHz | Type B | 1T8R | 1+1 (0,10) | MCS4 | TDLA30-10 | 14 | -12.5 | -10.5 |

Table 4. Ideal simulation results for PUSCH with TBoMS for 1T2R (to be updated)

| Case | SCS | slot | PRB | Mapping type | Tx/Rx | DMRS | MCS | Channel | Length | SNR(70%) (ideal) | SNR(70%) (impair) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz SCS FDD | 4 slots | 5 PRB | Type A | 1T2R | 1 +1 (2,11) | MCS2 | TDLB100-400 | 14 | -3.9 | -1.9 |
| 2 | 15KHz SCS FDD | 4 slots | 5 PRB | Type B | 1T2R | 1 +1 (0, 10) | MCS2 | TDLB100-400 | 14 | -3.7 | -1.7 |
| 3 | 30KHz SCS FDD | 4 slots | 5 PRB | Type A | 1T2R | 1 +1 (2,11) | MCS2 | TDLB100-400 | 14 | -4.3 | -2.3 |
| 4 | 30KHz SCS FDD | 4 slots | 5 PRB | Type B | 1T2R | 1 +1 (0, 10) | MCS2 | TDLB100-400 | 14 | -4.3 | -2.3 |
| 5 | 15KHz SCS TDD (3D1S1U, S=10D:2G:2U) | 2 slots | 5 PRB | Type A | 1T2R | 1 +1 (2,11) | MCS2 | TDLB100-400 | 14 | -3.8 | -1.8 |
| 6 | 15KHz SCS TDD (3D1S1U, S=10D:2G:2U) | 2 slots | 5 PRB | Type B | 1T2R | 1 +1 (0, 10) | MCS2 | TDLB100-400 | 14 | -3.7 | -1.7 |
| 7 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5 PRB | Type A | 1T2R | 1 +1 (2,11) | MCS2 | TDLB100-400 | 14 | -4.2 | -2.2 |
| 8 | 30KHz SCS TDD (7D1S2U, S=6D:4G:4U) | 2 slots | 5 PRB | Type B | 1T2R | 1 +1 (0, 10) | MCS2 | TDLB100-400 | 14 | -4.1 | -2.1 |
| 9 | 60KHz SCS TDD (3D1S1U, S=10D:2G:2U) | 2slots | 5 PRB | Type B | 1T2R | 1 +1 (0,8) | MCS2 | TDLA30-300 | 10 | -4.5 | -2.5 |
| 10 | 120KHz SCS TDD (3D1S1U, S=10D:2G:2U) | 2 slots | 5 PRB | Type B | 1T2R | 1 +1 (0, 8) | MCS2 | TDLA30-300 | 10 | -4.3 | -2.3 |

# 4	Conclusion

In this contribution, the view on the remaining issue of PUSCH requirement is provided. Meanwhile, initial simulation results of PUSCH JCE and PUSCH TBoMS are provided for alignment.

Proposal 1: No limitation for number of HARQ process for PUSCH JCE requirement

# References

Email discussion summary for coverage enhancement, China Telecomm