3GPP TSG-RAN WG4 Meeting #104-Bis-e												R4-2217732

Electronic Meeting, Oct. 10 – 19, 2022

Agenda item:			6.8.4

Source:	Apple

Title:	WF on System parameter assumption, UE architecture and test setup

Document for:	Information

# 1 System parameter assumption and UE architecture

## 1.1	On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO”

For UEs supporting single DCI, assume the following set of UE capabilities as pre-requisites:

- Support of simultaneousReceptionDiffTypeD-r16

- Support of singleDCI-SDM-scheme-r16

- Support of 4L DL MIMO

For UEs supporting multi-DCIs, assume the following set of UE capabilities as pre-requisites:

- Support of simultaneousReceptionDiffTypeD-r16

- Support of multiDCI-MultiTRP-r16

- Support of 4L DL MIMO

FFS if and how RAN4 aims to specify the same RF requirements to accommodate both types of UEs

Further consider the following options:

- Option 1: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used.

- Option 2: UE RF requirements shall not preclude UEs that do not support the single DCI scheme.

### 1.2	On using legacy beam management reference signal signaling for this feature

For core requirement and testing, it is assumed that both SSB and CSI-RS are configured to UE for beam management

### On UE panel assumption

UE panel assumption should follow implementation agonistic manner.

- FFS on how to determine a minimum benefit that a UE must demonstrate when configured for this feature.

# Test setup

### 2.1	On the term “panel” used in test configuration

The term “panel” is not used in test configuration.

### On relation between testing and core requirement

Requirement discussions need to consider testability issue so that the defined requirement can be properly verified.