3GPP TSG-RAN WG4 Meeting #104-bis-e                           		R4-2217737

Electronic Meeting, 10 October– 19 October, 2022

Source: 	Huawei, HiSilicon

Title: 	TP for TR 38.876 to capture some agreements on ATG coexistence study

Agenda Item:	6.13.2

Document for:	Approval

# 1. Introduction

In RAN4#104 meeting, RAN4 reached some agreements on the ATG co-existence study based on the WF [1]. In this paper, we’d like to capture these agreements into this TR.

# 2.	References

[1] R4-2214459, WF on co-existence evaluation for ATG, Huawei, HiSilicon

# Text proposal for TR 38.876

<<Start of TP for TR 38.872>>

6	Co-existence study

6.1 Co-existence simulation scenario

Table 6.1-1 summarizes the initial simulation scenarios for ATG coexistence study. Scenarios (No. 1, 2, 3, 4, 9, 10, 11 and 12) can be studied in phase 1.

Table 6.1-1: Simulation scenarios for ATG coexistence study

| No. | Combination | Aggressor |  | Victim |  | Simulation frequency | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | deployment scenario UL/DL | CBW duplex mode | deployment scenario UL/DL | CBW duplex mode |  |  |  |
| 1 | TN with ATG | ATG DL | 100MHz TDD | TN rural DL | 100MHz /TDD | 3.5 GHz |  | Phase 1 |
| 2 | TN with ATG | ATG UL | 100MHz TDD | TN rural UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 3 | TN with ATG | TN rural DL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 4 | TN with ATG | TN rural UL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | Phase 1 |
| 5 | TN with ATG | ATG DL | 100MHz TDD | TN rural UL | 100MHz /TDD | 3.5GHz |  | FFS |
| 6 | TN with ATG | ATG UL | 100MHz TDD | TN rural DL | 100MHz TDD | 3.5GHz |  | FFS |
| 7 | TN with ATG | TN rural DL | 100MHz TDD | ATG UL | 100MHz TDD | 3.5GHz |  | FFS |
| 8 | TN with ATG | TN rural UL | 100MHz TDD | ATG DL | 100MHz TDD | 3.5GHz |  | FFS |
| 9 | TN with ATG | ATG DL | 20MHz FDD | TN rural DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 10 | TN with ATG | ATG UL | 20MHz FDD | TN rural UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 11 | TN with ATG | TN rural DL | 20MHz FDD | ATG DL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 12 | TN with ATG | TN rural UL | 20MHz FDD | ATG UL | 20MHz FDD | 2 GHz |  | Phase 1 |
| 13 | TN with ATG | ATG UL | 20MHz FDD | TN rural DL | 20MHz TDD | 2 GHz | n1/n39 | FFS |
| 14 | TN with ATG | TN rural DL | 20MHz TDD | ATG UL | 20MHz FDD | 2 GHz | n39/n1 | FFS |

6.2 Co-existence simulation assumption

6.2.1 Network layout model

6.2.1.1 Co-existence between ATG and NR terrestrial network

6.2.2 System parameters

6.2.2.1 ATG parameters

The initial system parameters for ATG BS is assumed as below.

Table 6.2.2.1-1: system parameters for ATG BS

| ATG BS altitude | 30m |
| --- | --- |
| Carrier frequency | 2GHz, 4GHz |
| Frequency reuse factor | 1 |
| Duplex mode | FDD@2GHz, TDD@4GHz |
| Channel bandwidth | 20MHz@2GHz, 100MHz@4GHz |
| Subcarrier spacing (SCS) | 15k@2GHz, 30k@4GHz |
| Number of cells | TBC |
| Environment1 | Rural |
| UE distribution | TBC |
| Indoor UE percentage | 0% |
| Number of DL active UEs per cell (NOTE 2) | 1 |
| Number of UL active UEs per cell (NOTE 2) | 1 |
| DL scheduled bandwidth per UE | 1 |
| UL scheduled bandwidth per UE | 1 |
| Traffic model | Full buffer |
| ATG BS maximum output power | FFS |
| ATG BS noise figure | 5dB |
| Handover margin | 3dB |
| NOTE 1: ATG BS is assumed to serve UEs in the rural environment. NOTE 2: Same as the number of BS beam(s); NOTE 3: ATG BS max TX power is defined per polarization |  |

6.2.2.2 ATG UE parameters

The system parameters for ATG UE is assumed as below.

Table 6.2.2.2-1: system parameters for ATG UE

| ATG UE altitude | UE altitude (upper boundary): 10km UE altitude (lower boundary): 3~7km |
| --- | --- |
| Carrier frequency | 2, 4GHz |
| ATG UE max TX power in dBm | FFS |
| ATG UE min TX power in dBm | FFS |
| ATG UE noise figure | FFS |

6.2.2.3 TN BS and UE parameters

The system parameters for TN BS and TN UE is assumed as below.

Table 6.2.2.3-1: system parameters for TN BS and UE

| Parameters | Rural | Rural |  |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz | 4GHz |  |
| Channel bandwidth | 20MHz | 100MHz |  |
| Scheduled channel bandwidth per UE (DL) | 1 | 1 |  |
| Scheduled channel bandwidth per UE (UL) | 3 | 3 |  |
| The number of active UE (DL) (Note 1) | 1 | 1 |  |
| The number of active UE (UL) (Note 1) | 3 | 3 |  |
| Traffic model | full buffer | full buffer |  |
| DL power control | No | No |  |
| UL power control | Yes | Yes |  |
| TN BS-UE min distance in meters | 30m | 30m |  |
| TN BS max TX power in dBm | FFS | FFS |  |
| TN UE max TX power in dBm | 23dBm | 23dBm |  |
| TN UE min TX power in dBm | -40dBm | -40dBm |  |
| TN BS Noise figure in dB | 5dB | 5dB |  |
| TN UE Noise figure in dB | 9dB | 9dB |  |
| Handover margin | 3dB | 3dB |  |
| Note 1 Same as the number of BS beam(s); Note 2: TN BS max TX power is defined per polarization |  |  |  |

6.2.3 Antenna and beamforming pattern modelling

6.2.3.1 ATG BS antenna model

6.2.3.2 ATG UE antenna model

6.2.3.3 TN BS antenna model

6.2.3.4 TN UE antenna model

The following assumption for TN UE antenna is shown as below.

Table 6.2.3.4-1: TN handheld UE antenna charateristic

| Characteristics | Handheld |
| --- | --- |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Tx/Rx Antenna gain | 0 dBi per element |
| the number of Tx and Rx | 1T2R |

6.2.4 ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL.

Therefore, it is assumed that both ACLR ( or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as:

(Assuming ACLR, ACS and ACIR to be linear).

6.2.5 Propagation model

6.2.5.1 Propagation model between TN UE and ATG UE

Referring to section 6.6 in TR 38.811, the propagation model between TN UE and ATG UE can be simplified and summarized as below.

LOS probability

Line-Of-Sight (LOS) probability depends on UE environment and elevation angle, and is obtained from Table 6.2.5.1-1. Reference elevation angles are considered from 10° to 90° with a 10° step. For an ATG UE-to-ATG BS, the LOS probability is taken from the nearest reference elevation angle.

Table 6.2.5.1-1 LOS probability

| Elevation | Suburban and Rural scenarios |
| --- | --- |
| 10° | 78.2% |
| 20° | 86.9% |
| 30° | 91.9% |
| 40° | 92.9% |
| 50° | 93.5% |
| 60° | 94.0% |
| 70° | 94.9% |
| 80° | 95.2% |
| 90° | 99.8% |

Path loss and Shadow fading

The signal path between ATG UE and ATG BS undergoes several stages of propagation and attenuation. The path loss (PL) is composed of components as follows:

PL=PLb=FSPLd, fc+SF+CLα, fc,	(6.2.5.1-1)

where	is the total path loss in dB,

is the basic path loss in dB,

This section specifies the basic path loss model () which accounts for the signal's free space propagation, clutter loss, and shadow fading.

The free space path loss (FSPL) in dB for a separation distance d (between ATG UE and ATG BS) in meter and frequency fc in GHz is given by

(6.6-2)

is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~. When the UE is in LOS condition, clutter loss is negligible and should be set to 0 dB in the basic path loss model.

Table 6.2.5.1-2: Shadow fading and clutter loss for suburban and rural scenarios

| Elevation | 2GHz 3.5GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | (dB) | (dB) | (dB) |
| 10° | 1.79 | 8.93 | 19.52 |
| 20° | 1.14 | 9.08 | 18.17 |
| 30° | 1.14 | 8.78 | 18.42 |
| 40° | 0.92 | 10.25 | 18.28 |
| 50° | 1.42 | 10.56 | 18.63 |
| 60° | 1.56 | 10.74 | 17.68 |
| 70° | 0.85 | 10.17 | 16.50 |
| 80° | 0.72 | 11.52 | 16.30 |
| 90° | 0.72 | 11.52 | 16.30 |

6.2.5.2 Propagation model between TN BS and TN UE

Referring to section 7.4 in TR 38.901, the propagation model between TN BS and TN UE can be summarized as below, which is same as RMa scenario.

Pathloss:

The pathloss models are summarized in Table 6.2.5.2-1 and the distance definitions are indicated in Figure 6.2.5.2-1 and Figure 6.2.5.2-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 6.2.5.2-1.

| Figure 6.2.5.2-1: Definition of d2D and d3D for outdoor UTs | Figure 6.2.5.2-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs. |
| --- | --- |

Note that

(7.4-1)

Table 6.2.5.2-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 6 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| RMa | LOS | , see note 5 |  | h = avg. building height W = avg. street width The applicability ranges: |
|  | NLOS | for |  |  |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by , where . Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: PLUMi-LOS = Pathloss of UMi-Street Canyon LOS outdoor scenario. Note 5: Break point distance dBP = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0 108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively. Note 6: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 6.2.5.2-2.

Table 6.2.5.2-2 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| RMa |  |

O2I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

(6.2.5.2-2)

where  is the basic outdoor path loss given in Clause 6.2.5.2, where  is replaced by  .  is the building penetration loss through the external wall,  is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

is characterized as:

(6.2.5.2-3)

is an additional loss is added to the external wall loss to account for non-perpendicular incidence; , is the penetration loss of material i, example values of which can be found in Table 7.4.3-1;  is proportion of i-th materials, where ; and N is the number of materials.

Table 6.2.5.2-3: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass |  |
| IRR glass |  |
| Concrete |  |
| Wood |  |
| Note: f is in GHz |  |

Table 6.2.5.2-4 gives ,  and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated, and is added to the SF realization in the log domain.

Table 6.2.5.2-4: O2I building penetration loss model

|  | Path loss through external wall: in [dB] | Indoor loss: in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model |  | 0.5 | 4.4 |
| High-loss model |  | 0.5 | 6.5 |

is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UMa and UMi-Street Canyon, and between 0 and 10 m for RMa.  shall be UT-specifically generated.

Both low-loss and high-loss models are applicable to UMa and UMi-Street Canyon.

Only the low-loss model is applicable to RMa.

Only the high-loss model is applicable to InF.

6.2.5.3 Propagation model between ATG BS and TN BS

The propagation model between ATG BS and TN BS is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1.

6.2.5.4 Propagation model between ATG BS and TN UE

The propagation model between ATG BS and TN UE is same as the propagation model between TN BS and TN UE in clause 6.2.5.2.

6.2.5.5 Propagation model between TN BS and ATG UE

The propagation model between TN BS and ATG UE is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1.

6.2.5.6 Propagation model between ATG BS and ATG UE

The propagation model between ATG BS and ATG UE is same as the LOS propagation model between TN UE and ATG UE in clause 6.2.5.1.

6.2.6 Transmission power control model

6.2.6.1 TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = -40dBm,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1 For uplink scenario.

6.2.6.2 TN DL TPC

For downlink scenario, no power control scheme is applied.

6.2.6.3 ATG UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 is applied for TN with following parameters.

where:

-	Pmax = TBD dBm,

-	Rmin = TBD dBm,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + (-174+F+10*log(B)) )

-	SNRtarget = TBD, the target UL SNR.

-    F is BS noise figure (dB).

-    B is UL transmission BW (Hz)

-	γ = 1 for uplink scenario.

6.2.6.4 ATG DL TPC

For downlink scenario, no power control scheme is applied.

6.2.7 Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

6.2.8 Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For ATG, the average throughput loss and 5%-ile throughput loss should be less than 5%.

6.2.9 Link level performance for NR ATG coexistence

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR< SNIRMIN                                        ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

-	S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz

-					Attenuation factor, representing implementation losses

-	SNIRMIN		Minimum SNIR of the code set, dB

-	SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 4.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 4.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 6.2.9-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 4.2.7-1 are targeted for eMBB coexistence scenario.

6.3 Co-existence simulation methodology

6.4 Co-existence simulation results

<<End of TP for TR 38.872>>