3GPP TSG-RAN WG4 Meeting #104-Bis-e												R4-2217785

Electronic Meeting, Oct. 10 – 19, 2022

Agenda item:			6.8.4

Source:	Moderator (Apple)

Title:	Email discussion summary for [104-bis-e][133] FR2_multiRx_UERF_part2

Document for:	Information

# Introduction

This email thread treats the following two topics:

- System parameter assumption and UE architecture (6.8.2.1)

- Test setup (6.8.2.2)

As some contributions discuss issues that fall into email thread [104-bis-e][132] FR2_multiRx_UERF_part1, moderator has coordinated with moderator of thread [104-bis-e][132] FR2_multiRx_UERF_part1 to make sure all issues are being treated while minimizing the overlap between the two threads.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| vivo | Hao Du | duhao.txyjy@vivo.com |
| Anritsu | Hassen Chouli | hassen.chouli@anritsu.com |
| Verizon | Zheng Zhao | zheng.zhao@verizonwireless.com |
| LG Electronics | Yoonoh Yang | yoonoh.yang@lge.com |
| Murata | Hidefumi Ohira | hidefumi.ohira@murata.com |
| Xiaomi | Juan Zhang | zhangjuan8@xiaomi.com |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| Huawei | Xiang Gao | gaoxiang74@huawei.com |
| Samsung | Bozhi Li | bozhi.li@samsung.com |
| Rohde & Schwarz | Niels Petrovic | Niels.petrovic@rohde-schwarz.com |
| Sony | Kun zhao | Kun.1.zhao@sony.com |
| Qualcomm | Sumant Iyer | sumanti@qti.qualcomm.com |
| Ericsson | Christian Bergljung | Christian.Bergljung@ericsson.com |
| Keysight | Thorsten Hertel | Thorsten.Hertel@keysight.com |
| Lenovo | Colin Frank | colinfrank@motorola.com |
| Apple | Steven Chen | steven.x.chen AT apple.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: System parameter assumption and UE architecture

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215579 | Nokia, Nokia Shanghai Bell | Proposal 1: Consider both single-beam and multi-beam operations on one or more panels in defining the RF requirements and test for FR2-1 UE multi-Rx chain DL reception. Proposal 2: The concept of panel should not be explicitly used in core requirements and test configurations for FR2-1 UE multi-Rx chain DL reception. Proposal 3: The single panel with single-beam operation or multi-beam operation should not be excluded for FR2-1 UE multi-Rx chain DL reception. |
| R4-2215620 | Apple | Proposal 1: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used. Proposal 2: A panel is the hardware that consists of antenna array and the associated transceiver and BB unit that a UE uses to produce a TX/RX beam pointing to a particular direction in the spatial domain. From RX perspective, a panel is used to receive one and only one AoA or TCI state. Proposal 3: The concept of panel should not be explicitly used in core requirements and test configurations. Proposal 4: The scenario where a single panel is used to receive two AoAs should not be considered. Proposal 5: The following typical implementation options are considered in developing requirements: • Two panels having equal beamforming capabilities and non-overlapping scanning range • Two panels having unequal beamforming capabilities and non-overlapping scanning range • Two panels having equal beamforming capabilities and overlapping scanning range • Two panels having unequal beamforming capabilities and overlapping scanning range. |
| R4-2215701 | Samsung | Observation 1: FR2 multi-RX chain DL is featured as simultaneous DL reception from different TRPs in different directions Observation 2: It is not typical scenario for FR2 4 layer MIMO if UE is very far from TRP1 and very close to TRP2 due to large PSD difference Proposal 1: RAN4 to discuss “distance to TRPs” as one of system assumptions, and correspondingly a moderate PSD difference configuration in core requirement is expected. Observation 3: “different direction” implicitly indicates that there should be an applicable angle separation Observation 4: angle separation is needed to address not only core requirement issues but also testability issues Proposal 2: RAN4 to discuss “angle separation” as one of system assumptions, and correspondingly the requirements for FR2 multi-RX chain DL do not apply when angle separation smaller than a minimum threshold. Proposal 3: the concept of panel should not be explicitly used in core requirements and test configurations. Proposal 4: single panel implementation should not be excluded. Proposal 5: UE panel assumption should follow implementation agonistic manner. |
| R4-2215778 | Murata | Observation 1: There was no clear view how the single panel meets the requirement. Observation 2: In the WID in RAN plenary, multi-RX was assumed providing RF spherical coverage improvement. Proposal 1: We do not consider the single panel in the discussion. Observation 3: We do not have clear view for future technology, and we should not limit implementation by specifications. Proposal 2: We should not exclude single panel in the specification to not limit implementation Observation 4: There seems to be no limitation on relative position among antennas on UE and base stations, so it may be hard selecting some use cases under the implementation flexibility in reality. Observation 5: From the viewpoint of antenna, there may be four representative patterns between beam direction and radiation patterns on UE. The target signal is received at main lobe of antenna, and the non-target signal is received below patterns. - The non-target signal is shut out by the formfactor. - The non-target signal is received at null point of antenna. - The non-target signal is received at side lobe of antenna. - The non-target signal is received at main lobe of antenna. Observation 6: The multiplexing correction may make the EIS difference between the representative patterns smaller. This may relate to connection sequence because we need to estimate the channel matrix for this correction. Proposal 3: We suggest checking the difference of EIS between representative beam patterns with multiplexing correction. If we select beam pattern based on worst EIS case and make discussion, spherical coverage in the ordinary use case will be better than our assumption. |
| R4-2216125 | vivo | Observation 1: The beam pattern will be distorted when multiple panels activated simultaneously. Observation 2: The antenna gain of multiple panels activaed simultaneously is not always better than multiple panel switching, no matter for peak or 50% sperhical coverage. Observation 3: The mutual impact between panels is rely on the UE design, and it will significantly exacerbate the development overhead if we try to avoid it. Proposal 1: FFS whether the mutual impact between two panels need to be considered. Proposal 2: Single panel perform dual-polarized MIMO to receive 2-layer data from 2 AoAs is allowed. Proposal 3: Independent RF chain for each polarization should be the baseline for the multi-Rx RF requirement discussion. |
| R4-2216352 | Xiaomi | Proposal 1: The UE RF requirements for the UE supporting simultaneous DL reception with two different QCL TypeD RSs should be UE implementation agnostic, the concept of panel should not be explicitly used in core requirements and test configuration. |
| R4-2216445 | OPPO | Observation 1: It is probably feasible for UE to simultaneously receive FR2 dual-polarized electromagnetic waves from two directions. The side condition for this RF capability is that the antenna elements in the FR2 RF panel can be treated as sub-group to perform phase shift operation for independent beamforming. Proposal 1: The scenario of the single panel to receive the 4-layer DL MIMO should not be excluded when specifying the RF requirement. Proposal 2: The small separation between AoAs should be considered in the scenario of 4-layer DL reception with one single panel. Proposal 3: The minimum separation between the two AoAs for 4-layer DL reception should be specified to guarantee the two AoAs are workable for UE. Proposal 4: The UE should not be required to disclose the work mode of one panel reception or two panel reception for 4-layer DL MIMO. |

Other related contributions

| R4-2216253 | Sony, Ericsson | Observation 1: if the UE supports simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16 the test can be carried our with an RMC with different layers on the two probes, if the latter is not supported, the test is also possible by carried out with an RMC with one layer, the same on the two probes. Observation 2: full set of AoA1+ full set of AoA2 is not feasible from the testability aspect. However, it may still be used to derive the core requirement as it provides an overall assessment of device performance under arbitrary UE orientation and angle of incoming signals. However, a performance gap between the derived requirement and the actual test may appear. Observation 3: More than one AoA1 may need to be selected to ensure the device's performance in real life. In addition, testability issue may appear if one of the AoAs needs to have a fixed relative orientation towards the device while the device needs to be rotated. Proposal 1: The concept of antenna panel should not be explicitly used in core requirements. Proposal 2: RAN4 can discuss how to treat the UE which does not support singleDCI-SDM-scheme-r16 in UE RF test. Proposal 3: If full set AoA1 + full set AoA2 is selected, the performance difference between the derived requirement and the actual test setup defined in the end needs further study. Proposal 4: the testability of having one of the AoA fixed relative to the DUT must be confirmed before selecting this method. Proposal 5: RAN4 shall define the EIS metric for measuring the DL spherical coverage with simultaneous reception under different AoAs setups. One possibility is to use the total EIS from the two directions. Alternatively, the EIS is fixed from one direction at the 50%-ile for the eisting EIS spherical coverage requirement while the EIS spherical coverage is measured on the other. |
| --- | --- | --- |
| R4-2216589 | Huawei, HiSilicon | Observation 1: For the UE which is capable of worse spherical coverage capability, which could be reflected by the composite area, could have better DL receiving performance of the AoA pair with a small angular offset. Observation 2: The mTRP operation’s gain scenario should have the following characteristic: - The power imbalance between the two TRP-UE links is within an acceptable range. Observation 3: The definition of the requirement for simultaneous DL reception from two AoAs is strongly related to the test design. For instance, if the new requirement is still for spherical coverage, then no new RF requirement or test cases need to be introduced. Proposal 1: The concept of panel should not be explicitly used in core requirements and test configurations. Proposal 2: Any type of panel equipment considering reasonable physical limitation on feasibility due to e.g. heat dissipation should not be precluded, as long as such kind of UE implementation can meet the requirement for this feature. Proposal 3: The composite area can be introduce to distinguish the UE with different spherical coverage performance. - Within the composite area, when such UE is configured with simultaneous DL reception from 2 AoAs, shall achieve better EIS performance than the situation that it is not configured with this feature. - The UE with wider composite area could has worse spherical coverage performance. Proposal 4: Further consider how to accommodate the UE with different composite area for the derivation of RF requirements for simultaneous DL reception from two AoAs. Proposal 5: Identify the gain scenarios for m-TRP operation first, which is beneficial for multiple aspects like the discussion on RF/RRM/Demod requirements and the test design accordingly. Proposal 6: Consider the following two options to acquire the angular offset between two AoAs under the gain scenarios for m-TRP operation: - Alt. 1: Send an LS to RAN1 for asking more background info at least about: - All necessary SLS assumptions to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on. - The valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS. - Alt. 2: Align all necessary SLS assumptions within RAN4 to support m-TRP operation with up to 4 layers, like network topology, UE distribution and so on. Then find the valid range of angular offset between 2 AoAs so that obvious gain can be observed for enabling multi-panel simultaneous reception from different QCL Type-D RS. Proposal 7: For the RF requirements to support simultaneous DL reception from two AoAs, the range of power imbalance between two TRP-UE links shall be considered as side condition and should be further discussed. Proposal 8: Taking the current 50%-tile EIS spherical coverage requirement (for PC3) for single band as the baseline, the new spherical coverage requirement shall be further discussed at least considering the following system factors: - angular offset between two AoAs - reception power imbalance of the DL signals between two TRP-UE links Proposal 9: Clarify that the test design for verification of the RF requirement for simultaneous reception from 2 AoAs shall be based on single-layer reception for each DL direction with dual TCI configuration, i.e., total 2 layers for both directions. Proposal 10: Hold the discussion on test design in this WI and the SI for FR2 OTA before achieving concrete conclusion on RF requirement for setting UE RF requirement for simultaneous reception from 2 AoAs. |
| R4-2216786 | Qualcomm | On beam management: Proposal 1: The UE uses Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16 to convey to the network what QCL-D reference signal it can support for the multi-chain Rx feature. Proposal 2: For this feature, the TE configures 2-port CSI-RS QCL-D reference signals from each TRP when CSI-RS is required by the UE for beam management. On the UE RF requirement: Proposal 3: The RF requirement for any AoA pair is defined with assumption that TRP1 uses  polarization when TRP2 uses  polarization and vice-versa ( and  are the angular coordinates of the test system grid). Proposal 4: The RMC used for the proposed UE RF requirement for the 2-AoA rank 2 DL is shared with the RMC referenced in clause 7.3.2.1 of TS38.101-2 (single CC REFSENS) Proposal 5: The nominal or 100% throughput condition for the proposed UE RF requirement for the 2-AoA rank 2 DL is twice the maximum throughput determined for the REFSENS requirement for the single CC case for that band. Proposal 6: For each test point (AoA pair), the individual DL powers from each TRP are set in a ratio that enables a balanced sensitivity condition, where ‘balance’ refers to equal SNR metrics per TRP. Proposal 7: For a balanced sensitivity condition, sensitivity is the linear sum (mW domain) of the DL power levels at the UE location. On UE capability pre-requisites: Proposal 8: Support for simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16, along with support for 4L DL are pre-requisites for the UE to support ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier’. |
| R4-2215781 | LGE | Proposal 1: Consider 2 panels as baseline for UE RF requirements for enhanced FR2-1 UEs. Proposal 1-1: Consider which placement can be baseline for UE RF requirements among {Back2Back, Orthogonal, In-line} placement. Proposal 2: Do not define the concept of panel in UE RF core requirements. Proposal 3: Consider diversity gain of ‘0dB’ for EIS requirements of enhanced FR2-1 UEs. Proposal 4: Consider K sample(s) in the legacy spherical coverage of 50%-xile in one panel and all samples in other panel for evaluating CDF of multi-Rx. Proposal 4-1: Assume all K sample(s) to be selected at same point of CDF 50%-xile considering lowest received power. |

## Open issues summary

### On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO”

Issue 1-1-1: Is it agreeable: Support for simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16, along with support for 4L DL are pre-requisites for the UE to support ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier. (Qualcomm)

| Company | Comments |
| --- | --- |
| vivo | Ok with the proposal |
| Xiaomi | I think Supporting for simultaneousReceptionDiffTypeD-r16 is the pre-requisite for the UE to support ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier, since UE supporting ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier doesn’t means the UE must support 4L DL, it can only support 2L under supporting ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier for single DCI based FDM. Therefore, I thinks the proposal should modified as Support for simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16 are pre-requisites for the UE to support ‘simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4layer DL MIMO’ |
| Nokia | What about if the UE only support multi-DCI scheme as pointed out in issue 1-1-2? |
| Huawei | Not support. We don’t think the above mentioned two Rel-16 UE capabilities should be involved as the pre-requisite for this Rel-18 feature. |
| Samsung | We don’t think it necessary to exclude multi DCI for this 4 layer DL MIMO feature. |
| Sony | First, we agree that to support 2layer DL transmission (one layer per direction), simultaneousReceptionDiffTypeD-r16 and singleDCI-SDM-scheme-r16 are pre-requisites. Considering the scope of UE RF that we have agreed in last meeting, we are fine to focus on this case. On the other hand, we would also like to point out that singleDCI-SDM-scheme-r16 is not a mandatory UE capability. Therefore, there is case that UE only support simultaneousReceptionDiffTypeD-r16, and this can be tested by single layer test setup with two AoAs. This case should not be precluded as it is within the scope of the WI (up to 4 layers). We can take this case in the later stage of the WI once we have the case above in a stable shape. We believe most of test setup can be re-used but maybe with different RMC and requirements. |
| Qualcomm | We are the proponents. The idea is not to exclude mDCI. The proposal is just a logical extension of the agreement made in RAN4#104 that ‘(f)or setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline’ |
| Ericsson | The proposal is agreeable. UEs only supporting simultaneousReceptionDiffTypeD-r16 (support of SDM is optional) can be verified with an RMC with 1L (same on both probes). |
| Apple | If we assume that in order to support this feature, there is a RAN1 agreement that a UE has to support “singleDCI-SDM-scheme-r16,” we think this proposal is agreeable. However, given that there is no such agreement, we would like to have a bit discussion on whether a UE can support this feature by supporting the following capabilities as an alternative: - Support of simultaneousReceptionDiffTypeD-r16 - Support of multiDCI-MultiTRP-r16 Support of 4 DL MIMO layers through some signaling (still under RAN1 discussion) |

Issue 1-1-2: Is it agreeable: For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used. (Apple)

| Company | Comments |
| --- | --- |
| vivo | Generally OK, but if multi-DCI also need to be considered, some RF requirement discussions are also need to be involved, e.g., RMC. |
| Verizon | We do not agree single DCI scheme will be only defined in this Rel-18 work and don’t see a real reason to restrict this work to single-DCI! As required in WID, different implementation scenarios should be considered in this work at the UE to enable simultaneous receptions from different directions with different QCL Type-D RSs. The UE RF requirements should support the multi-TRP operation along with both single-DCI and multi-DCI modes, instead of a single DCI only. |
| Xiaomi | It need further clarify whether it means multi-DCI is used baseline for UE RF requirements, if the UE only support multi-DCI scheme, multi-DCI is used I think it is too early to decide there need two kinds of UE RF requirements for single DCI and multiple DCI separately. |
| Nokia | Do we need to cover the case when single DCI is used with single layer when setting the UE RF requirement? |
| Huawei | OK. |
| Samsung | If the common understanding is to diversify the requirements between single DCI and multi DCI, we would like to re-discuss which one should be the baseline between single DCI and multi DCI. We think operator request should be prioritized for the sake of practical deployment. |
| Sony | Generally fine with the proposal. However, we don’t think it is necessary to have separate requirements at this stage. |
| Qualcomm | We do not intend to exclude mDCI either in RAN4 or in the field. We would however like to retain the current agreement ‘sDCI as baseline’ for the sake of setting UE RF requirements. mDCI is supposed to be an enhancement over sDCI, but it brings some behavioral complications (see R4-2212332) that can be better evaluated once the simpler sDCI case is analyzed – this was the original motivation of establishing sDCI as baseline. We are ok to tackle the mDCI case after convergence on sDCI. |
| Apple | It is related to Issue 1-1-1. We think RAN4 should allow UEs only supporting multi-DCI scheme to support this feature. |

### On using legacy beam management reference signal signaling for this feature

Issue 1-2: Is it agreeable: The UE uses Rel-16 IEs beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16 to convey to the network what QCL-D reference signal it can support for the multi-chain Rx feature (Qualcomm)

| Company | Comments |
| --- | --- |
| vivo | The proposal seems a little ambiguous. If UE only support beamCorrespondenceSSB-based-r16, does this means that UE can only use SSB as QCL-D reference signal? In our understanding, the SSB only/ CSIRS only based BC is an optional enhanced feature, and SSB only means UE can meet BC requirement with SSB but doesn’t mean UE can only use SSB as QCL-D reference signal. |
| Xiaomi | Agree with vivo, these two capabilities are optional, if the UE supports neither beamCorrespondenceSSB-based-r16 nor beamCorrespondenceCSI-RS-based-r16, gNB can assume UE support both of SSB and CSI-RS. If UE reports support beamCorrespondenceSSB-based-r16, whether it means network can’t update the QCL-D to CSI-RS for PDSCH and PDCCH. |
| Nokia | Need further clarification on the meaning of this proposal on network operation. |
| Huawei | We would like to hear more about the intention of this proposal. Seems no need to agree anything about what the UE would do about these two optional capabilities? |
| Samsung | We agree that both SSB and CSI-RS can be BMRS, but it seems a little difficult to understand how beam correspondence capability is related with the 4 layer reception only. It is better to configure both SSB and CSI-RS |
| Sony | Fine with the proposal |
| Qualcomm | Our point with the proposal is that no new capabilities are required pertaining to BMRS. After reading the comments above, we think our initial proposal probably drew focus on unintended aspects. Our proposal perhaps can be restated as: ‘no new capabilities are required pertaining to BMRS for this WI’ Also agree with Xiaomi comment that if neither capability referenced in the proposals is supported, the UE would be tested with both CSIRS and SSB. |
| Ericsson | Is the requirement the same for a UE reporting these BC – and beam management – capabilities? What would be configured in the test? |
| Apple | We would like to clarify the intention of this proposal. In our understanding, a UE needs to support the R15 BC capability (bit 1 or bit 0). In addition, the UE can optionally support the two Rel-16 capabilities beamCorrespondenceSSB-based-r16 and beamCorrespondenceCSI-RS-based-r16. Overall, once a UE supports BC, there should be no restriction from the network as to what BMRS to configure, i.e., SSB or CSI-RS. |

### On CSI-RS beam management reference signal type

Issue 1-3: Is it agreeable: For this feature, the TE configures 2-port CSI-RS QCL-D reference signals from each TRP when CSI-RS is required by the UE for beam management. (Qualcomm)

| Company | Comments |
| --- | --- |
| vivo | OK with the proposal |
| Xiaomi | Support the proposal. |
| Nokia | Agreeable. |
| Huawei | Judging from the spec, currently the UE UL report such as RSRP/SINR is per measurement resource rather than per port. Even the UE would find a way to combine the measurement results on 2-ports which is labeled to a 2-port CSI-RS, what is the benefit from TRP perspective? |
| Samsung | For RF requirements, DL diversity for each panel is agreed, we’d like clarification why 2-port RS is needed for diversity reception. |
| Sony | Fine with the proposal |
| Qualcomm | On advantage of 2 port CSI-RS: This was discussed during the FR2 test method enhancement SI and applies here also. From our paper: ‘One significant advantage is that the CSI-RS signal can be configured as a 2-port signal, allowing the UE a much more complete view of the channel, and consequently a better shot at finding the optimal beam(s).’ 2 port CSIRS is better even for the legacy rank1 DL case, because it allows the UE a picture of all 4 elements in the UL channel matrix, rather than just 2. |
| Ericsson | Agreeable |
| Apple | In our understanding, for beam management UE support of 2-port CSI-RS is optional. RAN4 should not have such restriction in the test configuration to preclude UEs not supporting 2-port CSI-RS for beam management. We also appreciate further clarification from proponents. |

### On UE panel assumption

Issue 1-4: Please comment on the following proposal on panel assumptions:

- Proposal 1 (Apple): The following typical implementation options are considered in developing requirements:

  - •	Two panels having equal beamforming capabilities and non-overlapping scanning range

  - •	Two panels having unequal beamforming capabilities and non-overlapping scanning range

  - •	Two panels having equal beamforming capabilities and overlapping scanning range

  - •	Two panels having unequal beamforming capabilities and overlapping scanning range.

- Proposal 2 (Samsung): UE panel assumption should follow implementation agonistic manner.

- Proposal 3 (Murata): From the viewpoint of antenna, there may be four representative patterns between beam direction and radiation patterns on UE. The target signal is received at main lobe of antenna, and the non-target signal is received below patterns.

  - The non-target signal is shut out by the formfactor.

  - The non-target signal is received at null point of antenna.

  - The non-target signal is received at side lobe of antenna.

  - The non-target signal is received at main lobe of antenna.

- Proposal 4 (LGE): Consider 2 panels as baseline for UE RF requirements for enhanced FR2-1 UEs.

- Proposal 4 (LGE): Consider which placement can be baseline for UE RF requirements among {Back2Back, Orthogonal, In-line} placement

| Company | Comments |
| --- | --- |
| vivo | Proposal 2, any implementation should not be excluded only if it can meet the requirement |
| Verizon | Agree Proposal 2, and can use 4 as baseline. |
| LG Electronics | Fine with Proposal 4 and 2. |
| Murata | We need baseline for discussion. We made proposal 3 to clarify how much difference among each implementation assumptions. Proposal 4 may be able to be baseline, but we need further study. In this issue, we wonder whether multi-RX make spatial correction, because this correction may make the difference smaller. We also wonder what the worst use case is. For example, does unequal beamforming capabilities assumes worse specification than that of 2 Legacy UE panels? |
| Xiaomi | Support proposal 2 |
| Nokia | Agree with proposal 2. We consider the important aspect in the UE panel assumption, that all solutions that may comply with the requirements are valid solutions regardless single- or multi-panel solutions. We propose FFS to make a comparison of beam patterns that UE may support and in common come to an agreement across all solutions which are suitable for defining multi-RX chain requirements and tests. |
| Huawei | Option 2. |
| Samsung | We support proposal 2. Implementation agnostic is important. As shown in other proposals, if looking into so many detailed implementation scenarios, it is not possible to finalize requirements with so diversified scenarios. If panel means virtual concept as RAN1 understanding, we think following LGE proposal is also reasonable: - Proposal 4 (LGE): Consider 2 panels as baseline for UE RF requirements for enhanced FR2-1 UEs. |
| Sony | In general, we think the proposals for UE implementation assumption is pending on the panel definition. In any case, some comments are given below: Proposal 1: only two panels are assumed. This may depend on how the panel is defined. For a physical antenna array, it can be connected with one or more RF chain, for example, one RF chain per polarization. If each RF chain is assumed as one panel, then we think we need to assume at least four panels to support 4L DL. Proposal 2: generally fine. However, for single AoA spherical coverage, we had very detail UE implementation assumptions, it is unclear whether we can do the excise in a completely implementation agonistic manner. Proposal 3: In general, we agree with the observation. However, this observation is not only depending on UE implementation but also the AoA setup to our understanding. It might be hard to concluded UE implementation assumption from those observations. Proposal 4 similar comments as proposal 1. If each RF chain is assumed as one panel, then we need at least four panels to be assumed. |
| Qualcomm | General comment: If these assumptions will be used as basis for setting the UE RF requirement, it is important to streamline on one baseline architecture like some have already commented. It is not clear what path should be adopted if we have multiple sets of simulations associated with multiple antenna module distributions on the UE. Establishing a baseline architecture does not mean other architectures are precluded. Proposal 1: Please see our comment to proposal 2. There is some overlap. Proposal 2: If the proposal is about simulation assumptions to define UE RF requirements, this proposal needs further development, please see our general comment above. In its current form, the proposal unfortunately does not limit what may be admitted as a valid implementation. It is easy to construct a cynical UE implementation that would be admitted under this umbrella: A UE has just 2 logical panels on one face of the UE for dual TCI operation. These panels may each provide less sensitivity than say, for a single TCI case where they may be diversity combined in some way to meet the legacy spherical coverage requirements. Furthermore, assume that the implementation only provides one beam choice per panel, and their spatial coverages are overlapping. This kind of UE is expected to have a very small envelope of favorable 4L DL performance, i.e poor if any DL throughput gain over the legacy 2L case. How would we consider data from this type of UE towards UE RF requirement setting? So, some additional criteria must be established before this type of blanket proposal can be agreed. One example of the additional criteria could be ability of a UE to demonstrate some X% better DL throughput with 4L compared to the legacy 2L case in some spatially averaged sense. Proposal 3: This seems more like an observation rather than a proposal, but we agree all these scenarios are possible, possibly others too. Proposal 4: This proposal seems attractive to us from a viewpoint of streamlining the assumptions towards deriving UE RF requirements. |
| Ericsson | The requirement should not make any assumption on the UE implementation (the network should be able to use the capabilities advertised), but a reference architecture is (probably) needed for developing the requirement. |
| Apple | We support proposals 1 and 2. Would like to have more clarifications on proposal 3 whether the four representative patterns are equivalent to UE panel implementations. |

### On requirement to be specified in an implementation-agnostic manner

Issue 1-5: Is it agreeable:  The UE RF requirements for the UE supporting simultaneous DL reception with two different QCL TypeD RSs should be UE implementation agnostic. (Xiaomi)

| Company | Comments |
| --- | --- |
| vivo | Ok with the proposal |
| LG Electronics | Fine with the proposal. |
| Xiaomi | Support this proposal. |
| Nokia | Agreeable. |
| Samsung | Support the proposal. |
| Ericsson | Agreeable |
| Apple | We support the proposal. |

## Companies views’ collection for 1st round

### Open issues

See Section 1.2.

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1.2.1 | Issue 1-1-1: More discussion is needed in the second round to reach an agreement, given the views shared by some companies that UEs only supporting multi-DCI should be considered. Issue 1-1-2: There was good support in the proposal. Together with Issue 1-1-1, RAN4 needs to figure out how to avoid having to specify two different sets of requirements for UEs supporting single DCI and multi-DCI, respectively. Recommendations for 2nd round: Will be discussed in the WF in the second round. |
| Sub-topic #1.2.2 | It seems agreeable that this proposal is not needed. However, since this issue was brought up, it would be good if RAN4 can establish a common understanding on what BMRS needs to be configured for UE to support this feature. Recommendations for 2nd round: Will be discussed in the WF in the second round. |
| Sub-topic #1.2.3 | The issue that UE support of 2-port CSI-RS is optional was brought up. It is better to confirm if this is the case in the second round, and then seek to have some agreement. Recommendations for 2nd round: Will be discussed in the WF in the second round. |
| Sub-topic #1.2.4 | Option 2 received support from most companies. At the same time, there were questions how to specify requirement if no reference architecture or performance criterion is agreed. Recommendations for 2nd round: Will be discussed in the WF in the second round. |
| Sub-topic #1.2.5 | The proposal “The UE RF requirements for the UE supporting simultaneous DL reception with two different QCL TypeD RSs should be UE implementation agnostic” is agreeable. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

### 1.5.1	On UE capabilities for the feature ”simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO”

For UEs supporting single DCI, assume the following set of UE capabilities as pre-requisites:

- Support of simultaneousReceptionDiffTypeD-r16

- Support of singleDCI-SDM-scheme-r16

- Support of 4L DL MIMO

For UEs supporting multi-DCIs, assume the following set of UE capabilities as pre-requisites:

- Support of simultaneousReceptionDiffTypeD-r16

- Support of multiDCI-MultiTRP-r16

- Support of 4L DL MIMO

FFS how RAN4 aims to specify the same RF requirements to accommodate both types of UEs

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support the FFS part in its current form. An alternative for consideration: FFS if RAN4 aims to specify the same RF requirements to accommodate both types of UEs |
| vivo | Both sDCI and mDCI are optional feature, and they do have different performance in the field, so we think it is meaningless to force them have same requirement at this stage. Qualcomm’s proposal is better. |
| LG Electronics | Support 1st & 2nd & 3rd bullets. For 3rd bullet, fine also with QC’s alternative. |
| Sony | In general, we are in favor of having unified requirement for both cases, but since this may need further investigation, we support Qualcomm’s revision on the last point at this stage. |
| Nokia | We may consider defining the same RF requirements (e.g., xx dB EIS with 95 % throughput at yy % CDF) but using different RMCs (each has different maximum throughput) for single-DCI and multi-DCI. |
| Apple | For RF requirement, we think it is possible to have the same requirement for both types of UEs (with some conditions attached.) We can change the FFS part to “FFS if and how RAN4 aims to specify the same RF requirements to accommodate both types of UEs ” |
| Verizon | Same as companies, our target is to have the same RF requirements. This also may need to consider the specific condition as well, just as Nokia suggested example, in which use different RMCs for single-DCI and multi-DCI. |
| Ericsson | We support Qualcomm’s revision of the WF, the requirement need not necessarily be the same for single- and multiple-DCI but a unified requirement may be beneficial. |
| Samsung | Support the WF. Multi-DCI UE should not be excluded implicitly. When agree with ‘baseline’, it is expected that the derived requirements apply for both single DCI UE and multi-DCI UE, if multiple DCI is not considered at the beginning, final requirements are probably not applicable for multiple DCI UE. |

For setting the UE RF requirement when the UE is configured with 2 active TCI states, single DCI scheme is adopted as a baseline, if the UE supports single DCI scheme. If the UE only support multi-DCI scheme, multi-DCI is used.

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support. Have alternative for consideration. We prefer to retain the original agreement on baseline for deriving UE RF requirements. We support creating requirements for mDCI UEs also, but it cannot impact the baseline assumption as proposed above. We cannot change the existing agreement without due diligence on work scope change relative to sDCI assumption. There is also a notion of ‘single set of requirements’ which we are not sure is justifiable. It is not known how the behavior of mDCI UEs without joint demod across both layers will differ from that of sDCI UEs. Consequently, there is insufficient clarity on what other considerations are necessary while define UE RF requirements, like defining ‘exclusion zones’ where the twin AoAs are such that the UE cannot be reasonably expected to demodulate PDSCH. These exclusion zones would depend on beam overlap of the receivers assigned to each TCI state, and this can become very intrusive into the implementation details of each UE. Further, discussion is needed on how to treat the various overlap cases for mDCI, and how requirements will be generated for each. Perhaps companies could study the impact of mDCI assumptions in conjunction with UE RF test assumptions in [132] of this meeting, specifically issue 1.5 in that WF. Alternative proposal to go with existing agreement: UE RF requirements shall not preclude UEs that do not support the single DCI scheme. |
| vivo | After 1st round discussion, we wonder what the “baseline” here means, does this means that if a UE support both sDCI and mDCI capability, only sDCI need to be verified? The sDCI and mDCI are independent optional capabilities and have no clear inclusion relationship, we cannot conclude that mDCI verification can be replaced by the sDCI at this moment. The mDCI is raised in this meeting and we need more time to think about its details, e.g., interference reduction, decoding procedure. So we don’t support this proposal. |
| Sony | We think the baseline should still be singe DCI as we have agreed in the last meeting. Therefore, we prefer the alternative proposal from Qualcomm here. |
| Nokia | We may consider defining the same RF requirements (e.g., xx dB EIS with 95 % throughput at yy % CDF) but using different RMCs (each has different maximum throughput) for single-DCI and multi-DCI. |
| Apple | Given that the WID is not just focused on single DCI and UE support of single DCI is optional, without considering the multi-DCI case, we think the previous agreement needs to be re-discussed. We are also open to discussing what “baseline” means in the previous agreement. |
| Verizon | Both sDCI and mDCI need to be supported in this work! For moving this work forward, we can accept Nokia comment above as a high-level RMC requirement for sDMI and mDMI at this time. Based on more progresses achieved, we can further identify what the possible the various overlap conditions are for the mDCI. |
| Ericsson | The sDCI case should be considered first. |
| Samsung | Support the WF. Multi-DCI UE should not be excluded implicitly. When agree with ‘baseline’, it is expected that the derived requirements apply for both single DCI UE and multi-DCI UE, if multiple DCI is not considered at the beginning, final requirements are probably not applicable for multiple DCI UE. |

### 1.5.2	On using legacy beam management reference signal signaling for this feature

For core requirement and testing, it is assumed that both SSB and CSI-RS are configured to UE for beam management

| Company | Comments |
| --- | --- |
| Qualcomm | Agree: After discussions, we are ok to base the requirement on the Rel-15 understanding of what constitutes the BMRS. It is a scope reduction given that other options exist for BMRS, but we think this scope reduction is beneficial to completion of the WI. |
| Vivo | OK with this proposal. |
| Huawei | Agree. |
| Nokia | Agree |
| Apple | We support. |
| Samsung | Support the WF |

### On CSI-RS beam management reference signal type

As UE support of 2-port CSI-RS for beam management is optional, the TE configures single-port CSI-RS QCL-D reference signals from each TRP when CSI-RS is required by the UE for beam management.

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support. This agreement is not necessary if 1.2 is agreed as ‘Rel-15 BMRS’ |
| Huawei | Support. |
| Nokia | Agree |
| Apple | Even with the agreement of 1.2, it is still worth clarifying how CSI-RS for beam management is configured. |
| Samsung | Support the WF |

### On UE panel assumption

UE panel assumption should follow implementation agonistic manner.

To define requirement, further consider if it is necessary to use

- reference architecture(s)

  - Use two panels as a baseline

- performance criterion

| Company | Comments |
| --- | --- |
| Qualcomm | There seems to be some contradiction between ‘use 2 panels as baseline’ and ‘implementation agnostic’. Perhaps an alternative framing would be: To define requirement, further consider if it is necessary to use - reference architecture(s) - Use two panels as a baseline - performance criterion, UE panel assumption should follow implementation agonistic manner |
| LG Electronics | Fine with this proposal. |
| Sony | A clarification question here, is the sub-bullet are for FFS? If so, the proposal is okay. Otherwise, we think it is premature to decide use two panel as a baseline since we have no agreement on the panel definition yet. |
| Huawei | We are OK for following the implementation agonistic manner, the rest part needs further discussion. |
| Nokia | Ok that UE panel assumption should follow implementation agonistic manner. Other should be FFS. |
| Apple | We support “UE panel assumption should follow implementation agonistic manner.” We are open to further discussion on the following if it is agreeable To define requirement, further consider if it is necessary to use - reference architecture(s) - performance criterion |
| Verizon | We are ok for the Qualcomm proposal. |
| Ericsson | The Qualcomm proposal appears reasonable: a reference architecture can be considered for specification of the requirement. This is normal practice. The specification does not preclude any particular implementation per se (some may not be viable but up to the UE vendor). |
| Samsung | We support “UE panel assumption should follow implementation agonistic manner.” We are okay to further discuss other parts |

### On requirement to be specified in an implementation-agnostic manner

The UE RF requirements for the UE supporting simultaneous DL reception with two different QCL TypeD RSs should be UE implementation agnostic

Moderator: Based on first round discussion, it is agreeable.

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support The agreement that UE RF requirements will not use ‘panel’ in [132] is sufficient. This agreement is not necessary after agreement in GTW on ‘panel’ and UE RF requirements, see first topic in WF in [132]. |
| vivo | In [132], it is agreed that ‘panel’ is not referenced in the final UE RF requirements, and we think this agreement is clear enough. |
| LG Electronic | OK with this proposal based on the agreement in [132]. It seems a duplicated agreement. |
| Huawei | OK but it seems to be unnecessary. |
| Nokia | Not sure what is the different between this and the one in thread [132]. |
| Apple | We support. We would like to point out this proposal and the agreement in [132] “‘panel’ is not referenced in the final UE RF requirements” are not exactly the same. |
| Verizon | We should use one thread for the sufficient agreement. |
| Samsung | Support the WF. We see no confliction and redundancy with the agreement of “‘panel’ is not referenced in the final UE RF requirements” |

# Topic #2: Test setup

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215541 | Keysight | Observation 1: The independent positioning needs of the two probes yield larger system footprint/height, complexity and cost. Observation 2: The test effort for the 4-DL spherical coverage test based on Full Degree of Rotation Freedom for each AoA is tremendous. Observation 3: The test effort for the 4-DL multi-chain spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 is manageable. Proposal 1: A Spherical coverage test with full degree of rotation freedom for each AoA should not be considered due to test time limitations. Proposal 2: Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 with two performance metrics: CDF of EISAoA1 and CDF of the maximum TPAoA2. Proposal 3: Consider a wide range of AoA angular differences between AoA1 (full degree of freedom) and AoA2 (limited, fixed degree of freedom) |
| R4-2215580 | Nokia | Proposal 1: We propose that one or a small number of AoA1 + Full AoA2 are agreed for further test definition and that the main interest in the multi-Rx is AoA2 while AoA1 connection must remain. |
| R4-2216126 | vivo | Observation 1: The coverage performance of multiple panels work simultaneously is not definitely to be better than multiple panels switching. Proposal 1: The selected 2 AoAs only need to locate within the region that meet the single carrier spherical coverage requirement, e.g., top 50% of whole sphere for PC3. Observation 2: Fixing one of the AoAs will cause some panels’ performance not to be verified thoroughly. Observation 3: Fixed offset cannot reduce the test complexity of multi-Rx effectively. Proposal 2: Taking multiple Fixed AoA1+Fullset AoA2 as baseline for further discussion. - FFS on number of AoA1 - FFS on the principle of multiple AoA1 selection. |
| R4-2216444 | OPPO | Proposal 1: It is proposed that when appointing multi AoA1s for Candidate 2, relatively large separation between AoA1s should be adopted, e.g. the separation more than 120 degree. Proposal 2: It is proposed to down-select to Candidate 2 and Candidate 3 for further discussion. |
| R4-2216585 | Anritsu | Observation 1: At some point, the choice of the AoA pair for setting the UE RF requirements should be based on a preselection of the simplest AoA pairs allowing to set requirements that allow guarantying FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. It is understood that “Full set AoA1 + Full set AoA2” is the most complicated AoA pair. Observation 2: Due to test time, test system complexity and test system reuse, refinement of the RF requirements should be looked at such that one AoA pair can be chosen while still guarantying real-life performance of the FR2-1 multi-Rx chain UE for two AoAs simultaneous reception. That decision should happen after the preselection of AoA pairs have been studied properly. Observation 3: The best and final choice of an AoA pair may be influenced by discussions outside the “FR2 multi-Rx chain DL reception” WI ones, such as related as a to RF/RRM/Demodulation including Tx related requirements and may still vary within Rel-18 timeframe. It may be necessary to prepare specific UE RF requirements or one set of UE RF requirements compatible with different plausible AoA pairs. Observation 4: Choice of dedicated FR2 OTA test system configuration may still vary depending on assumptions of test conditions and scenarios for minimum requirements within Rel-18 timeframe. Observation 5: Sequential tests as in Option 6 (Sequential tests by introducing a new test command to fix an active antenna in the DUT) and Option 5 (IFF+ rotating UE and anchor probe as a whole) should be considered. Observation 6: To consider option 6 as equivalent to other test methods, further studies are necessary on the way to process the obtained sequential test results. Proposal 1: Sequential measurement principle should be studied at top level and for different AoA pairs, and conclusion should be made if it can be considered or not. |
| R4-2216787 | Qualcomm | Observation 1: RAN4 needs to determine test set feasibility before finalizing details of the FOM requirement for 2 TCI state spherical coverage. Proposal 1: RAN4 to continue work towards defining UE RF requirements on aspects that are not influenced by test-set design. |

## Open issues summary

### On the term ”panel” used in test configuration

Issue 2-1: Is it agreeable that the term “panel” is not used in test configuration? (Nokia, Apple, Samsung, Xiaomi, Huawei)

| Company | Comments |
| --- | --- |
| vivo | Support. Introducing the “panel” has the risk of restricting the UE implementation and behavior. |
| Verizon | We agree to adopt the definition of panel as used in RAN1 discussions, i.e., it is a radiating structure associated with one TCI-state’ |
| LG Electronics | Support. |
| Nokia | Agreeable. |
| Huawei | Support |
| Samsung | Support. |
| Sony | Fine with the proposal |
| Apple | Agree. |

### On relation between testing and core requirement

Issue 2-2: Please comment on the following proposals/observations

- Proposal 1 (Huawei): Hold the discussion on test design in this WI and the SI for FR2 OTA before achieving concrete conclusion on RF requirement for setting UE RF requirement for simultaneous reception from 2 AoAs..

- Proposal 2 (Qualcomm): RAN4 needs to determine test set feasibility before finalizing details of the FOM requirement for 2 TCI state spherical coverage. RAN4 to continue work towards defining UE RF requirements on aspects that are not influenced by test-set design.

| Company | Comments |
| --- | --- |
| vivo | For proposal 1, the RF requirement design still need to consider the test feasibility otherwise the requirement will be meaningless if it is untestable eventually. For proposal 2, the final detailed test setup/system/procedure should be decided in FR2 OTA SI. |
| Verizon | We agree RAN4 should consider the test feasibility and continue to discuss the detailed requirements |
| LG Electronics | Same view with vivo. |
| Nokia | We see both proposals as correct steps to collect further inputs and information before determining the core and test requirements – details aside. We need to agree on architectures and test capabilities before we can conclude on this issue. |
| Huawei | Actually we think one important principle is mentioned by both proposals, which is trying to decouple the discussion of RF requirements and test design. But till now we can find that these two aspects cannot be easily decoupled and that is the reason for our proposal 1. So insist on this proposal as the proponent. Otherwise we afraid that limited progress can be acquired. |
| Samsung | For proposal 1, more or less, RF requirements are related with test. And actually the OTA SI is also waiting for core requirement progress, we think it will delay the work if we rely on OTA SI. For proposal 2, we agree with principle of this proposal. On the other hand, if test set is not touched, many other requirements discussion is lack of basic understanding. |
| Rohde & Schwarz | Requirement discussions need to consider the testability aspect as well. At least it needs to understood which testing approaches are feasible and how they could be implemented and thus impact the requirements verification. |
| Sony | Proposal 2 is preferred. We would like to at least avoid the AoA setups that are not feasible to be tested. |
| Keysight | Ideally, the work in the WI and the SI should be coordinated as it would be unfortunate for the OTA SI to develop test systems/methodologies that do not correlate to the requirements and for the WI to define requirements with AoA assumptions that are not practical for test systems “for complexity, chamber footprint/height, lack of upgradeability of existing system, development lead time, increased measurement uncertainty/test tolerance, and test effort/test time reasons” |
| Apple | On proposal 1, we share vivo’s view that the RF requirement needs to consider testability. On proposal 2, can QC elaborate more on “RAN4 needs to determine test set feasibility before finalizing details of the FOM requirement for 2 TCI state spherical coverage”? Do you mean the selection of two AoAs depend on testability or something else? Thanks. |

### On how to determine candidate AoA pairs for setting the UE RF requirement

Issue 2-3: Please comment on the following options, with views extracted from contributions by moderator

- Full set AoA1 + Full set AoA2

  - Sony, Ericsson: full set of AoA1+ full set of AoA2 is not feasible from the testability aspect. However, it may still be used to derive the core requirement as it provides an overall assessment of device performance under arbitrary UE orientation and angle of incoming signals. However, a performance gap between the derived requirement and the actual test may appear.

  - Nokia: testing burden

  - Vivo: least preferred because this case is the most complicated

  - Keysight: A Spherical coverage test with full degree of rotation freedom for each AoA should not be considered due to test time limitations.

  - Anritsu: Full rotational degrees of freedom cannot be supported (reflectors proximity, interference issue)

- Fixed AoA1(s) + Full set AoA2 (supported by Nokia, OPPO, vivo, Keysight)

  - Sony, Ericsson:

    - More than one AoA1 may need to be selected to ensure the device's performance in real life. In addition, testability issue may appear if one of the AoAs needs to have a fixed relative orientation towards the device while the device needs to be rotated.

    - the testability of having one of the AoA fixed relative to the DUT must be confirmed before selecting this method..

  - Nokia:  We propose that one or a small number of AoA1 + Full AoA2 are agreed for further test definition and that the main interest in the multi-Rx is AoA2 while AoA1 connection must remain.

  - OPPO: It is proposed that when appointing several AoA1s for Candidate 2, relatively large separation between AoA1s should be adopted, e.g. the separation more than 120 degree.

  - vivo: Fixing one of the AoAs will cause some panels’ performance not to be verified thoroughly.

  - vivo: Taking multiple Fixed AoA1+Fullset AoA2 as baseline for further discussion.

    - FFS on number of AoA1

    - FFS on the principle of multiple AoA1 selection.

  - Anritsu: Keeping the AoA1 towards the UE beam peak direction is complicated without deteriorating the QZ as the UE is rotating

- Fixed offset(s) between AoA1 and AoA2 (supported by Sony, Ericsson, OPPO)

  - Sony, Ericsson: how to choose the offset? A narrow (30 degrees) and wide (135 degrees) angular probe separation to cover the cases of collocation and non-collocated TRP.

  - Nokia: An offset that would not resemble live network operation and would favor certain angular offset at the UE, but not guaranteed by the network propagation.

  - vivo: Fixed offset cannot reduce the test complexity of multi-Rx effectively.

  - OPPO:

    - Based on the Candidate 3, the RF requirement concept of the spherical coverage of one AoA can be reused in maximum extent.

    - The key issue for Candidate 3 is how to decide the fixed separation between the two AoAs.

- Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 with two performance metrics: CDF of EISAoA1 and CDF of the maximum TPAoA2. (newly proposed by Keysight)

  - Keysight: Consider a wide range of AoA angular differences between AoA1 (full degree of freedom) and AoA2 (limited, fixed degree of freedom)

| Company | Comments |
| --- | --- |
| vivo | Based on our simulation, the performance of multiple fixed AoA1+ fullset AoA2 can be closed to the ideal case if we can choose a set of suitable AoA1, so we support take fixed multiple AoA1+ fullset AoA2 as the starting point for further discussion. For fixed offset, we still need several offsets to verify the UE and we even need further discuss the relative position between 2 AoAs. We afraid it is hard to conclude which relative position configuration is more reasonable, so we do not prefer this option for now. |
| Anritsu | Thanks Keysight for your proposal described in R4-2215541. Is the test sequence described in Figure 3 of that contribution with the step “Measure TP (or EIS) on AoA2.m probe and record measurement, e.g. , TPAoA2,Pn, EISAoA2,Pn ” intended to avoid link failure during the spherical coverage test? Is the intent to create a map of the spherical coverage test result only when the UE could maintain the link between both AoA1 and AoA2? Is it the intention that if at a particular grid point none of the M AoA2.m probes allow to get a TP value, the EISAoA1,Pn measurement for that grid point is skipped? To seek for the possibility of sequential testing by the new test command, we’d like to ask views from companies on following points. - What is the minimum angle of separation between AoAs at which a UE is still considered to support Multi-Rx Chain DL? - Can it be considered high enough such that there is no inter-beam interference expected between the beam for AoA1 and the beam for AoA2? - Is there any coupling expected between the Rx chains used for receiving AoA1 and AoA2 such that simultaneous testing is mandatory? - Or can it be considered that based on EIS spherical coverage measurements for each Rx chain, the performance for simultaneous DL reception can be determined by data post-processing? - Can sequential testing (using a new test command to fix an active antenna in the DUT) be considered? |
| LG Electronics | Support multiple fixed AoA1+ fullset AoA2. In R4-2215781, we proposed as follows. Proposal 4: Consider K sample(s) in the legacy spherical coverage of 50%-xile in one panel and all samples in other panel for evaluating CDF of multi-Rx. Proposal 4-1: Assume all K sample(s) to be selected at same point of CDF 50%-xile considering lowest received power. Based on these, we suggest to consider selecting AoA1 within the legacy spherical coverage N%-xile (N=50, for PC3) when determining candidate AoA pairs. |
| Nokia | We propose that one or a small number of AoA1 + Full AoA2 as discussed in R4-2215580. |
| Samsung | First of all, we’d like to highlight the coordination system for the AoA discussion, it should make clear the AoA is with reference to UE coordination or with reference to test chamber coordination? i.e., is fixed AoA1 is a dedicated direction with reference to UE (which means probe rotating along with UE rotating to keep the fixed AoA with reference to UE), or is the fixed AoA1 a dedicated probe location inside the chamber? It seems that companies have different understanding. It is encouraged to make coordination system clear before discussing each options. |
| Rohde & Schwarz | • Full set AoA1 + Full set AoA2: As we are pointing out in our contribution to the testability SI, this setup is not feasible. We agree here with Keysight and Anritsu. • Fixed AoA1(s) + Full set AoA2: This seems to be a reasonable approach, but details need to be clarified further. Does this mean that AoAs are measured sequentially?I.e. measure throughput on AoA1, fix UE beam in direction of AoA1 and then measure AoA2? • Fixed offset(s) between AoA1 and AoA2: This is also a feasible approach if the fixed offsets are chosen appropriately. Current RRM setups offer various angular offsets (30, 60, 90, 120, 150 degrees). So one or multiple of those offsets could be chosen to verify the UE characteristics. • Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2 with two performance metrics: This seems to be related to the second approach. Further discussion is needed to evaluate the approach and how to calculate the CDF curve in the end. |
| Sony | - For Full set AoA1 + Full set AoA2: we agree that this is not feasible to test. However, it can be seen as a method to set the core requirement that is agonistic to the test method. It is still possible to derive the core requirement according to this method and later identify the gap between the core requirement and the test method. - For Fixed AoA1(s) + Full set AoA2: we would like to hear the feasibility of this test method, in particular how we can fixed the AoA1 towards UE while UE is rotating. As we discussed for issue 2-1, we would like to avoid discussing the method that is not feasible for test. - For clarification, in our understanding, fixed AoA1 means the AoA1 is fixed relatively towards UE, for example, AoA1 always point towards beam peak direction of the UE. - Fixed offset(s) between AoA1 and AoA2: this is the most feasible one from the testability aspect. We can consider two offset values, one small and one large to be representative cases for the test. |
| Qualcomm | We agree with Samsung’s comment and what Sony points out for clarification above. Perhaps this aspect can be established during an online session (GTW) |
| Ericsson | The “fixed offset(s) between AoA1 and AoA2” appears the most feasible for simultaneous reception of the two layers. A set of angular separations could be used and different polarizations of the two layers could be considered. We admit that all test aspects have not been considered for this setup, but it appears less complex than the case in which one of the directions is fixed relative to the DUT (if this is the proposal). |
| Keysight | Feedback to Anritsu: The intention of measuring the (total) TP while varying AoA2 is collect a second performance metric and to guarantee that the primary metric of EIS spherical coverage/CDF is optimized with the best set of AoA1 and AoA2s. Given the proposed large range of AoA angular differences between AoA1 (full degree of freedom) and AoA2 (limited, fixed degree of freedom), it is unlikely that no TP is measured. However, if no TP can be measured and thus no EIS can be determined, this will need to be captured and taken into account. Full set AoA1 + Full set AoA2 As discussed here and in the SI, the feedback from TE vendors is that such system is simply not practical for “for complexity, chamber footprint/height, lack of upgradeability of existing system, development lead time, increased measurement uncertainty/test tolerance, and test effort/test time reasons” Fixed AoA1(s) + Full set AoA2 As highlighted by Sony, additional clarifications on this approach is needed. - If Fixed AoA1(s) means that AoA1 can have arbitrary angular offset from AoA2, this approach essentially corresponds to the ‘Full set AoA1 + Full set AoA2’ approach since both AoAs require full degrees of freedom. - If the fixed AoA1(s) do not have full degrees of freedom, this approach likely matches the “Fixed offset(s) between AoA1 and AoA2.” As discussed here and in the SI, this methodology is a lot more practical and could readily be supported with either existing FR2 MIMO OTA systems or 2 AOA RRM systems with probe locations/directions defined/harmonized between system vendors. Fixed offset(s) between AoA1 and AoA2 See feedback provided above, i.e., this methodology is a lot more practical and could readily be supported with either existing FR2 MIMO OTA systems or 2 AoA RRM systems with probe locations/directions defined/harmonized between system vendors. |
| Lenovo | We agree with the following observation from Sony For Full set AoA1 + Full set AoA2: we agree that this is not feasible to test. However, it can be seen as a method to set the core requirement that is agonistic to the test method. It is still possible to derive the core requirement according to this method and later identify the gap between the core requirement and the test method. In our view, the FOM should be defined before the test method is selected, after which it can be determined to what extent the FOM is testable. As pointed out in our contribution R4-2216875, if N test points are used to determine single AOA spherical coverage, and if test pairs ϕ1,θ1,ϕ2,θ2 are limited to pairs for which ϕ1,θ1 and ϕ2,θ2 each meet the 50% EIS for single AOA reception, then no more than N2/8 measurement pairs need be considered. This number can be further reduced by limitations on the minimum angular separation. Additional observation made on the single AOA measurements may also further reduce the need for measurements. For example, in the case that EIS for ϕ1,θ1 and ϕ2,θ2 are each much less than the 50% EIS and the angular separation is large, it can be assumed that the pair ϕ1,θ1,ϕ2,θ2 pass and need not be tested. |
| Apple | To reduce the test complexity, the obvious “unqualified” test grid points should be excluded from the “full set” at least based on the following two criteria: • Minimum angular separation between AoA1 and AoA2. The exact value can be decided later. • R15-defined spherical coverage/R15 receiver sensitivity. We can use the R15 requirement to discuss if some EIS limit can be used to screen out some AoAs to ensure performance. Therefore, we think it is appropriate to use the term “limited set” instead of “full set” in the comments below. On Full set AoA1 + Full set AoA2 (or limited set AoA1 + limited AoA2): we share the view that testing time is an issue. We’d also like to confirm the testability issue raised by Anritsu. We believe this will be further discussed and concluded in the Rel-18 FR2 OTA test method enhancement SI. On Fixed offset(s) between AoA1 and AoA2: While this option may make testing simpler and easier because of the fixed offset of the two AoAs in the test set-up, we have several concerns if we consider it for requirement. First, it does not reflect the real network situation. It is hard to conceive that there is a fixed offset between AoA1 and AoA2 for a UE in the field. Second, it may strongly restricts the UE implementation options, i.e., the likelihood of the two AoAs supported by two panels having fixed offset is really small. On the other hand, if we want to accommodate different UE implementations, it is unclear how we can choose the fixed offsets for the requirement. On the proposal from Keysight “Adopt A spherical coverage test based on full rotational freedom for AoA1 with fixed and discrete AoA2,” we wonder if it is similar to “Fixed AoA1(s) + Full set AoA2,” as Thorsten clarified on the reflector. |

## Companies views’ collection for 1st round

### Open issues

See Section 2.2

### CRs/TPs comments collection

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2.2.1 | The proposal “The term “panel” is not used in test configuration” is agreeable. |
| Sub-topic#2.2.2 | Based on the discussion, there seems to be a consensus that requirement discussions need to consider testability issue so that the defined requirement can be properly verified. It is unclear if we need an explicit agreement on this. Recommendations for 2nd round: Will be discussed in the WF in the second round. |
| Sub-topic#2.2.3 | There were good inputs collected on the options. As we are probably not ready to make a decision, it is recommended to summarize the issues/concerns so we can come back with a focused discussion at the next meeting. Recommendations for 2nd round: Will be discussed in the WF in the second round. |

### CRs/TPs

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

### 2.5.1	On the term ”panel” used in test configuration

The term “panel” is not used in test configuration.

Moderator: Based on first round discussion, it is agreeable.

| Company | Comments |
| --- | --- |
| Qualcomm | Do not support The agreement that UE RF requirements will not use ‘panel’ in [132] is sufficient. For the test method, we do not want to foreclose future optimization by means of voluntary vendor declarations or similar. |
| LG Electronics | Agreeable. |
| Huawei | Support. |
| Nokia | Agree |
| Apple | We support. |
| Verizon | Agree with Qualcomm |
| Samsung | Support. test and requirement should be aligned. |

### 2.5.2 On relation between testing and core requirement

Requirement discussions need to consider testability issue so that the defined requirement can be properly verified.

| Company | Comments |
| --- | --- |
| Qualcomm | The proposal is reasonable. Support. |
| vivo | OK with the proposal |
| LG Electronics | Fine with this proposal. |
| Huawei | In general we are OK with this principle, but if this means that more alignment between this WI and the FR2 OTA enhancement SI should be expected, then we think how to handle the parallel discussion shall be further clarified. |
| Nokia | Agree |
| Apple | We agree. |
| Verizon | Support |
| Samsung | Support the WF |

### On how to determine candidate AoA pairs for setting the UE RF requirement

Coordinate system for the two AoAs:

- Option 1: UE coordinate system

- Option 2: Test chamber coordinate system

| Company | Comments |
| --- | --- |
| Keysight | These options are not very clear as the coordinate systems could be the same for different system options considered, e.g., when no DUT positioner is needed. If would be better to harmonize the AoA definitions with the SI, e.g., change to Offset between AoA1 and AoA2 o Option 1: Fixed Angular Offset(s) between AoA1 and AoA2 in the chamber, i.e., the angular separation between AoA1 and AoA2 is NOT changing during the testing o Option 2: Variable Angular Offset(s) between AoA1 and AoA2 in the chamber, i.e., AoA2 is fixed with respect to the UE and rotating together with the DUT. The angular separation between AoA1 and AoA2 is changing during the testing |
| Qualcomm | We think this discussion will benefit from down-selection of the test set up. It is too early to make this choice. Also as KS mentions, further details for each will be useful. |
| Vivo | Prefer keep this issue as FFS |
| LG Electronics | Fine with FFS. |
| Sony | Fine to FFS. Just to share our understanding: since we likely to also calculate the coverage based on the AoA setup, it might be more convenient to use UE as a reference for the coordinate system for UE RF requirement discussion. The most important thing is to clarify if the AoA is relative to the UE orientation or antenna chamber when companies brought up their analysis. |
| Rohde & Schwarz | We share similar views as Keysight since the two options proposed can be confusing. We are fine with the rewording proposed by Keysight, although we think that Option 1 (fixed angular offset between AoA1 and AoA2) is the only practical option. In our understanding, the coordinate system will be referred to the DUT eventually, but for test setup / testability discussions, it is easier to refer to the chamber. |
| Huawei | Seems too early to have discussion on this issue. |
| Nokia | AoA is angle of arrival, so it should be reference from UE (receiver) perspective. |
| Apple | We think this needs to be clarified, and the earlier, the better. We are OK to further discuss it. |
| Verizon | Agree with FFS! |
| Ericsson | FFS. The specification of the AoA relative to the DUT is already very confusing. |
| Samsung | Okay for further study. It should be highlighted that this issue is also bottleneck in FR2 OTA SI. This issue should be resolved as early as possible. |

Identified issues and further investigation for the following options

Moderator: The purpose is to collect more inputs for discussions at the next meeting. No plan to down-select any option at this meeting.

- Full set AoA1 + Full set AoA2

  - Long test time

  - Testability: Full rotational degrees of freedom cannot be supported (reflectors proximity, interference issue)

- Fixed AoA1(s) + Full set AoA2

  - The offset between AoA1 and AoA2

    - Fixed (If the fixed AoA1(s) do not have full degrees of freedom, this approach likely matches the “Fixed offset(s) between AoA1 and AoA2.” As discussed here and in the SI, this methodology is a lot more practical and could readily be supported with either existing FR2 MIMO OTA systems or 2 AOA RRM systems with probe locations/directions defined/harmonized between system vendors. )

    - Variable (If Fixed AoA1(s) means that AoA1 can have arbitrary angular offset from AoA2, this approach essentially corresponds to the ‘Full set AoA1 + Full set AoA2’ approach since both AoAs require full degrees of freedom. )

    - And how large?

  - How many AoA1s to select and their directions

  - Testability:

    - Having one of the AoA fixed relative to the DUT must be confirmed before selecting this method.

    - Keeping the AoA1 towards the UE beam peak direction is complicated without deteriorating the QZ as the UE is rotating

- Fixed offset(s) between AoA1 and AoA2

  - How to choose the offset?

    - A narrow one (e.g., 30 degrees)

    - A wide one (e.g., 135 degrees)

  - Would not resemble live network operation.

  - Fixed offset cannot reduce the test complexity

  - Testing is practical and could readily be supported with either existing FR2 MIMO OTA systems or 2 AOA RRM systems with probe locations/directions defined/harmonized between system vendors

| Company | Comments |
| --- | --- |
| Keysight | It would be better to clarify/change the three options to: - Full set AoA1 + Full set AoA2 (can stay as is) - Fixed AoA1(s) + Full set AoA2: given the ambiguity (are the offsets between AoA1 and AoA2 fixed or variable, it would be better to change to: Full set AoA1 + Set of AoA2s with variable offsets from AoA1 and remove the first bullet - Fixed offset(s) between AoA1 and AoA2: suggest clarifying slightly and to change to: Full set AoA1 + Set of AoA2s with fixed offsets from AoA1 - The comment “Fixed offset cannot reduce the test complexity” is not clear as the TE/TS vendors at least believe the test system complexity is greatly reduced with this approach. - It is suggested to remove the two testability bullets under “Fixed AoA1(s) + Full set AoA2”, i.e., the first bullet should no longer be applicable if we rename this option and the second is included in “this approach essentially corresponds to the ‘Full set AoA1 + Full set AoA2’ approach since both AoAs require full degrees of freedom” |
| LG Electronics | Support moderator’s proposal. |
| Sony | We disagree with statement that “Would not resemble live network operation.” For Fixed offset(s) between AoA1 and AoA2. In our view, fixed offset method can represent the real-life scenario well enough, especially if we can test under different AoA offsets. In general, we understand the sub bullet points here just to capture the concern raised by some companies in this meeting but not RAN4 common understanding, therefore, it should be clarified those are just for record tracking but not RAN4 agreement. |
| Rohde & Schwarz | We agree with the clarification and rewording proposed by Keysight. |
| Nokia | We would like to clarify if ‘Fixed offset(s) between AoA1 and AoA2’ can be combined with either ‘Fixed AoA1(s) + Full set AoA2’ or ‘• Full set AoA1 + Full set AoA2’, as they seem not mutually exclusive based on Keysight explanation. |
| Apple | We welcome TE vendors’ input on testability. |
| Verizon | Based on Keysight explanation, more clarification should be needed |
| Ericsson | For ‘Fixed offset(s) between AoA1 and AoA2’ we do not understand the remark that this does not resemble operation in the field. Is not this exactly what this option does? The UE is receiving from two TRPs at different direction and the performance is measure while the UE is rotated (spherical coverage)? The AoA should be clearly defined. |
| Samsung | Option 1 (Full set AoA1 + Full set AoA2), we would like to clarify the testability issue is not “cannot be supported” but “feasible but not practical”. And this option in FR2 OTA SI is concludes as not pursued in R18. Option 2 has many variants for further discussion. Option 3 (Fixed offset(s) between AoA1 and AoA2) seems a special case of option 2’s sub-option depending on coordination system. To avoid misunderstanding, it is highly suggested to align coordination system of AoA firstly as mentioned above. |
| Lenovo | We would like to point out that for Option1 (Full set AoA1 + Full set AoA2), we do not think a full set of AOA’s would really be required. Based on the single AOA measurements, AOA1 and AOA 2 could be limited to AOA’s for which individual AOA EIS is better than 50%. This and the fact that (AOA1, AO2) has the same performance as (AO2, AO1) can be used to reduce the number of pairs by a factor of 8 relative to N2 (still a large number). This same reduction can be applied for Option 2. |

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on System parameter assumption and UE architecture and test setup | Apple |  |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215579 |  |  | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215620 |  |  | Apple | Noted |  |
| R4-2215701 |  |  | Samsung | Noted |  |
| R4-2215778 |  |  | Murata | Noted |  |
| R4-2216125 |  |  | vivo | Noted |  |
| R4-2216352 |  |  | Xiaomi | Noted |  |
| R4-2216445 |  |  | OPPO | Noted |  |
| R4-2215541 |  |  | Keysight | Noted |  |
| R4-2215580 |  |  | Nokia | Noted |  |
| R4-2216126 |  |  | vivo | Noted |  |
| R4-2216444 |  |  | OPPO | Noted |  |
| R4-2216585 |  |  | Anritsu | Noted |  |
| R4-2216787 |  |  | Qualcomm | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |
| R4-2217732 |  | WF on System parameter assumption and UE architecture and test setup | Apple | Agreeable |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents