3GPP TSG-RAN WG4 Meeting #104-bis-e                                                      R4-2217800

Electronic Meeting, 10th – 19th October 2022

Source:	Vodafone

Title:	TP for TR 36.718-02-01 to add UL CA_1A-3A for CA_1A-3A-32A

Agenda item:	7.1.3.1

Document for:	Approval

# Background

This contribution provides a text proposal for adding UL configuration 1A-3A to CA_1A-3A-32A as requested in RAN4#104-e.

# Reference

# Text Proposal

##### ---Start of changes---

### 5.4.x	CA_1A-3A-32A

#### 5.4.x.1	Channel bandwidths per operating band for CA

Table 5.4.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 32 |  | N/A |  | 1452 MHz | – | 1496 MHz | FDD |

Table 5.4.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-32A | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |

#### 5.3.x.2	Co-existence studies

Coexistence for CA_1-3 has already been specified in TS 36101.

#### 5.3.x.3	∆TIB and ∆RIB values

Relaxation values already specified for CA_1-3-32.

#### 5.3.x.4	REFSENS Requirements

Table 5.3.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-3A-32A | CA_1A-3A | 1 | 1960 | 5 | 25 | 2510 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A |  | N/A |
|  |  |  |  | 32 |  | N/A |  | N/A |  | 25 |  | 1480 |  | 5 |  | 15.2 |  |  |  | IMD34 |  |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### ---End of changes---