3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2215392

Electronic Meeting, October 10 – 19, 2022

Title: 	Discussion on test cases for handover for NTN

Source: 	CATT

Agenda item:	4.2.6.4

Document for:	Discussion

- Introduction

In RAN4#104-e meeting, a way forward on NR NTN RRM requirements was approved [1]. Most agreements have been achieved, and some open issues have been remained. This document will further discuss these issues relative to test cases for handover and present our understandings and proposal.

- Discussion

In RAN4#104-e meeting, following agreements have been achieved relative to test cases design for handover:

Issue 2-1: Test cases for GSO and NGSO.

  - RAN4 to define TCs for both GSO and NGSO scenarios.

  - If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases.

  - The TCs for GSO and NGSO should be captured in the same clauses.

Issue 2-3: Test coverage regarding numerology and duplex mode.

  - Test configurations for TDD and numerology higher than 15kHz are not considered.

Issue 2-5: Serving and Neighbour Satellite configurations.

  - Agreement:

  - Take option 1 as baseline to further discuss how to configure the satellite parameters.

  - Option 1: RAN4 to define the following NTN specific configurations for GEO and LEO at 600km altitude in a common section, which can be referred to from NTN RRM test cases:

    - ……

  - Cells belonging to the same satellite can have different parameters for, e.g.

    - Koffset, Kmac, beam footprint information, cell service time, etc.

  - FFS on exact values and parameters

    - The values should be derived based on a realistic satellite constellation with respect to UE position

  - In order to alleviate the impact of uncertainty on UE uplink transmission timing due to quantized feeder link delay information, the feeder link delay can be set to zero.

  - RAN4 to define the following ephemeris format:

    - PVT information;

    - Orbital parameters.

……etc.

The agreements for the test case design for handover:

  - The test cases of Intra- and inter-frequency HO with known cell can refer to legacy TN test cases.

  - Regarding test cases for CHO with time condition, set the time instant fulfilling t1-Threshold-r17 to be at T2, and set the time instant fulfilling duration-r17 to be at (T2+ Tmeasure).

  - Regarding test cases for CHO with location condition, set the time instant fulfilling t1- condEvent D1 to be at T2.

There are following open issues as in WF [1]:

  - Regarding test cases for CHO, T2 is the instant when the last TTI containing the RRC message implying conditional handover command is sent to UE, others shall follow A.6.3.1.2.

  - RAN4 shall check the necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario.

In current specification, the test cases for conditional handover in section A.6.3.3 are designed as “NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2”. It is proposed that same way should be used in test cases for conditional handover for NTN.

Proposal 1: It is proposed that same way as in current specification for TN system should be used in test cases for conditional handover for NTN,  i.e. the network shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.

We think it is not necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, due to RAN4 does not define the requirements for this scenario. The abnormal UE’s behaviour should not been checked by the test cases.

Proposal 2: It is not necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario.

Based on the agreements and proposal 1, the CRs for handover for NTN are represented with some modifications in [2][3].

- Conclusion

This document discussed the open issues for test cases for conditional handover and presented the following proposals:

Proposal 1: It is proposed that same way as in current specification for TN system should be used in test cases for conditional handover for NTN,  i.e. the network shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.

Proposal 2: It is not necessity of adding test cases in which settings don’t fulfill power based events and time/location based events simultaneously, to examine UE’s behavior in this type of scenario.

- Reference

[1]	R4-2214471, WF on NR NTN RRM requirements, Qualcomm Incorporated

[2]	R4-2215393, Test cases for Intra- and inter-frequency HO with known cell for NTN, CATT

[3] 	R4-2215394, Test cases for Intra- and inter-frequency CHO for NTN, CATT