3GPP TSG-RAN WG4 Meeting #104-bis-e 	       R4-2215547

# Electronic Meeting, 10 - 19 October, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results on PDSCH demodulation requirements for NTN

Agenda item: 	4.2.7.3.1

Document for:	Approval

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present simulation results for PDSCH demodulation requirements.

# Discussion

We discuss open issues related to the PDSCH demodulation requirements in our accompanying paper [2].

## Simulation parameters

Agreements from [3]:

Issue 1-1: Channel model for NTN-TDLA (NLOS)

- Proposals: to confirm the following tentative agreements:

  - Option 1: DS =100ns, Doppler = 200Hz (residual Doppler after UE compensation)

  - Option 2: specify other option if any.

- Agreement:

  - DS =100ns, Doppler = 200Hz

Issue 1-2: Channel model for NTN-TDLC (LOS)

- Proposals: continue to discuss DS and K-factor for NTN-TDLC (LOS)

  - Option 1: DS = 10ns, Doppler =200Hz, K-factor=3.59dB

  - Option 2: DS=12ns, Doppler=200Hz, K-factor=3dB

  - Option 3: DS=100ns, Doppler=200Hz, K-factor=21.6dB

  - Option 4: DS=3.5ns, Doppler=200Hz, K-factor=8.05dB

- Agreement:

  - Assuming elevation angle: 30 degree to decide K_offset and K-factor

    - DS = [3.5ns],   Doppler =200Hz, K_factor= 8.05 dB , K_offset = [8 slots]

Simulation assumptions have been discussed already for a few meeting rounds and there are agreed assumptions available in way forward [3]. Agreements of the channel model parameters including delay spread, Doppler, K-factor and K-offset are shown above. These assumptions are used also in this document.

Simulated test cases are shown in Tables 1 and 2 for NLOS and LOS cases, respectively. Rest of the assumptions are shown in Tables 3 and 4 for HARQ disabled and enabled cases.

Table 1 Simulated test cases for NTN-TDLA channel model

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 1-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |

Table 2 Simulated test cases for NTN-TDLC channel model

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) DMRS |
| 2-1 | R.PDSCH.1-1.1 FDD (QPSK) | 10 / 15 | QPSK, 0.30 | NTN-TDLC3.5-200 | 1x2, ULA Low | 70 | TBD |
| 2-2 | R.PDSCH.1-2.1 FDD (16QAM) | 10 / 15 | 16QAM, 0.48 | NTN-TDLC3.5-200 | 1x2, ULA Low | 70 | TBD |

Table 3 Simulation assumptions for PDSCH performance with disabled HARQ

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | Disabled |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | N/A |
| Maximum number of HARQ transmission |  |  | 1 |

Table 4 Simulation assumptions for PDSCH performance with 32 HARQ processes

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 32 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2+K_offset |
| Maximum number of HARQ transmission |  |  | 4 |

## Simulation results

Simulation results for QPSK/16QAM and NTN TDL-A/C for HARQ disabled case are shown in Figures 1-4. Both BLER and normalized throughput plots are provided.

Normalized throughput results for QPSK/16QAM are shown in Figures 5 and 6 for NTN-TDLA and NTN-TDLC cases with HARQ, respectively.

These results are proposed to be taken into account when defining requirements for NTN demodulation.

Proposal 1:  Take results shown in this document into account when defining requirements for NTN demodulation.

Figure 1 BLER and normalized throughput for QPSK and NTN-TDL-A 100ns

Figure 2 BLER and normalized throughput for QPSK and NTN-TDL-C 3.5ns

Figure 3 BLER and normalized throughput for 16QAM and NTN-TDL-A 100ns

Figure 4 BLER and normalized throughput for 16QAM and NTN-TDL-C 3.5ns

Figure 5 Normalized throughput for QPSK and 16QAM, NTN-TDL-A 100ns, HARQ

Figure 6 Normalized throughput for QPSK and 16QAM, NTN-TDL-C 3.5ns, HARQ

# Conclusion

This contribution provides simulation results of PDSCH demodulation requirements for NTN and has the following proposal:

Proposal 1:  Take results shown in this document into account when defining requirements for NTN demodulation.

# References

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2215546, Discussion on PDSCH demodulation requirements for NTN, Nokia, Nokia Shanghai Bell

- R4-2214386, WF for NTN demodulation requirements - general and PDSCH, Qualcomm