3GPP TSG-RAN4 Meeting #104bis-e	R4-2215594

# Online, , 10th  - 19th October 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 2596 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for inter-cell beam management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some requirements are captured in “[ ]”. Based on agreements and discussions they need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additional applicability fo requirements for known state was deleted. - Editorial changes to equations with sharing factors. - Confirmed sharing factor for cell with diff PCI. - Deleted editors note - Clarified applicability of scheduling restriction – the scheduling restrictions are for PDCCH/PDSCH/PUCCH/PUSCH on serving cell and cell with diff PCI if applicable due to measurements on cell with different PCI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for unified TCI will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.13.3, 9,13.4, 9.13.5, 9.13.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------Change 1---------------------

### 9.13.3	Measurement Reporting Requirements

The UE shall send L1-RSRP reports only for report configurations configured for the active BWP.

The UE shall report the L1-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1dB step size according to clause 10.1.19 for FR1 and 10.1.20 for FR2 if nrofReportedRS is configured to one. If nrofReportedRS is configured to be larger than one, or if groupBasedBeamReporting-r17 is enabled, the UE shall use differential L1-RSRP based reporting as defined in clause 10.1.19 for FR1 and 10.1.20 for FR2. The differential L1-RSRP is quantized to a 4-bit value with 2dB step size. The mapping between the reported L1-RSRP value and the measured quantity is described in 10.1.6.

In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

#### 9.13.3.1	Periodic Reporting

Reported L1-RSRP measurements contained in periodic L1-RSRP measurement reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send periodic L1-RSRP measurement reports for an active BWP.

The UE shall transmit the periodic L1-RSRP reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.13.3.2	Semi-Persistent Reporting

Reported L1-RSRP measurements contained in a Semi-Persistent L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. This requirement applies for semi-persistent L1-RSRP reports send on PUSCH or PUCCH.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUSCH, if a DCI request has been received.

The UE shall only send semi-persistent L1-RSRP measurement reports on PUCCH, if an activation command [7] has been received.

The UE shall transmit the semi-persistent L1-RSRP reporting on PUSCH or PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

#### 9.13.3.3	Aperiodic Reporting

Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall only send aperiodic L1-RSRP measurement report if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

-----------------Change 2---------------------

### 9.13.4	L1-RSRP measurement requirements

#### 9.13.4.1	Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according 9.13.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP.

The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-1 for FR1, The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR1,

-	P=11-TSSB_CDPMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P2=11-TSSB_CDPTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod).

-	P2= 111-TSSB_CDPMGRP-TSSB_CDPTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB_CDP < 0.5*TSMTCperiod

-	P2= 111-TSSB_CDPmin(TSMTCperiod,MGRP),when SSB is partially overlapped with measurement gap (TSSB_CDP <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	If SSB resource from serving cell is configured for L1-RSRP measurements, and P1 is valid according to 9.5.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-   P = P21-P2*TSSB_CDPP1*TSSB_SC ,   if P2*TSSB_CDP < P1*TSSB_SC.

-   P = P2, if P2*TSSB_CDP> P1*TSSB_SC.

-   P = 2*P2, if P1*TSSB_SC = P2*TSSB_CDP.

-	Otherwise, P = P2

Where:

-	TSSB_CDP = SSB periodicity of the cell with PCI different from serving cell

-	TSMTCperiod = the configured SMTC period

-	TSSB_SC = ssb-periodicityServingCell of the serving cell

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 cell with PCI different from serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell with PCI different from serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 9.13.4.1-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: [K = 1 when TSSB_CDP ≤ 40 ms and highSpeedMeasFlag-r16 are configured; otherwise] K = 1.5. [Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16]] [Editor’s Note: Whether inter-cell L1-RSRP measurement requirements are applicable in HST scenario] |  |

Table 9.13.4.1-2: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

-----------------Change 3---------------------

### 9.13.5	Measurement restriction for L1-RSRP measurement

The UE is required to be capable of measuring SSB for L1-RSRP without measurement gaps. The UE is required to perform the SSB measurements with measurement restrictions as described in the following clauses.

Unless explicitly stated, the SSB to be measured for L1-RSRP measurement is transmitted from cell(s) with PCI different from serving cell(s).

#### 9.13.5.1	Measurement restriction for SSB based L1-RSRP

For FR1,

when the SSB for L1-RSRP measurement is in the same OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement is in the same OFDM symbol as SSB for L1-RSRP measurement,

-	UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

when the SSB for L1-RSRP measurement is in the same OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement,

-	If SSB and CSI-RS have same SCS, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If SSB and CSI-RS have different SCS,

-	If UE supports simultaneousRxDataSSB-DiffNumerology, UE shall be able to measure the SSB for L1-RSRP measurement without any restriction;

-	If UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2,

when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB transmitted from serving cell(s) for RLM, BFD, or CBD measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both the two SSBs. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as SSB for L1-RSRP measurement on the different CCs in the same band, UE is required to measure one of but not both the two SSBs. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

when the SSB for L1-RSRP measurement on one CC is in the same OFDM symbol as CSI-RS transmitted from serving cell(s) for RLM, BFD, CBD or L1-RSRP measurement on the same CC or different CCs in the same band, UE is required to measure one of but not both SSB for L1-RSRP measurement and CSI-RS. Longer measurement period for SSB based L1-RSRP measurement is expected, and no requirements are defined.

For FR2, if the network configures same or mixed numerology between SSB for L1-RSRP measurement on one FR2 band and CSI-RS for RLM, BFD, CBD, L1-RSRP or L1-SINR measurement on the other FR2 band, UE shall be able to perform the related SSB based measurements in one band without any measurement restrictions in the other band, provided that UE is capable of independent beam management on this FR2 band pair.

-----------------Change 4---------------------

### 9.13.6	Scheduling availability of UE during L1-RSRP measurement

Scheduling availability restrictions described in the following clauses

when UE is performing L1-RSRP measurement on cell(s) with PCI different from serving cell, for serving cell and/or cell(s) with PCI different from serving cell from which UE is receiving the PDCCH/PDSCH.

#### 9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1

There are no scheduling restrictions due to L1-RSRP measurement performed on SSB as RS for L1-RSRP measurement with the same SCS as PDSCH/PDCCH in FR1.

#### 9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to L1-RSRP measurement based on SSB as RS for L1-RSRP measurement. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to L1-RSRP measurement based on SSB configured for L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR1 is configured, the scheduling restrictions apply to cell(s) in the same band on the symbols that fully or partially overlap with restricted symbols. When inter-band carrier aggregation within FR1 is configured, there are no scheduling restrictions on FR1 cells configured in other bands than the bands in which the cell where L1-RSRP measurement is performed is configured.

#### 9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2

The following scheduling restriction applies due to L1-RSRP measurement.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols corresponding to the SSB indexes configured for L1-RSRP measurement.

When intra-band carrier aggregation in FR2 is performed, the scheduling restrictions is performed apply to cell(s) in the band on the symbols that fully or partially overlap with restricted symbols.

When inter-band carrier aggregation in FR2 is performed, there are no scheduling restrictions on FR2 cells in the bands due to L1-RSRP measurement performed on FR2 cell(s) in different band(s), provided that UE is capable of independent beam management on this FR2 band pair. Additionally, there is no scheduling restriction if the UE is configured with different numerology between SSB on one FR2 band and data on the other FR2 band provided the UE is configured for IBM operation for the band pair.

If following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type 2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for L1-RSRP measurement; and

For the SSB and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for L1-RSRP measurement.

#### 9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA

There are no scheduling restrictions on FR1 cell(s) due to L1-RSRP measurement performed on FR2 cell(s).

There are no scheduling restrictions on FR2 cell(s) due to L1-RSRP measurement performed on FR1 cell(s).